3GPP TSG-RAN2 Meeting #113bis-e	draftR2-2104387

Online, April 12 – 20, 2021

Agenda Item:	9.1.4

Source:			Huawei, HiSilicon

Title:				Offline 403 - RSRQ measurements when RSS is used

Document for:	Discussion and decision

- Introduction

This document is the summary of the offline email discussion “[AT113bis-e][403][ eMTC R16] RSRQ measurements when RSS is used (Huawei)”, as indicated below:

[AT113bis-e][403][eMTC R16] RSS based RSRQ (Huawei)

Status: Started

Scope: Check whether RSRQ measurements should be defined for RSS,
collect initial comments and draft an LS reply.

Intended outcome: Report in R2-2104387

Deadline: Thursday 2021-04-15 10:00 UTC

- Discussion

The options proposed by RAN4 are:

- Option 1: Remove RSRQ from the cell selection and cell re-selection criterion when a cell is measured using RSS.

- Option 2: Define RSRQ for RSS measurements.

Companies are requested to provide comments in the table below (one row for each new comment to better keep track of the discussion – please don’t edit the previous comments).

| Company | Option 1 or Option 2? | Detailed comments |
| --- | --- | --- |
| Qualcomm | Option 1 | RAN1/RAN had already concluded not to support RSRQ with RSS, see R2-2103013. The proposed draft reply LS for option 1 says RAN2 has agree CRs for Option 1 but there are no CRs submitted to this meeting. Therefore, the response in this LS should be modified as follows: “RAN2 have discussed the options listed in the LS from RAN4 and has agreed the attached CR for to implement option 1 (i.e. remove RSRQ has been removed from the cell selection and cell re-selection criterion when a cell is measured using RSS).” |
| ZTE | Option 2 | We understand that the WID update was to exclude the RSRQ improvement through the use of RSS, not to exclude the RSRQ measurement. According to the following description in TS 36.214 “E-UTRA Carrier Received Signal Strength Indicator (RSSI), comprises the linear average of the total received power (in [W]) observed only in certain OFDM symbols of measurement subframes”, we understand the measurement signal density will not increase the RSSI measured value, it may only improve the measurement accuracy. If the Option 1 is selected, the cell selection and/or reselection performance may be impacted and should be re-evaluated. Furthermore, the RAN2 specification may be impacted much (e.g. both the TS 36.304 and TS 36.331 will be impacted). So, RSS based measurement should not deactivate RSRQ based cell selection/reselection criterion. Therefore, we suggest to response the LS with the following points: RAN2 has discussed the two options in the LS related to RSS based RSRQ for LTE-MTC. Based on the discussion, RAN2 provide the following understanding: • The Option 1 is not preferred from RAN2’s perspective. • Whether Option 2 is necessary should be decided by RAN1.. |

- Conclusion

TBD

- Reference

[1]	R2-2103491, RSRQ measurements when RSS is used, Huawei, HiSilicon

[2] R2-2103013, Whether to support RSRQ with RSS, Qualcomm Incorporated

[3] R2-2104182, Consideration on LS related to RSS based RSRQ for eMTC, ZTE Corporation, Sanechips

# Appendix A Draft Reply LS for Option 1

3GPP TSG-RAN2 Meeting #113bis-e	R2-21xxxxx

Online, April 12 – 20, 2021

Title:	[draft] Reply LS related to RSS based RSRQ for LTE-MTC

Response to:	R4-2103728

Release:	Release 16

Work Item:	LTE_eMTC5-Core

Source:	[Huawei – to be] RAN2

To:	RAN4,

Cc:	RAN1

Contact Person:

#### Name:	Brian Martin

#### E-mail Address:	brian DOT alexander DOT martin AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: [Agreed CR]

1. Overall Description:

RAN2 thanks RAN4 for the LS in R4-2103728.

RAN2 have discussed the options listed in the LS and have agreed the attached CR for option 1 (i.e. RSRQ has been removed from the cell selection and cell re-selection criterion when a cell is measured using RSS).

2. Actions:

To RAN4:

RAN2 kindly asks RAN4 to take the above into account.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #114-e		  			19 – 27 May, 2021

TSG-RAN2 Meeting #115-e		16 – 27 Aug, 2021

# Appendix B: Draft Reply LS for Option 2

3GPP TSG-RAN2 Meeting #113bis-e	R2-21xxxxx

Online, April 12 – 20, 2021

Title:	[draft] Reply LS related to RSS based RSRQ for LTE-MTC

Response to:	R4-2103728

Release:	Release 16

Work Item:	LTE_eMTC5-Core

Source:	[Huawei – to be] RAN2

To:	RAN4, RAN1

Cc:

Contact Person:

#### Name:	Brian Martin

#### E-mail Address:	brian DOT alexander DOT martin AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

1. Overall Description:

RAN2 thanks RAN4 for the LS in R4-2103728.

RAN2 have discussed the options listed in the LS and have decided that from a RAN2 perspective option 2 is preferred.

2. Actions:

To RAN1, RAN4:

RAN2 kindly asks RAN1 and RAN4 to take the above into account, and update the specifications accordingly.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #114-e		  			19 – 27 May, 2021

TSG-RAN2 Meeting #115-e		16 – 27 Aug, 2021