3GPP TSG-RAN WG2 Meeting #114 electronic				R2-2104968
Online, May 19 – May 27, 2021

Agenda item:		8.6.5

Source:			Asia Pacific Telecom, FGI

Title:		Beam selection and failure handling for CG-SDT

Document for:	 	Discussion and decision

Introduction

In RAN2#112, we agreed that the association between the CG resource and the SSBs is need for CG-SDT. The UE should select one of the SSBs with SS-RSRP above the threshold and use the associated CG resource for SDT.

Agreements in RAN2#112 - From RAN2 point of view: An association between CG resources and SSBs is required for CG-based SDT. FFS up to RAN1 how the association is configured or provided to the UE. Send an LS to RAN1 to start the discussion on how the association can be made. Mention that one option RAN2 considered was explicit configuration with RRC Release message - A SS-RSRP threshold is configured for SSB selection. UE selects one of the SSB with SS-RSRP above the threshold and selects the associated CG resource for UL data transmission.

In RAN1#104bis, they agreed that the CG resources per CG configuration are associated with a set of SSB(s) configured by explicit signalling, while the details, e.g., the resource mapping, are FFS.

Agreements in RAN1#104bis - CG resources per CG configuration are associated with a set of SSB(s) configured by explicit signalling. - FFS how to define an SSB-to-PUSCH resource mapping within the CG configuration. - FFS specific changes to the CG configuration to support the additional SSB-to-PUSCH mapping, if any.

On top of that, two issues on beam operation should be discussed. The first issue is to discuss when to select the SSB, i.e., the UE should select the SSB for each CG new transmission or only need to select the SSB for the very first CG new transmission in the CG-SDT procedure. The second issue is to discuss how to handle the beam failure condition (i.e., in case that none of SSBs with SS-RSRP above the RSRP threshold).

Discussion

Beam selection

For the CG-SDT, one issue is about when to select the SSB for CG transmission. In our view, there are two options for the UE to select the SSB, as shown in Figure 1.

- Option 1: The UE only selects the SSB once for the very first CG new transmission (i.e., the UE does not select the SSB again in subsequent data transmission period)

- Option 2: The UE selects SSB for each CG new transmission (i.e., the UE should select SSB in subsequent data transmission period)

Figure 1: SSB selection for CG

For option 1, after the UE initiates a CG-SDT procedure, the UE will select the SSB only for the very first CG transmission. After that, the UE does not need to select SSB again, and the UE will use the selected SSB for the subsequent data transmissions until the NW updates the beam for the UE. Thus, the maintenance of the beam, e.g., for PDCCH and PDSCH, should rely on NW. Moreover, in option 1, since the UE only selects the SSB once, the UE uses the same identity of the CG occasion for the CG configuration based on the SSB selected for the first CG transmission in the following CG transmissions.

For option 2, after the UE initiates a CG-SDT procedure, the UE will select the SSB for each CG new transmission. The benefit of option 2 is that the UE could provide the latest channel condition to NW by repeatedly selecting SSB for each CG new transmission. Option 2 is closer to the beam operation in the legacy RA procedure. (Note: in the legacy RA procedure, the UE selects the SSB for each RA preamble transmission). Moreover, since the UE selects the SSB for each CG transmission, the UE may perform transmissions on different CG occasions with different identities for the CG configuration based on the SSB selected recently in the following CG transmissions.

In comparison with option 1 and option 2, option 2 can provide the latest information of qualified beam to NW during the subsequent data transmission period without further impact. Thus, option 2 is preferred.

Proposal 1: UE performs SSB selection for each CG new transmission in the SDT procedure (including the subsequent data transmission period) and performs the CG new transmission via a CG occasion associated with the latest selected SSB.

Beam failure handling

In some situations, it is likely that the UE suffers from poor channel condition. That is, none of SSBs with SS-RSRP above the RSRP threshold. There are some options to handle it. In the legacy RA procedure, the UE selects any SSB to attempt the preamble transmission even if the quality of the SSB is lower than the RSRP threshold.

One approach is to inherit the same method as RA procedure, i.e., the UE selects any SSB and performs the CG transmission anyway based on the selected SSB. However, there are some differences between legacy RACH transmission and CG-SDT transmission. For instance, the UE would transmit larger size of data via CG transmission than RA, which needs better channel quality. Hence, if the DL channel condition is poor (i.e., no beam is qualified), it is very likely that the CG transmission will fail at the end. Therefore, the UE should avoid performing UL transmission via CG-SDT if none of qualified SSB can be detected. From this point of view, the following two options can be considered.

Observation 1: It is very likely that the CG transmission will fail at the end if the UE selects any SSB to perform the CG transmission when none of SSBs with SS-RSRP above the RSRP threshold.

Option 1 is to switch to RA-SDT procedure from CG-SDT directly. That is, in case that none of SSBs with SS-RSRP above the RSRP threshold, the UE should release the CG-SDT resource and initiate a RA-SDT procedure (if the RA-SDT is configured). Nonetheless, option 1 may be too overkilled to directly abandon the CG-SDT only based on detecting the poor beam for once. In some conditions, the UE just suffers from the beam blockage temporarily, i.e., the UE may relieve this situation soon.

To provide more opportunities for attempting the CG transmission in poor beam condition:

Option 2 is that the UE can skip the CG transmission in case none of SSBs with SS-RSRP above the RSRP threshold for SSB. However, for option 2, the UE may need to maintain a counter and count how many times that the UE skips the CG transmission due to poor beam within a period. If the value of the counter reaches to a maximum value, the UE should release the CG resource and initiates a RA-SDT procedure (if the RA-SDT is configured). This mechanism is somehow like beam failure detection and recovery procedure. But the drawback is to increase the specification complexity.

Proposal 2: RAN2 down selects one of the following options for CG-SDT when none of SSBs with SS-RSRP above the RSRP threshold:

- Option 1: UE releases the CG-SDT resource and initiates a RA-SDT (if configured).

- Option 2: UE skips the CG transmission and increments a counter by 1. If the counter reaches to a maximum value, UE releases the CG-SDT resource and initiates a RA-SDT (if configured).

Conclusion

In summary, we have the following proposals.

Proposal 1: UE performs SSB selection for each CG new transmission in the SDT procedure (including the subsequent data transmission period) and performs the CG new transmission via a CG occasion associated with the latest selected SSB.

Observation 1: It is very likely that the CG transmission will fail at the end if the UE selects any SSB to perform the CG transmission when none of SSBs with SS-RSRP above the RSRP threshold.

Proposal 2: RAN2 down selects one of the following options for CG-SDT when none of SSBs with SS-RSRP above the RSRP threshold:

- Option 1: UE releases the CG-SDT resource and initiates a RA-SDT (if configured).

- Option 2: UE skips the CG transmission and increments a counter by 1. If the counter reaches to a maximum value, UE releases the CG-SDT resource and initiates a RA-SDT (if configured).

Reference