3GPP TSG-RAN WG2 Meeting #114-e 	R2-2105221

Electronic, 17th – 27th May, 2021

Agenda item:	8.11.4

Source:	Huawei, HiSilicon

Title:	TP for TS 38.305: on-demand PRS

Document for:	Approval

- Proposed changes

*** Unchanged text is omitted ***

7.6	Procedures for on-demand PRS

7.6.1	General

On-demand DL PRS functionality supports the following two modes:

-	UE-initiated request of on-demand DL PRS transmission

-	LMF (network)-initiated request of on-demand DL PRS transmission

7.6.2	On-demand PRS Procedure

The general procedure for UE-initiated or LMF (network)-initiated on-demand PRS is described in Figure 7.6.2-1.

Figure 7.6.2-1 The procedure for UE/LMF initiated on-demand DL PRS

1a.	Either: UE sends an on-demand PRS request through an LPP message (e.g. LPP RequestAssistanceData) including the requested or recommended PRS configuration, e.g. specific PRS pattern, ON/OFF, periodicity, BW, etc. Note that this step is optional and only applicable for UE-initiated on-demand PRS request.

1b.	Or: LMF decides on its own (e.g. Inactivity of positioning methods based on DL-PRS) that LMF-initiated on-demand PRS procedure is instigated.

2.	LMF sends an on-demand PRS request to the gNBs. This request may be carried in an NRPPa message (e.g. TRP INFORMATION REQUEST).

3.   The serving and neighbouring gNBs return a response to indicate whether to support the requested PRS configuration. This response may be carried in an NRPPa message (e.g. TRP INFORMATION RESPONSE).

4.	LMF sends a notification for the on-demand PRS request to UE through an LPP message (e.g. LPP ProvideAssistanceData), which may include the updated PRS configuration.

NOTE 1: The above procedure is only applicable for RRC_CONNECTED UE.

*** Unchanged text is omitted ***