3GPP TSG RAN WG2 Meeting #114-e                                                           R2-2105765

e-Meeting, 19th – 27th May, 2021                                            (Revision of R2-2100360)

Agenda item:	8.1.2.4

Source:	Samsung

Title:	SDAP/PDCP/RLC Aspects for MBS

Document for:	Discussion and Decision

1	Introduction

This document discusses remaining open issues for SDAP/PDCP/RLC layers.

2	Discussion

2.1 SDAP

SDAP is a newly added layer of Rel-15 NR connected to 5GC. Two main functions of NR SDAP are mapping of QoS flow to DRB for a PDU session consisting of multiple QoS flows and uplink reflective QoS flow mapping. For MBS, SA2 is considering a similar concept of MBS session consisting of multiple MBS flows. Then SDAP should be able to map MBS flow to an MRB (multicast RB). Reflective QoS mapping was introduced to support the same QoS between uplink and downlink. However, an MBS session does not have any uplink data except a few control signalling in RAN. Thus, reflective QoS is not necessary for MBS.

Proposal 1. Reflective QoS is not necessary for MBS.

SDAP header is used to identify each packet’s QFID for gNB to understand received packet’s QF. In case that no uplink SDAP SDU, the SDAP header is not necessary.

Proposal 2. SDAP header is absent.

In unicast, gNB may remap a QF from one DRB to different DRB, according to gNB’s internal policy considering QoS requirements. However, no specific mechanism for downlink is supported in Rel-15/16 NR. It should be noted that end marker packet can be used only for uplink. The reason not to have end marker in downlink is that gNB can fully control the exact transmission time and the end of transmission of certain QF via old DRB. Thus, DL QF remapping can be performed by gNB implementation without specification impact.

Proposal 3. DL QF remapping for MBS can be performed by gNB implementation without specification impact, as in unicast.

2.2 PDCP and RLC

In RAN2#112-e, ROHC U-mode was agreed to support. Also, UMTS MBMS supports ROHC-U mode. We may not need to go further than that. In common PDCP (one PDCP with one PTP bearer and one PTM bearer), uplink transmission and bi-directional mode may be possible. But it may not be essential. ROHC continue may be supported for intra-gNB HO. Also, if two gNBs have synchronized PDCP operation, ROHC continue may be used for inter-gNB HO. There seems no critical an issue.

Proposal 4. ROHC continue can be configured.

In MBS, UE may join an ongoing MBS service and receive the data from the middle of the sequence number (SN). However, the current PDCP and RLC operations for unicast assume receiving data from SN0. Thus, the existing unicast behaviour would not work in MBS, so we need to consider how to support the MBS feature. One simple way would be that gNB indicates the initial values of PDCP and RLC state variables which UE should use for MRB. The initial values can be delivered when gNB provides MBS-related configurations by dedicated RRC signalling, which RAN2 already agreed.

One may argue that the state variables can be determined by V2X rule using the first received packet. However, there are reordering delay in the V2X rule by the SN gap between RX_DELIV and RX_NEXT, which is set because of absence of any reordering information. In Uu interface between gNB and UE, this unnecessary reordering delay can be avoided by gNB to provide the initial values appropriately.

Proposal 5. Initial values of PDCP/RLC state variables are provided to UE by dedicated RRC message.

3	Conclusion

RAN2 is kindly requested to discuss and agree the following proposals:

Proposal 1. Reflective QoS is not necessary for MBS.

Proposal 2. SDAP header is absent.

Proposal 3. DL QF remapping for MBS can be performed by gNB implementation without specification impact, as in unicast.

Proposal 4. ROHC continue can be configured.

Proposal 5. Initial values of PDCP/RLC state variables are provided to UE by dedicated RRC message.