3GPP TSG-RAN WG2 #114-e	R2-2105955

Electronic meeting, 19th – 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0378 | rev |  | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Description of PDCCH features introduced for NR-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the RAN1 LS, R1-2104094, RAN1 asked RAN2 to add missing description of PDCCH features in TS 38.300 that have been introduced as part of the NR-U work item. Some of these features are also applicable for operation in licensed spectrum. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description for the following PDCCH features for operation also in licensed bands: - Triggering one-shot HARQ codebook feedback Add description for the following PDCCH features for operation with shared spectrum channel access: - Triggering search space set group switching - Indicating available RB sets and COT duration - Indicating downlink feedback information for configured grant (CG-DFI) PUSCH Impact analysis Impacted 5G architecture options: Standalone, MR-DC Impacted functionality: PDCCH features Inter-operability: - If the network is implemented according to the CR and the UE is not, there are no interoperability issues. If the UE indicates capability bits for these PDCCH features, the network may use them accordingly. - If the UE is implemented according to the CR and the network is not, there are no interoperability problems. The network may not use these PDCCH features. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing description of PDCCH features introduced in Rel-16, which may therefore not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 5.2.3	Physical downlink control channels

The Physical Downlink Control Channel (PDCCH) can be used to schedule DL transmissions on PDSCH and UL transmissions on PUSCH, where the Downlink Control Information (DCI) on PDCCH includes:

-	Downlink assignments containing at least modulation and coding format, resource allocation, and hybrid-ARQ information related to DL-SCH;

-	Uplink scheduling grants containing at least modulation and coding format, resource allocation, and hybrid-ARQ information related to UL-SCH.

In addition to scheduling, PDCCH can be used to for

-	Activation and deactivation of configured PUSCH transmission with configured grant;

-	Activation and deactivation of PDSCH semi-persistent transmission;

-	Notifying one or more UEs of the slot format;

-	Notifying one or more UEs of the PRB(s) and OFDM symbol(s) where the UE may assume no transmission is intended for the UE;

-	Transmission of TPC commands for PUCCH and PUSCH;

-	Transmission of one or more TPC commands for SRS transmissions by one or more UEs;

-	Switching a UE's active bandwidth part;

-	Initiating a random access procedure;

-	Indicating the UE(s) to monitor the PDCCH during the next occurrence of the DRX on-duration;

-	In IAB context, indicating the availability for soft symbols of an IAB-DU;

- 	Triggering one shot HARQ-ACK codebook feedback;

-	Triggering search space set group switching for operation with shared spectrum channel access;

-	Indicating one or more UEs about the available RB sets and COT duration for operation with shared spectrum channel access;

-	Indicating downlink feedback information for configured grant PUSCH (CG-DFI) for operation with shared spectrum channel access.

A UE monitors a set of PDCCH candidates in the configured monitoring occasions in one or more configured COntrol REsource SETs (CORESETs) according to the corresponding search space configurations.

A CORESET consists of a set of PRBs with a time duration of 1 to 3 OFDM symbols. The resource units Resource Element Groups (REGs) and Control Channel Elements (CCEs) are defined within a CORESET with each CCE consisting a set of REGs. Control channels are formed by aggregation of CCE. Different code rates for the control channels are realized by aggregating different number of CCE. Interleaved and non-interleaved CCE-to-REG mapping are supported in a CORESET.

Polar coding is used for PDCCH.

Each resource element group carrying PDCCH carries its own DMRS.

QPSK modulation is used for PDCCH.

End of changes