3GPP TSG-RAN WG2 Meeting #114-e	R2-2105988

Online, 19 -27 May Apr 2021

Agenda item:	8.2.3.1

Source: 	Huawei, HiSilicon

Title: 	Inter-node message design (with draft reply LS to RAN3)

Document for: 	Discussion and Decision

# 1. Introduction

In the last meeting, RAN3 discussed the CPAC and sent one reply LS [1] to RAN2. RAN3 asked two questions on the inter-node messages.

- About the inter-node RRC container design - In case multiple PSCells are prepared in one CPAC procedure, RAN3 would like to ask RAN2 to feedback on the inter-node RRC container design: will one RRC container for one PSCell be used, or one RRC container for multiple PSCells? - About the SN initiated inter-SN CPC, RAN3 would like to ask RAN2 to feedback on the following two alternatives: - Alternative 1: MN performs the association between the execution condition received from the source SN and the RRC configuration of the candidate PSCell received from the candidate SN. - Alternative 2: MN forwards the execution condition received from the source SN to the candidate SN. The candidate SN sends the execution condition and the RRC configuration of the candidate PSCell to the MN.

In this contribution, we will discuss the inter-node message design from RAN2 point of view.

# 2. Discussion

## 2.1 inter-node RRC container

According to the reply LS [1] from RAN3, RAN3 has the WA to support multiple PSCells in one CPAC procedure.

RAN3 discussed how to support CPAC and would like to consider following information as relevant for RAN2. - Relevant RAN3 agreements and working assumptions: - Target SN to make the decision on the prepared PSCells. - WA: Prepare multiple PSCells in one CPAC procedure. - In CPA and MN initiated inter-SN CPC, MN does not send execution condition(s) to the Target SN, Target SN provides the prepared PSCell id(s) and the corresponding RRC container(s) (RRCReconfiguration) to the MN, and then the MN generates and transmits the conditional configuration message to the UE. - WA: In SN initiated inter-SN CPC, prepare multiple PSCells in one target SN by one SN Change procedure is the baseline.

Based on above RAN3 WAs, different from CHO in CPAC more than one candidate PSCells can be prepared in one CPAC procedure, i.e. the related information for multiple PSCells should be indicated in one SN addition request/request acknowledge message. Thus before discussing detailed inter-node RRC message design for CPAC, we could first look into the existing inter-node RRC messages of CG-ConfigInfo and CG-Config, and check which existing parameters are needed for CPAC, and the corresponding exchange granularity, i.e. per SN or per PSCell. We gives our analysis in the following tables.

Table 1 Existing parameters in CG-ConfigInfo to be reused for CPAC

| Existing parameters in inter-node message |  | MN initiated CPA | MN initiated CPC | SN initiated CPC |
| --- | --- | --- | --- | --- |
| CG-ConfigInfo | ue-CapabilityInfo, candidateCellInfoListMN, measResultCellListSFTD-NR, drx-InfoMCG, measConfigMN, mcg-RB-Config, mrdc-AssistanceInfo, ph-InfoMCG, drx-ConfigMCG, fr-InfoListMCG, sftdFrequencyList-NR, servFrequenciesMN-NR, drx-InfoMCG2 | Need. Per candidate SN | Need. Per candidate SN | Need. Per candidate SN |
|  | candidateCellInfoListSN, sourceConfigSCG, scg-RB-Config, ueAssistanceInformationSourceSCG-r16 | N/A | Need. Per candidate SN | Need. Per candidate SN |
|  | ScgFailureInfo, measResultReportCGI, alignedDRX-Indication, scgFailureInfo-r16, measResultReportCGI, | N/A(because the candidate SN configuration has not been executed) |  |  |
|  | configRestrictInfo | Need. Per candidate SN could work. |  |  |
|  | candidateCellInfoListMN-EUTRA, candidateCellInfoListSN-EUTRA, sourceConfigSCG-EUTRA, scgFailureInfoEUTRA, measResultReportCGI-EUTRA, measResultCellListSFTD-EUTRA, sftdFrequencyList-EUTRA, scgFailureInfoEUTRA-r16, sidelinkUEInformationEUTRA-r16 | N/A(because RAN2 has agreed to only support NR SCG in CPAC) |  |  |
|  | sidelinkUEInformationNR-r16 | N/A(because it is designed for the interaction between CU/DU, not for MR-DC) |  |  |

Table 2 Existing parameters in CG-Config to be reused for CPAC

| Parameters in inter-node message |  | MN initiated CPA | MN initiated CPC |  | SN initiated CPC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Candidate SN->MN | Source SN->MN | Candidate SN->MN | Source SN->MN | Candidate SN->MN |
| CG-Config | scg-CellGroupConfig | Need. Per candidate pSCell | Need. Per candidate SN | Need. Per candidate pSCell | Need. Per candidate SN | Need. Per candidate pSCell |
|  | scg-RB-Config | Need. Per candidate SN | Need. Per candidate SN | Need. Per candidate SN | Need. Per candidate SN | Need. Per candidate SN |
|  | configRestrictModReq, measConfigSN, reportCGI-RequestNR | N/A(because the candidate SN configuration has not been executed) |  |  |  |  |
|  | drx-InfoSCG, drx-ConfigSCG, drx-InfoSCG2 | Need. Per candidate SN | N/A | Need. Per candidate SN | N/A | Need. Per candidate SN |
|  | candidateCellInfoListSN, candidateServingFreqListNR, ueAssistanceInformationSCG-r16 | N/A | Need. Per candidate SN | N/A | Need. Per candidate SN | N/A |
|  | selectedBandCombination, fr-InfoListSCG, scellFrequenciesSN-NR, selectedToffset-r16, ph-InfoSCG | Need. Per candidate PSCell | N/A | Need. Per candidate PSCell | N/A | Need. Per candidate PSCell |
|  | pSCellFrequency | N/A(The MN can know the PSCellFrequency according to the candidate PSCell ID) |  |  |  |  |
|  | pSCellFrequencyEUTRA,scg-CellGroupConfigEUTRA candidateCellInfoListSN-EUTRA candidateServingFreqListEUTRA reportCGI-RequestEUTRA, needForGaps, scellFrequenciesSN-EUTRA | N/A(because RAN2 has agreed to only support NR SCG in CPAC) |  |  |  |  |

According to the above analysis, we think the highlighted parameters in the CG-Config/ CG-ConfigInfo should be sent per candidate PSCell.

The existing parameters in CG-ConfigInfo sent from MN to candidate target SN to be reused for CPAC are all per-SN.

The existing parameters in CG-Config sent from source SN to MN to be reused for CPC are all per-SN.

Some existing parameters in the CG-Config sent from candidate target SN to MN should be per candidate PSCell.

One way to provide the per-PSCell parameters is to add a new list to include the parameters for each cell on top of the existing parameters in the CG-Config inter-node message. However, if RAN2 needs to discuss and select parameters one by one, this may be time-consuming. In addition, if new parameters added for MR-DC in the future, RAN2 also needs to check if it should be added in the CPAC-specific list.

A simpler way is to take the full existing CG-Config for each candidate PSCell.

Both ways can work, but we slightly prefer the latter one for simplicity.

In order to exchange per-PSCell parameter by reusing existing inter-node RRC message for CPAC, a list of CG-Config associated to each candidate PSCell should be sent from candidate SN to MN.
FFS if a list of CG-ConfigInfo from MN to candidate SN is needed. FFS if a list of CG-Config from source SN to MN is needed.

In our understanding, there are two options to enable the transmission of these parameters per candidate PSCell:

Option 1: specify new inter-node message for the CPAC. These new inter-node messages include the list of CG-Config

Option 2: reuse the existing CG-Config message. RAN3 adds the list of RRC containers with each containing one CG-Config for the CPAC.

We feel it would be better to address this issue within RAN2 and avoid unnecessary RAN3 involvement. We prefer option1 slightly.

RAN2 to specify a new inter-node message to include a list of CG-Config with each associated to one candidate PSCell for CPAC.

If proposal 1 and 2 are agreed, RAN2 to inform RAN3 that one inter-node RRC container is to be needed for CPAC.

## 2.2 Execution condition interaction in SN initiated inter-SN CPC

RAN3 also asked the following question on the execution condition in SN initiated inter-SN CPC.

- About the SN initiated inter-SN CPC, RAN3 would like to ask RAN2 to feedback on the following two alternatives: - Alternative 1: MN performs the association between the execution condition received from the source SN and the RRC configuration of the candidate PSCell received from the candidate SN. - Alternative 2: MN forwards the execution condition received from the source SN to the candidate SN. The candidate SN sends the execution condition and the RRC configuration of the candidate PSCell to the MN.

In R15&R16, the target SN can decide the PSCell based on UE measurement results. The UE accesses the PSCell after receiving the configuration of the PSCell. In most cases, the signal quality in measurement results received by the target SN from the source SN is nearly the same to the signal quality when the UE access to the PSCell. Therefore, the target SN can select the suitable PSCell that has the best measurement results to ensure the success of the access. In addition, the target SN may have different selection criteria for different frequencies. For example, FR2 may have a higher requirement for the signal quality than the FR1. The target SN may not select one FR2 Cell as the PSCell even if the signal quality of the FR2 cell is better than the signal quality of all FR1 cells. The target SN will only select the FR2 cell as the PSCell only if the signal quality of the FR2 cell is good enough.

In legacy MR-DC, the target SN can select the suitable PSCell based on the signal quality, e.g. select the PSCell that has the best measurement result to ensure the success of the access of PSCell.

In CPAC, RAN3 has agreed that it is the candidate SN to make the decision on the prepared PSCells. In CPAC, the UE will not immediately access the candidate PSCell after receiving the configuration of candidate PSCell. The signal quality in the measurement results received by the candidate SN in the SN addition request can be very different from the signal quality when the UE access to the candidate PSCell so the candidate SN cannot select the suitable PSCell based on the measurement result received from the source SN.

In the last meetings, RAN2 has agreed to use the A3/A5 as the execution condition for conditional PSCell change. RAN2 also has agreed the source SN decides the execution condition. In addition, the source SN needs to forward the whole SN RRC configuration to the candidate SN in order to perform the delta configuration. This means that the candidate SN can know the entire measurement configuration in the UE. If the candidate SN can know the execution condition of each candidate PSCell, the candidate SN can choose one suitable candidate PSCell based on the execution condition of each candidate PSCell and the measurement configuration of each execution condition.

In SN initiated inter-SN CPC, the candidate SN can select the suitable PSCell based on the execution condition.

Therefore, we prefer that the source SN can forward the execution condition of each candidate PSCell to the candidate SN via the MN.

In alternative 1, the MN needs to save the execution condition of each candidate PSCell after receiving them from the source SN and to lookup the execution condition for each candidate PSCell after receiving the candidate PSCell ID from the candidate SN. In alternative 2, if the execution conditions are forwarded to the target SN for the purpose of PSCell selection, the target SN could also provide the execution conditions to the MN together with each CG-Config message.

If the execution conditions are provided to the target SN as assistance information to select candidate target PSCells, it is straightforward for the target SN to include the relevant execution condition in each CG-Config message for each prepared candidate target PSCell.

Answer RAN3 that RAN2 would like execution conditions to be forwarded to the candidate target SN, as it can be useful for the candidate target SN to select candidate target PSCell. In such a case, alternative 2 would just require one additional parameter per candidate target PSCell.

# 3. Conclusion

Based on the discussion in this paper, we have the following proposals:

- In order to exchange per-PSCell parameter by reusing existing inter-node RRC message for CPAC, a list of CG-Config associated to each candidate PSCell should be sent from candidate SN to MN.
FFS if a list of CG-ConfigInfo from MN to candidate SN is needed. FFS if a list of CG-Config from source SN to MN is needed.

RAN2 to specify a new inter-node message to include a list of CG-Config with each associated to one candidate PSCell for CPAC.

If proposal 1 and 2 are agreed, RAN2 to inform RAN3 that one inter-node RRC container is to be needed for CPAC.

Answer RAN3 that RAN2 would like execution conditions to be forwarded to the candidate target SN, as it can be useful for the candidate target SN to select candidate target PSCell. In such a case, alternative 2 would just require one additional parameter per candidate target PSCell.

# 4. Reference

- eR3-211338, Reply LS on Conditional PSCell Addition/Change agreements

# 5. Reply LS to RAN3

3GPP TSG-RAN WG2 Meeting #114-e	R2-2XXXXXX

# Electronic Meeting, 19 - 27 May 2021

Title:	Reply LS on Conditional PSCell Addition/Change agreements

Response to:

Release:	Rel-17

Work Item:	LTE_NR_DC_enh2-Core

Source:	RAN2

To:	RAN3

Cc:

Contact Person:

Name: David Lecompte

E-mail Address: david.lecompte at huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks RAN3 for the Reply LS R3-211338 on Conditional PSCell Addition/Change agreements.

RAN2 has discussed the inter-node message design in CPAC and has the following agreements.

- One inter-node RRC container for multiple PSCells is to be specified for CPAC in RAN2, including a list of legacy inter-node RRC message (CG-Config) associated to one prepared PSCell.

- For the SN initiated inter-SN CPC, both alternatives mentioned by RAN are possible. Besides, RAN2 would like execution conditions to be forwarded to the candidate target SN, as it can be useful for the candidate taget SN to select candidate target PSCell. Therefore, alternative 2 can be realised by adding one parameter, containing the exetion condition, in the CG-Config associated with each prepared PSCell.

2. Actions:

To RAN3:

ACTION: RAN2 respectfully asks RAN3 to take the above information into account.

3. Date of Next TSG WG RAN2 Meetings:

TSG-RAN WG2 Meeting #115-e	16 -27 Augst 2021	E-meeting

TSG-RAN WG2 Meeting #116-e	1 - 12 November 2021	E-meeting