3GPP TSG-RAN2 #114e	R2-2106433

# Electronic meeting, May 19 – May 27, 2021

Agenda item:		8.5.2 (NR_IIOT_URLLC_enh-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on enhancements for support of time synchronization

Document for:	Discussion and Decision

1.	Introduction

In RAN2-113e meeting, the followings are assumed for support of time synchronization [1].

Assumptions: - There is no UE clock drift issue to be addressed - The source and target gNB are tightly synchronized to the same master clock within the budget and there is no need to optimize anything for HO.

This document discusses a potential issue that system frame numbers (SFNs) of the source cell and the target cell are different even when they are tightly synchronized to the same master clock.

2.	Discussion

Timing information for 5G internal system clock is contained in ReferenceTimeInfo IE as shown in below box [2]. Reference time information which is signaled to UE indicates the time at the ending boundary of the system frame number (SFN) indicated along with the reference time information.

ReferenceTimeInfo-r16 ::= SEQUENCE { time-r16 ReferenceTime-r16, uncertainty-r16 INTEGER (0..32767) OPTIONAL, -- Need S timeInfoType-r16 ENUMERATED {localClock} OPTIONAL, -- Need S referenceSFN-r16 INTEGER (0..1023) OPTIONAL -- Cond RefTime } … referenceSFN This field indicates the reference SFN corresponding to the reference time information. If referenceTimeInfo field is received in DLInformationTransfer message, this field indicates the SFN of PCell.. time This field indicates time reference with 10ns granularity. The indicated time is referenced at the network, i.e., without compensating for RF propagation delay. The indicated time in 10ns unit from the origin is refDays*86400*1000*100000 + refSeconds*1000*100000 + refMilliSeconds*100000 + refTenNanoSeconds. The refDays field specifies the sequential number of days (with day count starting at 0) from the origin of the time field. If the referenceTimeInfo field is received in DLInformationTransfer message, the time field indicates the time at the ending boundary of the system frame indicated by referenceSFN. The UE considers this frame (indicated by referenceSFN) to be the frame which is nearest to the frame where the message is received (which can be either in the past or in the future). If the referenceTimeInfo field is received in SIB9, the time field indicates the time at the SFN boundary at or immediately after the ending boundary of the SI-window in which SIB9 is transmitted. If referenceTimeInfo field is received in SIB9, this field is excluded when determining changes in system information, i.e. changes of time should neither result in system information change notifications nor in a modification of valueTag in SIB1.. …

The UE measures the time based on the downlink frame timing, SFN, and the latest reference time information. Then, the time may need to be compensated using the propagation delay. When a UE moves to the target cell from the source cell, time synchronization would work well without any optimization for HO if downlink timing is aligned between the source cell and the target cell and the target cell SFN is the same with the source cell SFN. According to the assumptions of RAN2-113e, it is assumed that downlink timing or system frame boundaries of the source cell and the target cell are aligned within the time synchronicity budget. However, it is not guaranteed that the SFNs are synchronized between the source cell and the target cell.

Observation 1. SFNs of the source cell and the target cell may be different even when they are tightly synchronized to the same master clock.

To make it sure that there is no need to optimize anything for HO, it needs to be guaranteed that the SFNs of the target cell and the source cell are synchronized as well.

Proposal 1. RAN2 discusses whether the SFNs of the target cell and the source are assumed to be synchronized as well when they are tightly synchronized to the same master clock.

In addition, RAN1-104b-e meeting concluded that RAN1 leaves it to RAN2 to decide whether to support UE based compensation and/or gNB based compensation for any propagation delay compensation (PDC) method RAN1 may adopt for Rel-17, if applicable [3]. In case of UE based compensation, the common part of the time is known to UEs by using the existing ReferenceTimeInfo IE and each UE can compensate it further by using its own PD information given by the gNB. In case of gNB based compensation, each UE receives the time compensated for it. For updating PDC, UE based compensation requires PD information only and gNB based compensation requires whole information of the time with compensation for PD. Based on this comparison, UE based compensation is preferred.

Observation 2. UE based compensation requires PD information only for updating PDC. On the other hand, gNB based compensation requires whole information of the time with compensation for PD.

Proposal 2. RAN2 supports UE based compensation.

3.	Conclusion

In this contribution, we have discussed support of time synchronization.

Observation 1. SFNs of the source cell and the target cell may be different even when they are tightly synchronized to the same master clock.

Proposal 1. RAN2 discusses whether the SFNs of the target cell and the source are assumed to be synchronized as well when they are tightly synchronized to the same master clock.

Observation 2. UE based compensation requires PD information only for updating PDC. On the other hand, gNB based compensation requires whole information of the time with compensation for PD.

Proposal 2. RAN2 supports UE based compensation.

4.	References

[1] R2-2102601, Report of 3GPP TSG RAN WG2 meeting #113-e

[2] 3GPP TS 38.331 v16.4.1, “Radio Resource Control (RRC) protocol specification (Release 16)”.

[3] RAN1 Chair’s Notes of 3GPP TSG RAN WG1 meeting #104b-e