3GPP TSG-RAN WG2 Meeting #114e	R2-2106695

Electronic, 19 – 27 May 2021

Agenda item:	8.3.3

Source: 	Qualcomm Incorporated

Title:  	[AT114-e][241][MUSIM] LS to SA2/CT1 on network switching

Document for:	Discussion and decision

# Introduction

During first online session of Rel-17 MUSIM WI in RAN2#114e, the following has been agreed:

We support at least AS-based solution (with AS-based response) for network switching while leaving RRC_Connected state in NW A. FFS if this may include NAS information

- 	ZTE thinks CT1 is discussing paging filtering assistance via NAS. Charter thinks NAS-based solution is needed for selective suspend. Nokia thinks NAS information can be included in assistance information. Samsung wonders if we need to send LS to SA2.

Email [241] (QC): Discuss LS to SA2/CT1 to inform them of RAN2 decision (CB in 2nd week Thu session)

The Session Chair has later clarified the scope of the offline #241 as follows:

- [AT114-e][241][MUSIM] LS to SA2/CT1 on network switching for leaving RRC_CONNECTED (Qualcomm)

Scope:

- Draft LS to SA2/CT1 to inform them of the RAN2 decision to support at least AS-based solution (with AS-based response) for network switching while leaving RRC_Connected state in NW A (FFS if this may include NAS information).

Intended outcome:

- Discussion summary in R2-2106503 (by email rapporteur).

- Draft LS to SA2/CT1 in R2-2106504 (by email rapporteur).

This document will capture feedback from companies on network switching to leave Connected state in order to agree on the contents of the LS to SA2/CT1.

| Company | Contact Name, Email |
| --- | --- |
| OPPO | fanjiangsheng@oppo.com |
| Sharp | Fangying Xiao, Fangying.xiao@cn.sharp-world.com |
| Apple | Sethuraman Gurumoorthy, sethu@apple.com |
| Huawei, HiSilicon | Yiru Kuang, kuangyiru@huawei.com |
| ZTE | li.wenting@zte.com.cn |
| CATT Charter Communications | zhourui@catt.cn Reza Hedayat, reza.hedayat@charter.com |
| MediaTek Ericsson | Felix Tsai, chun-fan.tsai@mediatek.com lian.araujo@ericsson.com |
| DENSO | Tomoyuki Yamamoto, tomoyuki.yamamoto.j5c@jp.denso.com |
| Samsung Xiaomi | Sangyeob Jung, sy0123.jung@samsung.com Wei HONG, hongwei@xiaomi.com |
| LG Electronics | Hongsuk Kim, hassium.kim@lge.com |
| Nokia Sony | Srinivasan. Srinivasan.selvaganapathy@nokia.com Anders.Berggren@sony.com |
| China telecom Vodafone | zhangt77@chinatelecom.cn Manook Soghomonian , manook.soghomonian@vodafone.com |
| Lenovo | Wulh5@lenovo.com |

# Discussion

It would be good to have a common understanding what the agreed “AS-based solution” entails.

In Rapporteur’s understanding, the intended AS-based switching solution will be similar or based on the existing Rel-16 RRC signaling for leaving Connected state. In Rel-16, the UE can send releasePreference-r16 in UE Assistance Information (UAI), which can indicate a preference for Idle or Inactive state. Then, it is up to the gNB to release the UE to the requested RRC state.

Even though the stage-3 details of the “AS-based solution” for network switching has not been discussed and may be (slightly) different in the end, it would be beneficial to agree on the stage-2 structure and also include this in the LS to SA2/CT1. This can be summarized as follows:

- The UE sends an RRC message (e.g. UAI) to the gNB which includes a request to be released to Idle or Inactive state.

- The gNB response to the UE can release the UE (e.g. via RRC Release) to Idle or Inactive state.

Q1: Do you agree with the above steps for the “AS-based solution” for network switching?

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | Agree | The existing RRC signaling can work at least, no strong motivation to have another similar solution. |
| Sharp | Agree with comments | For the first point, UE should be also allowed just to indicate to be released from RRC_CONNECTED and has no preferred RRC state to transition to as in Rel-16. |
| Apple | Agree with comments | Existing RRC UAI signalling can be reused. But to distinguish between NR UE Power Save and MUSIM, the UAI can potentially signal the difference that this for a MUSIM Network switching case. But this can be further discussed agreed in Stage 3. |
| Huawei, HiSilicon |  | Generally the stage-2 structure can be 1) UE sends request message indicating to leave RRC connected state 2) NW can respond to the UE. However, for the stage-2 structure above, for Rel-16 releasePreference-r16 in UAI, preferred state (e.g. Idle or Inactive) is optional to be indicated by the UE, it is not clear whether the preferred state is also optional here, this is not discussed in RAN2. Besides, UE is allowed to enter RRC_IDLE state if it does not receive response message from network within a certain configured time period, so the above step 2 is not mandatory. We think there are many details which is not clear, so we just need to inform the RAN2 agreement to SA2/CT1. |
| ZTE | Agree | Same view as OPPO |
| CATT | Agree | Agree with comments above.We only need to inform the RAN2 agreement to SA2/CT1.details in RAN2 can be discussed further. |
| Charter Communications | Agree |  |
| MediaTek | Partial | We understand the agreement so far is <1> The UE sends RRC message to inidicate it want to leave RRC_CONNECTED state in network A for MUSIM purpose <2> The NW sends RRC Release message to the UE Further details (which RRC message to use, INACTIVE mode aspect) could be further discussed in RAN2. We only need to inform SA2/CT1 our current agreements. |
| Ericsson | Yes, with comments | We think the wording should be more aligned with handling of UE assistance information message in that case: - The UE may be configured to sends an RRC message (e.g. UAI) to the gNB which includes a request preference to be released to Idle or Inactive state. - The gNB response to the UE can may release the UE (e.g. via RRC Release) to Idle or Inactive state. |
| DENSO | Agree | We agree with the steps generally. The detail of the message can be discussed further. |
| Samsung | Agree with comments | We are OK to include stage-2 structure in the LS to SA2/CT1 but following stage-3 details should be discussed further in next RAN2 meetings: - Whether UE is allowed to send its explicit preferred RRC state - Whether to distinguish existing power saving feature and MUSIM feature of switching procedure for leaving RRC_CONNECTED state - And so on Thus, we share similar view with Huawei that the above steps can be modified as follows: - The UE sends an RRC message (e.g. UAI) to the gNB for leaving RRC_CONNECTED state. - The gNB response to the UE can release the UE (e.g. via RRC Release) to Idle or Inactive state. |
| Xiaomi | Agree | We should reuse exsiting RRC signaling procedures, e.g. UE Assistance Information procedure. |
| LGE | Agree |  |
| Nokia | Partial | We only need to inform that RAN2 agrees to support AS based solution for leaving RRC-CONNECTED state. Th transition to RRC-IDLE/RRC-INACTIVE are to be discussed as part of the solution discussion which requires further discussion. |
| Sony | Agree | Should reuse the existing RRC signaling when possible. |
| China Telecom | Agree with comments | We share the same view as some companies mentioned above that we only reach agreement on the following aspects: 1. The UE can send RRC message to indicate it want to leave RRC_CONNECTED state for MUSIM purpose 2. The NW can release UE on demand |
| Vodafone | Agree |  |
| Lenovo | Agree |  |

Summary: All the companies agree that the AS-based solution is based on this RRC request/response structure. Several companies wanted to keep it more stage-2 level for the LS. It is reasonable not to discuss any stage-3 details nor inform SA2/CT1.

Proposal 1: AS -based solution for network switching includes two steps: 1-) If configured, UE can send an RRC message to leave RRC_CONNECTED for MUSIM purpose 2-) gNB may release the UE to Idle/Inactive.

RAN2 has also agreed in RAN2#113bis-e meeting that the UE can leave the Connected state if it does not get a response within a configured time, as follows:

During switching procedure for leaving RRC_CONNECTED state, UE is allowed to enter RRC_IDLE state if it does not receive response message from network within a certain configured time period. FFS for RRC_INACTIVE state.

The Rapporteur thinks that it would help to include this agreement in the LS to SA2/CT1, at least for completeness. Since RAN2 has now agreed that the signaling for the switching is AS-based, it is natural to assume that the above “time period” should be configured by the gNB.

Q2: Do you agree including the above RAN2#113bis-e agreement in the LS to SA2/CT1?

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | No strong view | It seems that the spec impact is only limited to RAN, maybe RAN2 should also ‘cc’ RAN3 if RAN2 agrees to include this agreement in the LS. |
| Sharp | Yes |  |
| Apple | Yes | No harm in including this agreement if that provides more clarity on this topic. We do agree with Oppo view that the intended impact is limited only to RAN specifications. |
| Huawei, HiSilicon | Yes | OK to inform it as it is related to the switching procedure with leaving RRC_CONNECTED. |
| ZTE | Agree | OK to include this agreement into the LS |
| CATT | No strong view | Agree with OPPO |
| Charter Communications | Yes |  |
| MediaTek | Yes (No strong view) | Okay to inform SA2/CT1 |
| Ericsson | No need | Since it is a RAN timer and it is “transparent” to SA2 we do not see the need to include this in the LS. SA2 can also check RAN2 agreements if anything. |
| DENSO | Yes | Just for information, it is OK to inform. |
| Samsung | No | We understand that it does not impact SA2/CT1 specifications so there seems no required actions on SA2/CT1. Besides, there seem still some open issues to be resolved i.e. Q3 and FFS part. Thus, internal checking seems sufficient at this moment. If needed, we can send a LS later after finalizing our works on this. |
| Xiaomi | No need | No strong view. |
| LGE | Yes |  |
| Nokia | No strong view | This information is not needed. SA2/CT1 needs to know whether RRC level solution was considered for leaving RRC-CONNECTED state. Based on this information they can decide on further SA2/CT1 impacts related to NAS impacts of this decision. |
| Sony | Yes, | Good for SA2 to know as much as possible, it does not harm. |
| China Telecom | Yes |  |
| Vodafone | Agree |  |
| Qualcomm | Yes |  |
| Lenovo | Agree | This feature is mentioned in the LS from SA2. therefore, we should inform SA2. |

Summary: 3 companies do not see a need for SA2 to know while other companies either agree or have no strong view. Now that SA2 also have a working agreement on using NAS solution for Idle mode, this agreement can help SA2 in deciding whether to align their solution with RAN2.

Proposal 2: Include the following RAN2#113bis-e agreement in the LS:

During switching procedure for leaving RRC_CONNECTED state, UE is allowed to enter RRC_IDLE state if it does not receive response message from network within a certain configured time period. FFS for RRC_INACTIVE state

Q3: Do you agree that the “configured time”, for the UE to leave RRC_CONNECTED without a response, is configured by the gNB?

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | Yes but | This question is out of LS scope, better to postpone the discussion. |
| Sharp | Yes | This should be an optional function and release of the RRC Connection should rely on the RRCRelease message from gNB if the time is not configured. |
| Apple | Yes but.. | This needs some more online discussion in RAN2 e.g. on what the default UE behavior should be, whether if the timer is configured or not, but in our view the LS can still go without including the response to this question. |
| Huawei, HiSilicon | Yes but | It is reasonable that the “configured time” is configured by the gNB, but this does not need to be included in the LS. |
| ZTE | Yes but | Same view as Huawei |
| CATT | - | Agree with OPPO |
| Charter Communications | Yes |  |
| MediaTek | - | Not related to the LS content. |
| Ericsson | Yes | Agree with Huawei. |
| DENSO | Yes but | Same view with OPPO |
| Samsung | Yes but | Agree but as commented in Q2, there seems no need to include it in the LS to SA2/CT1. |
| Xiaomi | Yes, but | This should be discussed in RAN2 first and should not be included in the LS. |
| LGE | Yes |  |
| Nokia | Yes | GNB should configure this time via dedicated signalling. This information is not required in the LS. |
| Sony | Yes | No need to include it in LS |
| China Telecom | Yes, but | Agree with Huawei |
| Vodafone | Yes |  |
| Lenovo | Yes |  |

Summary: All companies agree that the timer should be configured by the gNB. But several companies do not see a need to include this in the LS.

Proposal 3: The “configured time” for the UE to leave RRC_CONNECTED without a response is configured by the gNB. However, this information is not needed to be included in the LS.

As mentioned during the online session, CT1 and SA2 are discussing proposals on NAS signaling for network switching. This NAS message from the UE can include a request for paging filtering, e.g. to be paged only for certain PDU sessions after the switching.

Such NAS signaling can be completed before the UE sends the AS-based switching request while the UE is still in the Connected state. In this case, it can work with the “AS-based” solution without causing any additional delay to the switching. However, if this NAS message is piggy-backed on the AS-based request or sent right before the AS signaling by the UE, the switching may be delayed if the UE has to wait for a NAS response. As agreed in the online session, the AS-based switching will be based on “with AS-based response”. The Rapporteur interprets this where, if a NAS message is sent by the UE to AMF as assistance information for the switching, a NAS response should not be necessary for the UE to perform switching. In other words, the UE should just follow the AS signaling for the switching and this should not cause any errors in the AS and NAS states and Idle/Inactive operation. Assuming RAN2 agrees on this understaning, it would also be good to include this in the LS since this can help SA2/CT1 to design such signaling accordingly, e.g. only one-way signaling from the UE.

Q4: Do you agree that, if SA2/CT1 agrees that the UE can send a NAS message (e.g. assistance information on paging filtering) to AMF for leaving the Connected state, the UE does not need to wait for a NAS confirmation message and will only follow the AS signaling response?

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | Maybe not | Firstly, RAN2 should ask SA2/CT1 whether they have the requirement to have NAS message (e.g. assistance information on paging filtering) piggy-backed into RRC signaling once AS based solution for network leaving is introduced. And then, on top of above, even if SA2/CT1 agrees to have NAS message piggy-backed into RRC signaling, RRC already introduces a configured timer before leaving, network implementation can configure a proper timer to allow UE to receive NAS response message before being released to idle/inactive. So no big issue identified for Q4. Based on above, it’s enough to ask SA2/CT1 whether they have the requirement to have NAS message (e.g. assistance information on paging filtering) piggy-backed into RRC signaling once AS based solution for network leaving is introduced. |
| Sharp | Yes |  |
| Apple | Maybe not | We are not sure if such a kind of unsolicited NAS signaling without waiting for NAS level response from NW is possible, as we always see a clean handshake with every NAS signaling message (request/response format). As Oppo indicated, we should ask SA2/CT1 if they are fine to send the NAS message piggy backed onto RRC signaling. That in our view would clean up both AS and NAS level signalling for this issue. |
| Huawei, HiSilicon | No | It seems far more than what we discussed online. We understand the LS is only to inform the RAN2 agreement. |
| ZTE | Maybe not | - According to the on-line discusson, whether include NAS information is FFS FFS if this may include NAS information About this LS, we think it can’t be determined by RAN2, some confirmation with SA2/CT1 is needed. To accelerate the discussion, we think we should include this FFS issue in the LS, meanwhile give RAN2’s understanding on this FFS. Maybe it would be better to give a clear signaling procedure for the case with/without NAS assistance information and confirm with CT1/SA2 that whether such kind of procedure is acceptable , otherwise, the CT1/SA2 may be still confused on the AS-based procedure detail especially for the case with NAS assistance information (only based on the above description). According to our colleagues feedback the similar thing has happened for the busy indication topic, companies have different understanding on the "NAS-based" procedure. For the case that with NAS assistance information, a NAS message would be involved as a NAS piggy-back, e.g. service request message, then it would be little strange that the Ran node forward this Nas message to the CN but meanwhile the RAN node send release MSG to the UE, at least we need to confirm CT1/SA2 whether such kind of processing is acceptable for them. Normally, when a NAS message was received by CN, it should be CN to decide whether to release or keep the connection. Otherwise, we think some problem maybe exist. For example, if this service request massage couldn't pass the NAS layer integrity check, the CN would release the connection and send Release command to the RAN. But if the RAN node has sent release MSG to the UE with suspend indication (inactive state) before any response from CN, the RAN has to paging UE again and then send Release MSG (without suspend) to the UE. |
| CATT | No | It can be discussed after SA2/CT1 concludes on the NAS based solution. |
| Charter Communications | Maybe not | SA2 has made the working assumption (from Show of Hand on May 24th) that NAS leaving will result to end-state = RRC-IDLE. We believe that the AS leaving solution is most suitable and meant for the situations where the UE need not send any NAS level info Core Network or vice versa. Therefore, we believe this question is perhaps bestto be revised as follows. Given RAN2 recent agreement on AS-based leave procedure, and the recent SA2 working agreement on specifying a NAS-based leave procedure, we sugeest that this LS to ask SA2 to specify the conditions on which the UE is allowed to use AS or NAS or any leaving procedure. |
| MediaTek | Maybe not | This could be discussed if SA2/CT1 agree to have this NAS information and want to piggyback to the RRC message that trigger NW swiching. |
| Ericsson |  | We think this should be up to SA2 to decide. |
| DENSO | Maybe not | Agree with OPPO. The necessity of receiving the confirmation message in the NAS layer is in the realm of SA2/CT1 and should be decided by them. |
| Samsung | See our comments | In general informing our agreement to SA2/CT1 seems sufficient. We are also under the impression that this discussion seems far more than what we discussed online. As indicated by the term 'at least', it does not mean that NAS-based solution is entirely ruled out as it is not within RAN2 scope but up to SA2/CT1. We support at least AS-based solution (with AS-based response) for network switching while leaving RRC_Connected state in NW A. FFS if this may include NAS information If we need to clarify FFS part, we may simply ask to SA2/CT1 whether NAS-based solution is also needed for 5GS, and if needed, whether there is a need to piggy-back NAS information (e.g. assistance information on paging filtering) into RRC message. |
| Xiaomi | No | Agree with CATT. |
| LGE | Maybe not | Since SA2 is still underdiscussion for this issue, RAN2 may can also have some more time to discuss. Furthermore, in our understanding, especially in the case of timer based leaving, since the UE AS can initiate to leave the RRC connection before reception of release indication from the UE NAS, there seems to be additional scenario that the UE AS firstly indicate to the UE NAS. |
| Nokia | No | This can be discussed later if SA2/CT1 insist on the need to convey some NAS level information at the time of leaving. This information is also only applicable for RRC-IDLE state. |
| Sony |  | Agree with Ericsson |
| China Telecom | Maybe not | We’d better first ask SA2/CT1 if AS based switching message should also support to include assistance information related to core network e.g. paging filter information. If the reply is yes, we could further discuss details. |
| Vodafone | Possible not | The UE needs to wait for release NAS message, also agree with comments from Charter Comm. |
| Lenovo | No | Wait for the SA2 input if SA2 have such agreement. |

Summary: Most companies do not want to include any points regarding the interaction of AS-based solution with NAS message or NAS-based solution until SA2 finalizes their discussion.

Proposal 4: The interaction between AS-based solution and any SA2 agreement on NAS messages or NAS-based solution for network switching is not included in the LS.

# Conclusion

This report captures the feedback from companies on the details of “AS-based” signaling for network switching. Based on this feedback, the following are proposed:

Proposal 1: AS -based solution for network switching includes two steps: 1-) If configured, UE can send an RRC message to leave RRC_CONNECTED for MUSIM purpose 2-) gNB may release the UE to Idle/Inactive.

Proposal 2: Include the following RAN2#113bis-e agreement in the LS:

During switching procedure for leaving RRC_CONNECTED state, UE is allowed to enter RRC_IDLE state if it does not receive response message from network within a certain configured time period. FFS for RRC_INACTIVE state

Proposal 3: The “configured time” for the UE to leave RRC_CONNECTED without a response is configured by the gNB. However, this information is not needed to be included in the LS.

Proposal 4: The interaction between AS-based solution and any SA2 agreement on NAS messages or NAS-based solution for network switching is not included in the LS.