3GPP TSG-RAN WG2 Meeting #115-e		R2-2107591

Electronic, August 16 – August 27, 2021

Agenda item:	8.22

Source:	Apple

Title:	Discussion on Rel-17 UL Tx Switching

Document for:	Discussion

# 1	Introduction

According to the WID [1], Rel-17 UL Tx switching need to support the following scenarios:

- Scenario1: 2Tx-2Tx switching between two uplink carriers for SUL and UL CA

- Scenario2: 1Tx-2Tx and 2Tx-2Tx switching between 1 carrier on band A and 2 contiguous aggregated carriers on band B for SUL and UL CA

Main issues regarding UE capability and NW configuration have been elaborated in another contribution [2]. Here we only presented one additional issue on power boosting.

# 2   Discussion on power boosting in UL Tx Switching

In Rel-16 UL Tx switching, one UE capability named “uplinkTxSwitching-PowerBoosting” was captured for UL CA. During previous RAN plenary discussion [3], it was agreed that power class for case 1 and case 2 are the same and does not change in UL Tx switching. Thus, if UE reports PC3 in UL CA, UE can exploit 3dB power boosting at carrier 2 with 2Tx upon Tx switching.

However, in Rel-17, RAN4 agreed to introduce PC2 in TDD intra-band contiguous and non-contiguous UL CA, which means the Rel-16 feature of power boosting in UL Tx switching is no longer needed.

Proposal: uplinkTxSwitching-PowerBoosting-r16 is no longer needed for Rel-17 UL Tx Switching.

# 3   Conclusion

Based on the discussion above, we have the following proposal.

Proposal: uplinkTxSwitching-PowerBoosting-r16 is no longer needed for Rel-17 UL Tx Switching.

# 4   Reference

[1] RP-210899 revised WID: RF requirements enhancement for NR frequency range 1 (FR1)

[2] R2-2108158	RAN2 impact to support R17 UL Tx switching enhancement

[3] RP-201338 CR to TS 38.101-1: Switching time mask between two uplink carriers in UL CA and SUL