3GPP TSG-RAN WG2 Meeting #115e	R2-2107665

# e-Meeting, 16th - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.323 | CR | 0080 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for the ciphering of EHC header |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT-Core |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN2#107bis, RAN2 made the following agreements: The EHC function is in PDCP The EHC header is located after the SDAP header, and it is ciphered However, the current PDCP specification does not capture that the EHC header is ciphered. In Rel-15 LTE UDC, the similar issue, i.e. the UDC header is ciphered, was specified as follows: 5.6 Ciphering and Deciphering 5.6.0 General The ciphering function includes both ciphering and deciphering and is performed in PDCP. For the control plane, the data unit that is ciphered is the data part of the PDCP PDU (see clause 6.3.3) and the MAC-I (see clause 6.3.4). For the user plane, the data unit that is ciphered is the data part of the PDCP PDU (see clause 6.3.3); ciphering is not applicable to PDCP Control PDUs. 6.3.3 Data Length: Variable The Data field may include either one of the following: - Uncompressed PDCP SDU (user plane data, or control plane data); or - Compressed PDCP SDU (user plane data only); or - UDC header and UDC Data Block if UDC is configured.. Given that Rel-17 NR UDC would be considered, the similar way is preferred for the ciphering of EHC header. |
| Summary of change: |  | To add a text “EHC header and compressed PDCP SDU if EHC is configured” to Section 6.3.3 to clarify that EHC header is ciphered. Impact analysis Impacted functionality: Ethernet header compression Inter-operability: If the network is implemented according to the CR while the UE is not, it is not clear which part of PDCP PDCP UE should cipher, i.e. the UE may not cipher the EHC header, which can cause the deciphering failure in the network side. If the UE is implemented according to the CR while the network is not, it is not clear which part of PDCP PDCP the network should cipher, i.e. the network may not cipher the EHC header, which can cause the deciphering failure in the UE side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether to cipher the EHC header in the current PDCP specification, which may cause unnecessary deciphering failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

6.3.3	Data

Length: Variable

This field includes one of the followings:

-	Uncompressed PDCP SDU (user plane data, or control plane data);

-	Compressed PDCP SDU (user plane data only);

-	EHC header and compressed PDCP SDU if EHC is configured.

<End of modification>