3GPP TSG-RAN WG2 Meeting #115 electronic             	R2-2107759

E-Meeting, 16th – 27th August 2021

Source:	vivo

Title:	Remaining issues on Relay Discovery

Agenda Item:	8.7.3.1

Document for:	Discussion and Decision

- Introduction

In RAN2 #114e meeting, it was agreed by companies that[1]:

- RAN2 understand that the L2/L3 common parts of the relay discovery and (re)selection objectives are complete at stage 2 level from RAN2 perspective.

This contribution would further discuss some stage-3 level issues on Relay discovery procedure, such as:

- The transmit power of discovery message to ensure range requirement

- How CBR would affect the discovery message

- The selection of dedicated pool and shared pool

- Discussion

2.1 Selection of dedicated pool and shared pool

In RAN2 #114e meeting it was agreed[1]:

Proposal 6 [discussion]: RAN2 agrees dedicated discovery resource pool is supported besides shared resource pool configuration, whether it is configured is based on network implementation. And PHY layer parameters and design shall reuse the Rel-16 legacy resource pool design (including resource allocation design). RAN2 agree that the UE selection between dedicated and shared pool can be discussed as a stage 3 issue after RAN#92-e.

As we already support dedicated resource pool for discovery, at least the UE’s behavior should be clarified when both dedicated pool and shared pool are configured. We list two main questions to be answered:

- Whether data can be transmitted in dedicated resource pool for discovery?

Logically, if we allow data to be transmitted in the dedicated resource pool for discovery, it is actually regarded as a shared pool. In other word, the network can just configure a pool as a shared pool to allow data transmission on it. Therefore, we propose that data is not allowed to be transmitted in a dedicated resource pool for discovery, and this only apply to the case that a UE is configured with only dedicated resource pool for discovery, meaning no data transmission/reception can be performed by the UE in this case. Therefore,

Proposal 1: Data is not allowed to be transmitted in dedicated resource pool for discovery.

- Whether discovery messages can be transmitted in shared pool(s) if UE is configured with dedicated discovery resource pool?

This question is already discussed during online and offline discussion in RAN2 #114e meeting[1][2]. In our understanding, if a dedicated pool for discovery is configured by the network, which means the resources have already been reserved, it is better for UE to prioritize using these dedicated pools with higher resource use efficiency, and achieve better performance as it is less likely to encounter congestion in that dedicated pool.

Observation 1: To prioritize using dedicated pools for discovery will result in higher resource use efficiency and achieve better performance for discovery message transmission because of less congestion.

Secondly, if a UE would prioritize or always choose to use dedicated pools, for receiving UEs they can just listen to these dedicated pools to achieve better power efficiency. There may even be some UEs which are configured with only dedicated pools that are for discovery, without any shared pool, and this will make them not miss any discovery message, because otherwise TX UE may select share pools to transmit discovery message.

Observation 2: To prioritize using dedicated pools for discovery transmission will make the RX UEs just need to monitor dedicated pools to improve power efficiency, and make RX UEs which are configured with only dedicated pools not miss any discovery message due to shared pool selected for discovery message at TX side.

Based on these observations, it seems much beneficial to prioritize or always to use the dedicated pools for discovery message transmission. We think a simple solution is to make the UE always use the dedicated discovery pool once configured by network, and with this solution we can relieve RAN2 of further discussions about potential pool selection between shared pool(s) and dedicated pool(s), which seems not much benefits but only more specification work. Also, the RX UE can just monitor the dedicated discovery pool without concern that the TX may select the shared pool for discovery transmission. Then a further question would be whether one dedicated pool is enough, or multiple dedicated pool can be configured by network, which would have an impact on our RRC signaling design. And if multiple dedicated pools can be configured, it should be left to UE implementation to select one of them. Therefore,

Proposal 2: The UE shall only use the dedicated discovery pool(s) for discovery message transmission, if configured, and is only allowed to use shared pool for discovery in case not any dedicated discovery pool is configured.

Proposal 3: RAN2 to discuss whether one dedicated discovery pool is enough, or multiple dedicated pool can be configured by network.

Proposal 4: If multiple dedicated pools can be configured by network, it is up to UE implementation to select one of them.

2.2 Transmit power of discovery message

In RAN2 #114e meeting, for power control mechanism, it was agreed that[1]:

- RAN2 agrees to reuse Rel-16 power control mechanism for transmission of discovery messages. - RAN2 do not specify a solution to the power imbalance issue for relay (re)selection in Rel-17.

It means that sidelink OLPC would also be applied to discovery message and we do not pursue further enhancement on power imbalance issue for relay (re)selection. However, there is still one issue left for the transmit power aspect, because in LTE, we have different range classes for sidelink discovery to set an upper bound for determined transmit power of discovery, on top of OLPC, as indicated in TS 36.300 as follows[3]:

TS 36.300 ## 23.11 Support for sidelink discovery 23.11.1 General <omitted…> There are three range classes. Upper layer authorisation provides applicable range class of the UE. Maximum allowed transmission power for each range class is signalled in SIB19. UE uses the applicable maximum allowed transmission power corresponding to its authorised range class. This puts an upper limit on the determined transmit power based on open loop power control parameters.

Observation 3: In LTE, three range classes are defined and the applicable range class for sidelink discovery TX is provided by upper layer authorisation.

Observation 4: The UE receives the TX power limit corresponding to the applicable range class in SIB19 and further determines the exact TX power under this limit based on open loop power control.

In NR, similar range requirement is also defined according to SA2 specification TS 23.304, as shown below[4]:

TS 23.304 5.1.2.1 Policy/Parameter provisioning for 5G ProSe Direct Discovery The following sets of information for 5G ProSe Direct Discovery over PC5 reference point is provisioned to the UE: 1) Authorization policy for 5G ProSe Direct Discovery: - When the UE is "served by NG-RAN": - For open 5G ProSe Direct Discovery: a) open 5G ProSe Direct Discovery Model A monitoring authorization policy: - PLMNs in which the UE is authorised to perform 5G ProSe Direct Discovery monitoring. b) open 5G ProSe Direct Discovery Model A announcing authorization policy: - PLMNs in which the UE is authorized to perform announcing; - Authorised discovery range for announcing per PLMN.

Observation 5: in NR, ‘range’ is also supposed to be provided by upper layer authorisation similar to LTE.

Therefore, based on the above two observations, RAN2 should discuss whether similar mechanism of range classes in LTE should be reused in NR. In our understanding, from the perspective of Authorization policy, there is not much difference between LTE and NR about this ‘Authorised discovery range’, where both are used to control the range of discovering/being discovered. So we suggest to reuse the LTE mechanism to help to determine the transmit power of discovery message, and it can be further confirmed by SA2. The form of ‘Authorised discovery range’ can also be asked to SA2 once we decide to have an LS to them, e.g. whether it is still three different range classes as in LTE.

Proposal 5: LTE mechanism of range class is reused in NR that, the transmit power of discovery message should be determined with considering the requirement of Authorised discovery range provided by upper layer.

Proposal 6: If proposal 5 is agreed, send an LS to inform SA2.

Moreover, if LTE mechanism of range class is agreed, which puts an upper limit on the determined transmit power based on open loop power control parameters, we should also inform RAN1 as the formula to define the power for a PSSCH transmission is captured in TS 38.213 as follows[5]:

TS 38.213 16.2.1 PSSCH A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f of serving cell c as: PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PMAX,CBR is determined by a value of sl-MaxTransPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTransPower-r16 is not provided, then PMAX,CBR=PCMAX; <omitted…>

For discovery message, this formula should further consider the impacts brought by range class, when we calculate the power for discovery message transmission. Therefore,

Proposal 7: If proposal 5 is agreed, the power for a discovery message transmission should be determined with further considering the impacts brought by range classes.

Proposal 8: If proposal 5/7 is agreed, send an LS to inform RAN1.

2.3 Impact of CBR on discovery message

In NR sidelink, the mapping between PSSCH transmission parameters, CBR and priority is supported by the configuration parameter SL-CBR-PriorityTxConfigList, as follows:

TS 38.331 6.3.5 Sidelink information elements SL-CBR-PriorityTxConfigList The IE SL-CBR-PriorityTxConfigList indicates the mapping between PSSCH transmission parameter (such as MCS, PRB number, retransmission number, CR limit) sets by using the indexes of the configurations provided in sl-CBR-PSSCH-TxConfigList, CBR ranges by an index to the entry of the CBR range configuration in sl-CBR-RangeConfigList, and priority ranges. It also indicates the default PSSCH transmission parameters to be used when CBR measurement results are not available, and MCS range for the MCS tables used in the resource pool. SL-CBR-PriorityTxConfigList information element -- ASN1START -- TAG-SL-CBR-PRIORITYTXCONFIGLIST-START SL-CBR-PriorityTxConfigList-r16 ::= SEQUENCE (SIZE (1..8)) OF SL-PriorityTxConfigIndex-r16 SL-CBR-PriorityTxConfigList-v1650 ::= SEQUENCE (SIZE (1..8)) OF SL-PriorityTxConfigIndex-v1650 SL-PriorityTxConfigIndex-r16 ::= SEQUENCE { sl-PriorityThreshold-r16 INTEGER (1..8) OPTIONAL, -- Need M sl-DefaultTxConfigIndex-r16 INTEGER (0..maxCBR-Level-1-r16) OPTIONAL, -- Need M sl-CBR-ConfigIndex-r16 INTEGER (0..maxCBR-Config-1-r16) OPTIONAL, -- Need M sl-Tx-ConfigIndexList-r16 SEQUENCE (SIZE (1.. maxCBR-Level-r16)) OF SL-TxConfigIndex-r16 OPTIONAL -- Need M } SL-PriorityTxConfigIndex-v1650 ::= SEQUENCE { sl-MCS-RangeList-r16 SEQUENCE (SIZE (1..maxCBR-Level-r16)) OF SL-MinMaxMCS-List-r16 OPTIONAL -- Need M } SL-TxConfigIndex-r16 ::= INTEGER (0..maxTxConfig-1-r16) -- TAG-SL-CBR-PRIORITYTXCONFIGLIST-STOP -- ASN1STOP

As we agreed in RAN2 #113bis-e[7] and RAN2 #114e[1] meeting that:

RAN2 #113bis-e Agreement: Proposal 8a: [Easy] One new SL-SRB4 is used for all discovery messages. Its parameters will be fixed and defined as SCCH configuration in 38.331. (FFS on the LCH priority in Proposal 8b) RAN2 #114-e Agreement: Proposal 8 [discussion]: RAN2 agrees to fix the priority value as 1 of sidelink discovery message in the specification.

It can be observed that discovery is also transmitted on PSSCH with a new SL-SRB4. And at the same time we also support dedicated pool and shared pool for discovery messages, so it is better for RAN2 to discuss whether/how this SL-CBR-PriorityTxConfigList is applied to discovery message in both dedicated pool and shared pool respectively.

Observation 6: It is unclear whether/how the configuration of SL-CBR-PriorityTxConfigList is applied to discovery message in both dedicated pool and shared pool respectively.

It can be discussed separately:

- For shared pool, as sidelink discovery message has the fixed priority value 1, a simple solution is to set the priority value to 1 in SL-CBR-PriorityTxConfigList and then apply the rest configuration in SL-CBR-PriorityTxConfigList for discovery message.

- For dedicated pool, however, it should be discussed when we still need to apply SL-CBR-PriorityTxConfigList, which in fact, is about whether we need to have congestion control in the dedicated pool. And even if we agree to perform congestion control for discovery message in dedicated pool, we may also have optimization on the configuration because only the mapping between CBR and PSSCH transmission parameters is needed, without priority anymore, because only discovery messages can be transmitted on the pool. Therefore we propose:

Proposal 9: For shared pool, current SL-CBR-PriorityTxConfigList is applied to sidelink discovery message transmission with UE always setting the Priority to 1.

Proposal 10: For dedicated pool for discovery, RAN2 to discuss whether/how PSSCH transmission parameters should be adjusted due to different CBR, considering following options:

- Option-1: use current SL-CBR-PriorityTxConfigList as in Proposal 9.

- Option-2: use current SL-CBR-CommonTxConfigList (i.e. w/o Priority configuration)

- Conclusion

We have the following observation and proposals:

Observation 1: To prioritize using dedicated pools for discovery will result in higher resource use efficiency and achieve better performance for discovery message transmission because of less congestion.

Observation 2: To prioritize using dedicated pools for discovery transmission will make the RX UEs just need to monitor dedicated pools to improve power efficiency, and make RX UEs which are configured with only dedicated pools not miss any discovery message due to shared pool selected for discovery message at TX side.

Observation 3: In LTE, three range classes are defined and the applicable range class for sidelink discovery TX is provided by upper layer authorisation.

Observation 4: The UE receives the TX power limit corresponding to the applicable range class in SIB19 and further determines the exact TX power under this limit based on open loop power control.

Observation 5: in NR, ‘range’ is also supposed to be provided by upper layer authorisation similar to LTE.

Observation 6: It is unclear whether/how the configuration of SL-CBR-PriorityTxConfigList is applied to discovery message in both dedicated pool and shared pool respectively.

Proposal 1: Data is not allowed to be transmitted in dedicated resource pool for discovery.Error! Reference source not found.

Proposal 2: The UE shall only use the dedicated discovery pool(s) for discovery message transmission, if configured, and is only allowed to use shared pool for discovery in case not any dedicated discovery pool is configured.

Proposal 3: RAN2 to discuss whether one dedicated discovery pool is enough, or multiple dedicated pool can be configured by network.

Proposal 4: If multiple dedicated pools can be configured by network, it is up to UE implementation to select one of them.

Proposal 5: LTE mechanism of range class is reused in NR that, the transmit power of discovery message should be determined with considering the requirement of Authorised discovery range provided by upper layer.

Proposal 6: If proposal 5 is agreed, send an LS to inform SA2.

Proposal 7: If proposal 5 is agreed, the power for a discovery message transmission should be determined with further considering the impacts brought by range classes.

Proposal 8: If proposal 5/7 is agreed, send an LS to inform RAN1.

Proposal 9: For shared pool, current SL-CBR-PriorityTxConfigList is applied to sidelink discovery message transmission with UE always setting the Priority to 1.

Proposal 10: For dedicated pool for discovery, RAN2 to discuss whether/how PSSCH transmission parameters should be adjusted due to different CBR, considering following options:

- Option-1: use current SL-CBR-PriorityTxConfigList as in proposal 9.

- Option-2: use current SL-CBR-CommonTxConfigList (i.e. w/o Priority configuration)

# Reference

- RAN2 #114e meeting chairman notes.

- R2-2106586, Offline discussion xxx, 3GPP TSG-RAN WG2 #114-e, Electronic Meeting, May 19 – 27, 2021

- 3GPP TS 36.300 V16.2.0 (2020-07)

- 3GPP TS 23.304 V1.0.0 (2021-06)

- 3GPP TS 38.213 V16.6.0 (2021-06)

- 3GPP TS 38.331 V16.5.0 (2021-06)

- RAN2 #113bis-e meeting chairman notes.