3GPP TSG-RAN WG2 Meeting #115-e		      	   		             R2-2107795

E-Meeting, 16th – 27th Aug. 2021

Source: 	vivo

Title:  	Discussion on PTP PTM Switch

Agenda Item:	8.1.2.1

Document for:	Discussion and Decision

Introduction

In the past RAN2 meetings, the topic on PTM/PTP switch has been discussed. Following is a summary of relevant agreements:

RAN2#111-e: For a UE, gNB dynamically decides whether to deliver multicast data by PTM or PTP (Shared delivery) FFS which layer(s) handles reliability (in general), inorder delivery / duplicate handling, and it is FFS how it works at PTM PTP switch. RAN2#112-e: RLC AM is supported for PTP transmission of NR MBS. RLC UM is supported for PTP transmission of NR MBS. RLC UM is supported for PTM transmission of NR MBS. RLC TM is not supported for PTP transmission of NR MBS. RLC TM is not supported for PTM transmission of NR MBS. Working assumption: RLC-AM for PTM is not supported (can be revisited but it means that proponents of RLC-AM for PTM need to demonstrate the need, to change this). RAN2#113-e: For the case that both PTM and PTP are RLC-UM, configuration with No L2 ARQ and with PDCP anchored PTM – PTP switching shall be supported (e.g. for services that would typically be configured with RLC UM for unicast). RAN2#113bis-e: For a given UE, if the MRB’s QoS requirements are not met via PTM, switching to PTP with RLC-AM shall be supported Chair: NOTE that the below agreements are only based on architecture decisions so far. The reliability discussion not concluded yet i.e. other cases than RLC UM + RLC UM. PTM PTP switch for such other cases is FFS Dynamic PTM/PTP switch is supported for a split MRB bearer (type) with a common (single) PDCP entity. As a baseline, no new UE based signalling is introduced to support gNB switch decision (e.g. PDCP SR for high reliability is still TBD) Assuming a split-MRB (as agreed during the online session) configured with a PTM leg and PTP leg, the usage of the PTP leg cannot be deactivated (i.e. the UE needs to always monitor C-RNTI) after the necessary split-MRB configuration. Assuming a split-MRB (as agreed during the online session) configured with a PTM leg and PTP leg, it is FFS whether the usage of the PTM leg of the split-MRB may be subject to activation or deactivation and the details of such. RAN2#114 - RLC-AM is not supported for PTM (for MBS R17 WI).

In this contribution, we will discuss the issues on the intra-cell PTM PTP switch for RRC connected UE and give our proposals.

Discussion

RLC configuration for PTM

During the post email discussion after RAN2#114 meeting, RAN2 has dicussed the RLC configuration for PTP and PTM. It is common understanding that RRC should support to configure DL only UM RLC configuration for PTM, both DL and UL AM RLC configuration for PTP, and DL only UM RLC configuration for PTP. There are different views on whether RRC should support both DL and UL UM RLC configuration for PTP.

In our understanding, both DL and UL UM RLC configuration should be supported for PTP at least for the following purposes.

- ROHC feedback when ROHC configuration with R mode or O mode is configured for MBS.

- PDCP SR to reduce data loss.

In RAN2#112 meeting, RAN2 has agreed that “RoHC (at least U-mode) can be configured for NR MBS bearers. This is applicable for Mcast, assume this is applicable also to broadcast. ”. This means that MBS can be configured with ROHC function with R mode or O mode, which needs RLC to be configured as biodirectional RLC.

When performing PTM to PTP switching, data loss may occurs. In oder to reduce the data loss as much as possible, the PTP should be configured with biodirectional RLC to allow UE reporting PDCP SR.

Proposal 1: Support both DL and UL UM RLC configuiration for PTP.

The activation/deactication of PTM

During MRB transmission, UE should always monitor for C-RNTI while in RRC connected state. However, if the UE is switched from PTM to PTP, UE can stop monitoring G-RNTI for power saving. Thus, UE shall explicitly know when NW performs switching from PTM to PTP. If UE stops monitoring G-RNTI by UE implementation, e.g. UE will stop monitor G-RNTI if UE dose not receive any packets via PTM, UE may miss packets transmitted via PTM when NW switches the packet transmission back to PTM. This is because UE does not know when the NW will switch PTP back to PTM. For the purpose of power saving and data reception, explicit indication for PTM PTP switch shall be provided to UE.

On the other hand, if dynamic PTM/PTP switching is transparent to UE, the PTM RLC window may discard some valid MBS data by mistake. Specifically, due to the bad channel condition, the PTM RLC may not receive data from MAC layer for a long time and the RLC state variables will not change. After that there may be a valid packet received, but the newly received packet may be discarded if the SN of the newly received packet meets (RX_Next_Highest – UM_Window_Size) <= SN < RX_Next_Reassembly. Some companies think thath this issue can be avoid by releasing of the PTM RLC by RRC signaling. If UE wants to switch back to PTM when the channel condition become better, RRC should reconfigure UE with PTM, which will introduce a large delay. Moreover, this go against the original intention of introducing dynamic PTP PTM switch. Thus, we cannot assume that the above discarding valid packets can always be solved by releasing the PTM. Then, explicit command should be introduce for activating/deactivating PTM. The explicit command can be DCI or MAC CE. Considering this explicit activation/deactivation command can always transmitted via PTP leg, MAC CE is more suitable than DCI as the reliability of MAC CE carrying activation/deactivation command can be guaranteed by the HARQ feedback.

Proposal 2: MAC CE based activation/deactivation command of PTM is introduced.

When UE is configured with both PTP and PTM for the MBS reception, dynamic PTM PTP switch can be easier to realize as the common PDCP is used. Thus, it is possible that NW may configure UE with both PTP and PTM but deliver packets to UE via PTP only or PTM only. This is because that NW want to facilitate dynamic PTP PTM switch subsequently thus configure UE with both PTP and PTM in advance. Under this case, the initial state on activation/deactivation of PTP PTM reception should be configured by RRC, which can avoid unnecessary RNTI monitoring (e.g. G-RNTI) at UE side.

Proposal 3: The initial state on activation/deactivation of PTM when MRB is setup can be configured by RRC.

The trigger of Status Report

When RRC reconfigure a MRB, there is possible that some packets are still experiencing RLC ARQ or HARQ via the old leg (e.g. via PTM). Then, there may be data loss, e.g. in the following cases:

Case 1: Reconfiguration between PTP only and PTM only;

Case 2: Reconfiguration from split MRB to PTM only or PTP only;

Case 3: Reconfiguration from PTM only to split MRB with PTM deactivation;

Case 4: Reconfiguration from PTM only to split MRB with PTM deactivation;

Under the above cases, PDCP status report can be triggered in order to reduce packet loss as much as possible.

Proposal 4: PDCP status report can be triggered when RRC reconfigures a MRB.

No matter which use cases above, triggering PDCP SR is meaningful only when PTP is configured. The reason is that PTM is unidirectional channel. Thus, there is no way to report PDCP SR via PTM RLC bearer.

Proposal 5: PDCP SR can be triggered if PTP is configured.

In the current spec, PDCP SR can be triggred when PDCP re-establishem is requested. However, PDCP re-establishment is usually used when PDCP anchor changes. For the case that RRC reconfigures a MRB, PDCP anchor does not change, then PDCP re-establishement can be not needed.

Proposal 6: PDCP SR can be triggered without PDCP re-establishment when RRC reconfigures a MRB.

Conclusions

Based on the analysis given above, we have the following proposals:

Proposal 1: Support both DL and UL UM RLC configuiration for PTP.

Proposal 2: MAC CE based activation/deactivation command of PTM is introduced.

Proposal 3: The initial state on activation/deactivation of PTM when MRB is setup can be configured by RRC.

Proposal 4: PDCP status report can be triggered when RRC reconfigures a MRB.

Proposal 5: PDCP SR can be triggered if PTP is configured.

Proposal 6: PDCP SR can be triggered without PDCP re-establishment when RRC reconfigures a MRB.

Reference

- RP-201038, WID revise: NR Multicast and Broadcast Services.

- 23.757 v040, Study on architectural enhancements for 5G multicast-broadcast services.