3GPP TSG-RAN WG2 Meeting #115e	R2-2107919

Online, August 9th – 27th 2021

Agenda item:	   8.1.2.1

Source:		Lenovo, Motorola Mobility

Title:		Discussion on MRB type change and PTM/PTP switch

Document for:		Discussion and Decision

1	Introduction

It has been discussed in the [Post114-e072] email discussion some issues related to MRB type change via RRC signalling and PTP and PTM switch for split MRB[1]. In this paper, we further discuss the relevant issues.

- Dynamic PTM/PTP switch is supported for a split MRB bearer (type) with a common (single) PDCP entity.

- Assuming a split-MRB (as agreed during the online session) configured with a PTM leg and PTP leg, the usage of the PTP leg cannot be deactivated (i.e. the UE needs to always monitor C-RNTI) after the necessary split-MRB configuration.

- Assuming a split-MRB (as agreed during the online session) configured with a PTM leg and PTP leg, it is FFS whether the usage of the PTM leg of the split-MRB may be subject to activation or deactivation and the details of such.

2	Discussion

2.1 MRB type change

Besides split-MRB, there are two other bearer types: PTM only MRB and PTP only MRB. It is beneficial that Network has the flexibility to configure a UE receiving MBS data via PTM only MRB, PTP only MRB or split MRB for one specific MBS session. For example, if only one UE is receiving an MBS traffic, it is better to configure PTP only MRB to the UE. The bearer type should be configured by RRC signalling.

One MRB can be configured by RRC signalling as either a PTP only MRB, PTM only MRB, or a split MRB.

There are two ways to realize bearer type change between PTM-only MRB, PTP-only MRB, and split MRB as following:

-	Option 1: Separate PDCP entities are used for PTM-only MRB, PTP-only MRB, and split MRB

-	Option 2: Common PDCP entity is used for PTM-only MRB, PTP-only MRB, and split MRB

The option 1 have more spec impact e.g. it needs new functionalities in SDAP layer such as re-ordering, retransmission. Option 2 can reuse the existing PDCP functions as much as possible.

A common PDCP entity is used for bearer type change between PTM-only MRB, PTP-only MRB and split MRB.

The existing triggers of PDCP status report are specified as:

For AM DRBs configured by upper layers to send a PDCP status report in the uplink (statusReportRequired in TS 38.331 [3]), the receiving PDCP entity shall trigger a PDCP status report when:

-	upper layer requests a PDCP entity re-establishment;

-	upper layer requests a PDCP data recovery;

-	upper layer requests a uplink data switching;

-	upper layer requests a PDCP entity reconfiguration and the associated RLC entity is released for a radio bearer.

When bearer type change among PTM-only MRB, PTP-only MRB, and split MBR, one issue is that the PDCP status report may not be triggered according to existing triggers due to:

a) The PTM can only be configured as RLC-UM mode.

b) The PDCP entity re-establishment may not be needed e.g. the security may not be needed for PTP-only MRB as well.

c) The PDCP data recovery is not applicable of RLC-UM mode.

In order to minimize the data loss during bearer type change, it is beneficial to support PDCP status report once the MRB is changed. Considering a bidirectional PTP leg is required to transmit the PDCP status report, the PDCP status report could be triggered if the new MRB has a bidirectional PTP leg, e.g. when a PTM-only MRB is changed to a PTP-only MRB of RLC AM, or a PTM-only MRB is changed to a split MRB with RLC AM PTP leg.

For MRB configured by upper layers to send a PDCP status report in the uplink (statusReportRequired IE in RRC), the receiving PDCP entity shall trigger a PDCP status report in case of MRB type change. It is upon NW to ensure a bidirectional PTP leg (e.g. either PTP-only MRB or split MRB) is configured if statusReportRequired is provided.

2.2 PTM activation/deactivation in split MRB

Since the UE should always monitor for C-RNTI in RRC_CONNECTED state, the deactivation of PTP leg may not be useful. The dynamic PTM-PTP switch can be achieved by activation and deactivation of PTM leg. There are two different solutions on PTM activation and deactivation depending on the transparence to UE:

- Option 1: UE always monitors both G-RNTI and C-RNTI for MRB traffic for both PTM and PTP. The activation and deactivation are implemented by gNB’s scheduling decision and transparent to UE.

- Option 2: Explicit PTM activation/deactivation are sent to UE. Upon receiving PTM deactivation/activation command, the UE shall stop/re-start monitoring the G-RNTI.

The big benefit of option 2 is that it can reduce the power consumption when PTM is deactivated, since the UE can avoid monitoring the G-RNTI, assuming C-RNTI and G-RNTI have different DRX operations. Since UE power saving is an important feature as what have been working in other work items, the option 2 should be adopted.

For a split-MRB, explicit PTM activation/deactivation command is beneficial for UE power saving.

Lossless switch between PTM and PTP should be supported e.g. for some services with high reliability requirement. For an example, at the beginning, the UE receives an MBS traffic via PTM leg. Due to radio link quality change, stricter QoS requirement and other conditions change, the network may decide to deactivate PTM leg. When performing PTM deactivation, the UE need to send PDCP Status Reporting via PTP leg, and the network can retransmit the missing PDCP PDUs via PTP leg for lossless MBS data reception.

In order to reduce data loss for a split-MRB, the receiving PDCP entity shall trigger a PDCP status report when PTM activation/deactivation status is changed and if the PTP leg is a bidirectional RLC UM or RLC AM.

One issue worth clarifying is that, whether PTM activation/deactivation is only about PTM activation/deactivation for a split-MRB, or it is in general about the activation/deactivation of both PTM-only MRB and the PTM leg of the split-MRB. In our understanding, it should be the later case, since dynamic PTM activation/deactivation is mainly due to the channel condition degradation and the transmission via PTM is temporarily disabled. If NW believe the PTM transmission is no longer suitable at all, the NW can simply reconfigure the split-MRB to a PTP-only MRB, or reconfigure the PTM-only MRB to a PTP-only MRB.

Therefore, in our understanding, PTM activation/deactivation is essentially about start/stop the corresponding G-RNTI monitoring and DRX operation, which is applicable to both PTM leg of split-MRB and PTM-only MRB.

PTM activation/deactivation is essentially about start/stop the corresponding G-RNTI monitoring and DRX operation, which is applicable to both PTM leg of split-MRB and PTM-only MRB.

In [Post114-e072] email discussion, it was compared whether the PTM activation/deactivation can be sent via MAC CE or DCI. In our view, MAC CE is preferred given that

- Indicating PTM activation/deactivation via DCI has impact on RAN1 group and demands extra feedback mechanism for gNB to understand UE has received the relevant DCI

- It could be possible that multiple PTM corresponding to different G-RNTIs are deactivated at the same time. So the gNB may indicate the deactivation of multiple G-RNTIs in the same signalling. MAC CE is more flexible to carry contents of different size.

Besides, we believe the MAC CE for PTM/G-RNTI activation/deactivation shall be sent in unicast only, because

- If it’s sent via multicast, it is possible that the HARQ feedback is disabled and the gNB will not know if UE has received the PTM/G-RNTI activation/deactivation or not.

- In the case UE has stopped monitoring the G-RNTI already, then UE will not be able to receive any further PTM/G-RNTI activation message from gNB sent in multicast way.

MAC CE is used to carry the PTM/G-RNTI activation or deactivation for one or more G-RNTIs.

The MAC CE for PTM/G-RNTI activation/deactivation is sent in unicast manner.

In terms of the MAC CE design for PTM/G-RNTI activation/deactivation, in order to reduce the signalling overhead, we can make use of the order of multicast session configurations provided by the gNB to UE. Since the multicast session configurations (including G-RNTI) is provided by the gNB to UE in a certain order, in the MAC CE for PTM/G-RNTI activation/deactivation, the activation/deactivation command can be listed in the same order and each is mapped to a given G-RNTI. For example, a bitmap design similar as SCell activation/deactivation MAC CE can be used. As in Figure 1, each bit represents whether the corresponding PTM/G-RNTI is activated or deactivated, and in case of more than one G-RNTI is configured to UE, those bits are listed in the same order as the G-RNTI configured to UE and each bit is mapped to one G-RNTI.

Figure 1: Exemplary MAC CE for PTM/G-RNTI activation/deactivation

A bitmap design can be used for the MAC CE for PTM/G-RNTI activation/deactivation, wherein bits are listed in the same order as the G-RNTI configured to UE and each bit represents the activation/deactivation state for the corresponding G-RNTI.

3	Conclusion

Based on the discussion above, we propose:

Proposal 1	One MRB can be configured by RRC signalling as either a PTP only MRB, PTM only MRB, or a split MRB.

Proposal 2	A common PDCP entity is used for bearer type change between PTM-only MRB, PTP-only MRB and split MRB.

Proposal 3	For MRB configured by upper layers to send a PDCP status report in the uplink (statusReportRequired IE in RRC), the receiving PDCP entity shall trigger a PDCP status report in case of MRB type change. It is upon NW to ensure a bidirectional PTP leg (e.g. either PTP-only MRB or split MRB) is configured if statusReportRequired is provided.

Proposal 4	For a split-MRB, explicit PTM activation/deactivation command is beneficial for UE power saving.

Proposal 5	In order to reduce data loss for a split-MRB, the receiving PDCP entity shall trigger a PDCP status report when PTM activation/deactivation status is changed and if the PTP leg is a bidirectional RLC UM or RLC AM.

Proposal 6	PTM activation/deactivation is essentially about start/stop the corresponding G-RNTI monitoring and DRX operation, which is applicable to both PTM leg of split-MRB and PTM-only MRB.

Proposal 7	MAC CE is used to carry the PTM/G-RNTI activation or deactivation for one or more G-RNTIs.

Proposal 8	The MAC CE for PTM/G-RNTI activation/deactivation is sent in unicast manner.

Proposal 9	A bitmap design can be used for the MAC CE for PTM/G-RNTI activation/deactivation, wherein bits are listed in the same order as the G-RNTI configured to UE and each bit represents the activation/deactivation state for the corresponding G-RNTI.

4	Reference

- [1] Chairman notes for RAN2#114 meeting