3GPP TSG-RAN WG2 #114-e electronic	R2-210xxxx

Electronic, 16 – 27 August 2021

Agenda Item:	6.1.4.1.5 Other

Source:	Ericsson

Title:	[AT115-e][027][NR16] CP Other & LTE (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This contribution summarizes the following discussion:

[AT115-e][027][NR16] CP Other & LTE (Ericsson)

Scope: Determine agreeable parts and agree CRs, For R2-2107285-7288 await on-line treat remaining part if needed, Treat R2-2108291, R2-2107129, R2-2107482, R2-2106911, R2-2108268, R2-2107485, R2-2106996, R2-2108434, R2-2108275, R2-2108189, R2-2108190, R2-2108569, R2-2108679,

Intended outcome: Report, Agreed CRs.

Deadline: A first round with Deadline for comments Thursday Aug 19 1200 UTC

CandidateBeamRSList

R2-2107285	Report of email discussion [Post114-e][071][NR16] CandidateBeamRSList set to release (MediaTek)	MediaTek Inc.	discussion	Rel-16	NR_eMIMO-Core	Late

R2-2107286	Handling of candidateBeamRSListExt-v1610 (option A1)	MediaTek Inc.	draftCR	Rel-16	38.331	16.5.0	F	NR_eMIMO-Core	Late

R2-2107287	Handling of candidateBeamRSListExt-v1610 (option B)	MediaTek Inc.	draftCR	Rel-16	38.331	16.5.0	F	NR_eMIMO-Core	Late

R2-2107288	Handling of candidateBeamRSListExt-v1610 (option C)	MediaTek Inc.	draftCR	Rel-16	38.331	16.5.0	F	NR_eMIMO-Core	Late

Misc Corrections

R2-2108291	Miscellaneous non-controversial corrections Set XI	Ericsson	CR	Rel-16	38.331	16.5.0	2763	-	F	NR_newRAT-Core, TEI16

R2-2108587 Correction on RRC multiplicity and type constraint definitions Huawei, HiSilicon        CR       Rel-16           38.331 16.5.0  2782    -           F          NR_newRAT-Core

eCall over IMS

R2-2107129	Early implementation of eCall over IMS in NR	Qualcomm Incorporated, Ericsson, Vodafone	CR	Rel-16	38.331	16.5.0	2714	-	F	TEI16

NR-U

R2-2107482	Correction on description of lbt-FailureInstanceMaxCount in LBT-FailureRecoveryConfig	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.5.0	2727	-	F	NR_unlic-Core

2-step RACH

R2-2106911	LS on the description of RRC parameter p0-AlphaSets (R1-2106168; contact: ZTE)	RAN1	LS in	Rel-16	NR_2step_RACH-Core	To:RAN2

R2-2108268	Correction to 38.331 on field description of the MsgA-TransMax	ZTE Corporation, vivo, LG Electronic, OPPO, Samsung	CR	Rel-16	38.331	16.5.0	2760	-	F	NR_2step_RACH-Core

R2-2107485	Correction to description of po-AlfphaSets	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.5.0	2728	-	F	NR_2step_RACH-Core

R2-2106996	Correction on msg1-SubcarrierSpacing and msgA-SubcarrierSpacing	vivo	CR	Rel-16	38.331	16.5.0	2707	-	F	NR_2step_RACH-Core

Moved from 6.1.4.1.1

Redirection with MPS indication

R2-2108434	Correction on Redirection with MPS Indication	Peraton Labs, CISA ECD, T-Mobile US, Ericsson , Qualcomm, NTT DoCoMo, AT&T, Verizon	CR	Rel-16	36.331	16.5.0	4714	-	F	NR_newRAT-Core, TEI16

LTE changes - Mobility

R2-2108375	Correction on ULInformationTransferMRDC(R16)	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.5.0	4713	-	F	TEI16

LTE changes - ASN.1 on SCG Failure report

R2-2108189	ASN.1 misalignment for the SCGFailureInformationNR message	Ericsson	CR	Rel-16	36.331	16.5.0	4709	-	F	LTE_NR_DC_CA_enh-Core, NR_unlic-Core, NR_IAB-Core, NR_Mob_enh-Core

Moved from 6.1.4.1.1

R2-2108190	ASN.1 misalignment for the SCGFailureInformationNR message	Ericsson	CR	Rel-16	38.331	16.5.0	2758	-	F	LTE_NR_DC_CA_enh-Core, NR_unlic-Core, NR_IAB-Core, NR_Mob_enh-Core

Moved from 6.1.4.1.1

R2-2108569	Discussion on compatibility issue and solutions for Rel-15 failure type definition	Huawei, HiSilicon	discussion	Rel-16	TEI16

R2-2108679	Discussion on compatibility issue on failure type for NR SCG failure	CATT	discussion	Rel-15

Contact person(s) for each participating company:

| Company | Contact Name, Email |
| --- | --- |
| Ericsson | hakan.l.palm@ericsson.com |
| MediaTek | nathan.tenny@mediatek.com |
| Lenovo | hchoi5@lenovo.com |
| Samsung | seungri.jin@samsung.com |

# 2	Discussion

## 2.1	Phase 1: Intended to determine agreeable parts

### 2.1.1	CandidateBeamRSList

R2-2107285	Report of email discussion [Post114-e][071][NR16] CandidateBeamRSList set to release (MediaTek)	MediaTek Inc.	discussion	Rel-16	NR_eMIMO-Core	Late

R2-2107286	Handling of candidateBeamRSListExt-v1610 (option A1)	MediaTek Inc.	draftCR	Rel-16	38.331	16.5.0	F	NR_eMIMO-Core	Late

R2-2107287	Handling of candidateBeamRSListExt-v1610 (option B)	MediaTek Inc.	draftCR	Rel-16	38.331	16.5.0	F	NR_eMIMO-Core	Late

R2-2107288	Handling of candidateBeamRSListExt-v1610 (option C)	MediaTek Inc.	draftCR	Rel-16	38.331	16.5.0	F	NR_eMIMO-Core	Late

This topic was discussed at Monday online session. The following conclusions were captured in the chair’s notes:

We go for option A1 (for this and future rel)

-	MTK wonder if this is now the principle for the future (for other fields). Samsung think it is only for this case and current principle in RRC can be kept. Ericsson think we just discuss case by case, right now we don’ t need to discuss the future. Chair: seems that the interest to change/discuss principle is limited. Can disucss at later time, if found to be a general issue.

CRs by email

Q1. Companies are asked to provide their comments on the draft CR in R2-2107285 (option A1).

| Company | Comments |
| --- | --- |
| MediaTek | The unchanged sections in annex A can be deleted (they were provided in case we wanted to make modifications to capture the general case). |

### 2.1.2	Misc Corrections

R2-2108291	Miscellaneous non-controversial corrections Set XI	Ericsson	CR	Rel-16	38.331	16.5.0	2763	-	F	NR_newRAT-Core, TEI16

Q2. Companies are asked to provide their comments on the proposed changes in the draft CR, and provide further findings on typos etc.

| Company | Comments |
| --- | --- |
| MediaTek | One additional typo: in section 5.3.13.2, third level 3 bullet, „resumeCause“ and „mps-PriorityAccess“ should be in italics. |
| Lenovo | The changes are ok but further issues can be fixed as well: - R15 issue: In 6.4 the description of maxBarringInfoSet is not correct, it should say "Maximum number of access control parameter sets“. maxAccessCat-1 INTEGER ::= 63 -- Maximum number of Access Categories minus 1 maxBarringInfoSet INTEGER ::= 8 -- Maximum number of Access Categories - R16 issues: - 5.5.5.1: in the condition below there is a typo, should say „average“. 1> if avareage uplink PDCP delay values are available: - 5.5.2.10: typo in the paragraph below, missing letter „t“ to be added in the word "reselecion": If smtc2-LP is present, for cells indicated in the pci-List parameter in smtc2-LP in the same frequency (for intra frequency cell reselection) or different frequency (for inter frequency cell reselection), the UE shall … - 5.5.2.10a: in the paragraph below, „measDuration” to be replaced by “measDurationSymbols”: On the frequency configured by rmtc-Frequency, the UE shall not consider RSSI measurements outside the configured RMTC occasion which lasts for measDuration for RSSI and channel occupancy measurements. - 6.2.2, LoggedMeasurementConfiguration field descriptions: in the description of reportType the word should say “configuration”. Parameter configures the type of MDT configuration, specifically Periodic MDT conifguraiton or Event Triggerd MDT configuration. - 6.2.2, ConnEstFailReport field descriptions: the description of numberOfPreamblesSent can be removed since the field does not exist in IE ConnEstFailReport. numberOfPreamblesSent This field is used to indicate the number of random access preambles that were transmitted. - 6.3.2, RSSI-ResourceConfigCLI field descriptions: "scs" in field name rssi-scs should be set in capital letters to be aligned with ASN.1. rssi-scs Reference subcarrier spacing for CLI-RSSI measurement. … - 6.3.2, ServingCellConfig field descriptions: "iab-mt" in field name tdd-UL-DL-ConfigurationDedicated-iab-mt should be set in capital letters to be aligned with ASN.1. tdd-UL-DL-ConfigurationDedicated-iab-mt - 6.3.2, SlotFormatCombinationsPerCell field descriptions: in the description of enableConfiguredUL the word "channels" should be added as shown below (PUCCH and CG-PUSCH are channels). enableConfiguredUL If configured, the UE is allowed to transmit uplink signals/channels (SRS, PUCCH, CG-PUSCH) in the set of symbols of the slot when the UE does not detect a DCI format 2_0 providing a slot format for the set of symbols (see TS 38.213 [13], 11.1.1). |
| Samsung | Fine for the changes but I want to know the intention of CatF for this CR. It seems this CR has more minor corrections than Rel-15 CR. In this case, we think Rel-16 CR category could be A instead of F based on the MCC guideline i.e. If there are more substantial changes in the later release CR, we should normally split the CRs to Cat A + Cat F parts so this is clear. But in this case the changes are anyway mostly editorial so we think it’s fine to just use Cat A for all of them. BTW, it is not really ciritical, we are fine either way. |

R2-2108587 Correction on RRC multiplicity and type constraint definitions Huawei, HiSilicon        CR       Rel-16           38.331 16.5.0  2782    -           F          NR_newRAT-Core

Q3. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| MediaTek | Yes | The CR has no normative impact, but it’s good to have from a spec cleanliness perspective. We should normalise on one convention or the other (-1-r16 or -r16-1). |
| Lenovo | Yes | For consistency reasons it might be good to fix the suffices of the „minus one“ constants. However, we think that the correct format is „-1-r16“, i.e. the opposite to what is proposed in the CR. This format is also used in LTE. As result, the suffices of the following constants need to be fixed (and can be merged into the rapporteur CR): - maxAI-DCI-PayloadSize-r16-1 - maxNrofAvailabilityCombinationsPerSet-r16-1 - maxNrofCG-SL-r16-1 - maxCI-DCI-PayloadSize-r16-1 - maxNrofCLI-RSSI-Resources-r16-1 - maxNrofConfiguredGrantConfig-r16-1 - maxNrofConfiguredGrantConfigMAC-r16-1 - maxNrofSPS-Config-r16-1 |
| Samsung | Yes, but | Intention for consistency in RRC spec would be fine. But, it should be kept the legacy cases in 36.331 (i.e. use the suffix of “-1-r16” instead of the suffix of “-r16-1”. This change can be merged in the Rap CR. |

### 2.1.3	eCall over IMS

R2-2107129	Early implementation of eCall over IMS in NR	Qualcomm Incorporated, Ericsson, Vodafone	CR	Rel-16	38.331	16.5.0	2714	-	F	TEI16

Q4. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| MediaTek | Y |  |
| Lenovo | Not necessarily | We are ok to allow early implementation of eCall over IMS in NR. However, this can be always done acc. to Annex D in TS 38.331. There is no stringent need to add the original CR to the table in Annex C. |
| Samsung | Yes |  |

### 2.1.4	NR-U

R2-2107482	Correction on description of lbt-FailureInstanceMaxCount in LBT-FailureRecoveryConfig	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.5.0	2727	-	F	NR_unlic-Core

Q5. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| MediaTek | Y | We understand this aligns with the MAC spec. |
| Apple | Y | We agree. |
| Samsung | Yes | Fine with the change. |

### 2.1.5	2-step RACH

R2-2106911	LS on the description of RRC parameter p0-AlphaSets (R1-2106168; contact: ZTE)	RAN1	LS in	Rel-16	NR_2step_RACH-Core	To:RAN2

R2-2107485	Correction to description of po-AlfphaSets	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.5.0	2728	-	F	NR_2step_RACH-Core

Q6. Companies are asked to provide comments/questions on the RAN1 LS, their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| MediaTek | Y |  |
| Lenovo | Yes | We are fine to follow RAN1 recommendation and the CR looks ok. However, on the cover page the typos in the title (po-AlfphaSets) should be fixed to “p0-AlphaSets”. |
| Apple | Yes | Ok as per LS. |
| Samsung | Yes | This CR is in line with RAN1 understanding. |

R2-2108268	Correction to 38.331 on field description of the MsgA-TransMax	ZTE Corporation, vivo, LG Electronic, OPPO, Samsung	CR	Rel-16	38.331	16.5.0	2760	-	F	NR_2step_RACH-Core

Q7. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| Apple | Yes | Agree |
| Samsung | Yes |  |

R2-2106996	Correction on msg1-SubcarrierSpacing and msgA-SubcarrierSpacing	vivo	CR	Rel-16	38.331	16.5.0	2707	-	F	NR_2step_RACH-Core

Moved from 6.1.4.1.1

Q8. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| Samsung | See Comments | Regarding the 1st change, this may cause backward compatible issue. Legacy r15 UE does not expect msg1-SubcarrierSpacing to be configured if prach-RootSequenceIndex is not L139. So configuring msg1-SubcarrierSpacing when prach-RootSequenceIndex is not L139 and msgA-PRACH-RootSequenceIndex is L139 may cause problem, as legacy UE may consider the configuration as incorrect. In our view there are two possible ways to handle: Option 1: msgA-PRACH-RootSequenceIndex is set to L139 only if prach-RootSequenceIndex is L139. Option 2: msgA-SubcarrierSpacing is also configured if msgA-PRACH-RootSequenceIndex is set to L139 and if prach-RootSequenceIndex is not L139. Note that currently msgA-SubcarrierSpacing is configured if msgA-PRACH-RootSequenceIndex is set to L139 and 4 step RA is not configured. Option 1 restricts the network configuration whereas option 2 does not. So we prefer option 2 and change can be as follows: msgA-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the values 15 or 30 kHz (FR1), and 60 or 120 kHz (FR2) are applicable. The field is only present in case of 2-step only BWP or in case msgA-PRACH-RootSequenceIndex L = 139 and prach-RootSequenceIndex L is not equal to 139. If absent, the UE applies the SCS as derived from the msg1-SubcarrierSpacing in RACH-ConfigCommon in case of msgA-PRACH-RootSequenceIndex L=139, otherwise, the UE applies the SCS as derived from the msgA-prach-ConfigurationIndex in RACH-ConfigGenericTwoStepRA (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]). The value also applies to contention free 2-step random access type (RACH-ConfigDedicated).. |

### 2.1.6	Redirection with MPS indication

R2-2108434	Correction on Redirection with MPS Indication	Peraton Labs, CISA ECD, T-Mobile US, Ericsson , Qualcomm, NTT DoCoMo, AT&T, Verizon	CR	Rel-16	36.331	16.5.0	4714	-	F	NR_newRAT-Core, TEI16

Q9. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| MediaTek | Y | We are not sure the CR exactly matches the problem. In our reading, the access attempt for MPS redirection will be allowed if *any one* of the ACs 12..14 is not barred, even if the actually assigned AC is barred. So in order to effectively bar access for MPS redirection, it seems that the network needs to bar all of ACs 12..14. We understand this may be unavoidable since the UE does not know what the assigned AC is, but it is still a bit counterintuitive and should perhaps be captured in a NOTE. |
| Lenovo | Yes but | We are ok to extend the applicable AC to 12, 13. However, on the proposed change we wonder how it is meant to work: - If the UE is configured with AC 12, 13, 14 and the NW sets any of the corresponding bits in the ac-BarringForSpecialAC to „0“ then the UE considers the access to the cell as not barred. - If the UE is configured with e.g. AC 12 only and the NW sets AC 12 to „1“ and AC 13, 14 to „0“ in the ac-BarringForSpecialAC, then does this UE still consider the access to the cell as not barred? Proponents should clarify whether scenario 2 is valid or not acc. to the CR. |
| Samsung | Yes | MPS service can be offered in a number of possible Access Classes (e.g. AC 12-14). |

### 2.1.7	LTE changes - Mobility

R2-2108375	Correction on ULInformationTransferMRDC(R16)	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.5.0	4713	-	F	TEI16

Q10. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| MediaTek | Maybe | We think the intention is fine. I guess we can just remove the word „only“ here as below. NR RRCReconfigurationComplete (transmitted upon CPC execution if only SRB1 is configured and the UE is operating in EN-DC) messages. Note 1 - WI code should be LTE_feMob-Core as it is mainly CR for CPC. Note 2 – There is typo „SBR1“ in the proposed text. Should be SRB1. |
| Samsung | Maybe yes | It seems good to be in line with the procedural texts of TS38.331. |

### 2.1.8	LTE changes - ASN.1 on SCG Failure report

R2-2108189	ASN.1 misalignment for the SCGFailureInformationNR message	Ericsson	CR	Rel-16	36.331	16.5.0	4709	-	F	LTE_NR_DC_CA_enh-Core, NR_unlic-Core, NR_IAB-Core, NR_Mob_enh-Core

Moved from 6.1.4.1.1

R2-2108190	ASN.1 misalignment for the SCGFailureInformationNR message	Ericsson	CR	Rel-16	38.331	16.5.0	2758	-	F	LTE_NR_DC_CA_enh-Core, NR_unlic-Core, NR_IAB-Core, NR_Mob_enh-Core

Moved from 6.1.4.1.1

R2-2108569	Discussion on compatibility issue and solutions for Rel-15 failure type definition	Huawei, HiSilicon	discussion	Rel-16	TEI16

R2-2108679	Discussion on compatibility issue on failure type for NR SCG failure	CATT	discussion	Rel-15

The above-listed documents deal with an issue postponed at RAN2#114e

–	SCGFailureInformationNR

FailureReportSCG-NR-r15 ::=		SEQUENCE {

failureType-r15						ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-reconfigFailure,

srb3-IntegrityFailure, other-r16},

measResultFreqListNR-r15				MeasResultFreqListFailNR-r15		OPTIONAL,

measResultSCG-r15						OCTET STRING						OPTIONAL,

...,

[[	locationInfo-r16				LocationInfo-r10						OPTIONAL,

logMeasResultListBT-r16			LogMeasResultListBT-r15					OPTIONAL,

logMeasResultListWLAN-r16		LogMeasResultListWLAN-r15				OPTIONAL,

failureType-v1610				ENUMERATED {t312-Expiry, scg-lbtFailure,

beamFailureRecoveryFailure, bh-RLF-r16, spare4,

spare3, spare2, spare1}	OPTIONAL

]]

}

The rapporteurs understanding is that the code point other-16 does not exist in in 36.331 Rel-15 (not even as a spare value), and will result in a transfer syntax error if received by eNb based on Rel-15 ASN.1.

In the following, the rapporteur tried (based on R2-2108679) to list the proposed solutions (sourcing companies are asked to verify and add/correct where needed).

Solution 1-1
Add a new code point into the failureType-r15 field in 36.331 Rel-15 SCGFailureInformationNR, e.g. “reserved”. 
Further, add a code point into the failureTyp-15 field within the TS 38.331 CG-ConfigInfo structure, e.g. “reserved”.

Solution 1-2:

Add a new code point into the failureType-r15 field in 36.331 Rel-15 SCGFailureInformationNR, e.g. “reserved”.

CG-ConfigInfo is not updated, an hence MeNB should instead trigger a release of SCG.

Solution 2-1: 
The value other-r16 should not be used for any case. The UE should always include a “similar” r15 failure type. There are two variants.

- Setting of failureType-r15 is specified in 38.331 procedure text.

- Setting of failureType-r15 is left to UE impl. This solution is covered in R2-2108569.

Solution 2-2: 
For the case of BFR failure, the Rel-16 UE should set the value failureType-r15 to randomAccessProblem Other cases are prevented by network implementation (“prevent Rel-16 UEs from encountering T312 expires, LBT failures and BH RLFs when connecting to a Rel-15 MeNB “).

Solution 3
Introduce a new field failureTypeOther in 36.331 Rel-16 SCGFailureInformationNR message, and dummify existing other-r16 code-point. Corresponding procedure text changes in 38.331 Rel-16. (This solution is covered by draft CRs in R2-2108189/R2-2108190).

In this Phase 1, companies are asked to provide their views on preferred solution alternatives. In a Phase 2, we can discuss CR details.

Q11. Companies are asked to provide their view on preferred solution alternative.

| Company | Preferred Solution | Comments |
| --- | --- | --- |
| MediaTek | See comment | This is an unfortunate bug in R16 ASN.1. We should never add new UL enum value in legacy field. We recognize this is a real problem and there is no backward compatible way to solve. Our Suggestion - Dummify the other-r16 code-point (The R16 UE shall never use it, note that it is NBC) - No need to add new failureTypeOther-r16 as proposed in solution 3. The existing failureType-v1610 will provide enough information to R16 eNB. - The UE set failureType-r15 to any legacy code-point (or some predefined code-point, e.g. randomAccessProblem, no strong view) while including the R16 field failureType-v1610. |
| Lenovo | See comment | We recall that when the value “other-r16“ was agreed to be introduced in failureType-r15 for a previously unknown codepoint it was assumed that legacy eNBs need to be upgraded to comprehend the “other-r16“. We wonder why it is not possible to do that. We understood that solution 2-1 and 2-2 also require an upgrade of legacy eNBs to comprehend the new mapping of the legacy R15 failure types. |
| Apple | Aligned with Mediatek’s view. |  |
| Samsung | Solution 3 or the variant of solution 3 | We share the problem so indeed it should be corrected. The cleanest solution could be the Solution 3 so we think it is better if all other solutions have NBC problem as well. |

# 3	Conclusion

- To be updated after discussion on Phase 1 -