3GPP TSG-RAN WG2 #114-e electronic	R2-210xxxx

Electronic, 16 – 27 August 2021

Agenda Item:	6.1.4.1.5 Other

Source:	Ericsson

Title:	[AT115-e][027][NR16] CP Other & LTE (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This contribution summarizes the following discussion:

[AT115-e][027][NR16] CP Other & LTE (Ericsson)

Scope: Determine agreeable parts and agree CRs, For R2-2107285-7288 await on-line treat remaining part if needed, Treat R2-2108291, R2-2107129, R2-2107482, R2-2106911, R2-2108268, R2-2107485, R2-2106996, R2-2108434, R2-2108275, R2-2108189, R2-2108190, R2-2108569, R2-2108679,

Intended outcome: Report, Agreed CRs.

Deadline: A first round with Deadline for comments Thursday Aug 19 1200 UTC

CandidateBeamRSList

R2-2107285	Report of email discussion [Post114-e][071][NR16] CandidateBeamRSList set to release (MediaTek)	MediaTek Inc.	discussion	Rel-16	NR_eMIMO-Core	Late

R2-2107286	Handling of candidateBeamRSListExt-v1610 (option A1)	MediaTek Inc.	draftCR	Rel-16	38.331	16.5.0	F	NR_eMIMO-Core	Late

R2-2107287	Handling of candidateBeamRSListExt-v1610 (option B)	MediaTek Inc.	draftCR	Rel-16	38.331	16.5.0	F	NR_eMIMO-Core	Late

R2-2107288	Handling of candidateBeamRSListExt-v1610 (option C)	MediaTek Inc.	draftCR	Rel-16	38.331	16.5.0	F	NR_eMIMO-Core	Late

Misc Corrections

R2-2108291	Miscellaneous non-controversial corrections Set XI	Ericsson	CR	Rel-16	38.331	16.5.0	2763	-	F	NR_newRAT-Core, TEI16

R2-2108587 Correction on RRC multiplicity and type constraint definitions Huawei, HiSilicon        CR       Rel-16           38.331 16.5.0  2782    -           F          NR_newRAT-Core

eCall over IMS

R2-2107129	Early implementation of eCall over IMS in NR	Qualcomm Incorporated, Ericsson, Vodafone	CR	Rel-16	38.331	16.5.0	2714	-	F	TEI16

NR-U

R2-2107482	Correction on description of lbt-FailureInstanceMaxCount in LBT-FailureRecoveryConfig	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.5.0	2727	-	F	NR_unlic-Core

2-step RACH

R2-2106911	LS on the description of RRC parameter p0-AlphaSets (R1-2106168; contact: ZTE)	RAN1	LS in	Rel-16	NR_2step_RACH-Core	To:RAN2

R2-2108268	Correction to 38.331 on field description of the MsgA-TransMax	ZTE Corporation, vivo, LG Electronic, OPPO, Samsung	CR	Rel-16	38.331	16.5.0	2760	-	F	NR_2step_RACH-Core

R2-2107485	Correction to description of po-AlfphaSets	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.5.0	2728	-	F	NR_2step_RACH-Core

R2-2106996	Correction on msg1-SubcarrierSpacing and msgA-SubcarrierSpacing	vivo	CR	Rel-16	38.331	16.5.0	2707	-	F	NR_2step_RACH-Core

Moved from 6.1.4.1.1

Redirection with MPS indication

R2-2108434	Correction on Redirection with MPS Indication	Peraton Labs, CISA ECD, T-Mobile US, Ericsson , Qualcomm, NTT DoCoMo, AT&T, Verizon	CR	Rel-16	36.331	16.5.0	4714	-	F	NR_newRAT-Core, TEI16

LTE changes - Mobility

R2-2108375	Correction on ULInformationTransferMRDC(R16)	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.5.0	4713	-	F	TEI16

LTE changes - ASN.1 on SCG Failure report

R2-2108189	ASN.1 misalignment for the SCGFailureInformationNR message	Ericsson	CR	Rel-16	36.331	16.5.0	4709	-	F	LTE_NR_DC_CA_enh-Core, NR_unlic-Core, NR_IAB-Core, NR_Mob_enh-Core

Moved from 6.1.4.1.1

R2-2108190	ASN.1 misalignment for the SCGFailureInformationNR message	Ericsson	CR	Rel-16	38.331	16.5.0	2758	-	F	LTE_NR_DC_CA_enh-Core, NR_unlic-Core, NR_IAB-Core, NR_Mob_enh-Core

Moved from 6.1.4.1.1

R2-2108569	Discussion on compatibility issue and solutions for Rel-15 failure type definition	Huawei, HiSilicon	discussion	Rel-16	TEI16

R2-2108679	Discussion on compatibility issue on failure type for NR SCG failure	CATT	discussion	Rel-15

Contact person(s) for each participating company:

| Company | Contact Name, Email |
| --- | --- |
| Ericsson | hakan.l.palm@ericsson.com |
| MediaTek | nathan.tenny@mediatek.com |
| Lenovo | hchoi5@lenovo.com |
| Samsung | seungri.jin@samsung.com |
| Nokia | jarkko.t.koskela@nokia.com |
| Qulacomm | mambriss@qti.qualcomm.com (Mouaffac) |
| Vodafone | chris.pudney@vodafone.com |
| Convida Wireless | sunell.kaierik@convidawireless.com |
| Peraton Labs | akogiantis@peratonlabs.com |

# 2	Discussion

## 2.1	Phase 1: Intended to determine agreeable parts

### 2.1.1	CandidateBeamRSList

R2-2107285	Report of email discussion [Post114-e][071][NR16] CandidateBeamRSList set to release (MediaTek)	MediaTek Inc.	discussion	Rel-16	NR_Emimo-Core	Late

R2-2107286	Handling of candidateBeamRSListExt-v1610 (option A1)	MediaTek Inc.	draftCR	Rel-16	38.331	16.5.0	F	NR_Emimo-Core	Late

R2-2107287	Handling of candidateBeamRSListExt-v1610 (option B)	MediaTek Inc.	draftCR	Rel-16	38.331	16.5.0	F	NR_Emimo-Core	Late

R2-2107288	Handling of candidateBeamRSListExt-v1610 (option C)	MediaTek Inc.	draftCR	Rel-16	38.331	16.5.0	F	NR_Emimo-Core	Late

This topic was discussed at Monday online session. The following conclusions were captured in the chair’s notes:

We go for option A1 (for this and future rel)

-	MTK wonder if this is now the principle for the future (for other fields). Samsung think it is only for this case and current principle in RRC can be kept. Ericsson think we just discuss case by case, right now we don’ t need to discuss the future. Chair: seems that the interest to change/discuss principle is limited. Can disucss at later time, if found to be a general issue.

CRs by email

Q1. Companies are asked to provide their comments on the draft CR in R2-2107285 (option A1).

| Company | Comments |
| --- | --- |
| MediaTek | The unchanged sections in annex A can be deleted (they were provided in case we wanted to make modifications to capture the general case). |
| Nokia, Nokia Shanghai Bell | Agree with MediaTek on removal of annex A. Additionally, we have some general wording proposals for the text: - „maintains awareness“ is correct but seems a bit circumspect: We would propose to use just „remembers“ for simplicity. - Some „only“ could be used added to the text to ensure it‘s clear the extension does not apply to the legacy list entries. This is what we would propose for the added text (highlighted parts are different from the original CR): The UE remembers which elements were configured by candidateBeamRSListExt-v1610, and the subsequently received contents of candidateBeamRSListExt-v1610 apply only to these entries (i.e., receiving candidateBeamRSListExt-v1610 set to release releases only the entries that were configured by candidateBeamRSListExt-v1610, and receiving candidateBeamRSListExt-v1610 set to setup replaces only the entries that were configured by candidateBeamRSListExt-v1610 with the newly signalled entries). For cover page, some suggestions: - Reason for change: Use „only“ instead of „all“ in the last sentence, i.e. as per below „ If candidateBeamRSListExt-v1610 is set to release, it is ambiguous whether the UE should release all entries >16 in the combined list or only entries that were originally configured by candidateBeamRSListExt-v1610.“ - Summary of change: If we use „remembers“ in the field text, then maybe usiong „remembers“ is also appropriate here: „ It is clarified in the field description that the UE remembers of which list entries were configured by which field. „. Similarly, „all“ may not be appropriate and „only“ could be easier to understand (same as above comment for the field description), i.e. „ When candidateBeamRSListExt-v1610 is set to release, the UE releases only entries that were originally configured by candidateBeamRSListExt-v1610, irrespective of their current position in the stored list; when candidateBeamRSListExt-v1610 is set to setup, the UE replaces only entries that were originally configured by candidateBeamRSListExt-v1610 with the newly signalled entries.“ |

### 2.1.2	Misc Corrections

R2-2108291	Miscellaneous non-controversial corrections Set XI	Ericsson	CR	Rel-16	38.331	16.5.0	2763	-	F	NR_newRAT-Core, TEI16

Q2. Companies are asked to provide their comments on the proposed changes in the draft CR, and provide further findings on typos etc.

| Company | Comments |
| --- | --- |
| MediaTek | One additional typo: in section 5.3.13.2, third level 3 bullet, „resumeCause“ and „mps-PriorityAccess“ should be in italics. |
| Lenovo | The changes are ok but further issues can be fixed as well: - R15 issue: In 6.4 the description of maxBarringInfoSet is not correct, it should say „Maximum number of access control parameter sets“. MaxAccessCat-1 INTEGER ::= 63 -- Maximum number of Access Categories minus 1 maxBarringInfoSet INTEGER ::= 8 -- Maximum number of Access Categories - R16 issues: - 5.5.5.1: in the condition below there is a typo, should say „average“. - if avareage uplink PDCP delay values are available: - 5.5.2.10: typo in the paragraph below, missing letter „t“ to be added in the word „reselecion“: If smtc2-LP is present, for cells indicated in the pci-List parameter in smtc2-LP in the same frequency (for intra frequency cell reselection) or different frequency (for inter frequency cell reselection), the UE shall … - 5.5.2.10a: in the paragraph below, „measDuration” to be replaced by “measDurationSymbols”: On the frequency configured by rmtc-Frequency, the UE shall not consider RSSI measurements outside the configured RMTC occasion which lasts for measDuration for RSSI and channel occupancy measurements. - 6.2.2, LoggedMeasurementConfiguration field descriptions: in the description of reportType the word should say “configuration”. Parameter configures the type of MDT configuration, specifically Periodic MDT conifguraiton or Event Triggerd MDT configuration. - 6.2.2, ConnEstFailReport field descriptions: the description of numberOfPreamblesSent can be removed since the field does not exist in IE ConnEstFailReport. NumberOfPreamblesSent This field is used to indicate the number of random access preambles that were transmitted. - 6.3.2, RSSI-ResourceConfigCLI field descriptions: „scs“ in field name rssi-scs should be set in capital letters to be aligned with ASN.1. rssi-scs Reference subcarrier spacing for CLI-RSSI measurement. … - 6.3.2, ServingCellConfig field descriptions: „iab-mt“ in field name tdd-UL-DL-ConfigurationDedicated-iab-mt should be set in capital letters to be aligned with ASN.1. tdd-UL-DL-ConfigurationDedicated-iab-mt - 6.3.2, SlotFormatCombinationsPerCell field descriptions: in the description of enableConfiguredUL the word „channels“ should be added as shown below (PUCCH and CG-PUSCH are channels). EnableConfiguredUL If configured, the UE is allowed to transmit uplink signals/channels (SRS, PUCCH, CG-PUSCH) in the set of symbols of the slot when the UE does not detect a DCI format 2_0 providing a slot format for the set of symbols (see TS 38.213 [13], 11.1.1). |
| Samsung | Fine for the changes but I want to know the intention of CatF for this CR. It seems this CR has more minor corrections than Rel-15 CR. In this case, we think Rel-16 CR category could be A instead of F based on the MCC guideline i.e. If there are more substantial changes in the later release CR, we should normally split the CRs to Cat A + Cat F parts so this is clear. But in this case the changes are anyway mostly editorial so we think it’s fine to just use Cat A for all of them. BTW, it is not really ciritical, we are fine either way. |
| Huawei, HiSilicon | Clauses affected needs to be populated in the final CR. |
| Vivo | Editorial changes, agree |
| Nokia | Same as Huawei. Also Cat F aseems bit strong for this one |
| Convida Wireless | It reads NR_newRAT-Core on the coversheet. It is a Rel-15 work item. Hence, this is a Rel-16 mirror CR to a Rel-15 correction? If so, the category must be A. Otherwise, if this is only TEI16, it is a Category F CR. Observe, Rel-16 CR cannot be submitted to TSG RAN approval with a Rel-15 WI code unless it is a mirror CR. Also 1) the changes are minor but still some text is added and removed, and 2) Rel-16 is already frozen but Category D is prohibited to frozen Releases. Therefore this cannot be a Category D CR even if Category F is perceived strong. |

R2-2108587 Correction on RRC multiplicity and type constraint definitions Huawei, HiSilicon        CR       Rel-16           38.331 16.5.0  2782    -           F          NR_newRAT-Core

Q3. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| MediaTek | Yes | The CR has no normative impact, but it’s good to have from a spec cleanliness perspective. We should normalise on one convention or the other (-1-r16 or -r16-1). |
| Lenovo | Yes | For consistency reasons it might be good to fix the suffices of the „minus one“ constants. However, we think that the correct format is „-1-r16“, i.e. the opposite to what is proposed in the CR. This format is also used in LTE. As result, the suffices of the following constants need to be fixed (and can be merged into the rapporteur CR): - maxAI-DCI-PayloadSize-r16-1 - maxNrofAvailabilityCombinationsPerSet-r16-1 - maxNrofCG-SL-r16-1 - maxCI-DCI-PayloadSize-r16-1 - maxNrofCLI-RSSI-Resources-r16-1 - maxNrofConfiguredGrantConfig-r16-1 - maxNrofConfiguredGrantConfigMAC-r16-1 - maxNrofSPS-Config-r16-1 |
| Samsung | Yes, but | Intention for consistency in RRC spec would be fine. But, it should be kept the legacy cases in 36.331 (i.e. use the suffix of “-1-r16” instead of the suffix of “-r16-1”. This change can be merged in the Rap CR. |
| Huawei, HiSilicon | Yes | Proponent |
| vivo | Yes, but | Agree with Lenovo and Samsung that we should stick to the legacy convention by using „-1-r16“. |
| Nokia, Nokia Shanghai Bell | Partly | The correct form is „-1-r16“, i.e. the release suffix comes last. This is because in case the constant name is referected in procedural text, the releases suffix „-r16“ can be dropped but the „-1“ cannot. Any changes like this can be merged to the rapporteur CR. |
| QCOM | Yes | Since there are 2 different opinions on how to fix the “-1“ suffix, it makes more sense to have it “-1-r16““ |

### 2.1.3	eCall over IMS

R2-2107129	Early implementation of eCall over IMS in NR	Qualcomm Incorporated, Ericsson, Vodafone	CR	Rel-16	38.331	16.5.0	2714	-	F	TEI16

Q4. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| MediaTek | Y |  |
| Lenovo | Not necessarily | We are ok to allow early implementation of eCall over IMS in NR. However, this can be always done acc. to Annex D in TS 38.331. There is no stringent need to add the original CR to the table in Annex C. |
| Samsung | Yes |  |
| Huawei, HiSilicon | No | The support of eCall has been discussed previously in Rel-16 triggered by the LS in R2-2002549 and it was decided to support it since Rel-16 as cited below "TSG SA believes that the changes required in order to allow support for eCall over IMS (NG-eCall) over NR are minimal. CRs should be prepared for TSGs #88 in June 2020 in order to maximise the possibility of including this in Release 16." Thus we don't see need to have early implementation. |
| vivo | Yes |  |
| Nokia | Yes |  |
| Qualcomm Incorproated | Proponent | To answer Lenovo’s question, the CR is necessary to clarify the test applicability, i.e. necessary information for RAN5. Annex D is to explain „how“ the early implementation can be done. |
| Vodafone | yes | To answer Huawei: The SA report is encouraging early implementation! The alternative was that this was done in rel 17. For us it is important that “5G cars“ can appear as soon as possible – and owing to the long life of cars, it is important that they all support eCall over NR. |

### 2.1.4	NR-U

R2-2107482	Correction on description of lbt-FailureInstanceMaxCount in LBT-FailureRecoveryConfig	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.5.0	2727	-	F	NR_unlic-Core

Q5. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| MediaTek | Y | We understand this aligns with the MAC spec. |
| Apple | Y | We agree. |
| Samsung | Yes | Fine with the change. |
| Huawei, HiSilicon | Maybe not | The coversheet contains an error that “Related RAN1 LS in R2-2106911”is not true. The field description already links to TS 38.321 for more details, so we can live without this text enhancement, as we see no room for ambiguity. |
| vivo | Yes |  |
| Nokia | Not critical | Editorial. There is not really room for misunderstanding. This could be just part of editorial rapporetuer CR as well if any. |
| QCOM | No strong view | More like an editorial change, MAC spec is already clearly stated how the procedure should work |

### 2.1.5	2-step RACH

R2-2106911	LS on the description of RRC parameter p0-AlphaSets (R1-2106168; contact: ZTE)	RAN1	LS in	Rel-16	NR_2step_RACH-Core	To:RAN2

R2-2107485	Correction to description of po-AlfphaSets	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.5.0	2728	-	F	NR_2step_RACH-Core

Q6. Companies are asked to provide comments/questions on the RAN1 LS, their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| MediaTek | Y |  |
| Lenovo | Yes | We are fine to follow RAN1 recommendation and the CR looks ok. However, on the cover page the typos in the title (po-AlfphaSets) should be fixed to “p0-AlphaSets”. |
| Apple | Yes | Ok as per LS. |
| Samsung | Yes | This CR is in line with RAN1 understanding. |
| Huawei, HiSilicon | Yes |  |
| vivo | Yes |  |
| Nokia | Yes | aligns with RAN1 |

R2-2108268	Correction to 38.331 on field description of the MsgA-TransMax	ZTE Corporation, vivo, LG Electronic, OPPO, Samsung	CR	Rel-16	38.331	16.5.0	2760	-	F	NR_2step_RACH-Core

Q7. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| Apple | Yes | Agree |
| Samsung | Yes |  |
| Huawei, HiSilicon | Not needed | The change in this CR changes nothing at the moment as the only place where msgA-TransMax is configured in RACH-ConfigDedicated is cfra-TwoStep, so there is no confusion. The change in MAC agreed last meeting was actually relevant as it changed the UE behaviour, but this one is not needed in our opinion. |
| Vivo | Yes | Proponent |
| Nokia | OK | not critical but OK to have |
| QCOM | Yes | CR aligns 38.331 and 38.321 on application of msgA-TransMax. Without this CR, UE can't fallback to 4-step RACH (if 2-step RACH fails) for CFRA based HO. |

R2-2106996	Correction on msg1-SubcarrierSpacing and msgA-SubcarrierSpacing	vivo	CR	Rel-16	38.331	16.5.0	2707	-	F	NR_2step_RACH-Core

Moved from 6.1.4.1.1

Q8. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| Samsung | See Comments | Regarding the 1st change, this may cause backward compatible issue. Legacy r15 UE does not expect msg1-SubcarrierSpacing to be configured if prach-RootSequenceIndex is not L139. So configuring msg1-SubcarrierSpacing when prach-RootSequenceIndex is not L139 and msgA-PRACH-RootSequenceIndex is L139 may cause problem, as legacy UE may consider the configuration as incorrect. In our view there are two possible ways to handle: Option 1: msgA-PRACH-RootSequenceIndex is set to L139 only if prach-RootSequenceIndex is L139. Option 2: msgA-SubcarrierSpacing is also configured if msgA-PRACH-RootSequenceIndex is set to L139 and if prach-RootSequenceIndex is not L139. Note that currently msgA-SubcarrierSpacing is configured if msgA-PRACH-RootSequenceIndex is set to L139 and 4 step RA is not configured. Option 1 restricts the network configuration whereas option 2 does not. So we prefer option 2 and change can be as follows: msgA-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the values 15 or 30 kHz (FR1), and 60 or 120 kHz (FR2) are applicable. The field is only present in case of 2-step only BWP or in case msgA-PRACH-RootSequenceIndex L = 139 and prach-RootSequenceIndex L is not equal to 139. If absent, the UE applies the SCS as derived from the msg1-SubcarrierSpacing in RACH-ConfigCommon in case of msgA-PRACH-RootSequenceIndex L=139, otherwise, the UE applies the SCS as derived from the msgA-prach-ConfigurationIndex in RACH-ConfigGenericTwoStepRA (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]). The value also applies to contention free 2-step random access type (RACH-ConfigDedicated).. |
| Huawei, HiSilicon | Not sure | We are wondering whether RAN1 really considered the case that L=839 for 4-step RA while L=139 for 2-step RA, which is the configuration which causes the issues. If RAN1 intention was to support this case (which cannot be derived directly from the current agreements), then perhaps RAN1 should discuss whether for this case msgA-SubcarrierSpacing or msg1-SubcarrierSpacing should be used to indicate the SCS. We think this needs to be confirmed by RAN1 first. Also, the current CR seems to cause an issue that in case msg1-SubcarrierSpacing is only configured by the network to indicate SCS for msgA, then the UE will mistakenly apply it also for msg1 SCS, even though the intention was to use SCS indicated by prach-ConfigurationIndex for 4-step RA. |
| vivo | Yes with comments | Thanks Samsung and Huawei for pointing out the backward compatible issue raised in the draft CR. To resolve this problem, we are fine with Samsung’s revision. Regarding Huawei’s concern on the potential case where L=839 for 4-step RA while L=139 for 2-step RA (i.e. 4-step RA with the long sequence for coverage performance and 2-step RA with the short sequence for radio-efficiency performance), in our memory, this case was not excluded during the previous RAN1 discussion. If companies think we should check with RAN1, we are also okay to send an LS to RAN1 for further confirmation. |
| QCOM | Yes | CR fixes the current inconsistency in 38.331 of determing msgA-SubcarrierSpacing when msgA-PRACH-RootSequenceIndex = l139 |
| Convida Wireless | No preference | Agree with Samsung’s proposal to fix backwards compatibility issues. |

### 2.1.6	Redirection with MPS indication

R2-2108434	Correction on Redirection with MPS Indication	Peraton Labs, CISA ECD, T-Mobile US, Ericsson , Qualcomm, NTT DoCoMo, AT&T, Verizon	CR	Rel-16	36.331	16.5.0	4714	-	F	NR_newRAT-Core, TEI16

Q9. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| MediaTek | Y | We are not sure the CR exactly matches the problem. In our reading, the access attempt for MPS redirection will be allowed if *any one* of the ACs 12..14 is not barred, even if the actually assigned AC is barred. So in order to effectively bar access for MPS redirection, it seems that the network needs to bar all of ACs 12..14. We understand this may be unavoidable since the UE does not know what the assigned AC is, but it is still a bit counterintuitive and should perhaps be captured in a NOTE. |
| Lenovo | Yes but | We are ok to extend the applicable AC to 12, 13. However, on the proposed change we wonder how it is meant to work: - If the UE is configured with AC 12, 13, 14 and the NW sets any of the corresponding bits in the ac-BarringForSpecialAC to „0“ then the UE considers the access to the cell as not barred. - If the UE is configured with e.g. AC 12 only and the NW sets AC 12 to „1“ and AC 13, 14 to „0“ in the ac-BarringForSpecialAC, then does this UE still consider the access to the cell as not barred? Proponents should clarify whether scenario 2 is valid or not acc. to the CR. |
| Samsung | Yes | MPS service can be offered in a number of possible Access Classes (e.g. AC 12-14). |
| vivo | Yes, but | Not sure why the cases when AC = 11 or 15 are excluded from the description, as the TS 22.011 clause 4.4a states that: “4.4a Multimedia Priority Service Multimedia Priority Service (TS 22.153 [16]) shall be assigned its own unique access class value (i.e., one of the special access classes 11 to 15). The assigned access class value for Multimedia Priority Service is based on regional/national regulatory requirements and operator policy.” |
| QCOM | Yes |  |
| Peraton Labs | Yes | In LTE, the SIB1 contains the ac-BarringForSpecialAC 5-bit field. When a bit for an AC is set to zero the UE of that AC is not subject to barring. When the bit is not set (to zero) that UE will be barred as a normal UE (using ac-BarringConfig). The UE is not aware of what Access Class corresponds to the MPS service. Thus, the barring check has to cover all the possibilities of Access Classes. In deployments it would be very rare to encounter undesirable behavior from the proposed CR since a combination of conditions would have to occur where the MPS AC would have to be subject to barring, while the other ACs would not, and simultaneously the network would experience redirection of UEs with MPS Indication, which would have to be subject to barring. Therefore a simple solution is preferred for the barring check. Answer to Mediatek: The example described shows correct behavior with the proposed CR. That is, the UE being redirected with MPS Indication will only be subject to barring if all three bits corresponding to AC 12, 13 and 14 have not been set (bit is 1). If any of the three bits is set to zero this UE will consider the cell as not barred. Answer to Lenovo: The CR addresses the case where the UE is not assigned a special AC 12-14, thus it is a UE unsubscribed to MPS service. It is understood that the intention of the Lenovo examples is to mean “if the MPS is configured with AC 12, 13, 14” instead of what was stated as “if the UE is configured with AC 12, 13, 14”. For both examples 1 and 2 the proposed CR would allow the UE to consider the cell as not barred. Answer to VIVO: TS 22.011 describes the Access Class allocation in Section 4.2 as follows: Class 15 - PLMN Staff; -"- 14 - Emergency Services; -"- 13 - Public Utilities (e.g. water/gas suppliers); -"- 12 - Security Services; -"- 11 - For PLMN Use. Therefore only 12-14 are considered to be used. |

### 2.1.7	LTE changes - Mobility

R2-2108375	Correction on ULInformationTransferMRDC(R16)	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.5.0	4713	-	F	TEI16

Q10. Companies are asked to provide their view on the need of the draft CR, and comments on the changes in the draft CR.

| Company | CR needed? | Comments |
| --- | --- | --- |
| MediaTek | Maybe | We think the intention is fine. I guess we can just remove the word „only“ here as below. NR RRCReconfigurationComplete (transmitted upon CPC execution if only SRB1 is configured and the UE is operating in EN-DC) messages. Note 1 - WI code should be LTE_feMob-Core as it is mainly CR for CPC. Note 2 – There is typo „SBR1“ in the proposed text. Should be SRB1. |
| Samsung | Maybe yes | It seems good to be in line with the procedural texts of TS38.331. |
| vivo | Maybe | Similar view with MTK. |
| Nokia | Maybe | Change SBR1 to SRB1 or even the proposal from MTK seems simpler |
| QCOM | May be | MTK proposal seems ok |

### 2.1.8	LTE changes - ASN.1 on SCG Failure report

R2-2108189	ASN.1 misalignment for the SCGFailureInformationNR message	Ericsson	CR	Rel-16	36.331	16.5.0	4709	-	F	LTE_NR_DC_CA_enh-Core, NR_unlic-Core, NR_IAB-Core, NR_Mob_enh-Core

Moved from 6.1.4.1.1

R2-2108190	ASN.1 misalignment for the SCGFailureInformationNR message	Ericsson	CR	Rel-16	38.331	16.5.0	2758	-	F	LTE_NR_DC_CA_enh-Core, NR_unlic-Core, NR_IAB-Core, NR_Mob_enh-Core

Moved from 6.1.4.1.1

R2-2108569	Discussion on compatibility issue and solutions for Rel-15 failure type definition	Huawei, HiSilicon	discussion	Rel-16	TEI16

R2-2108679	Discussion on compatibility issue on failure type for NR SCG failure	CATT	discussion	Rel-15

The above-listed documents deal with an issue postponed at RAN2#114e

–	SCGFailureInformationNR

FailureReportSCG-NR-r15 ::=		SEQUENCE {

failureType-r15						ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-reconfigFailure,

srb3-IntegrityFailure, other-r16},

measResultFreqListNR-r15				MeasResultFreqListFailNR-r15		OPTIONAL,

measResultSCG-r15						OCTET STRING						OPTIONAL,

...,

[[	locationInfo-r16				LocationInfo-r10						OPTIONAL,

logMeasResultListBT-r16			LogMeasResultListBT-r15					OPTIONAL,

logMeasResultListWLAN-r16		LogMeasResultListWLAN-r15				OPTIONAL,

failureType-v1610				ENUMERATED {t312-Expiry, scg-lbtFailure,

beamFailureRecoveryFailure, bh-RLF-r16, spare4,

spare3, spare2, spare1}	OPTIONAL

]]

}

The rapporteurs understanding is that the code point other-16 does not exist in in 36.331 Rel-15 (not even as a spare value), and will result in a transfer syntax error if received by eNb based on Rel-15 ASN.1.

In the following, the rapporteur tried (based on R2-2108679) to list the proposed solutions (sourcing companies are asked to verify and add/correct where needed).

Solution 1-1
Add a new code point into the failureType-r15 field in 36.331 Rel-15 SCGFailureInformationNR, e.g. “reserved”. 
Further, add a code point into the failureTyp-15 field within the TS 38.331 CG-ConfigInfo structure, e.g. “reserved”.

Solution 1-2:

Add a new code point into the failureType-r15 field in 36.331 Rel-15 SCGFailureInformationNR, e.g. “reserved”.

CG-ConfigInfo is not updated, an hence MeNB should instead trigger a release of SCG.

Solution 2-1: 
The value other-r16 should not be used for any case. The UE should always include a “similar” r15 failure type. There are two variants.

- Setting of failureType-r15 is specified in 38.331 procedure text.

- Setting of failureType-r15 is left to UE impl. This solution is covered in R2-2108569.

Solution 2-2: 
For the case of BFR failure, the Rel-16 UE should set the value failureType-r15 to randomAccessProblem Other cases are prevented by network implementation (“prevent Rel-16 UEs from encountering T312 expires, LBT failures and BH RLFs when connecting to a Rel-15 MeNB “).

Solution 3
Introduce a new field failureTypeOther in 36.331 Rel-16 SCGFailureInformationNR message, and dummify existing other-r16 code-point. Corresponding procedure text changes in 38.331 Rel-16. (This solution is covered by draft CRs in R2-2108189/R2-2108190).

In this Phase 1, companies are asked to provide their views on preferred solution alternatives. In a Phase 2, we can discuss CR details.

Q11. Companies are asked to provide their view on preferred solution alternative.

| Company | Preferred Solution | Comments |
| --- | --- | --- |
| MediaTek | See comment | This is an unfortunate bug in R16 ASN.1. We should never add new UL enum value in legacy field. We recognize this is a real problem and there is no backward compatible way to solve. Our Suggestion - Dummify the other-r16 code-point (The R16 UE shall never use it, note that it is NBC) - No need to add new failureTypeOther-r16 as proposed in solution 3. The existing failureType-v1610 will provide enough information to R16 eNB. - The UE set failureType-r15 to any legacy code-point (or some predefined code-point, e.g. randomAccessProblem, no strong view) while including the R16 field failureType-v1610. |
| Lenovo | See comment | We recall that when the value “other-r16“ was agreed to be introduced in failureType-r15 for a previously unknown codepoint it was assumed that legacy eNBs need to be upgraded to comprehend the “other-r16“. We wonder why it is not possible to do that. We understood that solution 2-1 and 2-2 also require an upgrade of legacy eNBs to comprehend the new mapping of the legacy R15 failure types. |
| Apple | Aligned with Mediatek’s view. |  |
| Samsung | Solution 3 or the variant of solution 3 | We share the problem so indeed it should be corrected. The cleanest solution could be the Solution 3 so we think it is better if all other solutions have NBC problem as well. |
| Huawei, HiSilicon | Solution 2-1 | This is aligned with our solution in R2-2108569. We think the first thing is to handle the value other-r16 in failureType-r15, as it results in a transfer syntax error according to 3GPP common rules. If the value other-r16 is to be disabled, removed or dummied, how the UE sets the failureType-r15 (when failureType-v1610 is included) needs discussions. In our paper R2-2108569, we think it can be left to UE implementation. We suggest that the UE should set an appropriate value, and then Rel-15 eNB can do the appropriate actions based on the value. We are also open for alternatives, e.g. define explicit mapping between some R16 failure types and R15 failure types. |
| Ericsson | Option 3 (proponent) | In general, we agree that whatever solution we go the change it may be NBC. However, with out Option 3 we want to keep the procedural text as semantically correct as possible. In this sense, we are basically trying to re-use the text that is already there. Of course the UE can randomly set a failure cause for the Rel-15 IE, or define a mapping between Rel-16 and Rel-15 failure causes but from a procedural point of view it will look quite strange. |
| NTTDOCOMO | Solution 2-1 or solution3 | - Either dummify other-r16 code-point or restrict UE shall not set other-r16 is fine. - The benefis of new failureTypeOther-r16 seems limited as existing failureType-v1610 already provides the information for r16 eNB, while agree it can mostly re-use the exisiting UE procedure - For solution2-1, a) is more preferable. |
| vivo | Solution 3 | Prefer to go with solution 3. |
| Nokia | None - See comments | Is this a problem observed in the field? We assume this can only be network misinterpretation issue. We would like note that the failureType-r15 is a mandatory field. If we were to remove the "other-r16", then what shall the UE fill in for the failure cause? It will have to provide something, and then the eNB not comprehending the extended cause will think this is a failure of other sort than it actually is, causing different kinds of problems as networks will interpret the reporting differently. So if we go this way, we make NBC change for both UEs (who would have to change th codepoint setting) AND networks (who will now have to cope with erroneous information, which is not easy to detect)! Hence, we really think the only way would be to add a note to the specification to clarify the correct handling. Networks can adapt to this and it will not cause incompatibility with any UEs in the field. Finally, we have disucssed network handling of UL spare values earlier, see e.g. R2-161903. The general understanding at that time was that network just ignores the content. That was also the discussion at the time when the Rel-16 decision to introduce the "other-r16" codepoint was done: The encoding doesn't change and all the codepoints that are unknown will trigger eNB to handle the message as "not comprehended" if received. Hence, network would most likely ignore the whole message (or at least the field value). Note that this whole discussion happened during Rel-16 LTE ASN.1 review, with the following agreements made at the time (see R2-2005752 for discussion details): Agreements (for LTE and NR) 8 For extension of failure types (which have mandatory R15 field) introduced in R16: - Introduce a value other/ unspecified within the legacy field; Use spares if defined and undefined code point otherwise - Include all new R16 values in an –v16xy extension - When signalling the –v16xy extension, the UE will set the legacy field to other/ unspecified |
| QCOM | Option-3 | Seems the proper way to rectify the issue in the spec. |
| Convida Wireless | Option 3 with modifications | The damage has already happened. Rel-15 and Rel-16 are not backwards compatible for this use case. There is surely an issue to fix. Upon reception of "other-r16", eNB implementation based on Rel-15 ASN.1 most likely throws a decoding failure which means that the decoding of the whole message fails. It is the behaviour that commercially available ASN.1 softwares produce for UPER decoders. It is enough to have one such implementation on the field. No comments on in-house solutions. It is true that the encoding of this field requires 3 bits in any case and there could potentially be 8 possible code points but only 6 were defined in Rel-15. Therefore these unspecified code points cannot be used as spare values (otherwise they would be called spare values). The handling of this case is different from that of spare values. If a spare value is received, the decoder decodes the message and then the receiver decides what to do with its fields. There is no such choice now because there are no spares and decoding fails. (Try it!) It is pointless to rename or dummify the value anymore because it would not change anything. It may even create another serious error. So, we propose to leave "other-r16" as is but add in the field description something like this "... in this version of the specification the value other-r16 shall not be used and the extension failureTypeOther should be used instead". It does not exclude or prohibit the possibility that by co-incidence both the UE and the eNB support the same Rel-16 ASN.1 version with the code point "other-r16". |

# 3	Conclusion

- To be updated after discussion on Phase 1 -