3GPP TSG-RAN WG2 Meeting #116-e	R2-2109481

Electronic meeting, November 1st – 12th, 2021

Agenda item:		8.11.2

Source:				CATT

Title:					Discussion on Enhancements for Latency Reduction

Document for:	Discussion and Agreement

1	Introduction

The objectives of WI NR_pos_enh include accuracy improvement in WID [1]:

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

The email discussion [Post115-e][605][POS] discussed and agreed that option A- based on a validity area (e.g. a list of cells) can be further discussed.

- [Post115-e][605][POS] Pre-configured assistance data (Intel)

Scope: Discuss signalling and validity criteria for pre-configured assistance data:

- Options for validity conditions:

    - Option A: Based on a validity area (e.g. a list of cells)

    - Option B: Based on a (configured) validity timer or a numerical limit on number of times it is utilized

    - Option C: Based on explicit modification or release from the LMF/NG-RAN

    - Option D: Based on the UE’s current location and/or the time

- Validity in relation to the duration of the positioning session

In this contribution, we discuss how to meet the validity condition for pre-configured assistance data based on a specific area in detail. And resubmit the potential solution to reduce the latency caused by handover.

2	Discussion

2.1 Validity criteria based on a validity area

Based on the provided inputs in Question 3-1, the rapporteur of [Post115-e][605][POS] observes that there is a majority consensus on the need for defining validity condition for pre-configured assistance based on a specified area, but further discussion is needed regarding the specification impact and whether new signaling needs to be defined to support it.

2.1.1 Cadidate solution

The reason why companies support validity area of pre-configured assistance data, e.g. DL-PRS is that DL-PRS are based on TRPs where the DL-PRS is send. The DL-PRS is something like SSB which is irrealteive with time validatity, but relative where it is sent.

The natural way to set the validity area is a list of cells for each DL-PRS: Option A: Based on a validity area (e.g. a list of cells).

The other way to set the validity area is the indication of area ID: Option B: area ID.

Figure 2-1 TRPs within areaID

Taking 256 TRPs of DL-PRS as an example, the signalling analysis on two options is below:

- Option A: each TRP carries a list of cells which are 16 cells in maximum with PhysCellID in 10 bits

- Option B: each TRP carries area ID which indicate what the area the TRP belongs.

| Candidate options | Signalling resources |
| --- | --- |
| Option A: Cell list for each TRP | 256(TRP)*16 (cells)*10(PhysCellID) = 40Mbit |
| Option B: Area ID for each TRP | 256 TRP * 24(area ID) = 6Mbit |

85% signalling with Option B (area ID) can be saved compared with option A (cell list).

Observation 1: Area ID can save 85% signalling resources compared with cell list for pre-configured assistance based on a specified area.

Proposal 1: RAN2 to agree to area ID within each TRP for pre-configured assistance based on a specified area.

2.1.2 Stage-3 impact

What the areaID?

The areaID can be systemInformationAreaID or RAN area ID which is broadcast in SI. UE can compared the systemInformationAreaID or RAN area ID which its serving cell belongs with the areaID in the pre-configured assistance. If the areaID in the pre-configured assistance is identical to the systemInformationAreaID or RAN area ID which is associated with the serving cell, it is considerd the associated pre-configured assistance as valid.

Proposal 2: RAN2 to discuss the candidate areaID: systemInformationAreaID（SIAID）or RAN area ID.

2.2 Latency reduction during handover

2.1.1 The challenges due to handover in NR

RRC reconfiguration to perform reconfiguration with sync includes re-establishment of RLC and PDCP [3].According to the existing PDCP entity re-establishment mechanism in clause 5.1.2 in TS 38.323[2]:

Observation 2: When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs [2];

Observation3: When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

- for SRBs, discard all stored PDCP SDUs and PDCP PDUs [2];

Thus, all RRC messages during handover will be discarded. As we know, LPP message are carried in ULInformationTransfer on SRB1 or SRB2, so all LPP message carried in ULInformationTransfer will be discarded during the handover.

The handover in NR happens more frequently than in 4G network assuming the same coverage KPIs because the cell of NR is smaller than 4G. If the LPP message (LPP LPI) from UE to LMF is lost because SRB is discarded during the handover, the challenge of losing LPP message during handover will be faced a lot in NR network.

Observation 4: All LPP message carried in ULInformationTransfer are discarded during the handover because of the exisiting mechanism of PDCP entity re-establishment.

The existing mechanism is shown in the annex for your reference.

2.2 The challenges of lost LPP during handover

According to the existing procedure of RAT-Dependent and RAT-Indepent positioning methods, what will happen if the LPP message is lost?

- When UE plans to send LPP Request Assistance Data for periodic assistance data but it is lost during the handover:

LMF won’t take any actions since it doesn’t receive LPP Request. AS will inform upper layers about the possible failure to deliver the information contained in the concerned ULInformationTransfer messages. This also introduces latency. UE won’t take actions until upper layer sends the request again or LCS fails finally which is up to implementation in UE.

- When UE plans to send LPP Provide Location Information but it is lost during the handover:

LMF won’t take any actions since it doesn’t receive LPP PLI, and UE won’t take actions as well until this LCS failes.

It seems there is a challenge of latency if the LPP messages from UE to LMF are lost during the handover. So do the messages from LMF to UE.

Observation 5: This is a challeng of latency if the LPP messages from UE to LMF or from LMF to UE are lost during the handover, not only for RAT-Independent but also for DL RAT-Dependent positioning methods.

Latency will be reduced if we take some actions to avoid this kind of lost during handover, especially when handover happens a lot in NR network. If there is a mechanism to make sure LPP message won't be lost during handover, then the latency which is caused by message lost during the handover will be reduced.

Proposal 3: Latency can be reduced if there is a mechanism to make sure LPP messages won’t be lost during the handover.

2.4 How to reduce the latency during handover

There is a candidate solution for UE to perform some actions upon reception of the RRCReconfiguration to face the challenge above. The actions on ulInformationTransfer message which carries LPP can learn from the existing mechnism on ueAssistanceInformation message, considering they are both in DCCH.

UL-DCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

measurementReport               MeasurementReport,

rrcReconfigurationComplete      RRCReconfigurationComplete,

rrcSetupComplete                RRCSetupComplete,

rrcReestablishmentComplete      RRCReestablishmentComplete,

rrcResumeComplete               RRCResumeComplete,

securityModeComplete            SecurityModeComplete,

securityModeFailure             SecurityModeFailure,

ulInformationTransfer           ULInformationTransfer,

locationMeasurementIndication   LocationMeasurementIndication,

ueCapabilityInformation         UECapabilityInformation,

counterCheckResponse            CounterCheckResponse,

ueAssistanceInformation         UEAssistanceInformation,

failureInformation              FailureInformation,

ulInformationTransferMRDC       ULInformationTransferMRDC,

scgFailureInformation           SCGFailureInformation,

scgFailureInformationEUTRA      SCGFailureInformationEUTRA

},

How the existing mechanism to solve the UEAssistanceInformation message lost during handover?

Here is the existing procedure on UEAssistanceInformation message. UEAssistanceInformation message which are sent during last 1 second will be retransmitted in the target cell. So the same mechanism may happen to ulInformationTransfer message with little specification update.

Below please find the RRC protocols on UEAssistanceInformation message for your reference in TS 38.331[3].

5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup; and

2>	if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group:

3>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

3>	start or restart the prohibit timer (if exists) associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration;

UE retransmists UEAssistanceInformation message for the corresponding cell group during the last 1 second to avoid the message lost during handover.

Proposal 4: UE can follow the existing mechanism to retransmisit ulInformationTransfer message for the corresponding target cell during the handover to reduce the latency so the LPP messages won’t be sent/receive correctly.

3	Conclusion

Based on the discussions in section 2, the following observations and proposals have been made:

Validity criteria based on a validity area aspect:

Observation 1: Area ID can save 85% signalling resources compared with cell list for pre-configured assistance based on a specified area.

Proposal 1: RAN2 to agree to area ID within each TRP for pre-configured assistance based on a specified area.

Proposal 2: RAN2 to discuss the candidate areaID: systemInformationAreaID（SIAID）or RAN area ID.

Latency reduction during handover aspect (resubmition):

Observation 2: When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs [2];

Observation 3: When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

- for SRBs, discard all stored PDCP SDUs and PDCP PDUs [2];

Observation 4: All LPP message carried in ULInformationTransfer are discarded during the handover because of the exisiting mechanism of PDCP entity re-establishment.

Observation 5: This is a challeng of latency if the LPP messages from UE to LMF or from LMF to UE are lost during the handover, not only for RAT-Independent but also for DL RAT-Dependent positioning methods.

Proposal 3: Latency can be reduced if there is a mechanism to make sure LPP messages won’t be lost during the handover.

Proposal 4: UE can follow the existing mechanism to retransmisit ulInformationTransfer message for the corresponding target cell during the handover to reduce the latency so the LPP messages won’t be sent/receive correctly.

4	References

- RP-210903 Revised WID on NR Positioning Enhancements Intel Corporation, CATT

- TS 38.323 Packet Data Convergence Protocol (PDCP) specification V16.2.0

- TS 38.331 Radio Resource Control (RRC) protocol specification V16.4.1

5	Annex

LPP message which is NAS message is carried in ULInformationTransfer by SRB.

--------------------------------Start of TS 38.331----------------------------------------------------------------------------------------

–	ULInformationTransfer

The ULInformationTransfer message is used for the uplink transfer of NAS or non-3GPP dedicated information.

Signalling radio bearer: SRB2 or SRB1 (only if SRB2 not established yet). If SRB2 is suspended, the UE does not send this message until SRB2 is resumed

RLC-SAP: AM

Logical channel: DCCH

--------------------------------Next of TS 38.331---------------------------------------------------------------------------

Figure 5.3.5.1-1: RRC reconfiguration, successful

RRC reconfiguration to perform reconfiguration with sync includes, but is not limited to, the following cases:

-	reconfiguration with sync and security key refresh, involving RA to the PCell/PSCell, MAC reset, refresh of security and re-establishment of RLC and PDCP triggered by explicit L2 indicators;

-	reconfiguration with sync but without security key refresh, involving RA to the PCell/PSCell, MAC reset and RLC re-establishment and PDCP data recovery (for AM DRB) triggered by explicit L2 indicators.

-	reconfiguration with sync for DAPS and security key refresh, involving RA to the target PCell, establishment of target MAC, and

-	for non-DAPS bearer: refresh of security and re-establishment of RLC and PDCP triggered by explicit L2 indicators;

-	for DAPS bearer: establishment of RLC for the target PCell, refresh of security and reconfiguration of PDCP to add the ciphering function, the integrity protection function and ROHC function of the target PCell;

-	for SRB: refresh of security and establishment of RLC and PDCP for the target PCell;

-	reconfiguration with sync for DAPS but without security key refresh, involving RA to the target PCell, establishment of target MAC, and:

-	for non-DAPS bearer: RLC re-establishment and PDCP data recovery (for AM DRB) triggered by explicit L2 indicators.

-	for DAPS bearer: establishment of RLC for target PCell, reconfiguration of PDCP to add the ciphering function, the integrity protection function and ROHC function of the target PCell;

-	for SRB: establishment of RLC and PDCP for the target PCell.

--------------------------------End of TS 38.331------------------------------------------------------------------------------------------

--------------------------------Start of TS 38.323--------------------------------------------------------------------------------------

5.1.2	PDCP entity re-establishment

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the ROHC protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the EHC protocol for uplink if drb-ContinueEHC-UL is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the clause 5.2.2.1;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

----------------- End of TS 38.323---------------------------------------------------------------------------------------------------------