3GPP TSG-RAN WG2 #116-e	R2-21xxxxx

E-meeting, 01 – 12 November 2021

Agenda item:	8.4.2

Source: 	Samsung (rapporteur)

Title: 	Summary of discussion [AT116-e][031][eIAB] MAC - LCG extension and BSR (Samsung)

Document for:	Discussion and decision

# Introduction

This document captures the following discussion:

- [AT116-e][031][eIAB] MAC: LCG extension and BSR (Samsung)

Scope: Progress MAC: LCG extension and BSR (preemtive) based on contributions to this meeting. Identify agreements, discussion points, can also capture open issues. Attempt to close open issues.

Intended outcome: Report

Deadline: Tuesday W2 (online CB)

Section 2 captures individual companies’ responses to questions drafted by the discussion rapporteur, which are in turn based on individual companies’ submissions to this meeting, captured in the References section, as well as rapporteur’s initial draft of the running MAC CR (R2-2110453), submitted to this meeting for discussion. Section 3 then lists proposals from the discussion rapporteur (based on input captured in Section 2) and companies’ views on these proposals. Section 4 (Conclusions) then captures the next iteration of the proposals, to be treated in the online session in Week 2 of the ongoing meeting.

# Collecting input on key outstanding issues

## Configuration and use of Extended BSR formats

A number of contributions [3], [4], [6], [7], [8], [9], [12] treat some aspects of this topic.

The first issue is whether the use of both formats should be allowed for IAB-MTs. While seemingly obvious, the rapporteur proposes to first confirm the following:

Q1. Should IAB-MTs support both legacy and Extended BSR? (NB: Extended BSR refers to the new formats designed to accommodate the LCG space extension. Configurability aspects and the actual choice between the two types of formats are covered in questions that follow.)

| Company | Yes/No | Additional comments (if any) |
| --- | --- | --- |

The second issue (not perhaps essential at this stage of the discussion, but worth covering in rapporteur’s view) is whether the support for extended BSR by IAB-MTs is mandatory, or optional.

Q2. Is the support of Extended BSR by IAB-MTs mandatory, or optional IAB-MT capability?

| Company | Mandatory/optional | Additional comments (if any) |
| --- | --- | --- |

Assuming an IAB-MT needs to support both legacy and Extended BSR, the next issue is which one is sent when both are configured/possible. The IAB-MT could have some autonomy in choosing the format when both are configured/allowed (this is especially useful for padding as it allows sending ‘old’ BSR formats when there would be no room for Extended BSR formats). And finally, if use of both formats are supported, the question is whether and how to maintain two different mappings of LCHs to LCGs (in one case, limited to 8 LCGs, in the other case, limited to 256 LCGs). The questions that follow cover these aspects.

Q3. When both legacy and Extended BSR formats are supported by IAB-MT, which option for you prefer:

- Option 1: Only one type of BSR format (either legacy or Extended) may be configured for an IAB-MT, and this is the format that an IAB-MT shall use (i.e. this is fully under network control);

- Option 2: Both types of BSR format (legacy and/or Extended) may be simultaneously configured for an IAB-MT. (NB: which one is used in specific scenarios is covered by questions that follow.)

| Company | Option 1/ Option 2 | Additional comments (if any) |
| --- | --- | --- |

Assuming Option 2 above is agreeable, the following question then deals with the choice between the two types of BSR format:

Q4. Assuming both types of BSR format (legacy and/or Extended) may be configured simultaneously for an IAB-MT, which is your preferred option of managing this issue (choosing between the two types of formats):

- Option 1: The choice between the two is fully left to IAB-MT implementation. (NB: The network knows which type of format is sent based on LCID/eLCID value used by the IAB-MT; the choice of specific format e.g. Long or Short, Extended Long or Extended Short, is still governed by the specs.)

- Option 2: This needs to be captured in our specs.

| Company | Option 1/ Option 2 | Additional comments (if any) |
| --- | --- | --- |

Assuming normative work is needed (Option 2 in Q4), please choose between the following options:

Q5. Assuming both types of BSR format (legacy and/or Extended) may be configured simultaneously for an IAB-MT, and the choice between the two types is NOT left fully to IAB-MT implementation, which is your preferred option for a normative solution:

- Option 1: When the Extended BSR format is configured, Extended BSR is always reported (regardless of the fact that in some case legacy BSR format could also be used);

- Option 1a: When the Extended BSR format is configured, Extended BSR is always reported except for the case of padding BSR, where the IAB-MT may send the legacy BSR due to padding size limitations;

- Option 2: If there are no more than 8 LCGs configured, legacy format is always sent; otherwise the Extended format is sent;

- Option 2a: If there are no more than 8 LCGs configured, legacy or Extended formats is sent (choice left to IAB-MT implementation); otherwise the Extended format is sent;

- Option 3: If the maximum configured LCGID is 7 or lower, legacy format is always sent; otherwise the Extended format is sent;

- Option 3a: If the maximum configured LCGID is 7 or lower, legacy or Extended formats is sent (choice left to IAB-MT implementation); otherwise the Extended format is sent;

- Option 4: Two groupings (one for the case of 8 LCGs, the other for the case of 256 LCGs) of logical channels into LCGs should be configured by the network, and it is then left to IAB-MT implementation which type of format is used (but with appropriate NW-configured grouping applied).

| Company | Option 1/ 1a/ 2 / 2a / 3 / 3a / 4 | Additional comments (if any) |
| --- | --- | --- |

## Extended BSR format for Long/Long Truncated BSR

Five options are being floated for Extended Long BSR format:

(a) Option 1								(b) Option 2			 						(c) Option 3

(d) Option 4

(e) Option 5

The summary of the resulting 7 options is given immediately below:

- Option 1: (8-bit LCG ID+8-bit Buffer Size for this LCG ID), (8-bit LCG ID+8-bit Buffer Size for this LCGID), (8-bit LCG ID+8-bit Buffer Size for this LCGID), …; buffer size field may be included only for the LCG(s) having data available for transmission.

- Option 2. Fixed size of 256 LCGi followed by variable number of (fixed size) Buffer Size fields; related buffer size field is added only when the corresponding LCG bit is set to 1 in the bitmap.

- Option 2a. As per figure (b) but – octet 1 is included only if there is at least one LCG configured, octet 2 is included only if at least one LCG above LCG7 is configured, octet 3 is included only if at least one LCG above LCG15 is configured, and so on, until octet 32 that is included only if at least one LCG above LCG247 is configured.

- Option 3. Variable size of the LCGi bit-map and variable number of the Buffer Size(s) fields; the fixed 4 bytes of bitmap can be defined to indicate whether one octet of LCGs is included or not.

- Option 4. We only include those LCGIDs which have been configured to be associated with LCHs in the Long (Truncated) BSR MAC CE.

- Option 5. LCGs are divided into 2 or more LCG sets and each new long (truncated) BSR MAC CE reports the buffer sizes for one LCG set.

- Option 6. The exact format is determined based on number LCGs that have data available.

Option 2 has a majority support [2], [8], [9], [11], [13]. Submission [4] appears to support both Option 2 and Option 1. Option 1 has two additional supporters [1], [5].

The remaining Options 2a/3/4/5/6 have one supporter each ([10], [12], [7], [3] and [6] respectively).

Q6. Please choose from the above 7 options for Extended Long BSR:

| Company | Option 1/ 2 / 2a/ 3 / 4 / 5 / 6 | Additional comments (if any) |
| --- | --- | --- |

The vast majority appear to be in favour of adopting an extended format for Long Truncated BSR as well – the following question aims to confirm this:

Q7. Do you agree that an Extended Long Truncated BSR should also be designed?

| Company | Yes / No | Additional comments (if any) |
| --- | --- | --- |

Q8. Assuming it is agreed that an Extended Long Truncated BSR should also be designed, do you agree that the Extended Long Truncated BSR format should be identical to the Extended Long BSR format?

| Company | Yes / No | Additional comments (if any) |
| --- | --- | --- |

## Identifying Extended BSR MAC CEs

As a reminder, the values of the two-octet eLCID space are dedicated uniquely to the identity of the logical channels on the backhaul. Therefore, in order to identify the new Extended BSR formats, we can either use the LCID space or the one-octet eLCID space. The latter seems to be the majority choice, but the former has some limited support.

Q9. Should values of LCID or one-octet eLCID space be used to identify the new, Extended BSR formats?

| Company | LCID space / one-octet eLCID space | Additional comments (if any) |
| --- | --- | --- |

## New triggers for BSR/Extended BSR

In [8] it is argued that if we want to ensure fairness – meaning that lower priority overall of a LCH/LCG could still mean highest priority LCH for an individual UE, we should be able to allocate resources with per-UE granularity, or even groups of UEs. Conventional BSR triggering would not satisfy this requirement. It is argued in [8] that – in the case where new data arrives in a LCG and there is already data in a LCG with higher priority – that a BSR should nevertheless be triggered if this data is of the highest priority for the relevant UE.

Q10. Should we introduce new triggering conditions for BSR/Extended BSR, to address above issues and/or any other issues?

| Company | Yes / No | Additional comments (if any) |
| --- | --- | --- |

Q11. Assuming an answer ‘yes’ to Q10, do you support one or more of: BSR triggering based on LCH data source and/or destination; BSR triggering for designated LCH/LCGs; any other new or adapted triggering rule?

| Company | Yes / No | Additional comments (if any) |
| --- | --- | --- |

## Pre-emptive BSR aspects

Legacy pre-emptive BSR uses the same format as legacy Long (Truncated) BSR and it makes sense to adopt the same approach for the Extended pre-emptive BSR format – the following two questions cover this issue:

Q12. Should the extended LCG space (max 256 LCGs) apply to pre-emptive BSR as well?

| Company | Yes / No | Additional comments (if any) |
| --- | --- | --- |

Q13. Assuming an answer ‘yes’ to Q10, do you agree that the Extended pre-emptive BSR format should be identical to the Extended Long BSR format?

| Company | Yes / No | Additional comments (if any) |
| --- | --- | --- |

And finally, RAN2 has in the past discussed standardizing buffer size calculations for pre-emptive BSR and triggering conditions for pre-emptive BSR. In Rel-16, this is left to implementation. This is an opportunity to revisit this for Rel-17:

Q14. Do you agree that RAN2 should standardize buffer size calculation for Rel-17 pre-emptive BSR?

| Company | Yes / No | Additional comments (if any) |
| --- | --- | --- |

Q15. Do you agree that RAN2 should standardize triggering conditions for Rel-17 pre-emptive BSR?

| Company | Yes / No | Additional comments (if any) |
| --- | --- | --- |

## Any other issues

Q16. Are there any issues not captured above which you think should be covered as part of this discussion? If so, please provide details.

| Company | Yes / No | Additional comments (if any) |
| --- | --- | --- |

# Initial set of proposals for RAN2’s consideration

…

# Conclusions

…

# References

- R2-2109582	Long BSR Format with Extended LCG	CATT

- R2-2109611	Discussion on remaining open issues of LCG range extension and congestion	Intel Corporation

- R2-2109748	BSR format and reporting in IAB	Fujitsu

- R2-2109782	LCG extension for eIAB	Huawei, HiSilicon

- R2-2109854	Discussion on LCG extension and UL hop-by-hop flow control feedback	ZTE, Sanechips

- R2-2110290	Discussion on LCG extension issues	vivo

- R2-2110306	Discussion on LCG extension and congestion migitation for IAB	Lenovo, Motorola Mobility

- R2-2110422	Finalizing enhancements to LCG space and BSR triggering including pre-emptive BSR	Samsung Electronics GmbH

- R2-2110807	BSR formats for LCG extension	Nokia, Nokia Shanghai Bell

- R2-2110883	On Topology-wide Fairness, Multi-hop Latency, and Congestion in IAB Network	Ericsson

- R2-2110897	On BSR formats for IAB	InterDigital

- R2-2111155	Stage-3 details of LCG extension	LG Electronics Inc.

- R2-2111174	Remaining issues -Fairness, latency, congestion	Rakuten Mobile, Inc