3GPP TSG-RAN WG2 #116bis-e								R2-2200365

E-meeting, January 2022

Agenda Item:	8.7.1

Source:	OPPO

Title:	Remaining Open issue list of R17 Sidelink Relay WI

Document for:	Discussion, Decision

Introduction

This is for the remaining open issues of R17 Sidelink Relay WI in RAN2.

Discussion

  - WID Scope

In RP-212819, the scope of the work item is described as follows:

The objective of this work item is to specify solutions to enable single-hop, sidelink-based, L2 and L3 based UE-to-Network (U2N) relaying.

Work Item objectives on aspects common to both L2 and L3:

- Specify mechanisms for U2N relay discovery and (re)selection for L3 and L2 relaying [RAN2, RAN4]

  - Re-use LTE relay discovery and (re)selection as baseline

- Specify mechanisms for Relay and Remote UE authorization for L3 and L2 relaying [RAN3]

  - Re-use LTE as baseline

Work Item objectives specific to Layer-2 (L2) relaying:

- Specify mechanisms for E2E, i.e. PC5 and Uu, QoS management [RAN2]:

- Specify mechanisms for service continuity

  - Limited to intra-gNB cases [RAN2]

- Specify mechanisms for U2N Adaptation layer design [RAN2]

  - For bearer mapping and Remote UE identification, incl. RAN related security aspects if any

- Specify Control Plane procedures for U2N, including RRC connection management, system information delivery, paging mechanism and access control for Remote UE [RAN2, RAN3]

Secondly, the objective of this work item also covers the non-relay discovery (i.e. 5G ProSe Direct Discovery).

- Specify mechanisms for 5G ProSe Direct Discovery [RAN2, RAN3, RAN4];

  - Open Issue list

Objective-1/7: Relay discovery and (re)selection, Non-relay discovery

NOTE: The issues below may be applicable to non-relay discovery (O7) as well.

| Issue Index | Description |
| --- | --- |
| O1.01 | FFS if network can also configure a setting where both shared and dedicated pools can be used for SL discovery.. |
| O1.02 | LCP impact due to dedicated pool for discovery traffic. |
| O1.03 | Whether any impact to SUI message report due to the discovery and relay. |
| O1.04 | Details on the new PC5-RRC signaling triggered by handover, Uu-RLF and cell (re)selection of relay UE |
| Q1.05 | How to differentiate a gNB that is relay-capable/relay-incapable and discovery-capable/discovery-incapable |

Objective-3: QoS

No major left issues.

Objective-4: Service Continuity

| Issue Index | Description |
| --- | --- |
| O4.01 | Support of RRC_IDLE / RRC_INACTIVE Relay UE during direct-to-indirect switching |
| O4.02 | Whether legacy PDCP behaviour can be reused for remote UE |
| O4.03 | Stopping condition of T304-like new timer for direct-to-indirect switching |
| O4.04 | Left issue on measure configuration and reporting (e.g., which ID to report for serving cell of relay UE (NCGI/NCI/PCI), allow/black-list configuration) |

Objective-5: Adaptation Layer

| Issue Index | Description |
| --- | --- |
| O5.01 | Data PDU format for adaptation layer over Uu hop and PC5 hop. |
| O5.02 | Further RRC configuration details, e.g., the adaptation layer field configuration for remote UE, dependent on the field for PC5 hop |
| O5.03 | Whether control PDU for adaptation layer is needed, and if yes, what is the format. |

Objective-6: CP Procedure

| Issue Index | Description |
| --- | --- |
| O6.01 | Uu RLC configuration for SRB0/1 message |
| O6.02 | Detailed stage-3 signaling format on paging acquisition message from remote UE to relay UE, a |
| O6.03 | Cause value setting for relay UE access due to remote UE traffic |
| O6.04 | Whether/how to support minimum/essential SI |
| O6.05 | Handling of new T30x-like timers that used by SL-relay scenario |

Reference

- RP-212819, revised WID on SL Relay, OPPO, CMCC