3GPP TSG RAN WG2 Meeting #116bis-e                                                    R2-2200502

# Electronic meeting, 17th-25th January 2022

Agenda item:	8.6.1

Source:	Intel Corporation

Title:	UE capabilities for Rel-17 SDT WI

Document for:	Discussion and decision

# Introduction

This document discusses the new UE radio capabilities required/desirable in relation to Rel-17 SDT WI [1].

# Discussion

Rel-17 SDT WI  [1] is defining the following mechanisms for a UE in RRC_INACTIVE state:

- UL small data transmissions for RACH-based schemes, i.e. RA-SDT.

- Transmission of UL data on pre-configured PUSCH resources, i.e. CG-SDT.

A single capability could be sufficient for UE to indicate the support of SDT feature. However, we understand that it might be preferable for a UE to indicate the support of each one separately with the clarification that to support Rel-17 SDT, UE has to support RA-SDT (i.e. a UE supporting CG-SDT has to always support RA-SDT).

- Two new UE capabilities are defined as ‘optional with capability signalling’ for UE to indicate its support of RA-SDT and CG-SDT.

- To support Rel-17 SDT mechanism, UE shall always support RA-SDT, i.e. a UE supporting CG-SDT shall also support RA-SDT.

In addition, it should be discussed whether any feature previous to Rel-17 needs additional/separate granularity in relation to UE’s capabilities when are combined/used with Rel-17 SDT mechanism. For example:

- For 2-step RACH, i.e. whether (a) we rely on legacy support of 2-step RACH (twoStepRACH-r16) or (b) new capabilities specific to 2-step RA-SDT is preferable (in which case we would define one for 2-step RA-SDT and another for 4-step RA-SDT).

- For SUL support for RACH, i.e. whether (a) we rely on legacy support of SUL (msgA-SUL-r16) or (b) new capabilities specific to SUL for SDT is preferable. If (b) is preferable, we would need to discuss whether further differentiation is needed for SUL operation with RA-SDT and with CG-SDT.

It is not clear whether different UE capacities might be needed for previous optional Rel-17 features that can operate with Rel-17 SDT, for example for 2-step RACH operation, or for SUL operation.

- To discuss whether any pre-Rel-17 features (e.g. 2-step RACH or SUL) requires additional/separate UE capabilities when used in combination to Rel-17 SDT mechanism.

It should also be discussed whether these new capabilities for SDT need to be different for FDD/TDD and FR1/FR2. In our view, there is no need to have this level of differentiation based on current functionality.

- The new capabilities for SDT feature (as defined in proposals 1 and 2) are defined per UE and do not need xDD and FRx differentiation.

# Conclusion

The observations captured are the following:

Observation 1.	It is not clear whether different UE capacities might be needed for previous optional Rel-17 features that can operate with Rel-17 SDT, for example for 2-step RACH operation, or for SUL operation.

The proposals captured are the following:

Proposal 1.	Two new UE capabilities are defined as ‘optional with capability signalling’ for UE to indicate its support of RA-SDT and CG-SDT.

Proposal 2.	To support Rel-17 SDT mechanism, UE shall always support RA-SDT, i.e. a UE supporting CG-SDT shall also support RA-SDT.

Proposal 3.	To discuss whether any pre-Rel-17 features (e.g. 2-step RACH or SUL) requires additional/separate UE capabilities when used in combination to Rel-17 SDT mechanism.

Proposal 4.	The new capabilities for SDT feature (as defined in proposals 1 and 2) are defined per UE and do not need xDD and FRx differentiation.

# Reference

- RP-212594, Work Item on NR small data transmissions in INACTIVE state, Rel-17, ZTE Corporation.