3GPP TSG RAN WG2 Meeting #116bis-e                                                         R2-2200505

# Electronic meeting, 17th-25th January 2022

Agenda item:		8.6.3

Source:			Intel Corporation

Title:		Control Plane leftover issues on SDT procedure

Document for:	 	Discussion and decision

# Introduction

This document is a revision of [1] considering the latest agreements and progressed. It includes delta signaling for SDT configuration, SDT configuration signaling, UE behaviour during SDT, RNAU during SDT, SDT failure detection timer and RRC signaling allowed.  It also addresses open points raised during the email discussions [Post116-e][506][SDT] RRC running CR update [2], [Post116-e][507][SDT] MAC running CR update [3], [Post116-e][510][SDT] CCCH and DCCH [4] and [Post116-e][509][SDT] CG open issues [5].

# Discussion

## Conditions to initiate SDT

The following open points are raised during the review of the stage-3 running CR:

During email discussion [Post116-e][506][SDT] RRC running CR update [2], Rapporteur included the following question in relation to one of the comments “whether the data volume threshold is UE specific or cell specific”.    The data threshold was defined for the network to restrict UEs of using this feature when large amount of data is available at the initiation of the SDT session. From RRC perspective, we see the question a bit differently – as whether it is broadcast in SIB or provided over dedicated signalling. Normally a configuration is provided in SIB if it is required for initial access.  In this particular context, the question then becomes whether this varies from cell to cell or is common across the cells (at least of an RNA).  If it is common across cells, this can be part of the SDT dedicated configuration.  Otherwise, it has to be provided in SIB.   To cater for different cell types, it would be useful to provide more flexibility to be able to configure it differently for each cell. Therefore, it would be helpful to configure it via SIB signaling.

- The sdt-RSRP-Threshold is provided in Sdt-ConfigCommonSIB IE via SIB signaling as it can take different value for different cells.

It was captured “FFS when sdt-RSRP-Threshold is not configured”. In our understanding this discussion is different depending on whether this parameter were provided via SIB or dedicated signaling. If sdt-RSRP-Threshold is agreed to be sent via SIB msg (as proposed in previous Proposal 1) but it is absent (i.e. not configured) and the usage of SDT feature is allowed in that given cell, our understanding is that the UE should be allowed to continue with the SDT initiation phase independent of its RSRP value. However, if sdt-RSRP-Threshold is agreed to be sent via dedicated signalling, this parameter should always be configured if SDT feature is also configured (to avoid the use of Need S in dedicated signalling).

- If Proposal 1 is agreed (sdt-RSRP-Threshold is provided via SIB signalling), when sdt-RSRP-Threshold is absent (i.e. not configured) and the usage of SDT feature is allowed in a given cell, UE is allowed to continue with the initiation phase of the SDT session (independent of its RSRP).

- If Proposal 1 is not agreed (sdt-RSRP-Threshold is provided via dedicated signalling), network always configure sdt-RSRP-Threshold in UE when SDT feature is configured for that UE.

It was also captured “FFS the procedure when sdt-RSRP-ThresholdSSB-SUL is not configured”. During RAN2#113-bis-e, the related agreement said “RSRP threshold for carrier selection is specific to SDT (i.e. separately configured for SDT).  This is optional for the network”. Therefore, when the SDT specific RSRP threshold for carrier selection is not configured (i.e. absent), our understanding is that UE re-uses the value of legacy RSRP-ThresholdSSB-SUL.

- When sdt-RSRP-ThresholdSSB-SUL is absent (i.e. not configured), UE reuses the value of legacy RSRP-ThresholdSSB-SUL for the SDT procedure (when having to do the carrier selection check).

“FFS whether the RSRP threshold for UL carrier selection is common for both CG and RA-SDT”. During RAN2#113-bis-e, RAN2 agreed in an inter-related topic that “RSRP threshold to select between SDT and non-SDT procedure is used for both CG-SDT and RA-SDT” and “RSRP threshold to select between SDT and non-SDT procedure is same for both CG-SDT and RA-SDT”. Similarly, there is no need to have different RSRP threshold for UL carrier selection for CG-SDT and RA-SDT.

- The same value of the RSRP threshold for UL carrier selection is applicable for both CG and RA-SDT (i.e. same value is used for both mechanisms).

## SDT related configuration

The following agreements might be relevant to this discussion on the required signaling for SDT related configuration:

- “Non-SDT radio bearers are only resumed upon receiving RRCResume (same as today)

- Whether to support ROHC continuity is explicitly configured by the network

- PDCP duplication is not supported for SDT

- Connected mode DRX is not supported for SDT

- The UE performs PDCP re-establishment implicitly, i.e. without explicit indication for PDCP re-establishment, when the UE initiates SDT procedure

### Delta signalling for SDT configuration

SDT from UEs will be frequent small data transmissions at short intervals.   As RAN2 has agreed to use RRC signalling based approach as the baseline, each SDT session must involve an RRCResumeRequest and RRCRelease message.  There could be a fair amount of configuration associated with SDT such as CG, I-RNTI, short-I-RNTI and signalling associated with normal Resume.  If these have to be signalled at every SDT session, they can be a significant relative to the volume of actual data sent in the SDT session.

Configuration associated with legacy Resume is also provided by RRCRelease message.  The normal behaviour in the absence of delta signalling will be that both SDT and legacy Resume will need to be provided at the end of each SDT session if RRC Release message is reused for SDT.

Since these configurations are likely to be the same over several SDT sessions, delta signalling of both the normal Resume and SDT configuration could be very beneficial.  Considering that the UE may go through a sequence of SDT and full resume, this delta signalling of the SDT configuration can continue across all of these sessions.  That is the SDT configuration can be retained as long as the INACTIVE configuration is also retained and form the baseline for subsequent delta configuration.

Delta signalling of legacy Suspend and SDT configuration can reduce signalling overhead compared to volume of data

- Delta signaling is supported for the SDT related configuration. This delta signaling applies across different SDT sessions and when resuming the RRC connection (i.e. SDT related configuration is released when UE enters RRC_IDLE or when the network explicitly releases the SDT configuration).

### General configuration applicable to SDT operation

When the UE was moved to INACTIVE, it stored the previous configuration.  During the SDT transfer, some of that configuration, such as the configuration related to the radio bearers, is restored and used as the configuration for the SDT data transfer as agreed in the last meeting.  The stored configuration may also include many other configurations such as UE connected mode measurements.  To minimize additional complexity, by default, none of the other UE configuration should be restored during the SDT session and UE shall not perform any additional procedures beyond what is essential for receiving and transmitting SDT data.

- During the SDT session, UE only restores the configurations for SDT, suspend and radio bearers (i.e. other UE stored configurations are not restored during the SDT session).

Legacy resume mechanism explicitly acts to certain configurations and timers that might not be relevant for SDT operation. For simplicity, previous proposals suggest not to enable any related actions on UE side during SDT session (regardless on whether it is for release restore or even update). Therefore, UE’s stored configurations in UE Inactive AS context that may not be relevant for SDT operation may include at least:

- delayBudgetReportingConfig,

- overheatingAssistanceConfig,

- idc-AssistanceConfig,

- drx-PreferenceConfig for all configured cell groups,

- mrdc-SecondaryCellGroup

- Timers T346a, T346b, T346c, T346d, T346e, T346f

- During the SDT session, UE does not act (e.g. release, restore or update) at least any of the following configurations which are stored in UE Inactive AS context (but are not relevant to SDT operation): a) delayBudgetReportingConfig, b) overheatingAssistanceConfig, c) idc-AssistanceConfig, d) drx-PreferenceConfig for all configured cell groups, e) mrdc-SecondaryCellGroup, and f)  Timers T346a, T346b, T346c, T346d, T346e, T346f.

### Update of SDT specific configuration while in RRC_INACTIVE

RAN1 LS [R1-2102125] indicated the following on the CORESET and search space for monitoring the PDCCH addressed to the C-RNTI after successful completion of the RACH procedure during RA-SDT:

“From RAN1 perspective, the existing type-1 PDCCH CSS used for random access can be reused but it may impose higher PDCCH blocking rate and impact to the legacy UE performing random access, so at least a new search space that is different from the existing CSS should be supported for RA-SDT. It is feasible from RAN1 perspective to use either a new common search space or a UE-specific search space, thus it can be up to RAN2 to make the decision. The configuration of CORESET will be further discussed, basically following the same design logic for search space.”

Possible ways for the network to update SDT related configuration are:

- Broadcast signalling: SDT related configuration can be updated outside the SDT session (before/after), while UE is INACTIVE and when UE reselects to a new cell.

- Dedicated signalling: SDT related configuration is updated at the beginning of an SDT session. This can be done via dedicated signaling which can provide more flexibility and minimize the impact on the legacy system (e.g., reduction of the blocking probability compared to common search space/CORESET). However, it can add signaling overhead to each SDT session where the main motivation was to reduce any unnecessary signaling. This would allow enabling the “UE specific search space”.

To avoid overheads during SDT session, option (A) looks preferable from RAN2 point of view. Option (B) should only be considered if option A is not sufficient based on RAN1 input. Latest RAN1 agreements confirm RAN2 preference on this regard as shown below.

“- RAN1 confirms the working assumption in RAN2 that UE-specific search space is configured for UEs performing CG-SDT. This does not exclude the configuration of CSS  for UEs performing CG-SDT.

- CORESET for UE performing RA-SDT should be a common CORESET.””

Therefore, for SDT procedure UE only gets its required configuration via RRCRelease msg or via broadcast signaling.

From RAN2 point of view, broadcast signaling is preferable when updating SDT related configuration. RAN1 latest agreements confirm RAN2 preference on this for SDT operation.

- To confirm that for SDT procedure, a UE only gets SDT related configuration/parameters via broadcast signaling (e.g., common search space and CORESET) or via RRCRelease msg. I.e., RRCReconfiguration message is never used during an ongoing SDT session.

Update of the other stored UE configuration is not essential during an SDT session and should not be supported unless explicitly indicated. For example, RAN2 already agreed for positioning that “gNB can configure the UE with periodic SRS (assuming periodic SRS is supported in RRC_INACTIVE) by RRCRelease with suspendConfig at least when periodic event is configured for deferred MT-LR.  Other cases can be further discussed”.

- gNB cannot update any of the stored configuration for a UE in RRC_INACTIVE that is not re-stored during the SDT session (i.e., not related to SDT transfer) or not used while UE is in RRC_INACTIVE.

## UE behaviour during SDT session

This remaining UE’s operations during an ongoing SDT session are the following:

- Paging monitoring

  - RAN2 agreed that “The UE needs to monitor paging after UE initiates SDT for system information change, PWS.  FFS for other cases”

  - For UE’s specific paging, our understanding is that if there is DL data for the UE during an SDT session (even for non-SDT DRBs), network can move the UE to RRC_CONNECTED using Fallback to resume.

- When there is no specific behaviour defined in specification (e.g. for measurements), the UE should behave like for a legacy UE in RRC_INACTIVE.

  - SDT aims to be simple mechanism that benefit of not having to enable all functionally in used by UEs in RRC_CONNECTED, therefore it should focus only addressing the additional operation required taken RRC_INACTIVE as baseline.

- To agree on the following behaviour for a UE in RRC_INACTIVE during an ongoing SDT session:

  - UE-specific paging is not monitored.

  - UE behaves like for a legacy UE in RRC_INACTIVE when specification does not indicate different or SDT-specific operation (e.g. for measurements).

Another open topic raised during the review of the stage-3 running CRs is “whether UL carrier selection are performed for subsequent uplink transmission.”. On this topic, it was agreed “UL carrier selection is performed before CG-SDT selection” in RAN2#113-bis-e, and in addition, when capturing expected flow of events, UL carrier selection was only considered in the initiation phase. Therefore, we suggest keeping this assumption understanding that SDT session aims to be short in order to have the maximum benefit considering the UE’s power consumption, potential impact if change of cell were to occur, etc.

- UL carrier selection is not performed for subsequent uplink transmission (i.e. this is only performed during the initiation phase of the SDT session),

## RNAU during SDT session

The handling and/or interaction of RNAU procedure and the SDT session needs to be discussed. Email discussion [Post116-e][510][SDT] CCCH and DCCH [4] also raised this scenario but only in relation to one of the solutions. However our understanding is that this topic should be discussed by RAN2 independently on which solution is chosen to handle the non-SDT traffic during an SDT session.

Current RNAU can be triggered (A) upon expiry of periodic RNAU timer (i.e. t380 set with PeriodicRNAU-TimerValue) or (B) upon UE reselects a cell that does not belong to the UE’s configured RNA (i.e. serving cell does not belong to the configured ran-NotificationAreaInfo).

For scenario (A), T380 is currently started upon reception of RRCRelease and stopped upon reception of RRCSetup, RRCResume and RRCRelease, as shown in TS 38.331 reference below.

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T380 | Upon reception of t380 in RRCRelease. | Upon reception of RRCResume, RRCSetup or RRCRelease. | Perform the actions as specified in 5.3.13. |

During SDT operation, the following behaviours are possible for the periodic RNAU timer (T380) for previous scenario (A):

- T380 is stopped upon start of the SDT session.

- T380 continues running upon of the SDT session.  T380 will be restarted as legacy on receipt of RRC Release message.

For both options, T380 is started again at the end of the SDT session (upon reception of RRCRelease including the updated suspendConfig). Our suggestion is to follow option 2 that seems more aligned to legacy behaviour (as T380 is currently stopped upon reception of RRCResume msg but not upon sending RRCResumeRequest). For SDT session, the problem is that there is no L3 acknowledgment after sending the 1st UL msg including the RRCResumeRequest. Therefore, if T380 were stopped upon sending RRCResumeRequest there could be some problems when the SDT session is not successfully started e.g. if T380 timer associated with a given UE expires while the UE is trying to initiate an SDT session (i.e. SDT failure detection timer is running).

Legacy T380 timer is stopped upon reception of RRCResume msg (i.e. not upon sending RRCResumeRequest msg). For SDT session, there is no L3 acknowledgment after sending the 1st UL msg including the RRCResumeRequest msg, and if T380 is stopped upon sending RRCResumeRequest, there could be some problems when the SDT session is not successfully started.

In addition, if T380 were to expire during the SDT session, UE would not need to notify the network as UE already communication ongoing with the network. Moreover, if it is needed, network can always update the suspendConfig at the end of the SDT session which includes the value of this timer. However if SDT session is not successfully initiated (i.e. UE has not received the ACK to the 1st UL SDT (which includes RRCResumeRequest & UL SDT data)), UE shall prioritize initiating legacy RNAU instead than SDT. The reason being that otherwise network may release UE AS Inactive Context autonomously if it thinks that the UE in INACTIVE is not there anymore.

On summary, our preference is that periodic RNAU timer (T380) continues running during the SDT session. However, if T380 expires during an ongoing SDT session, UE does not need to notify the network as UE already communication ongoing with the network while SDT session is ongoing. Therefore, there is no need to send RNAU cause value to the network while the SDT session is ongoing.

- Periodic RNAU timer (T380) continues running upon initiating the SDT session.

  - T380 is always (re)started at the end of the SDT session based on legacy RRCRelease procedure. This is part of legacy RRCRelease operation and do not require any change for SDT operation.

  - If T380 expires during an ongoing SDT session, UE does not need to take any action to notify the network. However, if SDT session is not successfully initiated (i.e. UE has not received the ACK to the 1st UL SDT msg.) when T380 expires, UE shall initiate legacy RNAU instead than SDT.

For scenario (B), UE does RNAU to notify the network that its location has change (which would guarantee UE’s DL reachability via paging) but at the same time UE has SDT data to send. If UE has SDT data to be sent at the same time that UE detects being outside of the configured RNA, there should not be any issue for the UE to initiate the SDT mechanism (understanding that network always updates the suspendConfig at the end of the SDT session). Therefore, if SDT procedure can be used when UE requires to perform RNAU and there is SDT data available, the only impact foreseen to the SDT proc. is that that the resumeCause set to rna-Update.

- When at the same time UE requires to perform RNAU and there is SDT data available, UE is allowed to initiate SDT session. The only impact to the SDT proc. is that that the resumeCause set to rna-Update.

## SDT failure detection timer

On the SDT failure detection timer, it was agreed the following during RAN2#113bis-e:

- SDT failure detection timer is started upon initiation of SDT procedure

- T319 legacy is not started if RRCResumeRequest or RRCResumeRequest1 is transmitted …

- T319 legacy stop conditions also apply to SDT failure detection timer

- FFS Upon SDT failure detection timer expiry, the same procedure as T319 expiry is used (e.g. transition to IDLE as in the case of expiry of the T319 timer and attempts RRC connection setup)  (18/8)

- UE start a window after CG/DG transmission for CG-SDT.   FFS whether to design a new timer or to reuse an existing timer.

On the CG-SDT specific discussion, it was agreed the following during RAN2#116-e:

- The “CG-SDT timer” starts at the first “valid” PDCCH occasion from the end of the CG-SDT PUSCH transmission. The first “valid” PDCCH occasion is defined in RAN1

- The “CG-SDT timer” can be started/restarted during for initial and subsequent transmissions

- The UE restarts the “CG-SDT timer” at least:

  - upon the PUSCH retransmission indicated by the CS-RNTI PDCCH

  - after each CG-SDT transmission

- The “CG-SDT timer” stops at least:

  - When the UE receives RRC feedback messages (e.g. RRCResume, RRCSetup, RRCRelease and RRCReject)

The FFS on the expiry of the SDT failure detection timer is already addressed in previous sections. However, RAN2 still needs to discuss:

- Q1) whether/when the SDT failure detection timer is (re)started during a given SDT session.

- Q2) Relation between the SDT failure detection timer and the window of time after CG/DG transmission for CG-SDT (also referred in RAN2#116e agreements as “CG-SDT Timer”). This question is related to Q11 of the email discussion [Post116-e][509][SDT] CG open issues [5], as we also explained below.

Q1) whether/when the SDT failure detection timer is (re)started during a given SDT session.

Two options are proposed for the (re)start condition of the SDT failure detection timer:

- option (a) this timer only starts at initiation of an SDT session (upon sending RRCResumeRequest msg)

- option (b) this timer (re)starts with every tx/rx during an given SDT session.

The timer for option (a) would need to have a range of values that allow the exchange of multiple DL/UL SDT during a given SDT session. This requires the network to estimate how long the SDT session is limited to be. The timer for option (b) would allow the UE to detect earlier failure of data transmission than the one for option (a), as associated timer most likely would have a shorter value, and it would not impose any limitation to the duration of the SDT session. Both options are feasible, but option (b) seems less restricted.

- The SDT failure detection timer (re)starts with every tx/rx during a given SDT session, only stops only at the end of SDT session (i.e. upon reception of RRCRelease msg) and upon expiry, UE triggers an abrupt termination of the SDT session (i.e. failure).

Q2) Relation between the SDT failure detection timer and the “CG-SDT timer”

In high level, the two new timers under discussion, the SDT failure detection and the “CG-SDT timer” seems to have a similar purpose, i.e. not to keep the UE trying to monitor PDCCH unnecessarily (e.g. in case gNB had sent RRCRelease and UE had not received it). Although the handling upon their expiry might be slightly different depending on expected operation as it is discussed in the following cases 1) and 2) below.

During email discussion [Post116-e][509][SDT] CG open issues [5], the following proposals is suggested for discussion in a potential additional condition to stop the CG-SDT timer:

- [Q11] Proposal 10: CG-SDT timer should be stopped when PDCCH addressed to C-RNTI and CS-RNTI is received (12/19)

However in our understanding for CG-SDT operation, it needs to also be considered whether (re)transmissions via subsequent CG-SDT resources are or are not allowed during a CG-SDT procedure (which is a topic also discussed in email discussion [Post116-e][509][SDT] CG open issues [5] as part of Q7):

- Case 1) Subsequent CG-SDT resources are not used after the initiation of the CG-SDT session.  For this case 1), “CG-SDT timer” seems to have the same functionality/purpose as the SDT failure detection timer, i.e. monitor future tx/rx or trigger otherwise a failure of the SDT session. Therefore, a single timer is sufficient to trigger a failure of the SDT session (independently if it is RA-SDT and CG-SDT).

- Case 2) Subsequent CG-SDT resources can be used after the initiation of the CG-SDT session.  For this case 2), both timers are needed during CG-SDT operation “CG-SDT timer” and SDT failure detection timer, understanding that CG-SDT timer controls triggering of retransmission(s) which is necessary when subsequent CG-SDT resources are to be used.

On summary, SDT failure detection timer is required/applicable for both RA-SDT and CG-SDT and if RAN2 agrees to use subsequent CG resources during an ongoing CG-SDT session, an additional “CG-SDT timer” is also needed during a CG-SDT procedure.

- SDT failure detection timer is applicable to any both RA-SDT and CG-SDT.

- If Proposal 14 is agreed, whether “CG-SDT timer” is the same as the SDT failure detection timer or defined in addition to it, depends on whether subsequent CG resources can be used during an ongoing CG-SDT session

  - If subsequent CG resources can be used during an ongoing CG-SDT session, both timers (“CG-SDT timer” and SDT failure detection timer) are used during a CG-SDT procedure. The expiry of “CG-SDT timer” triggers retransmissions on UE side while the SDT failure detection timer triggers an abrupt termination of the SDT session (i.e. a failure).

  - If subsequent CG resources cannot be used during an ongoing CG-SDT session, one timer is sufficient (i.e. “CG-SDT timer” and SDT failure detection timer are the same). The expiry of this timer triggers an abrupt termination of the SDT session (i.e. a failure), i.e. retransmissions are only handled via DG operation.

## Signaling allowed during SDT session

Table 2 summarizes the RRC messages potentially allowed during an SDT session. The following sub-sections provide further details on the actual operation however this Table 2 describes whether the allowed messages are using at any time of the SDT session or only at the initiation (as clarified in note 1 below).

Table 2. RRC message allowed at any time or only at initiation of SDT session

| RRC message | Purpose | Only at initiation of SDT session (note 1) | At any time of the SDT session |
| --- | --- | --- | --- |
| RRCResumeRequest | To initiate an SDT session | Yes |  |
| RRCRelease | To end an SDT session and potentially update SDT related configuration and location related configuration (e.g. SRS) |  | Yes |
| RRCReject | Not to start an SDT session under congestion scenarios | Yes (note 3) | NA |
| RRCResume | To fallback into legacy resume | NA | Yes |
| RRCSetup | To fallback into legacy setup | Yes | NA |
| DLInformationTransfer, ULInformationTransfer | To transfer NAS PDU (as SRB can be used during SDT ongoing session) e.g. for positioning | NA | Yes |
| RRCReconfiguration | Not needed (note 2) | NA | FFS |

Note 1: initiation of SDT session refers to the 1st UL SDT and 1st DL SDT exchanges.

Note 2: Rel-17 positioning WI already agreed that “gNB can configure the UE with periodic SRS (assuming periodic SRS is supported in RRC_INACTIVE) by RRCRelease with suspendConfig at least when periodic event is configured for deferred MT-LR.  Other cases can be further discussed”, and RAN1 also confirmed RAN2 WA on this as previously explained. Further discussion is added below on the signaling details discussed during positioning SDT related discussion.

Note 3: It is important to consider that RRCReject msg should be sent over SRB0 and therefore should only be used in response to 1st UL SDT msg (i.e. during the initiation phase).

- To confirm that the following RRC messages are allowed during an SDT session (as shown in Table 2): only at the initiation phase of a given SDT session (i.e. 1st UL or DL SDT messages), RRCResumeRequest, RRCReject, and RRCSetup; and, at any time of the SDT session, RRCRelease, RRCResume, DLInformationTransfer, and ULInformationTransfer.

The following RAN2 agreements were taken in positioning discussion with potential impact to SDT operation:

For positioning in RRC_INACTIVE state, the positioning assistance data can be delivered to UE through the following ways:

-     positioning system information, i.e. posSIB;(12/13)

-     pre-configure assistance data when UE in RRC_CONNECTED state;(11/13)

-     send to UE in RRC_INACTIVE during ongoing SDT procedure. (9/13)

SRS for positioning in RRC_INACTIVE state can be configured through the following ways:

-     RRCRelease with SuspendConfig (13/13)

-     SDT DL RRC message, i.e. Msg B / Msg 4 of RA-SDT (9/13)

-     WA: pre-configure positioning SRS in RRC_CONNECTED (9/13)

FFS detailed signalling for these approaches.

Support SP SRSp for positioning in RRC_INACTIVE state. (12/13)

SP Positioning SRS Activation/Deactivation MAC CE is reused for triggering SRSp transmission in RRC_INACTIVE. (12/12)

AP SRSp is not supported for positioning in RRC_INACTIVE state. (11/13)

As previously explained these agreements indicate that MAC CE based SP-SRS activation/deactivation needs to be allowed during an SDT session and support providing positioning reconfiguration via Msg.B/Msg.4 if it were sent e.g. when RRCRelease us sent in response to the 1st UL SDT msg, however our understanding is that there is no need to support any other RRC msg to provide corresponding positioning configuration.

- MAC CE based SP-SRS activation/deactivation is supported during an SDT session.

- RRCReconfiguration and RRCReconfigurationComplete are not supported during an SDT session.

### RRCRelease message

A UE can receive RRCRelease msg. at any time during an ongoing SDT session to successfully end an ongoing SDT session. This RRCRelease msg would be ciphered and integrity protected with the security keys generated by the NCC provided in a previous RRCRelease msg. Three scenarios seem possible:

- Scenario (1) represents when UE is allowed to continue using SDT feature i.e. UE stays in RRC_INACTIVE with a valid stored suspendConfig and SDT related configuration.

- Scenario (2) represents when UE is not allowed or is de-configured to continue using SDT feature i.e. UE stays in RRC_INACTIVE with a valid stored suspendConfig (but stored SDT related configuration is released).

- Scenario (3) represents when UE is transition into RRC_IDLE, i.e. stored suspendConfig and SDT related configuration are both released.

- At any time during an SDT session, a UE can receive RRCRelease msg. indicating either of the following scenarios: (1) UE stays in RRC_INACTIVE with a valid stored suspendConfig and SDT related configuration, (2) UE stays in RRC_INACTIVE with a valid stored suspendConfig (i.e. stored SDT related configuration is released), and (3) UE is transition into RRC_IDLE (i.e. stored suspendConfig and SDT related configuration are both released).

Legacy operation allows sending waitTime when RRCRelease includes suspendConfig. Therefore under congestions scenarios during SDT session, gNB could also reuse this functionality if it were necessary.

If network suffers congestion during an ongoing SDT session, legacy operation is sufficient as it is allowed for the network to send waitTime as part of RRCRelease message includes suspendConfig.

### RRCReject and RRCSetup messages

UE should only receive RRCReject and RRCSetup messages at the beginning of the SDT session, i.e.  when network responds to UE’s ResumeRequest for SDT. In both cases, the SDT session is terminated abruptly but the network controls the end of the SDT session. However, the transfer of 1st UL SDT data should be treated as a failure by the UE (i.e., not processed by the network).

For all of these cases, next Resume attempt could be considered as a new attempt. Therefore, for the UL Data handling when SDT session terminates, the gNB may not even be aware of the actual data that was sent (or wanted to be sent) in UE’s 1st UL SDT. Therefore, it would be up to UE control to handle the retransmission of the data.

It is important to differentiate that RRCReject msg triggers the UE to remain in RRC_INACTIVE (with its stored UE AS Inactive Context, including its SDT configuration), but RRCSetup msg triggers to fallback mechanism to set up a new RRC connection and get the UE into RRC_CONNECTED.

Different ways are possible on how to specify the retransmission of the data after UE receives an RRCReject and RRCSetup message:

- Leave the failure handling up to UE implementation.

LTE PUR handles similar scenarios with the following note added in TS 36.331 upon PUR fallback or PUR failure “NOTE: For transmission using PURmiro, further UE actions upon reception of PUR fallback or PUR failure indication from lower layers (see TS 36.321 [6]) is left up to implementation”. Same handling could apply here with the difference that in LTE UE had only sent one packet vs here a UE may have sent multiple packets before the failure scenario happens.

- Specification defines UE’s behavior.

The specification defines UE behaviour to guarantee the data continuity by having the UE to retransmit all the not-confirmed data after its failure.

Both options are feasible, and this could be discussed after the design further progresses.

- gNB can only send RRCSetup and RRCReject as immediate response to UE’s ResumeRequest for SDT operation.

  - When UE detects the termination of the SDT session for congestion, or fallback to setup, it is left up to UE the decision on how to handle any retransmission of the not acknowledge UL data. Discuss whether TS needs to specify or clarify (e.g. with a NOTE) the related handling of the data for either scenario.

### RRCResume message

UE can receive RRCResume msg at any time during the SDT session (i.e. initiation phase or during an ongoing SDT session) and previous unconfirmed UL data does not need to be resent unless network indicates otherwise.  This operation could allow the UE to start sending traffic belonging to DRBs configured for SDT and non-SDT upon resuming the RRC connection. Therefore, the fallback is done via RRCResume, and data continuity is guaranteed.  During a network-controlled fallback to Resume, the network should successfully process the UL data sent before sending RRCResume (e.g. in case of initiation of SDT session, this would refer to msg 3/A without loss).

This same mechanism would be re-used when non-SDT is triggered during an ongoing SDT session and the corresponding handling is done via DCCH based approach. Upon gNB receives this request or information from UE that indicates that non-SDT data becomes available in UE’s buffer, gNB could trigger a fallback to resume the RRC connection directly from the ongoing SDT session, i.e. SDT session continues after getting RRC_CONNECTED.

Note that even though RRCResume msg can be received at any time during the SDT session, majority companies preferred during email discussion#507 that this mechanism is not used when SDT session was ongoing but the UE AS Context in the anchor gNB was not relocated to the serving gNB.

- UE can receive RRCResume msg. at any time after UE sends the 1st UL SDT. If so, UE follows legacy fallback to resume mechanism which guarantees the data continuity from the ongoing SDT session.

  - When fallback to Resume is used, network ensures that the data received before triggering the fallback to resume is not lost (i.e. UE does not need to retransmit it) and data transfers can continue after UE gets RRC_CONNECTED.

  - This same mechanism is used for DCCH-based switch when non-SDT data is detected during an ongoing SDT session.

# Conclusion

The observations captured are the following:

Observation 1.	Delta signalling of legacy Suspend and SDT configuration can reduce signalling overhead compared to volume of data

Observation 2.	From RAN2 point of view, broadcast signaling is preferable when updating SDT related configuration. RAN1 latest agreements confirm RAN2 preference on this for SDT operation.

Observation 3.	Legacy T380 timer is stopped upon reception of RRCResume msg (i.e. not upon sending RRCResumeRequest msg). For SDT session, there is no L3 acknowledgment after sending the 1st UL msg including the RRCResumeRequest msg, and if T380 is stopped upon sending RRCResumeRequest, there could be some problems when the SDT session is not successfully started.

Observation 4.	If network suffers congestion during an ongoing SDT session, legacy operation is sufficient as it is allowed for the network to send waitTime as part of RRCRelease message includes suspendConfig.

The proposals captured are the following:

Proposal 1.	The sdt-RSRP-Threshold is provided in Sdt-ConfigCommonSIB IE via SIB signaling as it can take different value for different cells.

Proposal 2.	If Proposal 1 is agreed (sdt-RSRP-Threshold is provided via SIB signalling), when sdt-RSRP-Threshold is absent (i.e. not configured) and the usage of SDT feature is allowed in a given cell, UE is allowed to continue with the initiation phase of the SDT session (independent of its RSRP).

Proposal 3.	If Proposal 1 is not agreed (sdt-RSRP-Threshold is provided via dedicated signalling), network always configure sdt-RSRP-Threshold in UE when SDT feature is configured for that UE.

Proposal 4.	When sdt-RSRP-ThresholdSSB-SUL is absent (i.e. not configured), UE reuses the value of legacy RSRP-ThresholdSSB-SUL for the SDT procedure (when having to do the carrier selection check).

Proposal 5.	The same value of the RSRP threshold for UL carrier selection is applicable for both CG and RA-SDT (i.e. same value is used for both mechanisms).

Proposal 6.	Delta signaling is supported for the SDT related configuration. This delta signaling applies across different SDT sessions and when resuming the RRC connection (i.e. SDT related configuration is released when UE enters RRC_IDLE or when the network explicitly releases the SDT configuration).

Proposal 7.	During the SDT session, UE only restores the configurations for SDT, suspend and radio bearers (i.e. other UE stored configurations are not restored during the SDT session).

Proposal 8.	During the SDT session, UE does not act (e.g. release, restore or update) at least any of the following configurations which are stored in UE Inactive AS context (but are not relevant to SDT operation): a) delayBudgetReportingConfig, b) overheatingAssistanceConfig, c) idc-AssistanceConfig, d) drx-PreferenceConfig for all configured cell groups, e) mrdc-SecondaryCellGroup, and f)  Timers T346a, T346b, T346c, T346d, T346e, T346f.

Proposal 9.	To confirm that for SDT procedure, a UE only gets SDT related configuration/parameters via broadcast signaling (e.g., common search space and CORESET) or via RRCRelease msg. I.e., RRCReconfiguration message is never used during an ongoing SDT session.

Proposal 10.	gNB cannot update any of the stored configuration for a UE in RRC_INACTIVE that is not re-stored during the SDT session (i.e., not related to SDT transfer) or not used while UE is in RRC_INACTIVE.

Proposal 11.	To agree on the following behaviour for a UE in RRC_INACTIVE during an ongoing SDT session:

Proposal 11.1.	UE-specific paging is not monitored.

Proposal 11.2.	UE behaves like for a legacy UE in RRC_INACTIVE when specification does not indicate different or SDT-specific operation (e.g. for measurements).

Proposal 12.	UL carrier selection is not performed for subsequent uplink transmission (i.e. this is only performed during the initiation phase of the SDT session),

Proposal 13.	Periodic RNAU timer (T380) continues running upon initiating the SDT  session.

Proposal 13.1.	T380 is always (re)started at the end of the SDT session based on legacy RRCRelease procedure. This is part of legacy RRCRelease operation and do not require any change for SDT operation.

Proposal 13.2.	If T380 expires during an ongoing SDT session, UE does not need to take any action to notify the network. However, if SDT session is not successfully initiated (i.e. UE has not received the ACK to the 1st UL SDT msg.) when T380 expires, UE shall initiate legacy RNAU instead than SDT.

Proposal 14.	When at the same time UE requires to perform RNAU and there is SDT data available, UE is allowed to initiate SDT session. The only impact to the SDT proc. is that that the resumeCause set to rna-Update.

Proposal 15.	The SDT failure detection timer (re)starts with every tx/rx during a given SDT session, only stops only at the end of SDT session (i.e. upon reception of RRCRelease msg) and upon expiry, UE triggers an abrupt termination of the SDT session (i.e. failure).

Proposal 16.	SDT failure detection timer is applicable to any both RA-SDT and CG-SDT.

Proposal 17.	If Proposal 14 is agreed, whether “CG-SDT timer” is the same as the SDT failure detection timer or defined in addition to it, depends on whether subsequent CG resources can be used during an ongoing CG-SDT session

Proposal 17.1.	If subsequent CG resources can be used during an ongoing CG-SDT session, both timers (“CG-SDT timer” and SDT failure detection timer) are used during a CG-SDT procedure. The expiry of “CG-SDT timer” triggers retransmissions on UE side while the SDT failure detection timer triggers an abrupt termination of the SDT session (i.e. a failure).

Proposal 17.2.	If subsequent CG resources cannot be used during an ongoing CG-SDT session, one timer is sufficient (i.e. “CG-SDT timer” and SDT failure detection timer are the same). The expiry of this timer triggers an abrupt termination of the SDT session (i.e. a failure), i.e. retransmissions are only handled via DG operation.

Proposal 18.	To confirm that the following RRC messages are allowed during an SDT session (as shown in Table 2): only at the initiation phase of a given SDT session (i.e. 1st UL or DL SDT messages), RRCResumeRequest, RRCReject, and RRCSetup; and, at any time of the SDT session, RRCRelease, RRCResume, DLInformationTransfer, and ULInformationTransfer.

Proposal 19.	MAC CE based SP-SRS activation/deactivation is supported during an SDT session.

Proposal 20.	RRCReconfiguration and RRCReconfigurationComplete are not supported during an SDT session.

Proposal 21.	At any time during an SDT session, a UE can receive RRCRelease msg. indicating either of the following scenarios: (1) UE stays in RRC_INACTIVE with a valid stored suspendConfig and SDT related configuration, (2) UE stays in RRC_INACTIVE with a valid stored suspendConfig (i.e. stored SDT related configuration is released), and (3) UE is transition into RRC_IDLE (i.e. stored suspendConfig and SDT related configuration are both released).

Proposal 22.	gNB can only send RRCSetup and RRCReject as immediate response to UE’s ResumeRequest for SDT operation.

Proposal 22.1.	When UE detects the termination of the SDT session for congestion, or fallback to setup, it is left up to UE the decision on how to handle any retransmission of the not acknowledge UL data. Discuss whether TS needs to specify or clarify (e.g. with a NOTE) the related handling of the data for either scenario.

Proposal 23.	UE can receive RRCResume msg. at any time after UE sends the 1st UL SDT. If so, UE follows legacy fallback to resume mechanism which guarantees the data continuity from the ongoing SDT session.

Proposal 23.1.	When fallback to Resume is used, network ensures that the data received before triggering the fallback to resume is not lost (i.e. UE does not need to retransmit it) and data transfers can continue after UE gets RRC_CONNECTED.

Proposal 23.2.	This same mechanism is used for DCCH-based switch when non-SDT data is detected during an ongoing SDT session.

# Reference

- R2-2109620, Control Plane leftover issues on SDT mechanism, Intel Corporation, November 2021.

- R2-2200050, [Post116-e][506][SDT] RRC running CR update, January 2017.

- R2-2200032, Summary of [Post116-e][507][SDT] MAC running CR update (Huawei)review issue list, January 2022.

- R2-2200026, Report of [Post116-e][510][SDT] CCCH and DCCH (Nokia), January 2022.

- R2-2200033, [Post116-e][509][SDT]CG open issues (Huawei), January 2022.