3GPP TSG-RAN WG2 #116bis-e                                                                                  R2-2201904

eMeeting, 17th – 25th January, 2022

Agenda Item:	    8.0.3

Source: 	    MediaTek Inc.

Title:		    Report of [AT116bis-e][018][NR17] Gaps Coordination (Mediatek)

Document for:	    Discussion and decision

# 1 Introduction

This is report for the following AT116bis-e mail discussion.

[AT116bis-e][018][NR17] Gaps Coordination (Mediatek)

Scope: List the relevant gap features and potential opportunities regarding commonality, parts that need coordination (e.g. common capability/overall limitation). Collect comments, e.g. on feasibility, ambition levels, what to decide now, what to postpone etc. Consider proposals from tdocs submitted to 8.0.3.

Intended outcome: Report, ambition level up to rapporteur.

Deadline: For On-Line CB W2

Deadline – Friday Week1 UTC 2300

# 2 Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| MediaTek (Rapp) | Felix Tsai | chun-fan.tsai@mediatek.com |
| vivo | Xiaodong Yang | Yangxiaodong5g@vivo.com |
| OPPO | Jiangsheng Fan | fanjiangsheng@oppo.com |
| Huawei, HiSilicon | Lili Zheng | zhenglili4@huawei.com |
| Qualcomm | Mouaffac | mambriss@qti.qualcomm.com |
| Intel | Candy Yiu | Candy.yiu@intel.com |
| CATT | ShiJie | shijie@catt.cn |
| Samsung | Sangyeob Jung | sy0123.jung@samsung.com |
| Ericsson | Felipe Arraño Scharager | felipe.arrano.scharager@ericsson.com |
| Apple | Yuqin Chen | yuqin_chen@apple.com |
| ZTE | LiuJing | liu.jing30@zte.com.cn |
| Nokia | Tero Henttonen | tero.henttonen@nokia.com |

# 3 Discussion

## 3.1 List the relevant gap features

We starting from collecting company view on which features should be discussed in this gap coordination sections. The following is listed of gap related features (modified from [2]).

Feature 0 – Legacy measurement gap

- There are 26 measurement gap patterns defined in the specification

  - Each pattern is identified by an offset value, a measurement gap length (MGL), and a measurement gap repetition period (MGRP)

  - Some gap patterns are mandatory to be supported

  - Gap pattern 24 and 25 is used for positioning measurement only

- Two type of measurement gap is introduced.

  - (Mandatory) per UE gap, where the gap apply to all serving cells

  - (Optional) per FR gap, where one FR1 gap and one FR2 gap could be configured. FR1 gap apply to FR1 serving cells and FR2 gap apply to FR2 serving cell

- Measurement Gap is configured only by RRC and is always activated once configured

- For UE supports per FR gap, at most one FR1 gap and one FR2 gap could be configured. Otherwise, at most one per UE gap could be configured.

Feature 1 – Pre-Configured gap

- Pre-configured gap is a measurement gap that does not always activated once configured.

- Pre-configured gap status (activated or not) is determined by which BWP is activated and other pre-defined rules specified in RAN4 SPEC.

- Pre-configured gap is configured by RRC.

- MAC CE control of activation and deactivation of pre-configured gap is NOT support.

Feature 2 – Concurrent Gap

- Concurrent gaps are multiple measurement gaps, and each gap pattern could be associated with one or multiple frequency layers.

- No new gap pattern is introduced for concurrent gap, the existing R15/R16 gap pattern could be configured for the concurrent gaps.

  - Note that NTN is discussing to have more than one measurement gap, which seems to be covered by this work.

Feature 3 – Network control small gap (NCSG)

- NCSG is a kind of measurement gap but causes small interruption to the serving cell(s) (i.e. interruption only at start and end of the MGL, not whole MGL).

- R15 gap pattern is reused (Positioning gap pattern is NOT applicable)

- The UE has to report whether it support NCSG per target band per BC

Feature 4 – ePOS gap (Pre-configured MG for PRS)

- R1 intend to have multiple pre-configured measurement gaps (MG) for PRS measurement. RRC to provide the pre-configuration.

- UL MAC CE is used to request activate/deactivate of pre-configured PRS MG.

- DL MAC CE is used to activate/deactivate a pre-configured PRS MG.

- It seems that same R15/R16 measurement gap pattern will be reused

- Assuming only one activated at a given time

Feature 5 – NTN Gap

- The agreement is unclear at this moment

- Similar to concurrent gaps but two gaps could be used for same measured frequency

- How many numbers of gap could be configured is not clear.

Feature 6 – MUSIM gap

- MUSIM gap is the gap the UE will switch to do task from another SIM. The UE does not send or receive data from serving cell(s) of current SIM. (Similar to per-UE measurement gap)

- At most 3 MUSIM gaps (one aperiodic, two periodic) are supported

- R15/R16 gap pattern could be reused, and some additional pattern may be added for MUSIM

- The UE should request gap configuration (using assistance information) to the network

- RAN2 will not create MAC CE activation of gaps in MUSIM, but if the common gap discussion allows this anyway, RAN2 will not prevent that, either

Feature 7 – UL FR2 gap

- UL FR2 gap is similar to FR2 gap but it is used for self-calibration (not measurement). DL data reception is still possible within this gap.

- New gap pattern will be defined for UL FR2 gap pattern. Similar parameters as in measurement gap (i.e., offset, length, period) will be introduced.

- Activate/deactivate FR2 UL gap by RRC (no agreement in RAN2 for MAC CE control for now)

And the following table provide a quick summary on the gap related features

Table 1 – List the relevant gap feature

| Index | Gap Feature | Purpose | Activation/ Deactivation | Note |
| --- | --- | --- | --- | --- |
| 0 | Legacy gap | Measurement | By RRC | Periodic gap Per UE or Per FR Max 1 FR1 gap + 1 FR2 gap could be configured |
| 1 | Pre-Configured gap | Measurement | By RRC and RAN4 Rules |  |
| 2 | Concurrent gap | Measurement | By RRC | FFS Max 2 FR1 gap + 2 FR2 gap could be configured |
| 3 | NCSG | Measurement | By RRC |  |
| 4 | ePOS gap | PRS Measurement | MAC CE |  |
| 5 | NTN gap | Measurement | By RRC |  |
| 6 | MUSIM gap | MUSIM | By RRC | Only per UE gap Max 2 periodical and 1 aperiodic gap |
| 7 | UL FR2 gap | self-calibration for FR2 cells | FFS MAC CE | Only FR2 gap |

First, let check if companies agree that above features should be discussed in gap coordination item and any high-level comment on the listed features. The rapporteur thinks this table includes all the gap features and would like to confirm with other companies.

Question 1: Companies are invited to provide general comment on the listed features and Table 1 above. Any feature is missing or which feature is not needed to be included? Any high-level clarification on the each of the feature (with the understanding that details to be discussed in specified WI)?

| Company | Comments |
| --- | --- |
| vivo | For ePoS gap, RAN1 has agree MAC CE activation. Considering that the positioning requirement from UE, it is valid case, So RAN2 can just follow the agreement. FFS from MAC CE can be removed. |
| OPPO | Just correct the typo for MUSIM gap, we’re open to discuss the coordination for all the above gaps. |
| Huawei, HiSilicon | UL FR2 gap bears little relation to other gap features (others are all DL gaps and have different gap patterns with the UL FR2 gap, thus they will have separate configuration), so we think UL FR2 gap should not be discussed here. |
| QCOM | UL FR2/MUSIM/NTN/ePOS gap features, all belong to different WIs from the EMG features (concurrent/preconfigured/NCSG) and have different requirements/purpose, therefore they should be treated separately. In addition we need to make sure no collision will occur between non-EMG features and the EMG features. |
| Intel | Added notes for ePOS and NTN gap. Also agree with VIVO that RAN1 has agree MAC CE activation for ePOS. In addition, RAN2 ePOS also agreed as follow: Agreements: Proposal 5a:A new UL MAC CE for positioning measurement gap activation and deactivation request is introduced. Proposal 5b:The new UL MAC CE for positioning measurement gap activation and deactivation request includes at least the ID of the pre-configured positioning measurement gap configuration for which the activation/deactivation is requested. Other parameter are FFS. Proposal 5c (modified):A new DL MAC CE for positioning measurement gap activation and deactivation command is introduced for positioning latency reduction. LS to RAN1/4 indicating our conclusion, and confirming that DL MAC CE can also be used for positioning measurement gap deactivation as well as activation (to be drafted by email). Proposal 5d:The new DL MAC CE for positioning measurement gap activation and deactivation command includes at least the ID of the pre-configured positioning measurement gap configuration which has been configured/activated by the gNB. Other parameter are FFS. Proposal 5e:The Scheduling Request should be triggered when there is no PUSCH and UL MAC CE for positioning measurement gap activation/deactivation request is triggered. Removed FFS. |
| CATT | Agree with vivo, the MAC CE activation/deactivation has been agreed for ePoS gap. For UL FR2 gap, its purpose is different from the DL gap, it is independent with other types of MGs. In order to reduce the work load and unnecessary overlap, we agree with HW to decouple the UL FR2 gap from this AI. |
| Samsung | Same view with vivo and QC that for ePOS gap, it has different objective and the requirement / purpose with the MGE WI, and already the core of procedures is agreed in RAN1. So prefer not to treat in common. And also FFS from MAC CE should be removed. |
| Ericsson | We agree with Huawei and believe that UL FR2 gaps should be left out of the scope of this activity since they are to be configured separately. Comments for the other topics found below. |
| Apple | We also propose that UL gap can be discussed separately. Main reason is the new flags for NCSG gap, pre-MG would not be applied to UL gap. And differently from preMG/NCSG/concurrent gap, RAN plenary decided that MR-DC should be supported for UL gap. |
| ZTE | We prefer to at least exclude MUSIM aperiodic gap and FR2 UL gap from this discussion, because both of them have very different pattern design. |
| Nokia | MUSIM has two types of gaps: Periodic and aperiodic. Whatever is discussed here should focus on periodic gaps. When aperiodic gap is used, UE does not use its serving network for the duration of the aperiodic gap. FR2 UL gaps are also somewhat "special" in the sense that they only apply to FR2, and only impact UL operation. The main coordination should be on attempting to find common configuration aspects - trying for too wide a scope will easily fail. |
| MediaTek | We are open to discuss the coordination for all the gap features listed in Table 1. |

Summary 1:

Based on the comments from companies, there is no other gap related feature so far is identified that should be discussed in gap coordination section. Most companies also think FR2 UL gap could be developed independently and no need to coordinate with other gap features. There is some diverse view on whether to discuss MUSIM gap in this section.

Observation 1: Most companies think that UL FR2 gap is independent from other gap features. (i.e. separate configuration for UL FR2 gap and enabling the UL FR2 gap or not does not conflict with other gap features from RAN2 perspective).

## 3.2 How to do gap coordination

| Companies | Proposals |
| --- | --- |
| Ericsson [1] | Proposal 1 Use this AI to showcase the Measurement Gap Enhancements (MGE) WI progress and the solutions being discussed for each objective. Proposal 2 Use this AI to gather the views from other Rel-17 WIs requiring gap-related improvements as possible input for the design of the MGE WI solutions. Proposal 3 Use this AI to see to what extent it is possible to use the MGE WI solutions to address the needs of the other WIs requiring gap-related improvements. |
| MTK [2] | Proposal 2: RAN2 to work on individual RRC CR for the gap enhancement features first. |
| Huawei [4] | Proposal 1: The one thing in common among the WIs is the configuration of multiple gaps (concurrent gaps). Other features (pre-configured gaps, NCSG, gap activation/deactivation via MAC CE) are only valid in one WI, thus will not be discussed in this AI. |
| Apple [7] | Proposal 1: UL gap and MUSIM gap should be discussed separately from the other three gap features (NCSG, pre-MG, concurrent gap). |

On how to coordinate between different R17 gap features, some company suggest to start with individual RRC CR first and can discuss later on how to merge. Some company thinks MGE WI solutions may be able to address the gap features from other WI.

Question 2: Companies are invited to comment on how to coordinate the gap relevant features.

| Company | Comments |
| --- | --- |
| vivo | We can discuss how RRC configuration is harmonized for all gap types in 8.0.3 section, i.e., which IE is enhanced and what is IE structure in R17 for all gaps. We do not combine one topic to another topic. |
| OPPO | We believe the intention for gap coordination is trying to make the ASN.1 design simple, but this work should be carefully organised, not all type of gaps should be coordinated, for instance, MUSIM gap and UL FR2 gap are used for other purpose other than measurements, trying to combine the ASN .1 design with other type of gaps will even make the ASN.1 complex and unreadable, so in our view, the gap coordination can start within MG WI, i.e. the gap coordination for preconfigured gap, concurrent gap and NCSG. As for other type of gaps introduced in R17, individual RRC CR can be the baseline, we can further consider the ASN.1 optimization when we do the merging. |
| Huawei, HiSilicon | As we indicated in our paper, the one thing in common among the WIs is the configuration of multiple gaps (concurrent gaps). Other features (pre-configured gaps, NCSG, gap activation/deactivation via MAC CE) are only valid in one WI. Therefore, we can try to see how the concurrent gap feature can incorporate gap features from other WIs. |
| QCOM | Coordination can be divided into 2 topics: - ASN.1 design - Discuss if there is a need to combine the ASN.1 structure for some of the EMG features - For non-EMG features, each should have its own separate ASN.1 structure - procedural description to define co-existence and to prevent collision - define rules for the EMG features to co-exist, e.g. concurrent gap + preconfigured gap, and/or NSCG + legacy … - define rules to prevent the collision between non-EMG features and EMG features, and among the non-EMG features themselves. |
| Intel | In general, we think that we can consider one framework for gap configuration in RRC including pre-configured gap, NCSG and concurrent gap since they have a similar structure. But individual WI may have different activation/deactivation of gap, it can be discussed in individual WI. |
| CATT | One common framework on RRC configuration for all gap types is suggested for this AI, but the condition is a clear view to the relationship of concurrent/preconfigure/NCSG gap and ePOS/MUSIM/NTN is achieved. The other specific feature is better to be discussed in separately WI. |
| Samsung | We can discuss how to design ASN.1 structure in common after stabilizing each feature in each WI. Besides we think that at least FR2 UL gap/MUSIM gap should not be considered within the scope of MGE WI and our views on other features (e.g. NTN/ePOS) are given in the next sections. |
| Ericsson | We believe that the main questions we should try to answer within the framework of this activity are: - How many gaps patterns can run in parallel in total? - Which purposes can be shared on a gap pattern? - How are the gap patterns prioritised in case of collision? |
| Apple | - ASN.1 optimization can of course be considered. - Whether one gap can be configured with multiple flags, i.e., pre-MG flag, NCSG flag. - The maximum gaps or the maximum percentage of time duration of multiple gaps should be discussed. We suggest a guidance from RAN4 is needed. |
| ZTE | We think following issues need to be address in this common gap AI: 1) Which features can be merged into MGE? 2) Which features may be configured at the same time? (to identify the possible gap combinations) 3) Whether and how to define a common ASN.1 structure for all features, or at least most features? We think the discussion in this AI is parallel to the discussion of individual WIs. Which means one AI can provide inputs or guidance to the other AI. We are afraid that just waiting for the CRs from each WI and merge it will cause more problem. |
| Nokia | Agree with many of the previous comments: From RRC perspective, we should identify common configuration structures, but before we do that, it has to be clear which gaps can be configured simultaneously, and how many gap patterns can UE have at once. We assume aperiodic MUSIM gap and FR2 UL gap can be independent from all other cases. If we can also assume that Rel-17 does not define MAC CE-based activation for any gaps unless there is critical need, everything can be done via RRC, which helps in removing most MAC impacts. |
| MediaTek | As mentioned in our paper, we suggest to develop the individual RRC CR for the gap features first and consider how to do ASN.1 merge later. Except for common configuration part, we also want to discuss whether two gap features could be enabled simultaneously. |

Summary 2:

Some observation is made based on the comments above.

Observation 2: Several companies mentioned that common configuration structure could be considered. However, there is no much intention to do ASN.1 merge at this moment (individual feature is not stable enough).

Observation 3: Most companies want to clarify how many gaps could be configured simultaneously. For example, which features may be configured at the same time and is there any “collision” between different gaps features?

According to the contributions and companies’ input, Rapporteur thinks that the collision of gaps could be considered in two different aspects. It is better to clarify this before further discussion.

Observation 4: Two different aspects are mentioned for “collision” between gap features

- Case a: Two gap cannot be configured together. Simultaneous configuration may cause some issue in procedure part.

- Case b: Two gap could be configured together but UE behavior on the overlapping period in time domain may needed further clarification.

## 3.3 Commonality of gap features

In this section, we discuss the commonality of different gap features.

### 3.3.1 ePOS gap

The following are the proposals related to ePOS gap.

| Companies | Proposals |
| --- | --- |
| Ericsson [1] | Proposal 3 Use this AI to see to what extent it is possible to use the MGE WI solutions to address the needs of the other WIs requiring gap-related improvements. For example: a) could MUSIM or NTN gaps could be considered within Rel-17’s MGE concurrent gaps? b) could pre-configured positioning-specific gaps be covered by Rel-17’s MGE pre-configured gaps? |
| MTK [2] | Proposal 1: Specify that pre-configured PRS MG for positioning is not configured together with enhanced gap operation from MG enh. WI (i.e. pre-configured MG, concurrent gap, or NCSG). |
| Intel [3] | Proposal 3: RAN2 to discuss the two options to move forward for ePOS and MGE measurement gap work: - Option 1: Decouple ePOS with MGE for pre-configured gap and concurrent gap - Option 2: Reuse the same concurrent gap structure to support ePOS pre-configured gap Proposal 4: RAN2 to discuss NTN measurement gap in NTN WI. |

Several comments mentioned that ePOS gap may be considered as pre-configured gap or concurrent gap. Some company suggest that they should be decoupled or they should not be configured together. Rapporteur understands that ePOS gap is not discussed in RAN2 yet but maybe we can discuss based on RAN1 input.

Question 3: Do companies think ePOS gap could be consider as pre-configured gap or concurrent gap or both ? Any view on simultaneous enabling the ePOS gap and other MGE features?

| Company | Comments |
| --- | --- |
| Vivo | No, we do not combine one topic to another topic From our understanding, only two issues can be discussed in 8.0.3 section. 1) common RRC singling design for harmonize the ASN.1 and 2) simultaneous gaps configuration issue for all gap types. |
| OPPO | As mentioned in Q2, the gap coordination can start within MG WI, an individual RRC CR for ePOS gap is more desirable at this stage. |
| Huawei, HiSilicon | We think the pre-configured MG discussed in Positioning WI has the different meaning with the pre-configured MG discussed in MG enhancement. However, the concurrent gap has already taken PRS into account. In our understanding, the positioning gap can be considered as one of the concurrent gap. |
| QCOM | We’re against combining those 2 features. [Rapp] The intention is not to combine the 2 features. The intention is to discuss whether common ASN.1 configuration could be used and whether it could be configured simultaneously. |
| Intel | We think it is kind of both. It has an activation/ deactivation using MAC CE so it is pre-configured gap. However, there can be multiple of them so it is also concurrent gap. For this reason, we think we can structure concurrent gap to include pre-configured gap flag (if it is in gapConfig), it will be by nature inside concurrent gap. That way, from structure pov, it will be supported. |
| CATT | Concurrent gap has already included the purpose for positioning based on RAN4 LS, so ePOS gap could be considered as concurrent gap. Pre-configured gap is this topic is associated to the BWP, it is necessary to firstly confirm whether the ePOS gap is associated to BWP, i.e, ePOS gap is activated/deactivated based on the activation/deactivation of BWP, which is better to be confirmed in ePOS WI or RAN4. Besides, ePOS gap is under discussion in Positioning session now. ePOS is designed for PRS measurement only. DL MAC CE and UL MAC CE is used for ePOS gap: - UL MAC CE is used to request activate/deactivate of pre-configured PRS MG. - DL MAC CE is used to activate/deactivate a pre-configured PRS MG. But MAC CE is not supported either in Pre-Configured gap or in Concurrent Gap. An ID is required in ePOS gap according to the LS from RAN1(R1-2112883) Agreement Preconfiguration of MG(s) in RRC is supported from RAN1 perspective. - Each MG in the preconfiguration is associated with an ID - The information in the UL MAC CE for MG activation request by the UE can be one ID associated with the preconfiguration of the MG Hence, we suggest that ePOS gap be decoupled or wait the progress in each separate WI. |
| Samsung | RAN4 indicated that pre-configured MG for positioning measurements beyond Rel-17 is not discussed under the MGE WI so ePOS gap should not be considered within the scope of MGE pre-configured gap. But ePOS gap can be considered as one of concurrent gap. |
| Ericsson | For the activation/deactivation of pre-configured ePOS gaps, RAN2 has followed the MAC CE approach already agreed in RAN1. Hence, it seems challenging to consider them as part of this “gaps coordination” activity. However, as mentioned above, ePOS gaps could eventually be covered by the structure adopted by concurrent gaps in the MGE WI. |
| Apple | I think RAN4 agreed pre-MG does not apply to Rel-17 ePositioning. For concurrent gap, RAN4 has no explicit agreement if it applies to ePositioning or not. We are open to take ePostitioning gap as one concurrent gap. |
| ZTE | RAN4 agreed that concurrent gap can be applied for PRS measurement, but RAN4 also agreed concurrent gaps are considered as activated when they are configured. So this is different from the pre-configured gaps discussed in ePOS session. Since ePOS gaps use MAC CE to activate/deactivate, we prefer to discuss it separately. But for ASN.1, it is better to define a common signalling to accommodate these features. |
| Nokia | Fine to consider these together with pre-MG and concurrent MG. |
| MediaTek | In our view, the first priority is to clarify whether the ePOS gap could co-exist with MGE features. It looks quite similar but there is clear difference on how to control the gap. |

Summary 3:

Some observation is made based on the comments above.

Observation 5: There is no clear consensus on whether we should consider ePOS gap as pre-configured MG or concurrent MG. It seems that the behavior of ePOS gap will be different from either pre-configured MG or concurrent MG.

In rapporteur’s view, we could have separate discussion on ePOS and MGE features. Whether to have harmonized ASN.1 configuration could be discussed later.

### 3.3.2 MUSIM gap

The following are the proposals related to MUSIM gap.

| Companies | Proposals |
| --- | --- |
| Ericsson [1] | Proposal 3 Use this AI to see to what extent it is possible to use the MGE WI solutions to address the needs of the other WIs requiring gap-related improvements. For example: a) could MUSIM or NTN gaps could be considered within Rel-17’s MGE concurrent gaps? b) could pre-configured positioning-specific gaps be covered by Rel-17’s MGE pre-configured gaps? |
| Intel [3] | Proposal 1: For MUSIM, aperiodic gap can be operated independently from MG WI Proposal 2: For MUSIM, periodic gap can be incorporated with concurrent gap framework (re-use the same configuration, activation and pattern) and can be discussed in MG WI |

It seems that MUSIM gap could be consider as concurrent gap. Not sure how to deal with the aperiodic gap. Anyway, Rapporteur would like check company’s understanding on this aspect.

Question 4: Do companies think MUSIM gap could be consider as concurrent gap? Any view on how to handle the aperiodic MUSIM gap?

| Company | Comments |
| --- | --- |
| vivo | No, we do not combine one topic to another topic From our understanding, only two issues can be discussed in 8.0.3 section. 1) common RRC singling design for harmonize the ASN.1 and 2) simultaneous gaps configuration issue for all gap types. |
| OPPO | No, as mentioned in Q2, the gap coordination can start within MG WI, an individual RRC CR for MUSIM gap is more desirable at this stage. Trying to combine the MUSIM gap ASN .1 design with other type of gaps will even make the ASN.1 complex and unreadable. Especially considering the aperiodic MUSIM gap is hard to do the coordination with other type of periodic gaps. |
| Huawei, HiSilicon | Yes for periodic gaps in MUSIM, no for aperiodic MUSIM gap. Aperiodic gap can be discussed in MUSIM WI. |
| QCOM | We’re against combining those 2 features. |
| Intel | Aperioidic gap is a one shot request by UE, therefore, we can just not discuss here and focus on perioid gap. As for periodic gap, it can be considered concurrent gap. But individual can have detail mechanism. |
| CATT | As feature description for Concurrent gap, it is multiple measurement gaps, and each gap pattern could be associated with one or multiple frequency layers. So, we can see Concurrent gap is not conflict with periodic gaps in MUSIM, which could be considered as Concurrent gap. For aperiodic MUSIM gap, it could to be discussed in MUSIM WI if it is majority view. |
| Samsung | MUSIM gaps are not used for RRM measurments but for other purposes i.e. paging monitoring/ SI acquisition. So we do not prefer to consider MUSIM gap within the scope of MGE concurrent gaps. |
| Ericsson | It will depend on the questions we placed as comment to Q2. However, MUSIM gaps could eventually be considered as concurrent gaps (at least the periodic ones as pointed out above). |
| Apple | We generally think MUSIM gap should be discussed separately. For MUSIM periodic gap, I think it is only about introducing more patterns thus can naturally be covered already. For MUSIM aperiodic gap, this is quite different so we suggest to discuss it in MUSIM. |
| ZTE | From RAN2 perspective, periodical gap can be considered as part of concurrent gap. But this may impact RAN4 when discussing the UE requirement, because MUSIM gap is not used for RRM measurement, so it will be quite complex if one gap pattern is associated with both MUSIM and RRM measurement. But the impact may be limited if the gap is only associated with MUSIM. For Aperiodic gap, we prefer to only discuss it in MUSIM. |
| Nokia | MUSIM can operate 2 periodic gaps, which have to be possible to be used with (at least one) "classic" measurement gap Then aperiodic MUSIM gap interrupts operation in serving network for a while, and occurs only once. |
| MediaTek | We tend to agree that periodical MUSIM is quite similar to concurrent gap. However, we think it would be better to discuss separately first. Whether to have common ASN.1 configuration is not first priority, and can be discussed later. |

Summary 4:

Some observation is made based on the comments above.

Observation 6: Some companies think periodic MUSIM gaps could consider as concurrent gap. However, there is also strong resistance to include MUSIM gap as part of MGE concurrent gap at this moment.

### 3.3.3 NTN gap

The following are the proposals related to NTN gap.

| Companies | Proposals |
| --- | --- |
| Ericsson [1] | Proposal 3 Use this AI to see to what extent it is possible to use the MGE WI solutions to address the needs of the other WIs requiring gap-related improvements. For example: a) could MUSIM or NTN gaps could be considered within Rel-17’s MGE concurrent gaps? b) could pre-configured positioning-specific gaps be covered by Rel-17’s MGE pre-configured gaps? |
| Intel [3] | Proposal 4: RAN2 to discuss NTN measurement gap in NTN WI. |
| Nokia [6] | Proposal 4: Model NTN gaps as concurrent gaps. |

It seems that NTN gap could be consider as concurrent gap. However, we may have to wait for more agreement from NTN section.

Question 5: Do companies agree that NTN gap could be consider as concurrent gap?

| Company | Agreed or not | Comments |
| --- | --- | --- |
| vivo | no | No, we do not combine one topic to another topic From our understanding, only two issues can be discussed in 8.0.3 section. 1) common RRC singling design for harmonize the ASN.1 and 2) simultaneous gaps configuration issue for all gap types. |
| OPPO | Maybe | As mentioned in Q2, the gap coordination can start within MG WI, an individual RRC CR for NTN gap is more desirable at this stage. Maybe considering the coordination when we clearly know how NTN gap looks like. |
| Huawei, HiSilicon | No | First, there haven’t been enough agreements on NTN gaps yet. Besides, in our understanding, the multiple NTN gaps can share one gap configuration by adding a list of gap offsets (modelled as single gap instead of concurrent gaps) since they could differ only in offsets. |
| QCOM | No | We’re against combining those 2 features. |
| Intel | Yes/ no | At this point, it is unclear what NTN gaps like. So it is really hard to say. The key in NTN gap is each of them may link to individual SMTC and there could be up to 4 of them (it is still under discussion). So we suggest to have the framework and let NTN to decide if they want to use the same framework. |
| CATT | Yes/no | There is an agreement in NTN item that, measurement gap related aspects for Rel-17 NTN will be addressed in Rel-17 NTN WI. Coordination and avoiding overlap with other WIs and WGs is recommended. But no detail design or agreement is achieved, so we suggest to wait for further progress of NTN group. |
| Samsung | See comments | Can be but we are not sure whether NTN gap can be considered within the scope of MGE concurrent gaps. Note that the details on NTN gap are still under discussion so we prefer to wait until we get a clear picture on it. |
| Ericsson | Maybe | Gap-related progress in the NTN WI might still not be sufficient as pointed out above. Perhaps we could wait for some further progress on that WI. |
| Apple | No | From our understanding based on NTN gap progress, it is about associating multiple SMTC with a gap. Then it is not related to gap enhancements. |
| ZTE | No | Agree with others that the progress in NTN WI is not sufficient. On the other hand, one gap pattern with multiple gapOffsets may still be considered as multiple gap patterns, so it might be counted in the total number of configured concurrent gaps. |
| Nokia | Maybe | Agree with Ericsson that let's wait until NTN progresses a bit more to determine this. |
| MediaTek | Maybe | We also think we should wait more progress on NTN gap design. |

Summary 5:

Some observation is made based on the comments above.

Observation 7: There is no enough agreement on NTN measurement gap yet.

## 3.4 Maximum number of parallel gaps

There are some proposals to clarify the maximum number of parallel gaps.

| Companies | Proposals |
| --- | --- |
| Ericsson [1] | Proposal 4 Use this activity as a means of clarifying the maximum number of parallel (independent) gap patterns that need to be supported by the specifications, considering all Rel-17 WIs. |
| Huawei [4] | Proposal 4: Total number of multiple gaps configured to the UE (regardless of the features enabled or the purpose of gaps) shall not exceed the limitation set in the MG enhancement WI for concurrent MGs. |
| Vivo [5] | Proposal4：Send LS to RAN4 to ask whether all kinds of gap can be configured together for the UE and how to handle the gap collision if different gaps are configured together and there are overlap between them. |
| Nokia [6] | Proposal 1: RAN2 to clarify whether UE capable of concurrent measurement gaps covers any case where measurement gaps can be used concurrently (i.e. NTN, MUSIM, UL gaps). |
| Apple [7] | Proposal 2: RAN2 aims for no impact to legacy measurement gap before RAN4 concludes on simultaneous configuration on legacy measurement gap and new gaps. Proposal 5: RAN2 should ask RAN4 the maximum number of gaps or maximum percentage of gap duration can be configured simultaneously to UE. |

Some companies think that we should ask RAN4 and some company suggest to use the limitation of concurrent gap from MGE WI. Rapporteur thinks it may be premature to decide an exact number at this moment but anyway let’s collect companies views first.

Question 6: Companies are invited to comment the maximum number of parallel gaps could be configured/activated (considering all R17 new gaps and legacy gap). Do you agree to ask RAN4?

| Company | Comments |
| --- | --- |
| vivo | We should give the initialized discussed about this question in 8.0.3 section and Send LS to ask RAN4 for the whole picture. |
| OPPO | Asking RAN4 is more desirable as RAN2 doesn’t know the co-existence rules among different type of gaps. |
| Huawei, HiSilicon | We think the maximum number shall not exceed the limitation set in the MG enhancement WI for concurrent MGs. No need to ask RAN4 since RAN4 may not be interested in coordinating the multiple features. |
| QCOM | The max number is determined by the concurrent gaps limit, which it seems still under discussion in RAN4. We should wait till we receive further details from RAN4. |
| Intel | I think this should be RAN4 decision. |
| CATT | Leave it to RAN4. |
| Samsung | Our understanding is that the concurrent gap limitation in the MGE WI is only for RRM measurements, not for other use cases (e.g. MUSIM, NTN, FR2 UL gap). Depending on which feature/WI can be considered within the scope of MGE concurrent gap objective we can then ask RAN4 if any input is needed. |
| Ericsson | A back-and-forth LS exchange with RAN4 might not be desirable given the time constraints we have. On this matter, RAN4 has agreed not to discuss MUSIM and NTN gaps together with concurrent gaps in the Rel-17 MGE WI. Perhaps, we can use this activity to discuss the max number based on the valid scenarios case by case. |
| Apple | We think this should be decided by RAN4. |
| ZTE | The number of parallel gaps should be decided by RAN4, but RAN2 can provide guidance to RAN4 that which features may be enabled at the same time. |
| Nokia | At least one "classic" measurement gap must be available regardless of which other gaps are configured to support legacy operation. If using any of the Rel-17 features would require NW to deconfigure "classic" measurement gaps, those feature would likely never be used. Overall, we think a limit of 2-3 gap "purposes" could be considered. It's best to keep this as simple as possible. Note also that it's unlikely that RAN4 discusses other gaps than MG enhancement aspects, so this may need to be a RAN2 decision in the end. |
| MediaTek | We think simultaneous configuration and maximum number of configured gap pattern should of course be discussed. However, we don’t think it is mature enough to ask RAN4 now. |

Summary 6:

Some observation is made based on the comments above.

Observation 8: There is no clear consensus on the maximum number of parallel gap patterns could be configured. Some companies suggest to ask RAN4 but there is no enough support to send LS at this meeting.

## 3.5 MAC CE Control on gap activation

The following are the proposals related to MAC CE control of gap activation/deactivation.

| Companies | Proposals |
| --- | --- |
| MTK [2] | Proposal 3: Do NOT introduce common MAC CE framework to activate/deactivate all (new) gaps. If needed, new MAC CE could be introduced in individual WI and control its own gap(s). |
| Vivo [5] | Proposal2: Whether MAC CE activation/deactivation is used can be discussed by separated WI. |

Currently, MAC CE activation/deactivation is discussed in ePOS gap and FR2 UL gap. It is not so clear whether other features should follow the same conclusion and use the MAC CE control. It seems not so straightforward to enable MAC CE control for other features. Two companies suggest that it should be discussed in related WI and should NOT simply enable MAC CE control for all gap features.

Question 7: Do companies agree that “Whether MAC CE activation/deactivation is used can be discussed by separated WI”?

| Company | Agreed or not | Comments |
| --- | --- | --- |
| vivo | yes |  |
| OPPO | Agree |  |
| Huawei, HiSilicon | Agree |  |
| Qcom | Yes |  |
| Intel | Yes |  |
| CATT | Agree |  |
| Samsung | Agree |  |
| Ericsson | Agree |  |
| Apple | Agree |  |
| ZTE | Agree |  |
| Nokia | Agree | MAC CE-based activation/deactivation delay reduction is ~7ms: RRC processing delay is 10ms and MAC CE delay is 3ms. Whether this is too much delay can depend on the gap use case. |
| MediaTek | Agree |  |

Summary 7:

Some observation is made based on the comments above.

Observation 9: All companies agree that “Whether MAC CE activation/deactivation is used can be discussed by separated WI”.

## 3.6 Other details

There are some other proposals that rapporteur think it belongs to ASN.1 code discussion. It is suggested to discuss this later.

| Companies | Proposals |
| --- | --- |
| Huawei [4] | Proposal 2: In Rel-17, the configuration signalling of MG should consider the various combinations of these features (MG enhancement, NTN, Positioning, MUSIM) except for the case of simultaneous enabling of NTN and Positioning. Proposal 3: Concurrent gaps can be associated to use cases. The use case can be RS type like SSB, CSI-RS, PRS, or RAT type, or specific scenario like MUSIM. |
| Vivo [5] | Proposal1: To discuss whether the common RRC gap configuration can include the following IEs - Measurement gap ID - Gap purpose (MUSIM, UL gap) or related RS type or Frequency or BWP/Cell - Gap parameters - [length, period, offset, interruption] - Timing reference |
| Nokia [6] | Proposal 3: Add an additional gap configuration for Rel-17 gaps. Proposal 5: Model pre-configured gaps as additional configuration within existing GapConfig. Proposal 6: Create new gap configuration for NCSG. Proposal 7: Model concurrent gaps using legacy per-UE/per-FR gaps together with an "additional" gap types for each case in the Rel-17 signalling. Proposal 8: RAN2 to discuss whether the proposed configuration structure in Annex B can be used as baseline for the common configuration development. |

Finally, companies could provide other comments on gap coordination if needed.

Question 8: Any other comments or suggestions on gap coordination?

| Company | Comments |
| --- | --- |
| vivo | We should give harmonized ASN.1 for all gap types. |
| QCOM | Very crucial item is to ensure no collision among features when more than one is configured. In addition each feature can be deployed independently. |
| Intel | I think HW proposal to add use case can be consider where other WI may be able to use the same framework. Vivo parameters look ok to us. Gap purpose is similar to vivo use case. |
| Nokia | The ASN.1 proposal we had was one example: We are open to discuss the exact details based on agreed principles. |

Summary 8:

Rapporteur believes that the ASN.1 details could be discussed later. The critical issue mentioned by QC (no collision among features) is some already discussed in previous questions.

# 4 Conclusions

In section 3, we discussed the following R17 gap related features.

Feature 1 – Pre-Configured gap

Feature 2 – Concurrent Gap

Feature 3 – Network control small gap (NCSG)

Feature 4 – ePOS gap (Pre-configured MG for PRS)

Feature 5 – NTN Gap

Feature 6 – MUSIM gap

Feature 7 – UL FR2 gap

The following observations are made.

Observation 1: Most companies think that UL FR2 gap is independent from other gap features. (i.e. separate configuration for UL FR2 gap and enabling the UL FR2 gap or not does not conflict with other gap features from RAN2 perspective).

Observation 2: Several companies mentioned that common configuration structure could be considered. However, there is no much intention to do ASN.1 merge at this moment (individual feature is not stable enough).

Observation 3: Most companies want to clarify how many gaps could be configured simultaneously. For example, which features may be configured at the same time and is there any “collision” between different gaps features?

Observation 4: Two different aspects are mentioned for “collision” between gap features

- Case a: Two gap cannot be configured together. Simultaneous configuration may cause some issue in procedure part.

- Case b: Two gap could be configured together but UE behavior on the overlapping period in time domain may needed further clarification.

Observation 5: There is no clear consensus on whether we should consider ePOS gap as pre-configured MG or concurrent MG. It seems that the behavior of ePOS gap will be different from either pre-configured MG or concurrent MG.

Observation 6: Some companies think periodic MUSIM gaps could consider as concurrent gap. However, there is also strong resistance to include MUSIM gap as part of MGE concurrent gap at this moment.

Observation 7: There is no enough agreement on NTN measurement gap yet.

Observation 8: There is no clear consensus on the maximum number of parallel gap patterns could be configured. Some companies suggest to ask RAN4 but there is no enough support to send LS at this meeting.

Observation 9: All companies agree that “Whether MAC CE activation/deactivation is used can be discussed by separated WI”.

With above observations, rapporteur does not think we could make some solid agreement. In general, it seems that we should wait more progress on each gap feature and discuss in next meeting.

The suggested proposal is

Proposal 1: Continue to discuss each gap feature in individual WI with the following understandings.

- Whether to support MAC CE activation/deactivation of the gap is discussed independently in each WI. There is no need to have common MAC CE framework.

- RRC configuration for gap feature could be progressed separately in each WI. However, RAN2 may use common RRC configuration structures for different gaps once the relation between each gap feature is clear.

Proposal 2: On gap coordination, RAN2 to discuss (but not limit to) the following aspects

- Could two gap features be configured together? Is there any collision on procedure part when more than one is configured ? (to identify the possible gap combinations)

- How many gaps could be configured/activated at the same time ? Does RAN2 have to consult RAN4 for this number ?

- Is there a need on common RRC configuration for all or at least some gap feature? If yes, which gap features are included ?

- Does RAN2 have to clarify the UE behavior while two gaps are overlapped in time domain ?

# 5 References

[1] R2-2201565	Gaps coordination	Ericsson

[2] R2-2201238	Discussion on gap coordination	MediaTek Inc.

[3] R2-2200221	Joint discussion for measurement gaps	Intel Corporation

[4] R2-2200292	Discussion on gaps coordination	Huawei, HiSilicon

[5] R2-2200588	Discussion on Gap coordination 	vivo

[6] R2-2201057	Commonalities with measurement gaps in Rel-17	Nokia, Nokia Shanghai Bell

[7] R2-2201109	Discussion on gap features	Apple