3GPP TSG-RAN WG2 Meeting #116bis electronic	R2-220xxxx

Online, Jan 17 – 25, 2022

Agenda item:	8.24.1

Source: 	China Telecom

Title:	Draft-Summary of [AT116bis-e][036][NR17] UL TX switching Enh (China Telecom)

WID/SID:	NR_RF_FR1_enh

Document for:	Discussion and Decision

# Introduction

This document is the report of the following email discussion:

[AT116bis-e][036][NR17] UL TX switching Enh (China Telecom)

Scope: Treat R2-2200120, R2-2201499, R2-2201500, R2-2201501, R2-2200516. R2-2200519, R2-2200517, R2-2200518, Take into account R2-2200095.

1: Determine agreeable parts, parts that need CB on-line if any 2: agree updated Running CRs that reflect agreeable parts / agreements.

Intended outcome: 1 Report, 2 endorsed running CRs

Deadline: 1 for online CB Mon W2 if CB is needed, 2 EOM

For Phase 1 discussion, rapporteur suggests companies provide comments before Thursday W1 UTC 13:00 (Jan 20), so that we can try to figure out the agreeable parts and start to discuss how to update the Running CRs earlier. The Phase 1 report will be submitted before online CB Mon W2 (Jan 24).

For Phase 2 discussion, rapporteur plans to start it at Friday W1 (Jan 21), which can focus on updating the Running CRs based on the agreeable parts discussed in Phase 1. For the parts that need further online discussion, we can continue the Running CRs updating work based on the online agreements afterwards.

Contact from companies

| Company | Contact: Name (E-mail) |
| --- | --- |
| Qualcomm Incorporated | Masato KITAZOE (mkitazoe@qti.qualcomm.com) |

# Phase 1 Discussion

In last RAN2 meeting, the discussion on RRC configuration of Rel-17 UL Tx switching enhancement was postponed for further RAN1 progress. Considering Rel-17 RAN1 part of RF requirements enhancement for NR frequency range 1 has completed as indicated in RAN1 status report RP-212642, RAN2 needs to discuss how to capture the RAN1 agreements and whether there are extra parameters to specify from RAN2 perspective.

Besides, in RAN4#101-e meeting, RAN4 introduced new UL MIMO coherence capabilities for Rel-17 UL Tx switching enhancements and sent an LS (R2-2200120-R4-2120039) to ask RAN2 to take into consideration the RAN4 agreements to introduce UE capability signalling.

Based on the current progress on Rel-17 UL Tx switching enhancements in RAN1 and RAN4, and the company contributions submitted in this meeting, the following issues may need further discussion.

- Regarding RRC configuration, the following aspects should be considered:

(1)	How network indicates/UE determines 2Tx-2Tx switching is configured (with differentiation from 1Tx-2Tx switching).

(2)	How network indicates/UE determines 2 CCs on band B is configured for 1Tx-2Tx switching/2Tx-2Tx switching.

(3)	Other configuration introduced by RAN1

- Regarding UE capability reporting, the leftover includes:

(1)	UL MIMO coherent capability for UL Tx switching introduced by RAN4.

(2)	FFS captured in the endorsed running CR, i.e. whether switching option can be reported differently for 1Tx-2Tx and 2Tx-2Tx.

## RRC configuration

### How to indicate 1Tx-2Tx or 2Tx-2Tx switching is configured

In RAN1#107-e meeting, RAN1 reached the following agreement to introduce a new RRC parameter for 2Tx-2Tx switching.

Agreement For a UE capable of 2Tx-2Tx switching and configured with UL Tx switching via uplinkTxSwitching, to differentiate the switching delay for 1Tx-2Tx switching from that for 2Tx-2Tx switching, a new RRC parameter is used to indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode. - If 1Tx-2Tx mode is derived by the new RRC parameter, then there is one uplink (or one uplink band in case of intra-band) configured with uplinkTxSwitching, on which the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources should be 1 and non-codebook based UL MIMO is not configured. RAN1 assume the uplink is configured with RRC parameter “carrier1” by RAN2. - The default value of the new RRC parameter is 1Tx-2Tx switching mode. - In a configured switching mode, the switching gap duration for a triggered uplink switching is equal to the switching time capability value reported for the switching mode. - Note: This RRC parameter doesn’t imply any restriction on application of non-codebook transmission together with UL Tx switching.

The above agreement is stable in RAN1 with only some remaining discussion on the detailed description of UE behaviours/NW restrictions after reception of the parameter. From RAN2 perspective the meaning of the parameter is quite straightforward, i.e. the UL Tx switching mode is 2Tx-2Tx switching to differentiate from 1Tx-2Tx switching.

Based on the above RAN1 agreement, R2-2201499 proposes to add a Rel-17 field in CellGroupConfig to indicate the Tx switching mode, and the existing uplinkTxSwitchingPeriodLocation and uplinkTxSwitchingCarrier within UplinkTxSwitching can be reused without any change.

Below is the corresponding TP to show the change to TS38.331.

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                        OPTIONAL,   -- Need N

rlc-BearerToReleaseList                    SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity                  OPTIONAL,   -- Need N

mac-CellGroupConfig                        MAC-CellGroupConfig                                                     OPTIONAL,   -- Need M

physicalCellGroupConfig                    PhysicalCellGroupConfig                                                 OPTIONAL,   -- Need M

spCellConfig                               SpCellConfig                                                            OPTIONAL,   -- Need M

sCellToAddModList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig                       OPTIONAL,   -- Need N

sCellToReleaseList                         SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex                        OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent                ENUMERATED {true}                                                   OPTIONAL    -- Cond BWP-Reconfig

]],

[[

bap-Address-r16                            BIT STRING (SIZE (10))                                                  OPTIONAL,   -- Need M

bh-RLC-ChannelToAddModList-r16             SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelConfig-r16 OPTIONAL,   -- Need N

bh-RLC-ChannelToReleaseList-r16            SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelID-r16     OPTIONAL,   -- Need N

f1c-TransferPath-r16                       ENUMERATED {lte, nr, both}                                              OPTIONAL,   -- Need M

simultaneousTCI-UpdateList1-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousTCI-UpdateList2-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList1-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList2-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

uplinkTxSwitchingOption-r16                ENUMERATED {switchedUL, dualUL}                                         OPTIONAL,   -- Need R

uplinkTxSwitchingPowerBoosting-r16         ENUMERATED {enabled}                                                    OPTIONAL    -- Need R

]],

[[

reportUplinkTxDirectCurrentTwoCarrier-r16  ENUMERATED {true}                                                       OPTIONAL    -- Need N

]]

[[

uplinkTxSwitching-2T2T-Mode-r17-r17     BOOLEAN                             OPTIONAL   -- Need R

]]

}

uplinkTxSwitching-2T2T-Mode Indicates 2Tx-2Tx UL Tx switching is configured for inter-band UL CA or SUL as specified in TS 38.214 [19].

Companies are welcome to give comments on P1 within R2-2201499 for the configuration of 2Tx-2Tx switching.

Q1: Do companies agree P1 within R2-2201499 as it is: To configure 2Tx-2Tx switching, the new RRC parameter of 2Tx-2Tx switching mode agreed by RAN1 is included in CellGroupConfig, and the existing UplinkTxSwitching can be reused without change?

| Company | Agree/ Not agree | Comments |
| --- | --- | --- |
| China Telecom | Agree | We understand that RAN1 agreement is stable and the meaning of the newly introduced parameter is quite straightforward. We support to include the new RRC parameter of 2Tx-2Tx switching mode agreed by RAN1 in CellGroupConfig, and the existing UplinkTxSwitching can be reused without change. |
| Qualcomm Incorporated | Not agree | It is not entirely clear to us why this parameter is needed if 1Tx-2Tx vs 2Tx-2Tx can be known based on the number of antenna ports. RAN1 text says “The default value of the new RRC parameter is 1Tx-2Tx switching mode”, but it is not clear from the proposed text above how 1Tx-2Tx mode is configured. NOTE: As we discussed previously, we should discourage RAN1 from defining “default” behaviour associated with RRC parameter in their specification. |

### UL Tx switching between 1 carrier in band A and 2 carriers in band B

Regarding the RRC configuration for UL Tx switching between 1 carrier in band A and 2 carriers in band B, R2-2201499 gives the following proposals.

Proposal 2: For UL Tx switching between 1 carrier in band A and 2 carriers in band B, 3 uplinks are configured in legacy way, i.e. one uplink band (Band A) is configured with 1 UplinkConfig, and the other band (Band B) is configured with 2 UplinkConfig.

Proposal 3: For UL Tx switching between 1 carrier in band A and 2 carriers in band B, the field uplinkTxSwitchingPeriodLocation is reused to configure period location. The configuration to the 2 uplinks in band B (i.e. the band capable of 2Tx) should be aligned.

Proposal 4: For UL Tx switching between 1 carrier in band A and 2 carriers in band B, the field uplinkTxSwitchingCarrier is reused. The configuration to the 2 uplinks in band B (i.e. the band capable of 2Tx) should be carrier2.

Below is the corresponding TP to show the change to TS38.331.

uplinkTxSwitchingPeriodLocation Indicates whether the location of UL Tx switching period is configured in this uplink carrier in case of inter-band UL CA, SUL, or (NG)EN-DC, as specified in TS 38.101-1 [15] and TS 38.101-3 [34]. In case of inter-band UL CA or SUL, network configures this field to TRUE for the uplink carrier(s) on one band one of the uplink carriers involved in dynamic UL TX switching and configures this field in the other carrier(s) on the other band to FALSE. In case of (NG)EN-DC, network always configures this field to TRUE for NR carrier (i.e. with (NG)EN-DC, the UL switching period always occurs on the NR carrier).

uplinkTxSwitchingCarrier Indicates that the configured carrier is carrier1 or carrier2 for dynamic uplink Tx switching between 2 uplink carriers, as defined in TS 38.101-1 [15] and TS 38.101-3 [34]. In case of inter-band UL CA or SUL, network configures one of the two uplink carriers involved in dynamic UL TX switching as carrier1 and the other as carrier2. In case of (NG)EN-DC, network always configures the NR carrier as carrier 2. For dynamic uplink Tx switching between 2 bands with 3 uplink carriers as defined in TS 38.101-1 [15], the field is set to carrier2 for the uplink carriers in the band capable of two transmit antenna connectors.

Companies are welcome to give comments on the proposals within R2-2201499 for UL Tx switching between 1 carrier in band A and 2 carriers in band B.

Q2: Do companies agree P2 within R2-2201499 as it is: For UL Tx switching between 1 carrier in band A and 2 carriers in band B, 3 uplinks are configured in legacy way, i.e. one uplink band (Band A) is configured with 1 UplinkConfig, and the other band (Band B) is configured with 2 UplinkConfig?

| Company | Agree/ Not agree | Comments |
| --- | --- | --- |
| China Telecom | Agree | We agree with the proposal. For UL Tx switching between 1 carrier in band A and 2 carriers in band B, 3 uplinks can be configured in legacy way, and only some clarifications are needed. |
| Qualcomm Incorporated | Agree |  |

Q3: Do companies agree P3 within R2-2201499 as it is: For UL Tx switching between 1 carrier in band A and 2 carriers in band B, the field uplinkTxSwitchingPeriodLocation is reused to configure period location. The configuration to the 2 uplinks in band B (i.e. the band capable of 2Tx) should be aligned?

| Company | Agree/ Not agree | Comments |
| --- | --- | --- |
| China Telecom | Agree | We support this proposal. For UL Tx switching between 1 carrier in band A and 2 carriers in band B, the field uplinkTxSwitchingPeriodLocation can be reused to configure period location, and the configuration to the 2 uplinks in band B should be aligned. |
| Qualcomm Incorporated | Agree, but | For uplinkTxSwitchingPeriodLocation, the proposed text does not sufficiently clarify the same value is configured for all carriers on one band in case of UL Tx switching with 3 UL carriers. |

Q4: Do companies agree P4 within R2-2201499 as it is: For UL Tx switching between 1 carrier in band A and 2 carriers in band B, the field uplinkTxSwitchingCarrier is reused. The configuration to the 2 uplinks in band B (i.e. the band capable of 2Tx) should be carrier2?

| Company | Agree/ Not agree | Comments |
| --- | --- | --- |
| China Telecom | Agree | We support this proposal. For UL Tx switching between 1 carrier in band A and 2 carriers in band B, the field uplinkTxSwitchingCarrier can be reused, and the configuration to the 2 uplinks in band B should both be carrier2. |
| Qualcomm Incorporated | Agree |  |

### Other configuration introduced in RAN1

In RAN1#106bis-e meeting, one RRC parameter was agreed for determination of the state of Tx chains for 2Tx-2Tx switching and captured in RAN1 RRC parameter list R1-2112976 as below.

| NR_RF_FR1_enh-Core | Uplink Tx switching enhancements | uplinkTxSwitchingdualULTxState | new | For UL-CA option 2 and 2Tx-2Tx switching, indicate the state of Tx chains if the state of Tx chains after the UL Tx switching is not unique. | [{1T, 2T}] | [2T] | UE-specific | Agreement: Ÿ For UL-CA Option2, if UL Tx switching is triggered for 1-port transmission on a carrier and the state of Tx chains after the UL Tx switching is not unique, introduce a new RRC parameter to configure between 1) and 2) ‐ 1) The state of Tx chains supporting 2Tx transmission on the carrier is assumed. ‐ 2) 1Tx on carrier 1 and 1Tx on carrier 2 is assumed. | New stable |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Regarding the configuration of the new RRC parameter introduced by RAN1, R2-2201499 gives the following proposal.

Proposal 5: The new RRC parameter uplinkTxSwitchingdualULTxState should be included in CellGroupConfig to configure the state of Tx chains for UL-CA option2 in case of 2Tx-2Tx switching.

Companies are welcome to give comments on P5 within R2-2201499 for other configuration introduced by RAN1.

Q5: Do companies agree P5 within R2-2201499 as it is: The new RRC parameter uplinkTxSwitchingdualULTxState should be included in CellGroupConfig to configure the state of Tx chains for UL-CA option2 in case of 2Tx-2Tx switching?

| Company | Agree/ Not agree | Comments |
| --- | --- | --- |
| China Telecom | Agree | We support to include the new RRC parameter uplinkTxSwitchingdualULTxState in CellGroupConfig to configure the state of Tx chains for UL-CA option2 in case of 2Tx-2Tx switching. |
| Qualcomm Incorporated | Agree |  |

## UE capability reporting

### UL MIMO coherence capability for Tx switching

- Baseline for the Rel-17 UL Tx switching coherence capability discussion

In RAN2#115-e meeting, an LS (R2-2106952) from RAN4 on UL MIMO coherence for Rel-16 Tx switching was noted and the corresponding CRs on introducing UL MIMO coherence capability for Tx switching were postponed to wait for potential RAN1 progress.

In RAN2#116-e meeting, the following agreement was reached in RAN2#116-e meeting.

- [013] Both endorsed, not for RP, final version later. The CRs R2-2110483 and R2-2110484 correctly captures RAN4 request in their LS. CRs can be revisited after RAN1 discussion on the legacy MIMO coherence capability.

In RAN4#101-e meeting, RAN4 reached the following agreements on UL MIMO coherence capabilities for Rel-17 UL Tx switching enhancements and sent an LS (R2-2200120-R4-2120039) to ask RAN2 to take into consideration the RAN4 agreements to introduce UE capability signalling.

Regarding the UL-MIMO coherence for the above Rel-17 Tx switching scenarios, RAN4 reached the following agreements: - For 3CC (within 2 bands) 1Tx-2Tx switching - The Rel-16 per BC UE capability for 2CC 1Tx-2Tx switching can be applied, i.e., the same capability applies to both Rel-16 and Rel-17 1Tx-2Tx switching. - If the per BC UL-MIMO capability for 1Tx-2Tx switching is absent, the existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the 2Tx-capable bands for 1Tx-2Tx switching. - For 2CC and 3CC 2Tx-2Tx (within 2 bands) switching - Introduce new per band per BC UL-MIMO coherence capability, and the same capability applies to both 2CC and 3CC (within 2 bands) 2Tx-2Tx switching. - If the per band per BC UL-MIMO coherence capability is absent, the per BC UL-MIMO coherence capability for 1Tx-2Tx switching is applicable to 2Tx-2Tx switching. - If both the per band per BC UL-MIMO coherence capability and per BC UL-MIMO coherence are absent, the existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the bands for 2Tx-2Tx switching. - The above UL-MIMO coherence capability for Tx switching applies when UE is configured with uplink switching, and uplink switching is triggered by the switching mechanisms specified in RAN1 between last transmitted SRS and scheduled PUSCH transmission.

In RAN1#107-e meeting, RAN1 discussed on the legacy MIMO coherence capability issues, but did not reach a consensus on whether to introduce a new per-FS capability indicating UL MIMO coherence at least in Rel-16. Based on the discussion, RAN1 sent an LS (R2-2200072-R1-2112778) to inform RAN2 the following conclusion on MIMO coherence capability issues.

Following are some notes regarding the updates on the NR features list: (R1-2112778) - RAN1 has discussed on whether to introduce a new per-FS capability of UL MIMO coherence based on R1-2112185, but RAN1 did not reach a consensus to introduce a new per-FS capability indicating UL MIMO coherence at least in Rel-16.

As per RAN1 discussion, the discussion scope is limited to 4Tx UL MIMO coherence with no impact on Rel-16 UL Tx switching, which is a different issue from RAN4 discussion on UL MIMO coherence capability for UL Tx switching. Besides, according to the RAN4 LS on Rel-17 UL Tx switching UL MIMO coherence capability, the Rel-17 capability has some relation with the Rel-16 capability.

Considering there is no plan to treat Rel-16 discussion in this RAN2 meeting, R2-2200519 suggests first taking the endoresed Rel-16 CRs as baseline, on top of which the Rel-17 capability can be discussed and formulated, and gives the following proposal.

Proposal 1: Taking the endorsed Rel-16 CRs R2-2110483 and R2-2110484 as baseline for the Rel-17 UL Tx switching coherence capability discussion.

Companies are welcome to give comments on P1 within R2-2200519.

Q6: Do companies agree P1 within R2-2200519 as it is: Taking the endorsed Rel-16 CRs R2-2110483 and R2-2110484 as baseline for the Rel-17 UL Tx switching coherence capability discussion?

| Company | Agree/ Not agree | Comments |
| --- | --- | --- |
| China Telecom | Agree | We support to take the endorsed Rel-16 CRs as baseline for the Rel-17 UL Tx switching coherence capability discussion. |
| Qualcomm Incorporated | Agree | This is fine for now. We understand RAN1 is discussing introducing per band per BC capability for MIMO coherence in non-Tx switching case, which may have to be taken into account later. |

- For Rel-17 1Tx-2Tx switching, i.e. 3CC (within 2 bands) 1Tx-2Tx switching

RAN4 has already agreed that the Rel-16 per BC UL MIMO coherence capability for 2CC 1Tx-2Tx switching can be applied for Rel-17 3CC (within 2 bands) 1Tx-2Tx switching.

Based on RAN4 agreement, R2-2200519 and R2-2201499 give similar proposals as below.

Proposal 2 (within R2-2200519): The Rel-16 capability uplinkTxSwitching-PUSCH-TransCoherence-r16 endorsed in RAN2#116-e can be used to report UL MIMO coherence capability for Rel-17 3CC (within 2 bands) 1Tx-2Tx switching as well.

Proposal 8 (within R2-2201499): The Rel-16 per-BC UL MIMO coherent capability introduced for 1Tx-2Tx switching between 2 uplinks applies to Rel-17 UL Tx switching between 2 bands with 3 uplinks.

An example of TP for TS 38.306 based on the endorsed CR R2-2110483 is given below:

| uplinkTxSwitching-PUSCH-TransCoherence-r16 Indicates support of the uplink codebook subset for the carrier(s) on the band capable of two antenna connectors for UL 1Tx-2Tx switching, when UE is configured with uplink Tx switching with parameter uplinkTxSwitching-r16 and uplink Tx switching is triggered between last transmitted SRS and scheduled PUSCH transmission, as specified in TS 38.101-1 [2]. UE indicated support of full coherent codebook subset shall also support non-coherent codebook subset. If the field is absent, the UE capability reported in pusch-TransCoherence is applied when the uplink switching is triggered between last transmitted SRS and scheduled transmission. | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

Companies are welcome to give comments on the above proposals for Rel-17 1Tx-2Tx switching UL MIMO coherence capability reporting.

Q7: Do companies agree that the Rel-16 per-BC UL MIMO coherent capability introduced for 1Tx-2Tx switching between 2 uplinks applies to Rel-17 UL 1Tx-2Tx switching between 2 bands with 3 uplinks?

| Company | Agree/ Not agree | Comments |
| --- | --- | --- |
| China Telecom | Agree | Based on RAN4 LS, the Rel-16 per-BC UL MIMO coherent capability introduced for 1Tx-2Tx switching between 2 uplinks can also apply to Rel-17 UL 1Tx-2Tx switching between 2 bands with 3 uplinks. |
| Qualcomm Incorporated | Agree | We understand the change is done only in release-17. |

- For Rel-17 2Tx-2Tx switching, i.e. 2CC and 3CC (within 2 bands) 2Tx-2Tx switching

Based on RAN4 agreements, a new per band per BC UL MIMO coherence capability shall be introduced for Rel-17 2Tx-2Tx switching, which can be applied to both 2CC and 3CC (within 2 bands) 2Tx-2Tx switching. Considering the potential spec maintenance work in the future, R2-2200519 gives the following proposal.

Proposal 3: Introduce a new field UplinkTxSwitchingBandParameters-v17xx to report the UL Tx switching specific band parameters for a given band combination, which comprises of the following parameters:

-	bandIndex-r17 indicates a band on which UE supports dynamic UL Tx switching with another band in the band combination.

-	uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 is used to report UL MIMO coherence capability for Rel-17 2Tx-2Tx switching.

An example of TP for TS 38.331 is given below:

BandCombination-UplinkTxSwitch-v17xx ::=    SEQUENCE {

supportedBandPairListNR-v17xx         SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-v17xx         OPTIONAL,

uplinkTxSwitchingBandParametersList-r17     SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF ULTxSwitchingBandParameters-r17         OPTIONAL

}

ULTxSwitchingBandParameters-r17 ::=       SEQUENCE {

bandIndex-r17                                      INTEGER(1..maxSimultaneousBands),

uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17     ENUMERATED {nonCoherent, fullCoherent}            OPTIONAL

}

R2-2201499 shares similar views and gives the following proposals.

Proposal 7: Add a new per-band per BC UE capability in BandCombination-UplinkTxSwitch to indicate UL MIMO coherent capability specific for 2Tx-2Tx switching.

Proposal 9: The Rel-17 per-band per-BC UL MIMO coherent capability applies to Rel-17 UL 2Tx-2Tx switching between 2 uplinks and switching between 2 bands with 3 uplinks.

Companies are welcome to give comments on the above proposals for Rel-17 2Tx-2Tx switching UL MIMO coherence capability reporting.

Q8: Do companies agree to add a new per-band per BC UE capability in BandCombination-UplinkTxSwitch to indicate UL MIMO coherent capability specific for 2Tx-2Tx switching?

| Company | Agree/ Not agree | Comments |
| --- | --- | --- |
| China Telecom | Agree | We support to add a new per-band per BC UE capability in BandCombination-UplinkTxSwitch to indicate UL MIMO coherent capability specific for 2Tx-2Tx switching. |
| Qualcomm Incorporated | Agree |  |

Q9: For the detail design of UL MIMO coherent capability reporting for 2Tx-2Tx switching, do companies agree P3 within R2-2200519 as it is: Introduce a new field UplinkTxSwitchingBandParameters-v17xx to report the UL Tx switching specific band parameters for a given band combination, which comprises of the following parameters:

-	bandIndex-r17 indicates a band on which UE supports dynamic UL Tx switching with another band in the band combination.

-	uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 is used to report UL MIMO coherence capability for Rel-17 2Tx-2Tx switching?

| Company | Agree/ Not agree | Comments |
| --- | --- | --- |
| China Telecom | Agree | We think the above proposals can correctly capture RAN4 agreements on UL MIMO coherence capability for Rel-17 2Tx-2Tx switching. |
| Qualcomm Incorporated | Not agree | ASN.1 detail requires more discussion. Given RAN1 is still discussing per band per BC capability for MIMO coherence in non-Tx switching case, it may be better to extend the BandParameters in BandCombinationList. |

### FFS captured in the endorsed running CR

In the endorsed running CR R2-2109225, there is one FFS on switching option reported for 2Tx-2Tx switching. R2-2201499 suggests that the FFS should be deleted from the running CR, since RAN1 already claimed the RAN1 work is completed and no new UE capability is agreed by RAN1.

Proposal 6: Remove the sentence of “FFS: whether switching option can be reported differently for 1T2T and 2T2T” from the running CR.

Companies are welcome to give comments on P6 within R2-2201499.

Q10: Do companies agree P6 within R2-2201499 as it is: Remove the sentence of “FFS: whether switching option can be reported differently for 1T2T and 2T2T” from the running CR?

| Company | Agree/ Not agree | Comments |
| --- | --- | --- |
| China Telecom | Agree | In our understanding, it is not necessary to introduce a new UE capability to report different switching option supported by the UE for 1Tx-2Tx and 2Tx-2Tx switching. If UE can support 2Tx-2Tx switching and Tx switching between 2 UL bands for Rel-17 Tx switching, it can easily support 1Tx-2Tx switching between 2 UL carriers for Rel-16 Tx switching for the same option. |
| Qualcomm Incorporated | Not agree | This is still under discussion in RAN1. |

## Any others issues

Moderator understand the R2-2201501 and R2-2200516 are resubmissions of the endorsed running CR ported on the latest version of TS 38.331 and TS 38.306.

If companies have any concerns on either contribution or any other issues, please comment in below table.

| Company | Comments |
| --- | --- |

# Phase 2 Discussion

TBD…

# Conclusion

TBD…

# Reference

R2-2200120	LS on UL-MIMO coherence for Rel-17 Tx switching (R4-2120039; contact: China Telecom)	RAN4	LS in	Rel-17	NR_RF_FR1_enh-Core	To:RAN2	Cc:RAN1

R2-2200516	Running CR to TS 38.306 to support Tx switching enhancements	China Telecom, Huawei, HiSilicon, Apple, CATT	draftCR	Rel-17	38.306	16.7.0	B	NR_RF_FR1_enh	R2-2110424

R2-2200517	Draft CR to TS 38.306 on UE capability for UL-MIMO coherence for Rel-17 Tx switching	China Telecom, Huawei, HiSilicon	draftCR	Rel-17	38.306	16.7.0	F	NR_RF_FR1_enh

R2-2200518	Draft CR to TS 38.331 on UE capability for UL-MIMO coherence for Rel-17 Tx switching	China Telecom, Huawei, HiSilicon	draftCR	Rel-17	38.331	16.7.0	F	NR_RF_FR1_enh

R2-2200519	Discussion on UL MIMO coherence for UL Tx switching	China Telecom, Huawei, HiSilicon	discussion	Rel-17	NR_RF_FR1_enh

R2-2201499	Remaining issues to support R17 UL Tx switching enhancement	Huawei, HiSilicon, China Telecom, CATT	discussion	Rel-17	NR_RF_FR1_enh

R2-2201500	RRC configuration to support R17 UL Tx switching enhancements	Huawei, HiSilicon, China Telecom, CATT	draftCR	Rel-17	38.331	16.7.0	NR_RF_FR1_enh

R2-2201501	Running CR to TS38.331 to support Tx switching enhancements	Huawei, HiSilicon, China Telecom, Apple, CATT	draftCR	Rel-17	38.331	16.7.0	B	NR_RF_FR1_enh	R2-2109225

R2-2200095	LS on updated Rel-17 LTE and NR higher-layers parameter list (R1-2112977; contact: Ericsson)	RAN1	LS in	Rel-17	NR_feMIMO, NR_ext_to_71GHz, NR_IIOT_URLLC_enh, NR_NTN_solutions, NR_pos_enh, NR_redcap, NR_UE_pow_sav_enh, NR_cov_enh, NR_IAB_enh, NR_SL_enh, NR_MBS, NR_DSS, LTE_NR_DC_enh2, NR_RF_FR1_enh, NR_SmallData_INACTIVE, NB_IOTenh4_LTE_eMTC6, LTE_NBIOT_eMTC_NTN, LTE_terr_bcast_bands_part1	To:RAN2, RAN3	Cc:RAN4