3GPP TSG-RAN WG2 Meeting #116is electronic	R2-2xxxxxx

Online Meeting, Jan 17th – 25th 2022

Agenda item:	8.2.2.3

Source:	Apple Inc

Title:            Report of [AT116bis-e][223][DCCA] MCG failure recovery (Apple)

WID/SID:	LTE_NR_DC_enh2-Core

Document for:	Discussion and Agreement

# 1	Introduction

This document is for summary of the following discussions:

[AT116bis-e][223][DCCA] MCG failure recovery (Apple)

Scope: Discuss whether it's possible to support MCG failure recovery via deactivated SCG (based on contributions to this meeting).

Intended outcome: Discussion summary in R2-2201703.

Deadline: Deadline 3

The participants are invited to leave their contact information in the following table.

| Company | Contact: Name (E-mail) |
| --- | --- |
| A Apple | N naveen.palle@apple.com |
| ZTE | liu.jing30@zte.com.cn |
| Nokia | jarkko.t.koskela@nokia.com |
| Ericsson | Stefan.wager@ericsson.com |

# 2	MCG Failure recovery in deactivated SCG

## 2.1	Any objections?

All the papers submitted by the companies to RAN2-116bis-e meeting have proposal on ways to support MCG link recovery using the deactivated SCG, and the rapporteur do not see papers objecting to support this. However, the rapporteur would like to see if there are companies actually intending to object to this with the below question.

Question 1: Companies are requested to list below, if they intend to object the MCG failure recovery using deactivated SCG procedure.

| Company | Reason for objection |
| --- | --- |
| Huawei, HiSilicon | I don't see how we can do that because: 1) There is only one meeting left 2) We have not finished the design of SCG deactivation/activation for normal cases 3) This MCG failure recovery raises fundamental questions, e.g. some of the TPs say the SCG is activated, others only SRB3, and this is not covered in this discussion |
| Ericsson | Same reasons as mentioned by Huawei |

Summary:

TBD

## 2.2	Approaches to solving MCG failure recovery in deactivated SCG

### 2.2.1 		Self activation of SN by the UE

The basic approach to allowing the UE report the MCG failure information to the MN using the deactivated SCG is with one of the below options. Note that some companies have provided views for the top two approaches below.

- UE activated the SCG by itself (proponents of this approach claim that the main reason is that the UE needs to monitor PDCCH on the PSCell as part of this procedure). There are details on the UE specifics after UE activated the SCG by itself [3][8][10][11]

- UE does not activate the SCG by itself and waits for a RRC message from the NW to activate, but this raised the question of UE monitoring the PDCCH (atleast for RAR if RACH is triggered for eg) [4][5]

- UE uses UAI based approach [2]

The rapporteur thinks that this is one of the main discussion points to resolve with designing this.

Question 2: For the MCG failure recovery with deactivated SCG, which option do you prefer?

- Option 1 The UE activates the SCG by itself

- Option 2 The UE informs the SN of the MCG failure information, but only activates the SCG if the NW provides a configuration (message) activating the SCG

- Option 3 The UE uses the UAI message to the MN to request for SN activation. This message has information for the reason for the request of SN activation: UL data arrival, MCG failure recovery etc.

| Company | Which option | Reasons/comments on your views |
| --- | --- | --- |

| Apple | Option 2 | We do not prefer UE self activation, as it violates the principles we agreed earlier. But we do acknowledge that the UE needs to monitor PDCCH for RAR (if the UE RACHes) for MSG3. But we think that MAC spec can be changed (if needed) to allow the UE to monitor PDCCH, while the UE is still SCG deactivated state. |
| --- | --- | --- |
| ZTE | Either Option 1 or Option 2 | For Option 1, we think the UE activates the SCG autonomously only because the UE has to monitor PDCCH for RAR and subsequent MN RRC message via SCG. The UE is not expected to transmit UL data during the MCG Failure Recovery procedure. After the MN receives the MCGFailureInformation message, if the MN wants to keep the SCG as deactivated state, the MN can response a MN RRCReconfiguration message with SCG-state set to “deactivated”. Option 2 is also acceptable to us. As commented by Apple, we can formulate UE behaviour to allow the UE to monitor PDCCH when MCG Failure Recovery is triggered. Then whether the SCG is activated or deactivated can based on the explicit indication in the RRC response message sent by the MN. |
| Nokia | Option | Agree with Apple |

Summary:

TBD

### 2.2.2 		Only RACH based or SR

Companies in [3][4][5][1][9][11] propose that the UE trigger SR (if valid) and use RACH in case SR is not available or TAT expired earlier. We already agreed that there would NOT be any data to be transmitted on the DRBs during SCG deactivated state, but SRB might need to be transmitted. It would be good to confirm if SR is valid for the UE at SCG deactivated state.

Question 3: If configured for SCG in SCG activated state, is the SR configuration valid to the UE while the SCG is deactivated, to be used for the purpose of transferring the SRB?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Apple | Yes | If the NW has provided SR configuration, the UE is allowed to use it. However, we do understand that NW might intend to have the SR resources used for other UEs. For this reason, a dedicated SCG deactivated config can be given to the UE (answer to Q5 below). Anyway NW is allowed to release all SR configurations in the same RRC message that is used to deactivate the SCG. So the signaling means is there, and so, if NW does NOT remove, the UE is allowed to use the resources in SCG deactivated state. |
| ZTE | Yes with comment | It is possible to use SR when TAT is running, beam failure does not happen and the SR resource is not released for this UE. In our understanding, if network intends to allocate the same SR resource for other UEs, the network can first explicitly release the SR resource by sending RRC message during SCG deactivation state. If majority companies have strong concern on triggering SR, we are also fine with RACH-based approach. |
| Nokia | No strong view | We think use of SR would imply that UE would then monitor PDCCH and therefore would activate SCG by itself, contrary to our response to the previous question. Since monitoring of PDCCH is an explicit part of RACH procedure, we think the specification impacts would be simpler if only RACH is used. |

Summary:

TBD

### 2.2.3 		Validity of the SR

Question 4:  If the answer to Q3 is yes, would you agree that the SR resource would not be considered valid, if the TAT expires in SCG deactivated state (similar to legacy operation)?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Apple | Yes, the UE would release the resources. |  |
| ZTE | Yes |  |

Summary:

TBD

### 2.2.4 		Dedicated configuration to the UE at SCG deactivation

Companies in [11][3] propose that the UE can be use the dedicated configuration provided to it for the purpose of faster MCG failure recovery. Rapporteur thinks an explicit agreement related to this also can help with progressing this procedure.

Question 5 Can the NW be allowed to provide the UE with a dedicated configuration at the time of SCG deactivation, to be used during the SCG deactivated state (for eg., with the purpose that the UE can use this for informing the SCG about MCG failure information)?

- The dedicated configuration includes dedicated RACH resources or SR configuration

- Any other config?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Apple | Yes | The flexibility should be given to the NW. |
| ZTE | Yes | We have the following RAN2 agreement: 9 While the SCG is deactivated, the MN RRC reconfiguration message and the embedded SN RRC reconfiguration message can reconfigure any parameter (any restriction requires an explicit decision). We see no benefit to limit network reconfiguration. So the network should be allowed to reconfigure parameters not only in the message that is used to deactivate SCG, but also the RRC message that sent while the SCG is deactivated. |
| Nokia | Yes |  |

Summary:

TBD

### 2.2.5 		PDCCH monitoring after SR

There is one open item in case the UE does NOT activate the SCG after triggering SR (in case RAN2 agrees to this approach). The rapporteur thinks it’s worth raising this to see where RAN2 stands.

Question 6:  In case the UE does NOT activate the SCG by itself, but is allowed to trigger SR for the SRB (for MCG failure information), should the UE monitor PDCCH on the PSCell?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Apple | Yes | To us, this is an alternative to full-fledged self-activation of the SCG by the UE. Since the MCG link has gone bad, the UE might not be able to receive the SN activation RRC message from MN, and it would have to be from the MN via the SN, which requires the UE to monitor PDCCH. But we do not want any additional UE actions that the UE is expected to do in SCG activated state (for eg., actions with the SCells of SCG etc). |
| ZTE | Yes | Same view as Apple. |
| Nokia | Yes | We see no other way in such a case. |

Summary:

TBD

## 2.3	T316 and other aspects

Companies in [3][11][1][2] propose that the legacy timer T-316 might need to be re-visited for MCG failure recovery while the SCG is deactivated (due to for eg., increased time needed for MN-SN co-ordination etc).

Rapporteur likes to collect feedback on this aspect for making progress.

Question 7:  Companies are requested to provide views on the below:

7.1 – A new timer similar to T-316 is needed for MCG failure recovery in SCG deactivated state.

7.2 – The existing T-316 needs to be extended.

7.3 – The current 2000ms should be enough and the NW can choose the needed configuration

| Company | Which among 7.1,7.2,7.3? | Additional comments |
| --- | --- | --- |
| Apple | 7.3 | We think the NW can decide the time of T-316 and 2000ms should be enough to resolve this. |
| ZTE | 7.3 | If SR can be used to trigger MCG failure recovery, there is no difference in delay compared with legacy procedure. If RACH is used to trigger MCG failure recovery, then there is additional delay caused by RACH procedure, however, from network perspective, it is not a big deal for RRC timers, so we think it is feasible to use current T316 to cover both cases. |
| Nokia | 7.3 |  |

Summary:

TBD

## 2.4	Comments on the TPs

Companies in [1][5][9][10][11][12] have provided TPs for their intended way of implementing this feature. While the content might need changes based on the RAN2 progress, it is worth collecting input on the company views on the TPs in parallel.

Question 8:  Companies are requested to provide views on the below:

8.1 – Any comments on the TP from [1].

| Company | Additional comments |
| --- | --- |
| Nokia | Just added another alternative in [12] |

Summary:

TBD

8.2 – Any comments on the TP from [5].

| Company | Additional comments |
| --- | --- |

Summary:

TBD

8.3 – Any comments on the TP from [9].

| Company | Additional comments |
| --- | --- |

Summary:

TBD

8.4 – Any comments on the TP from [10].

| Company | Additional comments |
| --- | --- |

Summary:

TBD

8.5 – Any comments on the TP from [11].

| Company | Additional comments |
| --- | --- |

Summary:

TBD

## 2.5	Any additional comments

Question 9:  Companies are requested to provide views on any additional aspects as part of this discussion.

| Company | Additional comments |
| --- | --- |

# 4	Conclusion

TBD

# 5	References

[1]  R2-2200388	Fast MCG recovery based on SCG deactivation	OPPO	discussion	Rel-17	LTE_NR_DC_enh2-Core

[2]		R2-2200896	Considerations for Fast MCG link recovery with deactivated SCG	CMCC	discussion	Rel-17	LTE_NR_DC_enh2-Core

[3]		R2-2201073	Other aspects of SCG activation/deactivation	Qualcomm Incorporated	discussion	Rel-17

[4]	R2-2201115	Simple MCG recovery procedure using deactivated SCG for Rel-17	Apple	discussion	LTE_NR_DC_enh2-Core	R2-2110092

[5]	R2-2201116	CR TP for MCG recovery procedure using deactivated SCG for Rel-17	Apple	discussion	LTE_NR_DC_enh2-Core

[6]	R2-2201295	Further discussion on TCI State indication in RRC	MediaTek Inc.	discussion	R2-2111192

[7]	R2-2201317	Deactivation of SCG	LG Electronics Finland	discussion	Rel-17	LTE_NR_DC_enh2-Core

[8]	R2-2201333	Discussion on SCG (de)activation	NTT DOCOMO, INC.	discussion	Rel-17

[9]	R2-2201394	Fast MCG recovery via deactivated SCG	vivo	discussion	LTE_NR_DC_enh2-Core

[10]	R2-2201432	Fast MCG link recovery via deactevated SCG	Sharp	discussion	Rel-17	LTE_NR_DC_enh2-Core

[11]	R2-2201575	Rest issues of SCG Activation	LG Electronics	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2111018

[12] R2-2200882	Open issues in activation of SCG	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core