3GPP TSG-RAN WG2 Meeting #116bis-e	R2-2201706

Online, 17 – 25 January 2022

Agenda item:	8.3.3

Source:	vivo

Title:	[AT116bis-e][231][MUSIM] MUSIM gap details

WID:	LTE_NR_MUSIM-Core

Document for:	Discussion and Decision

Introduction

This document is the summary of the following email discussion:

[AT116bis-e][231][MUSIM] MUSIM gap details (vivo)

Scope: Discuss the details of MUSIM gaps for the NW switching when UE does NOT leave RRC connection: 1) is there a need to define new MGL or MGRP for MUSIM purposes, or are the existing MGL/MGRP sufficient? 2) how to define the details of gap signalling (UE assistance + NW configuration) 3) are there any urgent RAN2 actions needed based on the RAN4 LS R2-2200132 (e.g. reply LS)

Deadline: Thursday W1, 0500 UTC (Deadline 2)

Deadline 2 (discussions for Fri online):

- Comment deadline, 1st phase: Thursday W1, 0500 UTC (for collecting views)

- Rapporteur proposals, 1st phase: Thursday W1, 2000 UTC (proposed resolution of issues)

- Document deadline, 1st phase: Friday W1, 0500 UTC (report, agreed CRs,final approved LS, etc.)

Discussion

To make it easier to find the correct contact delegate in each company for potential follow-up questions, the rapporteur encourages the delegates who provide input to provide their contact information in this table:

| Company | Contact: Name (E-mail) |
| --- | --- |
| vivo | yangxiaodong5g@vivo.com |
| OPPO | fanjiangsheng@oppo.com |
| Spreadtrum | Qufang.huang@unisoc.com |
| Ericsson | lian.araujo@ericsson.com |
| Huawei/HiSilicon | rama.kumar@huawei.com |
| NEC | wangda@labs.nec.cn |
| Nokia | Srinivasan.selvagaanpathy@nokia.com |
| ZTE | Li.wenting@zte.com.cn |
| Lenovo | Wulh5@lenovo.com |
| MediaTek | chun-fan.tsai@mediatek.com |
| Samsung | Aby.abraham@samsung.com |
| Charter Communications | reza.hedayat@charter.com |
| Intel | Sudeep.k.palat/@intel.com |
| Apple | sethu@apple.com |
| DENSO | tomoyuki.yamamoto.j5c@jp.denso.com |
| Futurewei | mazin.shalash@futurewei.com |
| Qualcomm | oozturk@qti.qualcomm.com |
| China Telecom | liujiaxiang6@chinatelecom.cn |
| LGE | han.cha@lge.com |
| Sharp | Fangying.xiao@cn.sharp-world.com |

The needed MUSIM Gap

RAN#94e[1] has agreed that “RAN4 is requested to capture in TS38.133 that legacy measurement gap patterns (as defined in TS 38.133 Table 9.1.2-1, and also including patterns #24 and #25) can be applicable for MUSIM operation and also to capture new gap patterns for MUSIM with MGRP equal to paging DRX cycles for IDLE/INACTIVE.”

We would like to discuss the supported gap patterns for MUSIM.

Supported periodic gap

Contributions [5][6][7][8][9][10][11] mentioned the measurement gap patterns for MUSIM.

- All legacy measurement gap patterns, including #24 and #25, are applicable to MUSIM [5][6][8][9][11]

- Introduce new gap periodicities to match paging cycles.[5][6][8][9][10][11]

- Longer gap durations may be introduced if needed.[6]

Since all legacy measurement gap patterns and new gap periodicities{320ms, 640ms, 1280ms, 2560ms } are applicable to MUSIM, as [7] proposed,  RAN2 could capture the following values in UAI and RRC reconfiguration for MUSIM gap:

- MGL: 1.5ms, 3ms, 3.5ms, 4ms, 5.5ms, 6ms, 10ms, 20ms

- MGRP: 20ms, 40ms, 80ms, 160ms, 320ms, 640ms, 1280ms, 2560ms.

Companies are invited to express their view on the following question.

- Whether the following MGL/MGRP values are applicable for MUSIM periodic gap:

- MGL: 1.5ms, 3ms, 3.5ms, 4ms, 5.5ms, 6ms, 10ms, 20ms

- MGRP: 20ms, 40ms, 80ms, 160ms, 320ms, 640ms, 1280ms, 2560ms.

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes | Based on RAN#94e[1] agreement, it’s fine to capture applicable MGL/MGRP values for MUSIM in TS38.331. It’s worth noticing that, the final applicable gap patterns need to be captured in TS38.133, including new gap patterns for MUSIM with MGRP equal to paging DRX cycles for IDLE/INACTIVE. |
| OPPO | Yes | This proposal is aligned with RAN4 agreements, so we support. |
| Spreadtrum | Yes | Aligned with RAN4 agreements, so we support. |
| Ericsson | Wait for RAN4 | Our understanding is that RAN4 is also discussing this, so we should wait for RAN4 conclusion. We should not jump in RAN2 to conclusion that would raise more work in RAN4 on this at this point. |
| Huawei/HiSilicon | Yes | Agree with Vivo |
| NEC | Yes |  |
| Nokia | Yes. But | If number of periodic gaps are restricted to 2, the gap for paging reception should consider SSB reception for synchronization before paging search space monitoring. Whether this can fit within 20 msec to be confirmed with RAN4. |
| ZTE | Yes | Agree with Vivo |
| Lenovo | Yes |  |
| MediaTek | Yes, but | We may also have to wait RAN4 for finial conclusion |
| Samsung | Yes |  |
| Charter Communications | Yes | But perhaps need to wait for RAN4 outcome after their first meeting on this WI. |
| Intel | Yes | The current gaps can be re-used also for MUSIM but wait for RAN4 input. |
| Apple | Yes | Agree with Vivo |
| DENSO | Yes | This proposal fits to the RAN4 agreements in the LS |
| Futurewei | Yes |  |
| Qualcomm | Yes, plus | These will be the minimum set, based on RAN4 response as well. RAN2 and RAN4 should continue discussing additional values. |
| China Telecom | Yes, but | Wait for RAN4 final conclusion |
| LGE | Yes | The purpose of MUSIM gap is mainly focused on to support the UE behavior of idle mode. The paging DRX on NW B can be utilized as enhancing efficiency of NW A service by configuring infrequent gap pattern. It is fine to capture the paging DRX cycle as the MGRP of MUSIM periodic gap. |
| Sharp | Yes |  |

- Summary:

20 companies shared their views, 19/20 companies agree that the above MGL/MGRP values are applicable for MUSIM periodic gap.   5 companies further suggest waiting for RAN4 conclusion.

In Rapporteur’s understanding, it’s fine from RAN2 perspective to capture applicable MGL/MGRP values for MUSIM in TS38.331. then, we propose that：

Proposal 1: [19/20] From RAN2 perspective, the following MGL/MGRP values are applicable for MUSIM periodic gap:

-	MGL: 1.5ms, 3ms, 3.5ms, 4ms, 5.5ms, 6ms, 10ms, 20ms

-	MGRP: 20ms, 40ms, 80ms, 160ms, 320ms, 640ms, 1280ms, 2560ms.

Supported aperiodic gap

Contributions [5][8][9][10] mentioned that aperiodic gap patterns are introduced for MUSIM purposes to cover Scenario 2 and Scenario 3.

[8] pointed out that, for aperiodic MUSIM gap, at least the MGL of legacy gap (i.e. R15/R16 gap) can be reused. It’s up to RAN4 to decide whether additional gap length should be introduced for aperiodic MUSIM gap.  Contributions [5][9] proposed that an aperiodic gap with length of 20ms can be configured for aperiodic (one-shot) switching.

In LS[2], RAN4 concludes that an aperiodic gap pattern can fulfill the task of MIB/SIB1 reading. For the question “What would be the feasible range of value(s) for gap cycle and duration that can allow the UE to stay in Connected mode in Network A for all 3 scenarios?” RAN4 concludes that at least no problem is identified in case legacy MGL and MGRP are used.

Therefore, the legacy MGL, i.e. {1.5ms, 3ms, 3.5ms, 4ms, 5.5ms, 6ms, 10ms, 20ms},  can be reused for MUSIM aperiodic gap.

Companies are invited to express their view on the following question.

- Whether the legacy MGL {1.5ms, 3ms, 3.5ms, 4ms, 5.5ms, 6ms, 10ms, 20ms} is applicable for MUSIM aperiodic gaps?

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes | It’s fine to capture legacy MGL value for aperiodic MUSIM gap in TS38.331. Whether extra MGL values for MUSIM aperiodic gaps need to be introduced is up to RAN4. |
| OPPO | Yes | This proposal is aligned with RAN4 agreements, so we support. |
| Spreadtrum | Yes | Aligned with RAN4 agreements, so we support. |
| Ericsson | Wait for RAN4 | Our understanding is that RAN4 is also discussing this, so we should wait for RAN4 conclusion. We should not jump in RAN2 to conclusion that would raise more work in RAN4 on this at this point. |
| Huawei/HiSilicon | Yes | Based on the reply LS from RAN4, legacy gap pattern with legacy MGL can be used. |
| NEC | Yes |  |
| Nokia | No | Aperiodic gaps are also needed for short signalling procedures like BUSY indication. Here RACH procedure and short time in connected mode will be required. RAN2 has already agreed that whether UE can have RRC connection during aperiodic gap is left to UE implementation. For BUSY indication if the UE is capable of completing this procedure, this duration is not sufficient. |
| ZTE | Yes (but prefer to wait for RAN4) | On this topic, we general agree, but we think we should wait for RAN4’s conclusion. |
| Lenovo | Yes |  |
| MediaTek | Yes, but | We may also have to wait RAN4 for finial conclusion |
| Samsung | Yes |  |
| Charter Communications | Wait for RAN4 | Need to wait for RAN4. Applicability of shorter MGLs for aperiodic gaps is not certain. |
| Intel | FFS – wait for RAN4 | Aperiodic gaps are one-off gaps and they also need to support longer gap lengths for longer tasks in PLMN-B – so these are not sufficient. The benefit of re-using these gap lengths from RAN4 perspective for aperiodic gap should be discussed in RAN4. |
| Apple | Yes – Wait for RAN4 | We have to wait for RAN4 conclusion on this. |
| DENSO | Yes | This proposal fits to the RAN4 agreements in the LS |
| Futurewei | Yes |  |
| Qualcomm | Yes | Same response as Q1 |
| China Telecom | Yes |  |
| LGE | Yes | According to past meetings, the main purpose of aperiodic gap is system information acquisition. It is enough to capture legacy MGL value for MUSIM aperiodic gap in TS 38.331. The additional MGL value would be captured after new scenario of aperiodic gap is introduced. |
| Sharp | Yes |  |

- Summary:

20 companies shared their views, 13/20 companies agree that the above MGL values are applicable for MUSIM aperiodic gap.

6 companies pointed out that we may also have to wait for RAN4 conclusion. 1 company thought that for BUSY indication if the UE is capable of completing this procedure, this duration is not sufficient.

In Rapporteur’s understanding, it’s fine to capture the above applicable MGL values for MUSIM in TS38.331 from RAN2 perspective.

Proposal 2: [13/20] From RAN2 perspective, the following MGL values are applicable for MUSIM aperiodic gap.

-	MGL: 1.5ms, 3ms, 3.5ms, 4ms, 5.5ms, 6ms, 10ms, 20ms

Signalling supports more periodic and aperiodic gaps for MUSIM or not

In RAN2#115e agreement, “Only a single aperiodic gap (for MUSIM) is supported in Rel-17. At most two periodic “gaps” (for MUSIM) and a single aperiodic gap (for MUSIM) is supported in Rel-17. FFS if signalling supports more.”

Contributions [5][6][10][11][12][13] mentioned the support of more than three gaps for MUSIM purposes in R17.

- Maximum periodic measurement gaps should not be limited to 2. [10][11]

- More than two periodic gap patterns should be supported. Using a single periodic gap configuration for paging reception will not be optimal when the SSB and PO are not in close proximity [6].

- More than 3 MUSIM gap configurations with active MUSIM gaps limited to 3 at the same time.[12]

- No extra work is needed to allow signalling supports more periodic and aperiodic gaps for MUSIM. Using gap list could support more gaps. This method is futureproofed. [5]

Contribution [7] proposed that signalling framework to support more than 3 gap patterns is not supported.

Companies are invited to express their view on the following question.

- Whether signalling supports more than three gaps for periodic and aperiodic gaps for MUSIM?

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes | Supporting more gaps is flexible for UE and NW to configure suitable gaps for specific scenarios with better gap efficiency, e.g. PO and SSB are not in close proximity or several gaps are needed for SI reception etc. Signaling could support more than three gaps for periodic and aperiodic gaps for MUSIM. If gap list is used as running CR does, it’s futureproofed and no extra work is needed. Maximum periodic gaps should be at least 4 to allow further extension. |
| OPPO |  | No strong view if no extra spec effort is needed. |
| Spreadtrum |  | It depends on RAN4. |
| Ericsson | No | We should stick with what so far agreed by RAN4. If further input is received, we can of course consider more gaps. Otherwise, we should keep the current agreement on 3 gaps. |
| Huawei/HiSilicon | No | Based on the reply LS from RAN4, RAN4 did not identify the need to support signaling framework of more than 3 gap patterns. |
| NEC | No | More than 3 active gaps only for MUSIM purpose are more complicated than necessary in reality from network operation point of view. We prefer to stick to current agreement to keep at most 3 gaps. |
| Nokia | Yes | It is better to define separate gaps for the cases specified in RAN4. There are 3 cases indicated in RAN4 response. If some gaps are combined it will require larger gap length than current MGL. This will require RAN4 confirmation again. Furthermore, combined gaps will remove the necessary flexibility for e.g. no. of SSBs needed for synchronization prior to paging monitoring. |
| ZTE | No | We share the view from Ericsson, and it may also depend on concurrent gap discussion. We’d better to wait for RAN4’s further confirmation about the aperiodic gap and also wait for some further progress on the concurrent gap |
| Lenovo | No | We need to keep the current agreement unless we have new motivation. |
| MediaTek | No | We also prefer to keep previous agreement that at most 3 gap is supported. It is always possible to extend the RRC signaling. ASN.1 details could be discussed based on CR. |
| Samsung | Yes | We think that it is better to support more than three gaps from signaling purpose for future enhancements, especially since there are limitations seen with current maximum of three gaps. |
| Charter Communications | Yes |  |
| Intel | No (but wait for further input from RAN4) | While this could be OK from RAN2 perspective, this should be discussed also in RAN4 and in the wider context of MGE. |
| Apple | Yes | From signalling aspect it should be ok, but prefer to wait for RAN4 input on this. |
| DENSO | Yes | Agree with vivo and Nokia. There seems to be the case where several gaps are needed for one use case, e.g. UE cannot finish paging monitoring and its prior SSB detection within one gap, etc. |
| Futurewei | No | No strong motivation at this point to change previous agreement. This can be revisited if needed based on RAN4 input. |
| Qualcomm | Yes | If we only use legacy gap patterns, it is impossible to support the scenarios we agreed on since we need at least three periodic gap occurrences will be more efficient, e.g. one for SSB, one for PO, and one for inter-frequency measurements. At least one aperiodic is needed for SI reception. |
| China Telecom | No | Stick to previous agreements |
| LGE | No but | We think no extra gaps are needed since sending busy indication, TAU, or RAU are excluded in the supported gap scenarios. But we agree that RAN2 can discuss again when RAN4 provides further input for this. |
| Sharp | Yes |  |

- Summary:

20 companies shared their views.   The views from the proponents [8/20] are following:

- Maximum periodic gaps should be at least 4 to allow further extension.

- Supporting more gaps is flexible for UE and NW to configure suitable gaps for specific scenarios with better gap efficiency.

- If we only use legacy gap patterns, it is impossible to support the scenarios we agreed on since we need at least three periodic gap occurrences will be more efficient, e.g. one for SSB, one for PO, and one for inter-frequency measurements. At least one aperiodic is needed for SI reception.

- It is better to define separate gaps for the cases specified in RAN4. There are 3 cases indicated in RAN4 response. If some gaps are combined it will require a larger gap length than the current MGL.

- it is better to support more than three gaps from signaling purpose for future enhancements, especially since there are limitations seen with current maximum of three gaps.

The views from the opponents [10/20] are following:

- RAN4 did not identify the need to support signaling framework of more than 3 gap patterns.

- If further input is received, we can of course consider more gaps. Otherwise, we should keep the current agreement on 3 gaps.

- This can be revisited if needed based on RAN4 input.

- More than 3 active gaps only for MUSIM purpose are more complicated than necessary in reality from network operation point of view.

- this should be discussed also in RAN4 and in the wider context of MGE. it may also depend on concurrent gap discussion. We’d better wait for RAN4’s further confirmation about the aperiodic gap and also wait for some further progress on the concurrent gap

- It is always possible to extend the RRC signaling. ASN.1 details could be discussed based on CR.

There seems to be no clear majority on signalling supporting more than three gaps for periodic and aperiodic gaps for MUSIM. Therefore, Rapporteur suggests discussing further.

Proposal 3: keep three gaps  agreement (i.e., 2 periodic gaps and 1 aperiodic gap) for MUSIM, can discuss if more RAN4 inputs are valid.

MUSIM assistance information

Configuration of UAI Reporting

This is covered by email discussion [AT116bis-e][232][MUSIM] MUSIM configured time for leaving RRC connection.

Gap preference

In RAN2#115e agreement, “The switching gap configuration will explicitly provide the gap starting position (e.g. offset value or start SFN and subframe explicitly), gap length and gap repetition period.”. In TS38.331 running CR[3], it’s an open issue how the gap offset is provided for periodic and aperiodic gaps in gap preference.

Contribution [5] mentioned below two potential options:

Option-1 : One field musim-GapOffset-r17 for both periodic and aperiodic gap, as in [4]

MUSIM-GapInfo-r17 ::=          SEQUENCE {

musim-GapOffset-r17                    ENUMERATED {FFS},

musim-GapLength-r17                    ENUMERATED {FFS},

musim-GapRepetitionPeriod-r17          ENUMERATED {FFS}            OPTIONAL,

...

}

Option-2: offset value for periodic gap, start SFN and subframe for aperiodic gap

MUSIM-GapInfo-r17 ::=          SEQUENCE {

musim-Starting-SFN-AndSubframe-r17     MUSIM-Starting-SFN-AndSubframe-r17         OPTIONAL, -- Cond aperiodic

musim-GapLength-r17                    ENUMERATED {FFS},

musim-GapRepetitionAndOffset-r17       CHOICE {FFS}         OPTIONAL -- Cond periodic

}

MUSIM-Starting-SFN-AndSubframe-r17 ::=            SEQUENCE {

starting-SFN-r17            INTEGER (0..1023),

startingSubframe-r17        INTEGER (0..9)

}

Companies are invited to express their view on the following question.

- How UE provides gap repetition and offset for MUSIM gaps?

| Company | Options (1, 2) | Comments |
| --- | --- | --- |
| vivo | Option-2 | First, Option-1 has a common gap offset field for both periodic and aperiodic gap, then the size of this gap offset field should at least be enough to explicitly indicate the start SFN and subframe for aperiodic gap, which is a waste for periodic gaps. Second, option-2 guarantees via ASN.1 coding that the gap offset is within the corresponding scope of Gap Repetition, which cannot be supported by option-1. Therefore, we support option-2 |
| OPPO | Option2 | Option2 is more clear from our side. |
| Spreadtrum | Option2, but comments | Option 2 is simple, if the start point could be predicted in advance. |
| Ericsson | Option-2 | We are fine to go with option 2. |
| Huawei/HiSilicon | Option-2 | Agree with Vivo. |
| NEC | Option 2 | Agree with vivo that if using offset to indicate the starting point of aperiodic gaps, the field of offset should be very large, so we prefer Option 2. |
| Nokia | Option 3 (New parameter needed based on RAN4 response) | As per RAN4 discussions, if the MUSIM gap and MG are overlapping MG of NW-A will be used for NW-A operation. This will lead to impacting the MUSIM operation if the gap is meant for paging reception. So the assistance information should include parameter “Gap-Priority” in Gap-info so that network may choose to reconfigure MG not to overlap with this. |
| ZTE | Option-2 | Agree with Vivo. |
| Lenovo | Option 2 |  |
| MediaTek | Option 2 | We are fine with option 2 |
| Samsung | See comments | We think that existing definition in RRC running CR can be reused, by following modifications. With respect to the start subframe for aperiodic MUSIM gap, there seems no real need to define separate field but to restrict the value of musim-GapOffset from 0 to 9 in the field description. MUSIM-GapInfo-r17 ::= SEQUENCE { musim-GapOffset-r17 ENUMERATED {FFS}, musim-GapLength-r17 ENUMERATED {FFS}, musim-GapRepetitionPeriod-r17 ENUMERATED {FFS} OPTIONAL -- Cond periodic, musim-GapStartSFN ENUMERATED {FFS} OPTIONAL -- Cond aperiodic, ... } Rapp: this method may partly solve the parameter size issue. However, musim-GapOffset-r17 cannot be guaranteed to be less than the corresponding musim-GapRepetitionPeriod (which is supported by option-2). |
| Charter Communications | Option 2 |  |
| Intel |  | The gap details has to be discussed first in RAN4 and on what parameters are needed for aperiodic gaps and whether there is real need to perfectly synchronise the network and UE for aperiodic gaps considering it could be much longer gap length. |
| Apple | Option 2 |  |
| DENSO | Option 2 | Option 2 is more optimized to support both periodic and aperiodic gaps |
| Futurewei | Option 2 |  |
| Qualcomm | Option 2 |  |
| China Telecom | Option 2 | Agree with Vivo |
| LGE | Option 2 |  |
| Sharp | Option 2 |  |

- Summary:

- 20 companies shared their views.   There is a clear majority [17/20] supporting Option 2.

Following the majority’s view, it’s proposed that：

Proposal 4: [17/20] In the gap assistance information, UE provides gap repetition period and offset for periodic gaps， and provides start SFN and subframe for aperiodic gaps.

UE indicates the release of gap pattern

Contributions [5][6][7][9][19][20][21][22][23][24][25[26] mentioned the ways for UE to indicate the release of the gap pattern(s).

Indicate the release of gap preference:

Option-1:  If the UEAssistanceInformation does not include a field for aperiodic or periodic gap preference, it indicates no preference for the corresponding field for aperiodic or periodic gap. [5][6][7][9][19][20]

Option-2: Use a size 0 list for MUSIM-GapRequestList-r17 to indicate the release of gap request.[21]

Indicate the release of configured gap(s):

Option-3: Each MUSIM gap configured by network A is associated with an index, UE can indicate which MUSIM gap should be released by including the corresponding MUSIM gap index into UEAssistanceInformation Msg.[8][25][26]

Therefore, UE could either indicate the release of gap preference(option 1, 2) or indicate the release of configured gap (option 3) with UEAssistanceInformation.

Companies are invited to express their view on the following question.

- How does UE request to release MUSIM gap pattern?

| Company | Options (1, 2, 3) | Comments |
| --- | --- | --- |
| vivo | Option-1 | If UE wants to release a configured gap, UE should firstly release the corresponding gap preference. Then the network could know the configured gap corresponding to the released gap preference is not needed any longer. Hence, UE doesn’t have to indicate the release of configured gaps to the network. To release a configured gap for MUSIM, the UE can send a UEAssistanceInformation without the preference information related to the configured gap to be released |
| OPPO | Option3 | UE MUSIM gap modification/release procedure is simpler and flexible, i.e. there is no need to add the previously requested MUSIM gap (if still needed by UE) into UAI message again even if the intention of the latest UAI message is to request new MUSIM gap and/or release part of the configured MUSIM gap. |
| Spreadtrum | Option 3 | For option3, the size of RRC message is small. |
| Ericsson | Option-1 | We think this is cleaner and how usually done to other fields in UE assistance information. Hence, a deviation from this principle, in case really needed, should be further motivated. We don’t think there is any size constraint with the RRC message for UAI case. Furthermore, it actually makes it more complex to create dependencies between a previous UE report and the current report, so we think it is actually simpler if the UE just sends all its preferences in every report. |
| Huawei/HiSilicon | Option-1 | Similar view as Vivo. Option-1 is simpler and aligned with other UAI procedures. There is no need to define size 0. |
| NEC | Option 1 | Agree with vivo. Option 1 is aligned with the other assistance information in UAI. |
| Nokia | Option 3 | Sperate field to indicate the configured gap index to be released is preferred |
| ZTE | Option-1 | Share the view as Ericsson |
| Lenovo | See comments | We assume this question is associated with releasing all configured gaps. In option1, the UE can transmit the UEassistanceinformation without gap request. according to the ASN.1 design, But the preferred state can be set as connected, which can explicitly indicate to network to stay at connected. MUSIM-Assistance-r17 will not be included. That means network receives the UAI without MUSIM information. How to deduce the UE wants to release all gaps? MUSIM-Assistance-r17 ::= SEQUENCE { musim-PreferredRRC-State-r17 ENUMERATED {IDLE, INACTIVE} OPTIONAL, musim-GapRequestList-r17 MUSIM-GapRequestList-r17 OPTIONAL, ... } MUSIM-GapRequestList-r17 ::= SEQUENCE (SIZE (1..3)) OF MUSIM-GapInfo-r17 MUSIM-GapInfo-r17 ::= SEQUENCE { musim-GapOffset-r17 ENUMERATED {FFS}, musim-GapLength-r17 ENUMERATED {FFS}, musim-GapRepetitionPeriod-r17 ENUMERATED {FFS} OPTIONAL, ... } |
| MediaTek | Option 1 | Same principle as other UE assistance information is preferred |
| Samsung | Option 1a: Option 1 only for periodic gaps. Aperiodic gaps can be implicitly released. | We propose to have a modified option 1- option 1a where option 1 is applied for periodic gaps, and aperiodic gaps are implicitly released without any UAI procedure. Option 1 is simpler and aligned with other UE assistance features for the release of periodic gaps. Since both UE and network know aperiodic gap is not used/needed anymore, UE needn’t trigger UAI procedure for release of aperiodic gaps and an aperiodic gap can be implicitly released once the configured gap period is over. |
| Charter Communications | Option 3 |  |
| Intel | Option 2 (or option 1) | We don’t see a big difference between option 1 and 2 – in both cases, the UE providing MUSIM UAI without a requested gap pattern indicates that UE no longer needs gaps. They are both based on the principle that whenever UE sends a UAI for MUSIM gap, UE will always request all the gaps needed and there is no delta signalling. |
| Apple | Option 3 | Using the gap index in straight forward and keeps the signaling compact. |
| DESNO | Option 3 | Agree with OPPO. This way is flexible in case UE needs to release or modify gap configuration(s) partially, e.g. UE wants to keep a gap for paging monitoring but wants to release a gap for SI reading, etc. |
| Qualcomm | Option 1 or 3 | Option 1 is the traditional way of UAI. But agree with others that Option 3 is more flexible and efficient. |
| China Telecom | Option3 | We think Option3 is more flexible way for gap release. |
| LGE | Option 3 | Using the gap index is more straightforward. There would be more benefits on this principle for future extension if more gaps are required in the whole UE operation not only for MUSIM operation but also for other features. |
| Sharp | Option 3 |  |

- Summary:

- 20 companies shared their views.  The following is a summary of company views:

- Option 1 [10/20]:

  - Option-1 is simpler and aligned with other fields in UE assistance information.

  - If UE wants to release a configured gap, UE should firstly release the corresponding gap preference. Then the network could know the configured gap corresponding to the released gap preference is not needed any longer.

  - Option-1 is simpler and aligned with other UAI procedures.

  - Aperiodic gap can be implicitly released once the configured gap period is over.

- Option 2 [1/20]:

- Option 3 [10/20]:

  - Option 3 is more flexible and efficient.

  - There is no need to add the previously requested MUSIM gap (if still needed by UE) into UAI message again even if the intention of the latest UAI message is to request a new MUSIM gap and/or release part of the configured MUSIM gap.

There seems to be no clear majority on option -1 or option-3. Therefore, Rapporteur suggests discussing further for down-selection.

Proposal 5: Further discuss and down-select between the following alternatives:

- Alt 1: If the UEAssistanceInformation does not include a field for aperiodic or periodic gap preference, it indicates no preference for the corresponding field for aperiodic or periodic gap.

- Alt 2: Each MUSIM gap configured by network A is associated with an index, UE can indicate which MUSIM gap should be released by including the corresponding MUSIM gap index into UEAssistanceInformation Message.

MUSIM UAI update

In RAN2#116e agreements, one open issue is “FFS if UE is allowed to update UAI message after the UE performs cell reselection in NW B or after the UE performs handover in NW A”. In the rapporteur’s understanding, the open issue is whether UE is allowed to update UAI message upon UE finishes cell reselection in NW B or handover in NW A, without the prohibit timer’s restriction.

Contributions [5][7][8][9][19][20][21][24][27] [28] mentioned the update of UAI message after the UE performs cell reselection in NW B or after the UE performs handover in NW A.

The proposals from the proponents are following:

- UE is allowed to update MUSIM UAI message. [5][24]

- UE is not allowed to resend the previous MUSIM UAI message in the same serving cell[5][27]

- UE is allowed to update the gap pattern by resending the UAI message after the UE performs cell reselection in NW B or after the UE performs handover in NW A [7]

- The attributes of previously-negotiated gaps may be revised by the UE due to cell reselection on NW B, in order to efficiently utilize the gap. Hence, RAN2 should avoid network-imposed restrictions.[11]

- After performing a handover in Network A, the UE is allowed to send an UEAssistanceInformation with MUSIM preference. [7][9][28].

The reason from the Opponent is following:

- There is no update of UEAssistanceInformation message including gap preference after performing cell reselection in Network B. [9]

And some companies think the issue can be left to UE implementaion:

- It’s up to UE implementation whether to update MUSIM gap configuration in network A after cell reselection in network B, i.e. no spec effort is needed from RAN2 perspective[8]

- The triggers for when the UE can request new MUSIM gap patterns are not specified and are left to the UE implementation[6]

In addition, contributions [5] [9][19][20][21] mentioned whether prohibit timer is used for switching notification message sending.

- Prohibit timer is Not used for switching notification message sending [5][21]

  - The prohibit timer may result in a long delay to send out updated gap preference after cell reselection in NW B, which may cause paging missing.[5]

- Introduce a prohibit timer.[9][19][20]

  - prohibit timer is used other assistance information within UEAssistanceInformation message to avoid frequent transmission of assistance information to the network.[20]

In the rapporteur’s understanding, the above two issues are related. If prohibit timer is used for switching notification message sending, UE could not update UAI for MUSIM gap preference upon UE finishes cell reselection in NW B if the prohibit timer is running. Paging message from network B could be lost if the configured MUSIM gap doesn’t overlap with the PO to be monitored in the new camping cell. If prohibit timer is not used, and UE could update UAI message whenever necessary, e.g. after the UE performs cell reselection in NW B or after the UE performs handover in NW A. But some mechanisms are needed to prohibit frequent UAI sending from UE.

There could be two potential proposals:

Option-1: UE is allowed to send MUSIM UAI message whenever necessary. But UE is not allowed to repeat the last MUSIM UAI message in the same serving cell.

Option-2: Introduce a prohibit timer, UAI message could not be triggered for MUSIM purposes when the timer is running

Companies are invited to express their view on the following question.

- On UAI update for MUSIM gap preference,  which option do you prefer?

Option-1: UE is allowed to send MUSIM UAI message whenever necessary. But UE is not allowed to repeat the last MUSIM UAI message in the same serving cell.

Option-2: Introduce a prohibit timer, UAI message could not be triggered for MUSIM purposes when the timer is running

| Company | Options (1, 2) | Comments |
| --- | --- | --- |
| vivo | Option-1 | For MUSIM, The prohibit timer may delay gap preference update after cell reselection in NW B, which may cause paging missing. Therefore, we propose that no prohibit timer is used for MUSM assistance info. UE should be allowed to update UAI message whenever necessary, e.g. after the UE performs cell reselection in NW B or after the UE performs handover in NW A. And, UE is not allowed to repeat the last MUSM assistance info reported in the same serving cell to avoid frequent UAI sending. |
| OPPO | Option1 with comments | Generally, we agree the comments from vivo, but also think the sentence ‘But UE is not allowed to repeat the last MUSIM UAI message in the same serving cell’ should be removed from option1, a good UE implementation will never repeat the last MUSIM UAI message in the same serving cell if no change happens in network B, so we prefer to revise option1 into the following: Option1: It’s up to UE implementation whether to update MUSIM gap configuration in network A after cell reselection in network B, i.e. no spec effort is needed from RAN2 perspective. The revised option1 is more clear from our side. |
| Spreadtrum | Option 1 | The update of UAI shall not be restricted, because the UAI updation depends on the other network. |
| Ericsson | Option-2 | Note a prohibit timer is a general handling adopted for messages in UE assistance info to avoid frequent reporting, together with the requirement for the UE to send a different indication from the one sent previously. Hence, if a prohibit timer is not introduced, RAN2 should discuss other means to avoid frequent reporting. |
| Huawei/HiSilicon | Option-2 with suggestions | Option-1 allows UE to update UAI whenever necessary. This kind of mechanism should be avoided as the UE behavior is untestable/uncontrollable. Normally too much flexible behavior in the spec should be avoided as it’s very difficult for the NW to adapt with frequent GAP requests from the UE (e.g.: a UE with bad implementation may repeatedly attempt different GAP configurations in a short time) and will have negative impact on scheduling policy of the NW. So we prefer Option-2 to be aligned with other UAI procedures with the possibility for the UE to update UAI for gap preference upon cell reselection in NW B or HO in NW A even if the prohibit timer is running. MUSIM gap configuration case is different from overheating. If the cell reselection or HO results in unsuitable gap and if the gap is not updated in time, resource waste/data loss will happen for a long time. So it’s necessary to allow the UE to update UAI upon cell reselection in NW B or HO in NW A even if the prohibit timer is running. As Ericsson commented, if prohibit timer is not introduced, RAN2 need to discuss methods to avoid frequent reporting. For example, specifying exact UAI conditions like cell reselection in NW B or HO in NW A. |
| NEC | Option 2 | Prohibit timer is used by the network to control frequent request from the UE. We prefer to reuse this to give network A the right to control the frequency of scheduling gap request for service of network B. If network A really cares about the delay of paging reception of network B, network A can configuring the prohibit timer value as 0. BTW, for Option 1, since UAI is triggered in the same serving only upon change of preferred gap information, “UE is not allowed to repeat the last MUSIM UAI message in the same serving cell” is not clear to us. |
| Nokia | Option 1 | As the trigger condition of UAI is not specified we need not restrict about the repetition. It can be left to UE implementation. |
| ZTE |  | We slightly prefer option 1 but we also see the concerns from the companies who support option 2, e.g. the UE may modify the gap info and send the UAI frequently even there is no new event triggered. If there is no other mechanism that can prevent UE from frequently reporting UAI, the prohibit timer shall be introduced. |
| Lenovo | Option 1 | Option is legacy. |
| MediaTek | Option 2, but | The maximum prohibit timer should be limited to a reasonable value. |
| Samsung | Option 2 | We think that network should have a means (e.g. prohibit timer) to control frequent MUSIM assistance information reports alike other UE assistance features. The value range of prohibit timer for UAI is in general in order of seconds so appropriate value range can be further discussed to avoid any side effects. |
| Charter Communications | Option 1 |  |
| Intel | Option 1 | We don’t think a prohibit timer should be used. The UE should be allowed to send UAI whenever needed based on the requirements for PLMN-B that could change at any time, due to cell reselection in PLMN-B, RF changes etc. If a badly implemented UE repeats UAI too often, network can disable MUSIM UAI for this UE. |
| Apple | Option 1 | Because UAI usage trigger is based on the other NW |
| DENSO | Option 1 | It can be left to UE implementation, as no spec effort is needed. |
| Futurewei | Option 2 |  |
| Qualcomm | Option 1 | Agree with Oppo and others that we don’t put such restrictions in UAI messages and leave it to the good UE implementation. |
| China Telecom | Option 1 |  |
| LGE | Option 2 | From the UE perspective, it is also necessary to request to change the MUSIM UAI again. If not, unexpected procedure delay may be caused by not being changed or not being updated. If the UE does not receive the MUSIM UAI configuration as requested, a mechanism is needed to request the same again. |
| Sharp | Option-1 but | We do not think an explicit restriction that “UE is not allowed to repeat the last MUSIM UAI message in the same serving cell” is needed. UE will retransmit the UAI only if the parameters are changed which may happen when cell reselection is performed in NW B. |

- Summary:

- 20 companies shared their views.  The following is a summary of company views:

- Option 1 [12/20];

- Option 2 [7/20]:

Regarding option-1, further comments are provided:

- As the trigger condition of UAI is not specified, we need not restrict the repetition. It can be left to UE implementation.

- It’s up to UE implementation whether to update MUSIM gap configuration in network A after cell reselection in network B, i.e. no spec effort is needed from RAN2 perspective.

- ‘But UE is not allowed to repeat the last MUSIM UAI message in the same serving cell’ is not clear, or should be removed from option-1.

Regarding option-2, further comments are provided:

- The maximum prohibit timer should be limited to a reasonable value to avoid any side effects.[2/20]

- If a prohibit timer is not introduced, RAN2 should discuss other means to avoid frequent reporting [2/20]

There seems to be no clear majority on the two options. Therefore, Rapporteur suggests discussing further for down-selection.

Proposal 6: Further discuss and down-select between the following alternatives regarding UAI update for MUSIM gap preference:

Alt 1: UE is allowed to send a UAI message with different MUSIM preference from the one sent previously whenever necessary.

Alt 2: Introduce a prohibit timer with a reasonable value for MUSIM UAI.

Contributions [5] [19] [20][21] proposed that UE is allowed to initiate UAI message with MUSIM preference in the target cell after handover if the UE has sent UAI during the last 1 second.

In the legacy UAI framework, UE is allowed to initiate a UAI message in the target cell after handover, if the UE has sent UAI during the last 1 second.

Companies are invited to express their view on the following question.

- Whether UE is allowed to initiate a UAI message with MUSIM preference in the target cell after handover, if the UE has sent UAI during the last 1 second?

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes | The source cell may fail to receive the latest UAI with MUSIM preference before/during handover and cannot send the latest UAI to the target cell. To resolve this issue, a simple way is to allow UE to initiate a UAI message with MUSIM preference in the target cell after the UE performs handover in NW A, as the legacy UAI framework does. |
| OPPO | Yes | Fine to reuse the legacy UE behavior defined for UAI. |
| Spreadtrum | Yes |  |
| Ericsson | Yes | Similar view as Vivo. |
| Huawei/HiSilicon | Yes | Agree with Vivo. |
| NEC | Yes | Agree to reuse the legacy behaviour for scheduling gap request. |
| Nokia | Yes | We don’t see reason to restrict this. If the received configuration after handover already has gap configurations, UE need not trigger again. But it is upto UE to decide on sending it after handover. |
| ZTE |  | We think it depends on whether the target cell has included the proper Gap configuration in the handover message. If included there is no need to send UAI again, otherwise, we think it should be allowed. |
| Lenovo | Yes | It is normal UAI issue rather than MUSIM specific issue. |
| MediaTek |  | We can follow the general UAI resending rule during handover |
| Samsung | Yes | Prefer to reuse the legacy behaviour |
| Charter Communications | Yes |  |
| Intel | Yes | Legacy behaviour is required for MUSIM UAI as well. |
| Apple | Yes | Legacy behaviour needs to be maintained for such cases |
| DENSO | Yes | No special reason not to apply the legacy spec for UAI |
| Futurewei | Yes |  |
| Qualcomm | Yes | Fine to align with legacy UAi |
| China Telecom | Yes |  |
| LGE | Yes |  |
| Sharp | Yes | This is the legacy UE behavior, UE should follow it to resolve the same issue happened to other UAI message. |

- Summary:

20 companies shared their views. The majority [19/20] agree that UE is allowed to initiate a UAI message with MUSIM preference in the target cell after handover, if the UE has sent UAI during the last 1 second. Most of companies think it’s fine to follow the general UAI resending rule during handover.

The following is proposed:

Proposal 7: [19/20] UE is allowed to initiate a UAI message with MUSIM preference in the target cell after handover if the UE has sent UAI during the last 1 second.

Gap Configuration

Contributions [5][26][27] mentioned whether adopt the list with ToAddModList/ToReleaseList for the scheduling gap configuration.

- Adopt the list with ToAddModList/ToReleaseList for the scheduling gap configuration. [5][27]

- Introduces addition/modification and release list for multiple gap handling.[26]

Furthermore, gap ID is proposed in [5][25][26] to identify each configured gap.

- Each MUSIM gap configured by network A is associated with an index, to support modification or release of gaps.[5]

- NW and UE can uniquely identify each configured gap pattern with a unique gap ID specific to the UE. [25]

- RAN2 introduces gap ID for multiple gap handling.[26]

[22] suggests that, nevertheless gap ID is configured for periodic gap pattern or not, the gap ID is not needed for the aperiodic gap pattern.

Companies are invited to express their view on the following question.

- Whether adopt the list with ToAddModList/ToReleaseList for the scheduling gap configuration?

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes | Network may modify or release the configured gaps. In order to facilitate the gap reconfiguration, we propose to adopt the list with ToAddModList/ToReleaseList for the scheduling gap configuration |
| OPPO | Yes |  |
| Spreadtrum | Yes |  |
| Ericsson | Depends on general gap handling discussion | This can be handled in the general gap configuration discussion. |
| Huawei/HiSilicon | Yes | Agree with Vivo. |
| NEC | Yes | Agree with vivo. |
| Nokia | Yes |  |
| ZTE | Yes | Agree with Vivo. |
| Lenovo | Yes |  |
| MediaTek | Yes, but | We also prefer to handle this in general gap configuration section |
| Samsung | See comments | We have no strong view but wonder whether network can change any part of MUSIM gap assistance information and configure it to the UE accordingly or not. |
| Charter Communications | Yes |  |
| Intel | Depends on MGE discussion | Wait for MGE discussion |
| Apple | Yes |  |
| DENSO | Yes | Agree with vivo, however, similar discussion is ongoing in MGE WI. Therefore, further discussion would be needed if we reuse the message definition or we define separate message. |
| Qualcomm | Yes |  |
| China Telecom | Yes |  |
| LGE | Yes |  |
| Sharp | Yes |  |

- Summary:

20 companies shared their views. The majority [17/20] support adopting the list with ToAddModList/ToReleaseList for the scheduling gap configuration.

Two companies thought that a similar discussion is ongoing in MGE WI (Measurement Gap Enhancement WI). Therefore, further discussion would be needed if we reuse the message definition or we define a separate message.

The following is proposed:

- Proposal 8: [17/20] Adopt the list with ToAddModList/ToReleaseList for the scheduling gap configuration

- Whether to introduce gap ID to identify each configured gap, and support modification or release of gaps via gap ID?

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes | Network may modify or release the configured gaps via the gap ID, assuming ToAddModList/ToReleaseList is agreed in Q8 for the gap configuration. |
| OPPO | Yes |  |
| Spreadtrum | Yes |  |
| Ericsson | Depends on general gap handling discussion | See comment on Q8. |
| Huawei/HiSilicon | No | The order of the entity in the requested “musim-GapRequestList” provided by the UE in UAI message implicitly indicates the gap ID. Based on this gap ID, NW may modify or release the configured gaps. The same method is used in current spec and there is no need to define explicit gap ID. |
| NEC | Yes | Agree with vivo. |
| Nokia | Yes |  |
| ZTE | Yes(with comments) | So this question is for the RRCReconfiguration message instead of the UAI, right? If it is, we agree . Rapp: Yes, it’s for the RRCReconfiguration message. |
| Lenovo | Yes |  |
| MediaTek | Yes, but | We also prefer to handle this in general gap configuration section |
| Samsung | See comments | We think that this depends on answer to Q8. If ToAddModList/ToReleaseList is agreed in Q8,gap ID is needed to identify each configured gap. |
| Charter Communications | Yes |  |
| Intel | Depends on MGE discussion | Wait for MGE discussion |
| Apple | Yes |  |
| DENSO | Yes | Agree with vivo |
| Qualcomm | Yes |  |
| China Telecom | Yes |  |
| LGE | Yes |  |
| Sharp | Yes |  |

- Summary:

19 companies shared their views. Most of companies[15/19] agree to introduce gap ID to identify each configured gap, and support modification or release of gaps via gap ID.

One company thought that the order of the entity in the requested “musim-GapRequestList” provided by the UE in UAI message implicitly indicates the gap ID. Based on this gap ID, NW may modify or release the configured gaps. The same method is used in the current spec and there is no need to define explicit gap ID.

Two companies thought this issue is related to MGE WI (Measurement Gap Enhancement WI).

The following is proposed:

Proposal 9: [15/19] Introduce gap ID to identify each configured gap, and support modification or release of gaps via gap ID.

In RAN2#115e agreement, “The switching gap configuration will explicitly provide the gap starting position (e.g. offset value or start SFN and subframe explicitly), gap length and gap repetition period.”.In the rapporteur’s understanding,  NW configures gap repetition and offset for periodic gaps, and configures start SFN and subframe for aperiodic gaps.

Companies are invited to express their view on the following question.

- Do companies agree that NW configures start SFN and subframe for aperiodic gaps?

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes | NW configures gap repetition and offset for periodic gaps, and configures start SFN and subframe for aperiodic gaps. |
| OPPO | Yes |  |
| Spreadtrum | Yes | If the start time of aperiodic gap could be predicted, NW could configures start SFN and subframe. |
| Ericsson | Yes | We think this also affects the modeling on question 4, i.e. if this is agreed then would be better to go with option 2 in question 4. |
| Huawei/HiSilicon | Yes | Similar view as Vivo. |
| NEC | Yes |  |
| Nokia | Yes But | It should be optional. If this is not given NW to start the aperiodic gap immediately on receiving the UAI. |
| ZTE | Yes | Similar view as Vivo. |
| Lenovo | Yes |  |
| MediaTek | Yes |  |
| Samsung | Yes |  |
| Charter Communications | Yes |  |
| Intel |  | Wait for RAN4 discussion on aperiodic gaps. For longer aperiodic gaps, synchronisation of the start between the network and UE may not be that relevant. |
| Apple | Yes |  |
| DENSO | Yes | This way is simple and straightforward |
| Futurewei | Yes | We can also support Nokia’s proposal to make start information optional |
| Qualcomm | Yes but | Assuming that the NW will follow the UE request |
| China Telecom | Yes |  |
| LGE | Yes |  |
| Sharp | Yes |  |

- Summary:

20 companies shared their views. The majority [19/20] agree that NW configures start SFN and subframe for aperiodic gaps.

Further views are provided:

- It should be optional. If this is not given NW to start the aperiodic gap immediately on receiving the UAI.

- Wait for RAN4 discussion on aperiodic gaps.  For longer aperiodic gaps, synchronization of the start between the network and UE may not be that relevant.

From RAN2 perspective, the following is proposed:

Proposal 10: [19/20] NW configures start SFN and subframe for aperiodic gaps.

The LS actions

In the rapporteur’s understanding, some of the above questions are related to the gap patterns which may lead to impact to RAN4. Hence, companies are invited to express their view on the following question.

- Whether a LS to RAN4 is needed? If Yes, any content?

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes | RAN2 agreements could be sent to RAN4, especially MGL/MGRP value range, which needs RAN4 to further define the applicable gap patterns. |
| OPPO | Yes | Fine to inform RAN4 of RAN2 agreements on MUSIM gap. |
| Spreadtrum | Yes |  |
| Ericsson | No | We should actually wait for RAN4 discussion on the details. |
| Huawei/HiSilicon | Yes | OK to send RAN2 gap related agreements to RAN4 |
| NEC | Yes | OK to send RAN2 gap related agreements to RAN4 |
| Nokia | Yes but | it depends on the conclusions and if it defers from RAN4 understanding as per received LS. Only the agreements which impacts RAN4 work to be included. |
| ZTE |  | We think it depends on the progress of online discussion |
| Lenovo | Yes |  |
| MediaTek | No | It depends on what is agreed in this meeting. So far, we don’t think it is really necessary. |
| Samsung | Yes |  |
| Charter Communications | Yes |  |
| Intel | Yes | We are OK to send an LS to RAN4 on MUSIM gap agreements. Coordination with MGE outcome may be needed. |
| Apple | Yes |  |
| DENSO | Yes | OK to send RAN2 agreements to RAN4. |
| Qualcomm | Maybe | If we agree on something which can help the RAN4 discussion on gap patterns. |
| China Telecom | Yes |  |
| LGE | Yes |  |
| Sharp | Yes |  |

- Summary:

19 companies shared their views. The majority [16/19] agree that a LS to RAN4 is needed.  The LS content could include RAN2 gap related agreements, which can help the RAN4 discussion on gap patterns.

One company thought that we should actually wait for RAN4 discussion on the details.

Following the majority’s view, it’s proposed:

Proposal 11: [16/19] A LS to RAN4 is needed if RAN2 gap related agreements are reached.

Other Comments

Companies are invited to express their view if any other comments or suggestions on the switching message details.

Any other comments or suggestions on the switching message details?

| Company | Comments |
| --- | --- |
| OPPO | Gap activation method should be addressed also considering the gap common AI is discussing the joint work among preconfigured MG, concurrent MG, NCSG and MUSIM gap, it’s necessary to define the MUSIM gap activation way to avoid any confusion. Based on above, we propose: Upon received by RRC signaling, all the configured MUSIM gap(s) will be activated immediately. |
| LGE | The MUSIM gap configuration may not be always activated by the gNB due to e.g. overlap with preconfigured MG and QoE requirement of service. In this case, gNB need to activate the requested gap partially. Based on above, we propose The UE prioritizes the MUSIM gap and indicate the priority in UAI message. |
| Sharp | When UE request to release a gap pattern, when UE can stop using it should be discussed. If the gap pattern is valid until released by gNB explicitly, then UE cannot be schedule during the gap after sending the UAI message to request release of the gap pattern even if it is no more needed. So, UE can be scheduled during the gap duration that requested to be released in the UAI message. |

Summary:

We may further discuss above questions based on contributions.

Conclusions

Based on the email discussion, we give the below proposals.

Proposal 1: [19/20] From RAN2 perspective, the following MGL/MGRP values are applicable for MUSIM periodic gap:

-	MGL: 1.5ms, 3ms, 3.5ms, 4ms, 5.5ms, 6ms, 10ms, 20ms

-	MGRP: 20ms, 40ms, 80ms, 160ms, 320ms, 640ms, 1280ms, 2560ms.

Proposal 2: [13/20] From RAN2 perspective, the following MGL values are applicable for MUSIM aperiodic gap.

-	MGL: 1.5ms, 3ms, 3.5ms, 4ms, 5.5ms, 6ms, 10ms, 20ms

Proposal 3: keep three gaps  agreement (i.e., 2 periodic gaps and 1 aperiodic gap) for MUSIM, can discuss if more RAN4 inputs are valid.

Proposal 4: [17/20] In the gap assistance information, UE provides gap repetition period and offset for periodic gaps，and provides start SFN and subframe for aperiodic gaps.

Proposal 5: Further discuss and down-select between the following alternatives:

- Alt 1: If the UEAssistanceInformation does not include a field for aperiodic or periodic gap preference, it indicates no preference for the corresponding field for aperiodic or periodic gap.

- Alt 2: Each MUSIM gap configured by network A is associated with an index, UE can indicate which MUSIM gap should be released by including the corresponding MUSIM gap index into UEAssistanceInformation Message.

Proposal 6: Further discuss and down-select between the following alternatives regarding UAI update for MUSIM gap preference:

- Alt 1: UE is allowed to send a UAI message with different MUSIM preference from the one sent previously whenever necessary.

- Alt 2: Introduce a prohibit timer with a reasonable value for MUSIM UAI.

Proposal 7: [19/20] UE is allowed to initiate a UAI message with MUSIM preference in the target cell after handover, if the UE has sent UAI during the last 1 second.

- Proposal 8: [17/20] Adopt the list with ToAddModList/ToReleaseList for the scheduling gap configuration

Proposal 9: [15/19] Introduce gap ID to identify each configured gap, and support modification or release of gaps via gap ID.

Proposal 10: [19/20] NW configures start SFN and subframe for aperiodic gaps.

Proposal 11: [16/19] A LS to RAN4 is needed if RAN2 gap related agreements are reached.

References

- RAN-94e Meeting Report

- R2-2200132	Reply LS on gap handling for MUSIM (R4-2120342; contact: vivo)	RAN4	LS in	Rel-17	LTE_NR_MUSIM-Core	To:RAN2	Cc:RAN

- R2-2108861 LS on gap handling for MUSIM

- R2-2200800	Running NR RRC CR for  MUSIM	vivo	draftCR	Rel-17	38.331	16.7.0	LTE_NR_MUSIM-Core

- R2-2200803	Remaining open issues on MUSIM Switching	 vivo	other	Rel-17	LTE_NR_MUSIM-Core

- R2-2200489	Configuration of MUSIM Gaps	Qualcomm Incorporated	discussion

- R2-2200920	Remaining issues for NW switching without leaving RRC_CONNECTED	Huawei, HiSilicon	discussion	Rel-17

- R2-2200230	Remaining Details for Periodic and Aperiodic Gaps	OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2201482	Discussion on switchover procedure without leaving RRC_CONNECTED state	Ericsson	discussion

- R2-2201633	Measurement Gaps Open issues	Rakuten Mobile, Inc	discussion	Rel-17

- R2-2201481	Remaining Issues for MUSIM Network Switching	Charter Communications, Inc	discussion

- R2-2200671	On remaining issues for MUSIM Gap configuration	Nokia, Nokia Shanghai Bells	discussion	Rel-17

- R2-2200950	Discussion on RAN4 Reply LS on MUSIM gaps	Samsung R&D Institute India	discussion

- R2-2201483	Discussion on switchover procedure for leaving RRC_CONNECTED state	Ericsson	discussion

- R2-2200231	Remaining Details on MUSIM Assistance Information for Leaving Case	OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2200904	Remaining issues for NW switching with leaving RRC_CONNECTED	Huawei, HiSilicon	discussion	Rel-17

- R2-2201234	Consideration on the Switching with Leaving Connected State	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2201316	Further details on network switching notification	MediaTek Inc.	discussion	R2-2111222

- R2-2200211	Remaining issues on network switching for MUSIM	Samsung Electronics Co., Ltd	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2200572	Remaining issues on scheduling gap for network switching	NEC	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2200359	Remaining open issues on network switching for MUSIM	Intel Corporation	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2201215	Release of MUSIM Gap	Sharp	discussion

- R2-2200754	Remaining issues for switching notification and busy indication	Lenovo, Motorola Mobility	discussion	Rel-17

- R2-2200522	Remaining issues of Network switching for MUSIM	China Telecom	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2201201	MUSIM Signaling aspects for Scheduling gap handling	Apple	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2201369	Remaining issues for MUSIM gap configuration	LG Electronics France	discussion	Rel-17

- R2-2201233	Further Consideration on the Scheduling Gap	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2200631	UE indication on switching	Spreadtrum Communications	discussion	Rel-17

Annex B: Agreements

In the previous RAN2 meetings, the following agreements were taken:

- Agreement (RAN2#113bis-e)

- 1	RRC signalling is used for switching procedure without leaving RRC_CONNECTED state in network A for UE temporarily switching to network B as a baseline. FFS on additional need of MAC signalling.

- Agreements (RAN2#114e)

- 1:	RRC signalling for network switching without leaving RRC_Connected state should allow multiple configurations of periodic “gaps” with different parameters (e.g. periodicities and durations). FFS is multiple can be active at the same time. FFS if multiple aperiodic gaps are supported.

- 4:	UE provides assistance information to the gNB of NW A in Connected state based on the configuration of USIM of NW B for the gNB to determine the necessary switching parameters. Up to network what is the action based on UE assistance information. FFS what assistance information is needed.

- Agreements (RAN2#115e)

- Only per UE level scheduling gap is supported in Rel-17 for non-DC. FFS if we support MR-DC.

- The network is allowed to configure at most 3 gap patterns (for any MUSIM purpose).

- Only a single aperiodic gap (for MUSIM) is supported in Rel-17. At most two periodic “gaps” (for MUSIM) and a single aperiodic gap (for MUSIM) is supported in Rel-17. FFS if signalling supports more.

- The SFN and subframe of the PCell of the network A is used in the gap configuration to calculate the gap.

- The switching gap configuration will explicitly provide the gap starting position (e.g. offset value or start SFN and subframe explicitly), gap length and gap repetition period.

- Switching Gaps (of any type) are configured or released by RRC signalling (e.g. RRCReconfiguration message) in Rel-17. FFS if gap can be released autonomously by UE after N repetitions.

- UE is allowed to include assistance information for setup or release of gaps for both 1) periodic gaps and 2) aperiodic gap in one UEAssistanceInformation Msg.

- To report the assistance information, the UE maps the timing info of the Gap on the network B  to the network A and reports the mapped timing info to the network A.

- For the gap assistance information, the Gap start time, Duration of the gap and gap repetition period (for periodic) may be included. FFS is other information is included (e.g. gap purpose).

- Do not support autonomous gaps for MUSIM in Rel-17.

- Agreements (RAN2#116e)

- - RAN2 will not specify MN-SN coordination of MUSIM gaps with MR-DC in Rel-17.

- - RAN2 will not create MAC CE activation of gaps in MUSIM, but if the common gap discussion allows this anyway, RAN2 will not prevent that, either.

- 1: RAN2 will not work in Rel-17 for the case that Dual-RX/Single-TX UE or Single-RX/Single-TX UE stays in RRC_CONNECTED mode in NW A while performing reception and transmission in NW B (in RRC_ CONNECTED or during RRC setup/resume period).

- 2: MR-DC is not supported in Rel-17.

- - Wait for RAN4 feedback on gap pattern support (can use FFS in RRC for maximum value)

- 4: RAN2 understands that the intent of aperiodic gap is as follows (no need to specify):

- -	If until the end of the aperiodic gap the UE still has not completed activity in NW B, e.g. due to the random access for on-demand SI request, the UE should stop the activity in NW B and switch to NW A. If needed, the UE can request another aperiodic gap in NW A.

- - RAN2 does not intend to specify any new signalling in Rel-17 for early return. If legacy signalling allows it, RAN2 does not intend to preclude it.

- 5: Do not introduce gap purpose for gap related MUSIM assistance information.

- 6: FFS how UE indicates release of gap pattern.

- 7: FFS if UE is allowed to update UAI message after the UE performs cell reselection in NW B or after the UE performs handover in NW A.

- 8: Autonomous release of MUSIM gap by UE after N repetitions is not supported.