3GPP TSG-RAN2 Meeting #117-e	R2-220xxxx

Online, 21 February – 03 March 2022

Agenda Item:	9.2.3

Source:		Huawei, HiSilicon

Title:		Report of [AT117-e][012][IOT-NTN] Control Plane (Huawei)

Document for:	Discussion and decision

# Introduction

This is the report of the email discussion on IOT NTN control plane open issues:

- [AT117-e][012][IoT-NTN] Control Plane (Huawei)

Scope: Based on R2-2203221 progress P5a and P7, address whether to move t-service to other SIB, address P5 from R2-22003721, Include OI 2.11 and OI 2.12 from AI 9.2.5. based on R2-2203220 progress the details, based on R2-2203457 (Ericsson), progress the details (proponent to drive the argumentation if any). Determine agreeable parts, Aim to agree offline, if needed identify CB points.

Intended outcome: Report.

Deadline: In time for on-line CB W2 Tuesday

In order to be able to have offline agreements and have a report for the online session on Tuesday, the following schedule is proposed

First round: Deadline for companies inputs: 		Friday W1 15:00 UTC

Rapporteur draft summary and list of proposals: 	Friday W1 18:00 UTC

Second round: Comments and offline agreements:	Tuesday W2 12:00 UTC

# Contact Information

| Company | Name | Email |
| --- | --- | --- |
| Huawei, HiSilicon | Odile Rollinger | odile.rollinger@huawei.com |
| Apple | Pavan Nuggehalli | pnuggehalli@apple.com |
| Lenovo | Min Xu | xumin13@lenovo.com |
| Spreadtrum | Xu Liu | xu.liu1@unisoc.com |
| Transsion Holdings | Wen wu | wen.wu5@transsion.com |
| Nokia | Ping Yuan | Ping.1.Yuan@nokia-sbell.com |
| Ericsson | Jonas Sedin | Jonas.sedin@ericsson.com |
| OPPO | Haitao Li | lihaitao@oppo.com |
| ZTE | Ting Lu | lu.ting@zte.com.cn |
| Xiaomi | Xiaolong Li | lixiaolong1@xiaomi.com |
| InterDigital | Brian Martin | brian.martin@interdigital.com |
| NEC | Yuhua chen | Yuhua.chen@emea.nec.com |
| CATT | Xiangdong Zhang | zhangxiangdong@catt.cn |

# First round: Discussion

## MAC & RRC actions upon expiry of the UL sync validity timer.

In RAN2#166bis-e, it was agreed:

- When SI used for UL synch (pre-compensation) is no longer valid, the UE autonomously tunes away and re-aquires the required SI, and then comes back

The UE actions upon expiry of the UL sync validity timer and the modelling have not been discussed in details.

During the Pre-offline discussion [1], it was commented that the meaning of ‘the UE autonomously tunes away and re-aquires the required SI’ was not clear and the following actions were suggested:

- Flushing HARQ buffers, clearing configured uplink grants etc (ZTE)

- When the timer expired, UE should stop UL transmission since its UL sync lost. When the UE UL sync recovered after re-acquiring the new SI, the UE can come back to service and able to perform UL transmission. The mechanism is similar to the maintenance of uplink time alignment when UL synchronization status is “non-synchronised” where it is only the UL synchronization lost, but the DL synchronization is kept. (Nokia)

- The handling of the validity timer is similar to PUR timing alignment timer, i.e. MAC does not initiate any UL transmission if the timer is not running. Nothing else is needed. (Huawei)

- After the expiry of the timer, MAC can suspend UL transmission, and perhaps other behaviours as mentioned by ZTE. (Xiaomi)

Based on the above, companies are invited to provide their views on the actions related to the UL:

- Upon timer expiry, UE stops all UL transmissions, flushes all HARQ buffers and releases all UL (PUCCH, SR, configured grants) resources. Upon recovery (timer is restarted), UE triggers RACH to resume UL operation

- Upon timer expiry, UE stops all UL transmissions, flushes all HARQ buffers and maintains all UL resources. Upon recovery (timer is restarted), UE resume UL operation (no RACH is needed).

- Other

Discussion point 1-1: UE actions in UL upon timer expiry and recovery

| Company | a/ b or c | Comments |
| --- | --- | --- |
| MediaTek | b | We think there is no need to release all UL (PUCCH, SR, configured grants) resources |
| Apple | b | Also OK with not doing anything |
| Lenovo | b or nothing |  |
| Spreadtrum | b | Maintaining all UL resource can ensure that UL operation can be resumed as soon as possible. |
| Transsion Holdings | b | We think maintain all UL resources is better for fast recovery |
| Qualcomm | b | Probably HARQ flushing is also not necessary. |
| Nokia | a with comment | Without RACH procedure, we are not sure whether UE can send PUSCH/PUCCH correctly with a valid TA based on UE’s self-estimation (with the new acquired ephemeris/common TA parameter). When the validity timer expired, the previously acquired ephemeris/Common TA parameters are considered to be outdated. UE should re-acquire SIB for new ephemeris and common TA parameters, then re-estimate the TA for UL compensation. This means the previous UE estimated TA and closed TA adjustment (via TA MAC CE) are not valid anymore. To maintain the TA between UE and eNB, we think UE should trigger RACH at least for new TA alignment and then NW can adjust that in RAR. Please note the maximum TA adjustment in RAR can be up to 2ms while the adjustment for TAC MAC CE in RRC Connected mode is only 0.017ms. RAR is needed for quick TA adjustment. For whether UE should release or maintain all UL UE resource, it is a separate issue. We slightly prefer to keep it simple to just follow TAT timer expiry handling (i.e. release UL resources and trigger RACH) to avoid more specification impact. |
| Huawei, HiSilicon | b | It was discussed last meeting that the network did not need to know, thus RACH is not needed and the UE resources should not be released. This is different for TimeAligment where the timer needs to be aligned between the NW and the UE |
| Ericsson | b | To keep things simple we are fine to not do RACH and no need to release UL resources. |
| OPPO | b with comment | Upon timer expiry, we agree that UE stops all UL transmissions, and maintain all UL resources, but UE doesn’t need to flush all HARQ buffers. And upon recovery, RACH is needed. Therefore, we suggest the modified option b: b1) Upon timer expiry, UE stops all UL transmissions, does not flush all HARQ buffers, and maintains all UL resources. Upon recovery (timer is restarted), UE triggers RACH. Similar to the legacy UL out of synchronization caused by the expiry of TAT, UL out of synchronization caused by the expiry of validity timer leads to the infeasible UL transmission, therefore stopping all UL transmissions is reasonable. Upon expiry of TAT, the UE flushes all HARQ buffers and release all resource configuration including PUCCH, SRS, CG, SPS, etc, so that network could allocate these radio resources to other UEs. Different from the case of TAT expiry, the validity timer expiry at the UE is not known to the network. That means even if the UE releases the all resources autonomously, since the network will not be aware of this, these resources would not be used by other UEs. So in our view, upon expiry of validity timer, UE doesn’t need to flush all HARQ buffers and release all UL resource configuration, only needs to stop UL transmission and suspend the resource configuration rather than release these resource configuration. When timer expiry, the UE autonomously tunes away and re-acquires the required SI, so UE might miss some scheduling from NW. Upon recovery, we think RACH is needed to let NW know that UE has come back. |
| ZTE | b | Even we are proponent for a), b) is acceptable to us. We think it’s obviously all HARQ buffers need to be flushed. We can understand the above concern that no TA adjustment to the re-estimated TA pre-compensation. This is different from that when UE enters connected mode from idle mode. But we think this may be not so critical. NW can adjust TA via TAC MAC CE later. If other issue is found, we can re-discuss whether PRACH procedure need to be triggered. |
| Xiaomi | a | If radio resources are not released, for recurring CG, UE doesn’t know whether to handle it or not, and UE doesn’t know how to handle the timers like sr-ProhibitTimer.(running or pending). But we are ok to go with b. |
| InterDigital | b | That is our understanding of the agreement i.e. “UE autonomously tunes away and re-aquires the required SI, and then comes back”, we don’t think additional actions need to be specified. |
| NEC | See comment | From RAN2 perspective, we understand that UE shall stops all UL transmissions, flushes all HARQ buffers, but no need to release UL resource Regarding whether RACH is required, should this be discussed in RAN1/RAN4 whether TA alignment/adjustment via RACH is required after TA compensation re-estimated |
| CATT |  | We can wait for the discussion progress of NTN, and this problem belongs to UP topic. |

Summary

TBC

Similarly, the actions related to the DL needs to be clarified. However, there were no suggestion in the pre-offline discussion [1].

Companies are invited to provide their views on the actions related to the DL

Discussion point 1-2: UE actions in DL upon timer expiry and recovery

| Company | Comments |
| --- | --- |
| MediaTek | Nothing is needed in DL |
| Apple | Agree with MediaTek that nothing needs to be specified |
| Lenovo | Not needed |
| Spreadtrum | Not needed. |
| Transsion Holdings | As the UE need to require the SIB ,the UE should assume no DL data transmission during the period. |
| Qualcomm | We are open to check for any impact/interruption in RLM and measurements or some running timers. |
| Nokia | We think UE’s DL synchronization is kept even the validity timer is expired. However, if UE receives the DL PDCCH for scheduling (while UE stop UL transmissions due to validity timer expiry), it will cause scheduling failure since the UE cannot send the PUCCH Ack for DL scheduling. NW may need to avoid scheduling the UE if the validity timer expired in UE. Please refer to our input in Discussion Point 1-5. |
| Huawei, HiSilicon | At least NB-IOT UE cannot receive system information in parallel of unicast transmission, so same as for UL, the UE shall stop DL unicast reception and flush the HARQ buffers before starting receiving system information. In addition, the DL carrier / narrowband may need to be reconfigured if different from the one in use. |
| Ericsson | It could be up to UE implementation if the UE happens to read any transmitted in the downlink. |
| OPPO | Not needed. |
| ZTE | Agree with Huawei that the UE shall stop DL unicast reception and flush the HARQ buffers before starting receiving system information. |
| Xiaomi | Capture in spec that UE is not required to monitor PDCCH for DL data. Then it is up to UE implementation whether to monitor PDCCH for DL data or not. |
| InterDigital | It’s not clear that there is any additional impact, but as QC mentioned we can check if there is impact e.g. to RLM etc. Also agree with HW that DL unicast would have to be temporarily stopped and UE tunes to another narrowband or carrier to receive SI but this seems to be covered by the agreement already i.e. “UE autonomously tunes away and re-aquires the required SI, and then comes back” |
| NEC | We share the similar understanding as Nokia and Huawei, but we do not need to specify much but leave to UE implementation |
| CATT | Nothing is needed in DL. |

Summary

TBC

The next point is whether the timer should be maintained in RRC or MAC.

Based on the understanding of the required actions in UL and DL, companies are invited to provide their views on where the timer should be maintained, RRC or MAC, and how to specify the interactions between RRC and MAC.

Discussion point 1-3: Whether the timer is maintained in RRC or MAC and how to specify the interactions

| Company | RRC or MAC | Comments |
| --- | --- | --- |
| MediaTek | RRC | RRC seems to be a better option. |
| Apple | MAC | But no strong preference |
| Lenovo | either is OK |  |
| Spreadtrum | either is ok |  |
| Transsion Holdings | RRC | No strong preference |
| Qualcomm | MAC | MAC can be in charge of UL sync timer like TAT. RRC can receive UL sync failure indication and upon which it can take actions. |
| Nokia | No strong view. |  |
| Huawei, HiSilicon | - | We think both works but it may be easier to capture the interactions between RRC and MAC if the timer is maintained in MAC |
| Ericsson | RRC | In the end, the timer is all about having read a certain system information, thus the action of starting, expiring and stopping the timer is all in RRC. There can be an indication that UE shall not transmit anything in the UL from RRC to upon expiry which I suppose is one section and one line in MAC. Another motivation is that this timer may start and run when the UE is idle mode, thus we think it makes more sense to model it in RRC. Then we guess that the timer should be given a T3xx name. |
| OPPO | RRC |  |
| ZTE | RRC | Such UL sync validity timer would (firstly) start when UE is in idle mode, e.g., upon acquiring of SIBXX before random access. Moreover, the expiry of this timer may be before or after UE enters into connected mode. Since the timer process would be related to UE state, we think RRC is the suitable place to maintain this UL sync validity timer. |
| Xiaomi | RRC | MAC doesn’t know when to start the timer. For implicit indication of the epoch time, epoch time is the ending position of SI window, MAC have no idea of this. |
| InterDigital | MAC | It is just a modelling issue, either way works, but MAC looks more convenient and it’s in line with how e.g. TAT is specified. It could also just be left to MAC and RRC CR specification rapporteurs to decide how to capture. |
| NEC | RRC | Since the timer is (re)started after reading the system information which is specify in RRC |
| CATT | RRC | The timer will trigger the SIBX updated, so it should be kept in RRC. |

Summary

TBC

Finally, there were some suggestions in [1] that SIBXX acquisition in RRC_CONNECTED mode should be fully integrated in section 5.2 System Information acquisition and that there was no need for any description in section 5.3.3 RRC connection establishment.

Companies are invited to provide their views on how to capture SIBXX acquisition in RRC_CONNECTED:

Discussion point 1-4: How to specify SIBXX acquisition in RRC_Connected mode

| Company | 5.2.2, 5.3.3., other | Comments |
| --- | --- | --- |
| MediaTek | 5.2.2 | We think 5.2.2 is better where all the SIBs are defined. |
| Apple | 5.2.2 |  |
| Lenovo | 5.2.2 |  |
| Spreadtrum | 5.2.2 |  |
| Transsion Holdings | 5.2.2 |  |
| Qualcomm | Other | Upon reception of UL sync failure indication from the lower layers. Then refer to 5.2.2. |
| Nokia | 5.3.3 | Prefer to have a dedicated section to cover UE actions upon indication of ul-SyncValidityTimer expiry. But it can refer to 5.2.2 for SIBXX (re)acquisition. |
| Huawei, HiSilicon | other | We think it is not possible to capture all in 5.2.2. We need a trigger and some MAC-RRC interactions before acquiring the SIB and after to resume normal operation, this needs to be captured in 5.3.3. |
| Ericsson |  | We are not sure exactly what is referred to, and what was the concern in [1]. There are 3 mentions of changes in last e-mail discussion that was proposed by Xiaomi. - One change of 5.2.2.3 that would force UE to read SIBxx in idle mode. - No need for this as SIBxx is not required in idle mode. - Change in 5.2.2.4 that states that UE shall connect without having read SIBxx. - Up to rapporteur if to pursue this or go with current text in 5.2.2.4. - Change to 5.2.2.5. - If we will have a guard timer we probably have to have own section for the behaviour upon ul-SyncValidityTimer. This could be defined under either 5.3.11.x or 5.3.xx. Up to rapporteur. Agree with Nokia, otherwise. |
| OPPO | 5.2.2 |  |
| ZTE | 5.2.2 | At least changes to 5.2.2 are needed. |
| Xiaomi | 5.2.2 | It is quite simple to capture it in 5.2.2.4 as below: 1> if the UE is in RRC_IDLE and enters a cell for which the UE does not have stored a valid version of the system information required in RRC_IDLE, as defined in 5.2.2.3: 2> acquire, using the system information acquisition procedure as defined in 5.2.3, the system information required in RRC_IDLE, as defined in 5.2.2.3; 1> if in RRC_CONNECTED and ul-SyncValidityTimer is not running; and 1> the UE is a BL UE or the UE is in CE or the UE is a NB-IoT UE; 2> acquire the required system information, as defined in 5.2.2.3; 1> following successful handover completion to a PCell for which the UE does not have stored a valid version of the system information required in RRC_CONNECTED, as defined in 5.2.2.3:. |
| InterDigital | other | Agree with QC and HW again. |
| NEC | 5.2.2 |  |
| CATT | 5.2.2 |  |

Summary

TBC

Companies are invited to indicate aspects that needs to be discussed.

Discussion point 1-5: Any aspects that have not been addressed above

| Company | Comments |
| --- | --- |
| Nokia | Once the validity timer is expired in UE, UE should stop UL transmissions including both PUSCH and PUCCH (for HARQ-Ack feedback). For DL, if eNB is not aware of validity timer expiry in UE hence continue scheduling the UE no matter in DL or UL, there will have scheduling failure after expiration of the validity timer and could be problematic for network performance. There may have several aspects need to be discussed: - Can we assume that UE’s sporadic short data transmission (assumption in Rel-17) can be completed within the validity timer thus NW don’t care about UE’s timer status at all ? - How long the UE will recover from validity timer expired status ? If the recovery period is very long, then the performance impact is quite big. Otherwise, maybe eNB can continue scheduling during the short out of sync period. - How eNB can know UE’s validity timer started and expired (and then stop scheduling) if needed ? In our understanding: For 1), even for sporadic short transmission, the data transmission may not be completed within the validity timer since it is totally up to NW scheduling decision based on cell load and UE status. Please consider there are many UEs to be scheduled in the cell, if the UE’s data transmission is started/scheduled near the end of the started validity timer, it may not be able to complete the data transmission within the validity timer. For 2), since IoT UE is half duplex and is not required to read SIB in RRC Connected mode in legacy, it is not clear how long time the UE requires to recover from validity timer expiry (e.g. how long UE need to tune away and then re-acquire the SIB, especially for poor coverage conditions where SIB with repetitions). For 3), from NW point of view, we would prefer UE and gNB have a common understanding on validity timer status to avoid scheduling failure. Hence, we have below proposal: Proposal: RAN2 to discuss whether sporadic short data transmission (assumption in Rel-17) can be completed within the validity timer in Rel-17. If not, RAN2 to decide if any enhancement is needed to avoid scheduling failure. |
| Xiaomi | The SIB requirement for connected mode UE and idle/inactive mode UE as below(we are ok to only capture connected mode requirement): - 5.2.2.3 System information required by the UE The UE shall: - ensure having a valid version, as defined below, of (at least) the following system information, also referred to as the ‘required’ system information: 2> if in RRC_IDLE: 3> if the UE is a NB-IoT UE: 4> the MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB and SystemInformationBlockType1-NB as well as SystemInformationBlockType2-NB through SystemInformationBlockType5-NB, SystemInformationBlockType22-NB, SystemInformationBlockTypeXX-NB (if broadcast); 3> else: 4> the MasterInformationBlock and SystemInformationBlockType1 (or SystemInformationBlockType1-BR depending on whether the UE is a BL UE or the UE in CE) as well as SystemInformationBlockType2 through SystemInformationBlockType8 and SystemInformationBlockType24 (depending on support of the concerned RATs), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC), SystemInformationBlockType29 (only for BL UE or the UE in CE depending on support of resource reservation), SystemInformationBlockTypeXX (only for BL UE or the UE in CE if broadcast); 2> if in RRC_INACTIVE: 3> the MasterInformationBlock and SystemInformationBlockType1 as well as SystemInformationBlockType2 through SystemInformationBlockType8 (depending on support of the concerned RATs), SystemInformationBlockType24 (depending on support of the concerned RATs), SystemInformationBlockType25, SystemInformationBlockTypeXX (only for BL UE or the UE in CE if broadcast); 2> if in RRC_CONNECTED; and 2> the UE is not a BL UE; and 2> the UE is not in CE; and 2> the UE is not a NB-IoT UE: 3> the MasterInformationBlock, SystemInformationBlockType1 and SystemInformationBlockType2 as well as SystemInformationBlockType8 (depending on support of CDMA2000), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC); 2> if in RRC_CONNECTED and T311 is running; and 2> the UE is a BL UE or the UE is in CE or the UE is a NB-IoT UE; 3> the MasterInformationBlock (or MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT), SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT), and for NB-IoT SystemInformationBlockType22-NB; 2> if in RRC_CONNECTED and ul-SyncValidityTimer is not running; and 2> the UE is a BL UE or the UE is in CE or the UE is a NB-IoT UE; 3> the SystemInformationBlockTypeXX (or SystemInformationBlockTypeXX-NB in NB-IoT, if broadcast); |
| Xiaomi | where does GNSS fix outdated indication come from. The current running CR captures that GNSS position fix outdate indication is from lower layer as follow: ### 5.3.3.x UE actions upon indication of out-of-date GNSS position fix Upon indication from lower layers that the GNSS position fix has become out-of-date while in RRC_CONNECTED, the UE shall: - perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’.. However, in our understanding, the GNSS position fix outdate indication is from the GNSS module. Thus, “upper layers” instead of “lower layers” should be used. ### 5.3.3.x UE actions upon indication of out-of-date GNSS position fix Upon indication from upper layers that the GNSS position fix has become out-of-date while in RRC_CONNECTED, the UE shall: - perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’.. |

Summary

TBC

## Provision of SIBXX in RRCConnectionReconfiguration for handover

As the result of pre-offline [012] [1], the following was agreed:

- All parameters needed to access the target cell are included in RRCReconfiguration message for handover.

- For simplicity, the whole SIBXX structure is included in RRCReconfiguration message for handover.

In the summary of the pre-offline discussion, it was proposed to discuss whether this should be included within mobilityControlInfo or outside similarly to SIB1/SIB2.

RRCConnectionReconfiguration-r8-Ies ::= SEQUENCE {

measConfig							MeasConfig						OPTIONAL,	-- Need ON

mobilityControlInfo					MobilityControlInfo				OPTIONAL,	-- Cond HO

dedicatedInfoNASList				SEQUENCE (SIZE(1..maxDRB)) OF

DedicatedInfoNAS			OPTIONAL,	-- Cond nonHO

radioResourceConfigDedicated		RadioResourceConfigDedicated	OPTIONAL, -- Cond HO-toEUTRA

securityConfigHO					SecurityConfigHO				OPTIONAL,	-- Cond HO-toEPC

nonCriticalExtension				RRCConnectionReconfiguration-v890-Ies	OPTIONAL

}

RRCConnectionReconfiguration-v1130-Ies ::= SEQUENCE {

systemInformationBlockType1Dedicated-r11	OCTET STRING (CONTAINING SystemInformationBlockType1)																			OPTIONAL,	-- Need ON

nonCriticalExtension				RRCConnectionReconfiguration-v1250-Ies	OPTIONAL

}

RRCConnectionReconfiguration-v1430-Ies ::= SEQUENCE {

sl-V2X-ConfigDedicated-r14		SL-V2X-ConfigDedicated-r14		OPTIONAL,	-- Need ON

sCellToAddModListExt-v1430		ScellToAddModListExt-v1430		OPTIONAL,	-- Need ON

9erc-GapIndicationRequest-r14	ENUMERATED{true}					OPTIONAL,	-- Need ON

systemInformationBlockType2Dedicated-r14	OCTET STRING (CONTAINING SystemInformationBlockType2)											OPTIONAL,	-- Cond nonHO

nonCriticalExtension			RRCConnectionReconfiguration-v1510-Ies		OPTIONAL

}

Companies are invited to provide their views on how to signal SIBXX in RRCConnectionReconfiguration message and indicate whether some procedure text is needed.

- within mobilityControlInfo

- outside mobilityControlInfo, similarly to SIB1/SIB2

- other

Discussion point 2: How to signal SIBXX in RRCConnectionReconfiguration and whether some procedure text is needed

| Company | a), b) or c) | Comments |
| --- | --- | --- |
| MediaTek | a) | Option a) seems simple |
| Apple | a) |  |
| Lenovo | a) |  |
| Spreadtrum | a) |  |
| Transsion Holdings | b) | As it is SIBXX, we think it can similarly to SIB1/SIB2 |
| Qualcomm | b) | Same as for SIB1 dedicated. |
| Nokia | b) | Similar to other SIBs in current specification |
| Huawei, HiSilicon | b) | we think both are OK |
| Ericsson | - | Up to rapporteur. Do not see much of a difference. I think we may need to discuss that epochTime is mandatory present when included in handover command, since the UE cannot rely upon the SI window on when the ul-SyncValidityTimer should start when performing a handover. In this way, perhaps it is more suitable to include ServingSatelliteInfo-r17 instead of the whole SIBxx. |
| OPPO | a) | We share the same views as Ericsson that epochTime is mandatory present when included in handover command. |
| ZTE | a) | Option a) seems simple and see no other need. |
| Xiaomi | a) |  |
| InterDigital | b) |  |
| NEC | b) |  |
| CATT | a) | It seems only the handover case needed. We need to discuss the possible case that needs SIBXX in RRCConnectionReconfiguration. |

Summary

TBC

## Location of t-Service parameter

t-service parameter is signalled withing SIBXX in the current RRC CR:

SystemInformationBlockTypeXX-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...

}

ServingSatelliteInfo-r17 ::= 	SEQUENCE {

ephemerisStateVectors-r17		EphemerisStateVectors-r17		OPTIONAL,	-- Need OR

ephemerisOrbitalParameters-r17	EphemerisOrbitalParameters-r17	OPTIONAL,	-- Need OR

nta-CommonParameters-17			SEQUENCE {

nta-Common-r17					INTEGER (0..8316827) 		OPTIONAL,	-- Need OP

nta-CommonDrift-r17				INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29470)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240},

epochTime-r17					SEQUENCE {

startSFN-r17					INTEGER (0..1023),

startSubFrame-r17				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17					INTEGER (0..1023),

k-Mac-r17						INTEGER (1..512) 				OPTIONAL,	-- Need OP

t-Service-r17              		TimeUTC-r17						OPTIONAL,	-- Need OR

...

}

During the pre-offline discussion about the handling of SIBXX in RRC_IDLE, it was highlighted that t-service was the only parameter used in RRC_IDLE while all other parameters were for use during access or during RRC_CONNECTED and that it might be better to move the parameter to another SIB.

Companies are invited to provide their views on whether to move t-service to another SIB and which one.

Discussion point 3: Whether to move t-Service to another SIB and which one

| Company | yes/ no | Comments |
| --- | --- | --- |
| MediaTek | No | No need to move this. Other ephemeris (neighbour cell), agreed for discontinuous coverage will also be there. |
| Apple | No | Prefer to be aligned with NR NTN |
| Lenovo | No | Align with NR NTN |
| Spreadtrum | No | Align with NR NTN. |
| Transsion Holdings | No | We don’t think t-service is the only parameter used in RRC_IDLE, for example the start-time of the incoming satellite’s coverage, or other potential parameters for discontinuous coverage predicting may still used in RRC_IDLE, so no need to move this now. |
| Qualcomm | Yes | TBS size in IoT is small. This SIB may need to be scheduled frequently. We prefer to separate static parameters to different SIB. |
| Nokia | No |  |
| Huawei, HiSilicon | Yes | It was agreed that information about neighbours will be signalled in another SIB, (e.g. SIBYY). Thus t-service is the only parameter in SIBXX applicable to UE in RRC_IDLE. In addition, it is not applicable to RRC_CONNECTED. Keeping t-service in SIBXX will require the UE to acquire SIBXX in RRC_IDLE for example at cell selection/reselection. This is unnecessary power consumption. W.r.t to which SIB, we have a preference for SIB3 as the parameter is used for measurements. |
| Ericsson | Yes | We see pros and cons of both. We would expect that new SIBxx should be signalled more often than other SIBs. So does not hurt to have in SIBxx. On the other hand, most other idle mode info are in SIB3 and SIB4. We would be fine to have in SIB3 as Huawei suggest. |
| OPPO | No | Align with NR NTN. |
| ZTE | Yes | We cannot clearly understand HW’s comment about UE in idle mode. We assume UE needs to (re)acquire t-service and other material in SIBXX after UE select/reselect to a cell. Meanwhile, after UE camps on a cell and before UE triggers random access, UE doesn’t need to require either t-service or material in SIBXX. From this aspect, we see no motivation to separate t-service from SIBXX. But we are thinking, to separate t-service from SIBXX may be beneficial for UE in connected mode. The t-service is not needed for UE in connected mode. Without t-service, UE in connected mode can require a SIBXX with less bits. SIB3 is fine. |
| Xiaomi | Yes | We assume the information for UE to predict the discontinuous coverage will be included in another NTN specific SIB (not current SIBx), otherwise UE needs to acquire the SIB for reselection purpose, instead of only required to acquire it before access. So we prefer to move t-service to the other SIB. |
| InterDigital | No | We agreed that SIBxx is an essential SIB, so there is no particular benefit to move this elsewhere, UE has to receive SIBxx in RRC_IDLE otherwise cell is barred. |
| NEC | Yes | Reason to move T-service: - For existing SIBs, information relevant to initial access and information relevant to cell reselection are in different SIBs, T-service is cell reselection information, others are initial access relevant information - we agreed that UE will be required to read SIBxx only before initial access (this is applicable to only information relevant to initial access), but this is not true for T-service - ul-SyncValidityDuration is applicable to other IE but not so suitable for T-service we think T-service should be in another SIB together with other cell (re)selection information |
| CATT | No | Align with NR NTN. |

Summary

TBC

## Guard timer to handle the case where the UE takes ‘forever’ reacquire the SIBXX

During the online session, the need for a guard timer to handle the case where the where the UE takes ‘forever’ reacquire the SIBXX was discussed and the following was captured in the chair’s notes:

- FFS if we Will have a guard timer to handle the case where the UE takes ‘forever’ reacquire the SIB. At timer expiry UE triggers RLF handling. (Note that it is expected that the timer will not expire in the normal case, and the UE can just come back acc to previous decision).

During the online, it seems that companies has different understanding of the purpose and handling of the potential guard timer, so it is proposed to clarify this first.

From the inputs, the rapporteur’s understanding of the UE behaviour is as follows:

- Upon UL synchronisation timer expiry, the UE shall:

  - Start the guard timer (TXXX)

  - Tune away and reacquire SIBXX

- Upon successful reception of SIBXX, the UE shall:

  - Stop the guard timer (TXXX)

  - Tune away and resume normal operation

- Upon expiry of the guard timer, the UE shall:

  - Trigger RLF and performs RRC Connection Re-establishment

Based on the above, companies are invited to provide their views on the following:

- Do you agree with the above description of the timer handling. Please provide comment if not

- If a guard timer is introduced, should a new UE timer be introduced or an existing one (and which one) be reused.

- Do you support introducing a guard timer for SIXX acquisition in connected mode. Please justify your answer.

Discussion point 4: Guard timer for SIBXX acquisition in connected mode

| Company | Point a: Yes/No | Point b: New/ existing | Point c: Yes/ No | Comments |
| --- | --- | --- | --- | --- |
| MediaTek | Yes | New | Yes | Point c) should be the baseline. |
| Apple | Yes | New | No | What is so special about SIBxx with respect to other SIBs for which we don’t have such handling? Can leave to UE implementation. |
| Lenovo | Yes | New | Yes |  |
| Spreadtrum | Yes | New | Yes | A guard timer can be applied to timing alignment. |
| Transsion Holdigs | No | New | Yes | a）Trigger RLF and performs RRC Connection Re-establishment seems contradict to the existing conclusion(RAN2#117e SIBXX is an essential SIB, i.e. the UE shall consider the cell barred if it is unable to acquire the SIB when scheduled. ) |
| Qualcomm | Yes | new | Yes | If UE cannot recover until the guard timer expiry, then there must be some issue which can be fixed by reestablishment. |
| Nokia | No. But reuse legacy procedure | Reusing legacy timer | No | For Point c). We don’t think an additional timer is needed to guard the UE to re-acquire the SIBXX within the defined timer. The legacy timer and procedure can handle the case. In our view, the UE’s validity timer expiry may be caused by, for example: - Case1) bad radio channel conditions of serving cell - Case2) UE is in UL PUSCH transmissions where UE cannot decode SIB for ephemeris due to half duplex. For Case1, the existing RLF mechanism (based on T310) is enough for UE trigger RLF and perform RRC Re-establishment, because it is not only the SIBXX cannot be decoded successfully but also PDCCH and other channels cannot be decoded successfully in bad radio channel conditions. For Case2, UE may tune away to re-acquire the new SIB after data transmissions. It seems not possible UE will take ‘forever’ reacquire the SIB if the radio channel is good enough, because the NW should broadcast ephemeris and common TA via SIB in a reasonable frequency/periodicity. Furthermore, if the UL validity timer expired, UE will stop UL transmission, eNB will anyway detect the UL radio link failure (since UL transmissions are lost for a duration). eNB may release the UE accordingly hence there is no ‘forever’ SIB acquiring in UE. Another possibility is that, NW will stop maintain the TAT timer due to no UL transmission available. UE’s TAT timer may be expired, UE will re-acquire SIBXX and trigger RACH for UL recovery. If RAN2 would like to go the way with the new guard timer introduced, we are wondering how UE behaviour to resume data transmission upon successful reception of SIBXX during the timer ? There is no existing procedure in legacy RLF procedure (e.g. in T310), should RAN2 define new procedure that UE should inform NW via RACH for TA alignment ? It will make the specification complex as well. |
| Huawei, HiSilicon | Yes | FFS | Yes | There will be case where the UE has changed cell and will be unable to reacquire the SIB. |
| Ericsson | Yes | New | Yes | Network needs this for predictability in UE actions. If these important actions are up to UE implementation it will be challenging for network to handle the vast array of options. Using values of T310 could be a reasonable baseline value range, but we can check reasonable values for next meeting. |
| OPPO | No | No need | No | We don’t see the need of guard timer. Anyway, the existing RLF and RRC re-establishment would ensure UE doesn’t take ‘forever’ reacquire the SIB. |
| ZTE | Yes | New | Yes | In LEO, the channel quality of UE may deteriorate. It is possible that UE cannot reacquire SIBXX. Hence, a guard timer for UE reacquiring SIBXX during connected mode is necessary. In most cases, we assume UE can normally back to connected mode after it reacquires the SIBXX. For a few other abnormal cases that UE cannot reacquire SIBXX, this guard timer would be beneficial to avoid that UE stuck in the reacquiring procedure. |
| Xiaomi | No | Legacy timer | No | We don’t see the difference between SIBx and SIB2 and there is no timer for UE to reacquire the SIB2, the guard timer is not needed for UE to reacquire the SIBx. |
| InterDigital | Yes | TBD | Yes | This doesn’t necessarily mean we define a new timer T3xx, just a time period could be specified. What isn’t very clear is how to decide how long this time should be. For autonomous gaps (e.g. report CGI) RAN4 defined that based on performance aspects, do we need to ask RAN4? If it turns out to be complicated we can consider re-using RLF timers, such that UE is allowed to re-tune to receive SIBxx within T310 |
| CATT |  |  | No | For connected UE, the RLM can avoid the UE to reacquire the SIBXX forever. When the RLF is detected, UE will trigger RRC reestablishment. So no need to introduce an guard timer. As for the idle/inactive UE, for SIB1 and other SIB, there is also no guard timer is used for UE to avoid reacquire SIB1 and other SIB forever. |

Summary

TBC

## Signalling of part-of-ARFCN indication in MIB in NB-IOT

Introducing signalling of part-of-ARFCN in MIB for NB-IoT was agreed in RAN2#117-e and the signalling details were pushed to offline.

In [2], it is proposed to have an indicator that part-of EARFCN is present in addition to the two LSB bits and the following ASN.1 was proposed

MasterInformationBlock-NB ::=	SEQUENCE {

systemFrameNumber-MSB-r13		BIT STRING (SIZE (4)),

hyperSFN-LSB-r13				BIT STRING (SIZE (2)),

schedulingInfoSIB1-r13			INTEGER (0..15),

systemInfoValueTag-r13			INTEGER (0..31),

ab-Enabled-r13					BOOLEAN,

operationModeInfo-r13			CHOICE {

inband-SamePCI-r13				Inband-SamePCI-NB-r13,

inband-DifferentPCI-r13			Inband-DifferentPCI-NB-r13,

guardband-r13					Guardband-NB-r13,

standalone-r13					Standalone-NB-r13

},

additionalTransmissionSIB1-r15	BOOLEAN,

ab-Enabled-5GC-r16				BOOLEAN,

earfcn-Indicator-17			BOOLEAN,

earfcn-LSB-r17					BIT STRING (SIZE (2)),

spare							BIT 	STRING (SIZE (69))

}

MasterInformationBlock-NB field descriptions

earfcn-Indicator, earfcn-LSB If earfcn-Indicator is set to TRUE, earfcn-LSB indicates the 2 least significant bits of the EARFCN. If earfcn-Indicator is set to FALSE, earfcn-LSB is set to ‘00’.

It was commented online that the indicator may not be needed.

Companies are invited to provide their views on whether a presence indicator is needed and comment on the proposed ASN.1.

Discussion point 5: Whether a presence indicator is needed and other comments on the ASN.1

| Company | indicator: Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes | A simple editor’s note should be enough. |
| Apple | No |  |
| Spreadtrum | No |  |
| Qualcomm | Depends on | What is the difference between - earfcn-Indicator is set to TRUE and earfcn-LSB is set to ‘00’ - earfcn-Indicator is set to FALSE and earfcn-LSB is set to ‘00’ Otherwise, spare bits are already zero and earfcn-LSB is set to ‘00’ has same meaning for all UEs (new and legacy) in which case new indicator is not needed. |
| Nokia | No | Presence Indicator not needed |
| Huawei, HiSilicon | Yes | we think there are three cases and that the UE is not able to distinguish from the signalling 1: The legacy 100 kHz channel raster is used and the two LSB are ‘00’ 2: The legacy 100 kHz channel raster is used and the two LSB are not signaled (legacy band/frequency) 3: the new 200 kHz channel raster is used If there is no TN-NTN overlapping band, then the UE can determine the band from the frequency and then the type of raster from RAN4 specification. However, this is not possible for an overlapping frequency. In addition, signalling of the signalling of the two LSB is an optimisation’ (the UE can perform blind decoding otherwise) so we should not mandate it. For the field description (regardless of the indicator), a reference to RAN4 spec is needed although it is unlikely to be specified in this release. |
| Ericsson | Yes | Presence indicator is needed |
| OPPO | Depends on RAN4’s feedback | Whether the presence indication is needed depending on whether RAN4 identifies any band for which 100 kHz and 200 kHz channel rasters can both be used. If this mixed scenario can be excluded by RAN4, then the presence indication is not needed. |
| ZTE | Depends on | Per our understanding, the main issue of only introducing two bits earfcn-LSB may be that, upon reception of MIB, the IoT NTN UE cannot differentiate whether this is a TN or NTN network. Even this two bits earfcn-LSB may be set to ‘00’, UE may always use this two bits earfcn-LSB to set the EARFCN. This may result error EARFCN for TN network. So an additional indicator in MIB can help UE to know whether this is a NTN network and whether the two bits earfcn-LSB should be applied. But as the bit in MIB is very precious, another way may be that, UE might use SIB1 to assist network differentiation and this can be left to UE implementation. |
| Xiaomi | Yes |  |
| InterDigital | Yes | Agree with HW, the presence bit is needed to distinguish between the different cases whereby the 2 bits are 0. |

Summary

TBC

## t-Service applicability to higher priority frequencies

From [3]:

In the CR the following is captured regarding t-service for LTE-M:

Editor’s Note: FFS whether t-Service applies to higher priority frequencies.

It was brought up during online discussions in RAN2#116-e, but it was not addressed by any contribution in RAN2#116bis-e. In the e-mail discussion it was mentioned that it can be resolved the way that it was resolved for NR NTN:

- For quasi-earth fixed cell, same as legacy, UE shall perform neighbour cell measurements of “higher priority NR inter-frequency or inter-RAT frequencies” regardless of the remaining serving time

Thus in the CR we resolved it to where the eMTC UE shall perform neighbour cell measurement of higher priority inter-frequency or inter-RAT frequencies regardless of the remaining serving time, where the full text on t-Service reads as follows:

If t-Service is present in SystemInformationBlockTypeXX of the serving cell, UE should start to perform intra-frequency or inter-frequency measurements, where the UE does not limit the needed measurements, before the time t-Service. UE shall perform measurements of higher priority inter-frequency or inter-RAT frequency regardless of the remaining service time of the serving cell.

Thus we propose that the above is agreed:

Proposal: UE shall perform neighbour cell measurement of higher priority inter-frequency or inter-RAT frequencies regardless of the remaining serving time.

Companies are invited to provide their views on the above proposal and the implemented text in [5].

Discussion point 6: Do you agree with the above proposal

| Company | Agree: Yes/No | Comments |
| --- | --- | --- |
| MediaTek | No | If there is no time to measure, the measurement will be un-necessary |
| Apple | Yes |  |
| Lenovo | Yes |  |
| Spreadtrum | Yes |  |
| Qualcomm | Yes |  |
| Nokia | Yes | There should not be any impact to ongoing measurements related to higher priority inter-frequency measurements due to t-service. This t-service is meant to start the measurements if not started earlier only. |
| Huawei, HiSilicon | yes | This is the same agreement as NR |
| Ericsson | Yes | The higher prio inter-frequency and inter-RAT frequencies need to be measured as if t-service does not exist, otherwise idle mode mobility to TN etc may not work. We should not break this functionality through t-service. |
| OPPO | Yes |  |
| ZTE | Yes |  |
| Xiaomi | Yes |  |
| InterDigital | yes |  |
| NEC | Yes |  |
| CATT | Yes | Align with NR NTN. |

Summary

TBC

## UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area.

From [3]:

In RAN2#116bis-e it was mentioned that there should be some text that describes the agreement:

- UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area.

Two examples where there may need to be some changes were mentioned:

----------------- 36.304 -----------------

## 3.1 Definitions

Location Registration (LR): UE registers its presence in a registration area, for instance regularly or when entering a new tracking area

4.2 Functional division between AS and NAS in Idle mode

| Idle Mode Process | UE Non-Access Stratum | UE Access Stratum |
| --- | --- | --- |
| ... | ... | .... |
| Location registration | Register the UE as active after power on. Register the UE's presence in a registration area, for instance regularly or when entering a new tracking area. Maintain lists of forbidden registration areas. Deregister UE when shutting down. Control and restrict location registration for a UE in eCall only mode. | Report registration area information to NAS. |

----------------- 36.304 -----------------

However, in both cases the text is more descriptive as it is described in section 3 and 4, and the specific case where it is stated “when entering a new tracking area” it is only an example of the case of registering the presence in a registration (of which there are many other cases as in 23.122). Rapporteur’s understanding is that the main specification on how location registration shall be performed should be in related NAS specification. However, we think it could be informative that in the section on Tracking Area (5.4) that the following can be added:

If multiple tracking areas per PLMN are broadcast, the UE does not perform location registration if the UE is registered to at least one of the tracking areas.

Companies are invited to provide their views on the above proposal and the implemented text in [5].

Discussion point 7: Do you agree with the above proposal

| Company | Agree: Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes |  |
| Apple | Yes |  |
| Lenovo | Yes |  |
| Spreadtrum | Yes |  |
| Transsion Holdings | Yes |  |
| Qualcomm | No | We see this is not necessary. Whether or not UE performs registration update is specified in NAS spec. |
| Nokia | Yes |  |
| Huawei, HiSilicon | No | This is NAS behaviour and we do not think this should be captured in TS 36.304. |
| Ericsson | Yes | Stage-2-like NAS behaviour is many times captured in idle mode, but if there are strong concerns we can skip it. |
| OPPO | No | NAS behaviour is not needed to be captured here. |
| ZTE | Yes |  |
| Xiaomi | Yes |  |
| InterDigital | No | Agree with QC and HW again. |
| NEC | No | section 5.4. does not look like the right place to capture this, and the highlighted sentence in section 4.2 also does not bring anything wrong due to the wording “for instance “ |
| CATT | No | Agree with Qualcomm and Huawei |

Summary

TBC

## Signalling range of positionX, positionY, positionZ

In the RRC CR [4], there is the following Editor’s Note:

Editor’s Note: The signalling range of positionX, positionY, positionZ is larger than the range indicated by RAN1. Need to check the exact signalling.

Proposal: Unless RAN1 updates the RRC parameters spreadsheet otherwise, align the value range with the number of bits. i.e.:

PositionStateVector-r17 ::= INTEGER (-33554432..33554431)

positionX, positionY, positionZ X, Y, Z coordinate of satellite position state vector in ECEF. Unit in meter. Value range -42200000…42200000 -43620761 .., -43620762 by step of 1.3. Actual value = IE value * 1.3.

Companies are invited to provide their views on the above proposal.

Discussion point 8: Do you agree with the above proposal

| Company | Agree: Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes |  |
| Apple | Yes |  |
| Lenovo | Yes |  |
| Spreadtrum | Yes |  |
| Qualcomm | Yes | As long as step size and range specified by RAN1 are covered, this should be fine. |
| Nokia | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Ericsson | No | No strong opinion, but from what we understand having a value range slightly large is not really a problem. |
| OPPO | Yes with comment | -33554432*1.3=-43620761.6 33554431*1.3=43620760.3 So the value range in meter should be rounded to (-43620762, 43620760)? |
| ZTE | Yes |  |
| Xiaomi | Yes |  |
| InterDigital | Yes |  |
| NEC | Yes |  |
| CATT | Yes |  |

Summary

TBC

## Signalling range and step size of velocityVX, velocityVY, velocityVZ

In the RRC CR [4], there is the following Editor’s Note:

Editor’s Note: The signalling range of velocityVX, velocityVY, velocityVZ is shorter than the range indicated by RAN1. Check with RAN1 whether to increase the range or adjust the step size.

Proposal: Unless RAN1 updates the RRC parameters spreadsheet otherwise, align the value range with the number of bits, i.e.:

VelocityStateVector-r17 ::= INTEGER (-131072..131071)

velocityVX, velocityVY, velocityVZ X, Y, Z coordinate of satellite velocity state vector in ECEF. Unit in meter/second. Value range -8000…8000 -7864 … 7863 by step of 0.06. Actual value = IE value * 0.06.

Companies are invited to provide their views on the above proposal.

Discussion point 9: Do you agree with the above proposal

| Company | Agree: Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes |  |
| Apple | Yes |  |
| Lenovo | Yes |  |
| Spreadtrum | Yes |  |
| Qualcomm | No | We prefer to be consistent with the RAN1 range. VelocityStateVector-r17 ::= INTEGER (-133334..133334) The size will be increased just by 3 bits from 54 to 57 bits. |
| Nokia | Yes but | The range in meter should be (-7864.32 .. 7864.26) m/s which can be rounded to (-7864 to 7864) ? |
| Huawei, HiSilicon | Yes |  |
| Ericsson | No | Here it becomes a bit more sensitive and I think we need some satellite operator input as the values being smaller than intended may have the consequence of certain scenarios not being supported. The velocity is in ECEF and for very low earth orbits at 300 km altitude, this might not be enough. |
| OPPO | No | The value ranges of velocityVX, velocityVY, velocityVZ specified by RAN1 (i.e., -8000…8000) should be covered. So the size of VelocityStateVector should be increased by 1 bit, e.g. from 18 to 19 bits. |
| ZTE | Yes |  |
| Xiaomi | Yes |  |
| InterDigital | Yes |  |
| NEC | Yes |  |
| CATT | No | Agree with Ericsson. |

Summary

TBC

## Signalling of cellBarred-NTN in NB-IoT

Introducing a new bit cellBarred-NTN was agreed in RAN2#117 with a FFS about the location of the bit, SIB1 or MIB, in NB-IoT:

- A new bit, e.g. cellBarred-NTN, is introduced in SIB1 to bar NTN UEs from accessing a NTN cell. FFS whether to consider MIB instead of SIB1 for NB-IoT. NTN UE ignores the legacy bit.

RRC rapporteur understands that if the cellBarred-NTN is located in the MIB then an additional bit intraFreqReselection-NTN should also be added to the MIB:

Companies are invited to provide their views on the location of cellBarred-NTN in NB-IOT.

Discussion point 10: location of cellBarred-NTN in NB-IOT

| Company | MIB/ SIB1 | Comments |
| --- | --- | --- |
| Apple | SIB1 |  |
| Spreadtrum | SIB1 | There should be careful consideration about the use of the remaining bits in MIB. |
| Transsion Holdings | SIB1 |  |
| Qualcomm | MIB | We agree with rapporteur, 2 bits will be needed. But there are 9 spares bits available. |
| Nokia | SIB1 |  |
| Huawei, HiSilicon | SIB1 | we think that the cell being barred is a rare case so the power saving of moving the bit to the MIB is small. In general, we prefer to keep the bits in the MIB for things that are really needed or really beneficial |
| Ericsson | SIB1 | We understand the potential enhancement, but we would prefer to potentially keep the spare bits for more crucial signalling. |
| OPPO | SIB1 | It is not essential to use the spare bit in MIB for the new bar bit. |
| ZTE | SIB1 |  |
| Xiaomi | SIB1 |  |
| InterDigital | SIB1 | Legacy cell barred is in SIB1, we don’t see a reason specifically to optimise for the NTN case |
| NEC | SIB1 |  |
| CATT | SIB1 | But we suggest this filed is optional, and we need to discuss the condition of the present or absent of this field. |

Summary

TBC

# Conclusion

## First round: Proposals

To be completed at the end of the first round.

## Second round: Proposals for agreements

RAN2 chair has indicated that the aim was to agree offline and identify Come-Backs.

The second round will be performed via interactive mode to reach consensus on the proposals identified in section 5.1

To be completed at the end of the second round

# Reference

- R2-2203221 Report of [Pre117-e][012][IOT-NTN] Control Plane Open Issues, Huawei, RAN2#117-e, February 2022

- R2-2203220 OI 2.10:  Signalling of part-of-ARFCN indication in MIB in NB-IOT, Huawei, HiSilicon, RAN2#117-e, February 2022

- R2-2203457 IoT NTN idle mode open issues resolutions, Ericsson, RAN2#117-e, February 2022

- R2-2203129 Support of Non-Terrestrial Network in NB-IoT and eMTC, Huawei, RAN2#117-e, February 2022

- R2-2203456 IoT NTN idle mode CR, RAN2#117-e, February 2022