3GPP TSG-RAN WG2 #117                                                                                          R2-220xxxx

eMeeting, 21st February - 3rd March, 2022

Agenda Item:	    8.22.1.3

Source: 	    MediaTek Inc.

Title:		    Report of 	[AT117-e][065][MGE] RRC (MediaTek)

Document for:	    Discussion and decision

# 1 Introduction

This is report for the following AT117-e mail discussion.

[AT117-e][065][MGE] RRC (MediaTek)

Scope: Treat R2-2202877. Determine agreeable parts, points for discussion, open issues if needed. Converge offline if possible. Can also open for comments on R2-2202868.

Intended outcome: Report

Deadline: In time for on-line CB W2 Tuesday

Deadline – 02/28 UTC 23:59 (for open issue C1-4, C1-5, C1-6), CR discussion will continue during and after the meeting

The agreement in first week MGE section is copied below for reference.

- RAN2 confirms that reporting of NCSG for E-UTRA target bands is supported. RAN2 assumes that support for EUTRA target bands can be a separate UE cap

- RAN2 confirms that NCSG could be configured as per FR gap.

- Reuse the legacy GapConfig with some extension for NCSG gap configuration.

- For additional gap configuration in concurrent gap, use ToAddModList and ToReleaseList structure for each gap type to add or release the additional gaps, and gap sharing configuration to be consistent.

- For concurrent gap, RAN2 confirms that there is no need to support coarse granularity association (i.e. per use case such as CSI-RS, SSB measurement) since the agreed fine granularity (per frequency layer) could cover this case.

- FFS the maximum number of measurement gap ID. This could be discussed in gap coordination section.

- Baseline assumption When multiple MOs (with the same SSB frequency) are configured, the network associates the same MG for the SSB measurement in each MO. Details sorted out in CR disc

- RAN2 introduces support of NW-Controlled activation/deactivation pre-configured gap

# 2 Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| MediaTek (Rapp) | Felix Tsai | chun-fan.tsai@mediatek.com |

# 3 Discussion

## 3.1 C1-4 - Simultaneously support of legacy gap and concurrent gap

| OI Index | Open issue | Rapporteur comment |
| --- | --- | --- |
| C1-4 | Simultaneously support of legacy gap and concurrent gap | RAN4 LS R4-2202604: - RAN4 response: From RAN4 requirement perspective, RAN4 would like to ensure that the association of frequency layers or dedicated use cases to measurement gaps shall be clearly understood by both UE and Network for all configured measurements. How the association is up to RAN2. |

For C1-4, it seems that the only RAN4 guideline (green highlight above) is that the association between MG and measured frequency should be clear for both NW and UE. It is rapporteur’s understanding that current ASN.1 definition in the running CR (R2-2201903) already ensure the association no matter the gap is configured by legacy field (i.e. gapUE) or new field (e.g. gapUEToAddModList). So, it is suggested that no need to further discuss C1-4.

Question 1: Do companies agree that MGE open issue C1-4 it is already addressed in the baseline MGE running CR? Is it okay that R2 continue to discuss the CR but no need to have specific agreement for C1-4?

| Company | Agreed or not | Comments |
| --- | --- | --- |
| MediaTek | Yes |  |

Summary: TBD

Proposal 1:

## 3.2 C1-5 - Simultaneously support of per-UE gap and per-FR gap

| OI Index | Open issue | Rapporteur comment |
| --- | --- | --- |
| C1-5 | Simultaneously support of per-UE gap and per-FR gap | RAN4 LS R4-2202604: - Up to 2 gaps can be configured to UE which does not support per-FR gap. - Up to 3 gaps cross all FRs can be configured to UE which supports per-FR gap in SA case. FFS for MR-DC case if it is supported. - In Rel-17, UE can be configured with per-UE gap and per-FR gap simultaneously when - UE is capable of per-FR gap and concurrent gaps, and - Per-UE gap is associated with PRS measurements |

For C1-5, there seems some copy-paste error in the open issue document R2-2202054. The correct reference text from R4 LS should be the green highlighted as above.

According to RAN4 response, simultaneous configuration of per-UE and per-FR gap is allowed while the per UE gap is associated with PRS measurement. This could be found in index 3 to 5 in below table (from RAN4 LS R4-2202604).

| Combinations of different gap types for per-FR gap capable UE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Index | # of simultaneous MG |  |  | RAN4 conclusion |
|  | Per-FR1 | Per-FR2 | Per-UE |  |
| 0 | 2 | 1 | 0 | Supported |
| 1 | 1 | 2 | 0 | Supported |
| 2 | 0 | 0 | 2 | Supported |
| 3 | 1 | 0 | 1 | Supported when per-UE gap is associated to PRS measurement |
| 4 | 0 | 1 | 1 |  |
| 5 | 1 | 1 | 1 |  |
| 6 | 0 | 0 | 1 | Supported |
| 7 | 1 | 1 | 0 | Supported |
| 8 | 1 | 0 | 0 | Supported |
| 9 | 0 | 1 | 0 | Supported |
| 10 | 2 | 0 | 0 | Supported |
| 11 | 0 | 2 | 0 | Supported |

The current ASN.1 define in the baseline running CR R2-2201903 already supported all combination in above table. The rapporteur suggest that we only have to capture the configuration limitation in field description.

Question 2: For MGE open issue C1-5, do companies agree to clarify in the MGE RRC CR that simultaneous configuration of per-UE and per-FR gap is allowed while PRS measurement is associated with the per UE gap.

| Company | Agreed or not | Comments |
| --- | --- | --- |
| MediaTek | Yes | The wording and other details could be further discussed in the CR discussion. |

Summary:

Proposal 2:

## 3.3 C1-6 - Support of gap sharing for concurrent gap

| OI Index | Open issue | Rapporteur comment |
| --- | --- | --- |
| C1-6 | Support of gap sharing for concurrent gap | RAN4 LS R4-2202604: - Each gap is configured with separate MeasGapSharingConfig which has the same configurable range of parameters. |

For C1-6, the replied LS from RAN4 is quite clear. For concurrent gaps, each gap could be associated with one gap sharing configuration (as in Legacy). And during the online discussion, it is also agreed that gap sharing configuration will be consistent with the ASN.1 structure (as below).

- For additional gap configuration in concurrent gap, use ToAddModList and ToReleaseList structure for each gap type to add or release the additional gaps, and gap sharing configuration to be consistent.

Therefore, it seems quite straightforward that we should introduce the gap sharing configuration and details could be discussed in the CR.

Question 3: For MGE open issue C1-6, do companies agree to add the gap sharing configuration for each additional current gap in the MGE RRC CR?

| Company | Agreed or not | Comments |
| --- | --- | --- |
| MediaTek | Yes | The wording and other details could be further discussed in the CR discussion. |

Summary:

Proposal 3:

## 3.4 RRC CR

An updated running 38.331 CR is provided in the draft folder.

Main change compared to R2-2202868 is summarized as below.

<1> Remove the following FFS as it is concluded

Editor Note: It is FFS whether to support the reporting of NCSG for E-UTRA target bands

Editor Note: It is FFS whether to support use case association that associated a gap to SSB measurement, CSI-RS measurement, or E-UTRAN measurement

Editor Note: It is FFS on how to configure the NCSG gap and whether the NCSG could be per FR gap

Editor Note: It is FFS whether to use ToAddMod and ToRelase to add the additional GapConfig for per UE, FR1 gap, and FR2 gap

<2> Add NCSG gap pattern configuration.

GapConfig ::=                  SEQUENCE {

gapOffset                      INTEGER (0..159),

mgl                            ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp                           ENUMERATED {ms20, ms40, ms80, ms160},

mgta                           ENUMERATED {ms0, ms0dot25, ms0dot5},

...,

[[

refServCellIndicator           ENUMERATED {pCell, pSCell, mcg-FR2}   OPTIONAL   -- Cond NEDCorNRDC

]],

[[

refFR2ServCellAsyncCA-r16      ServCellIndex                        OPTIONAL,   -- Cond AsyncCA

mgl-r16                        ENUMERATED {ms10, ms20}              OPTIONAL    -- Cond PRS

]],

[[

measGapId-r17                   MeasGapId-r17                    OPTIONAL,   -- Cond ConcurrentGap

preConfigInd-r17                ENUMERATED {true}                OPTIONAL,   -- Need R

nscgInd-r17                     ENUMERATED {true}                OPTIONAL,   -- Need R

mgta-r17                        ENUMERATED {ms0dot75}            OPTIONAL,   -- Cond NCSG

mgl-r17                         ENUMERATED {ms1, ms2}            OPTIONAL,   -- Cond NCSG

gapAssociation-r17              MeasGapAssociation-r17           OPTIONAL    -- Need R

gapSharing                      MeasGapSharingScheme             OPTIONAL    -- Need R

]]

}

MeasGapAssociation-r17 ::=          SEQUENCE {

prsMeas-r17                         ENUMERATED {true}            OPTIONAL    -- Need R

}

<3> Introduces support of NW-Controlled activation/deactivation pre-configured gap

BWP-DownlinkDedicated ::=           SEQUENCE {

pdcch-Config                        SetupRelease { PDCCH-Config }                  OPTIONAL,   -- Need M

pdsch-Config                        SetupRelease { PDSCH-Config }                  OPTIONAL,   -- Need M

sps-Config                          SetupRelease { SPS-Config }                    OPTIONAL,   -- Need M

radioLinkMonitoringConfig           SetupRelease { RadioLinkMonitoringConfig }     OPTIONAL,   -- Need M

...,

[[

sps-ConfigToAddModList-r16          SPS-ConfigToAddModList-r16                         OPTIONAL,   -- Need N

sps-ConfigToReleaseList-r16         SPS-ConfigToReleaseList-r16                        OPTIONAL,   -- Need N

sps-ConfigDeactivationStateList-r16 SPS-ConfigDeactivationStateList-r16                OPTIONAL,   -- Need R

beamFailureRecoverySCellConfig-r16  SetupRelease {BeamFailureRecoverySCellConfig-r16}  OPTIONAL,   -- Cond SCellOnly

sl-PDCCH-Config-r16                 SetupRelease { PDCCH-Config }                      OPTIONAL,   -- Need M

sl-V2X-PDCCH-Config-r16             SetupRelease { PDCCH-Config }                      OPTIONAL    -- Need M

]],

[[

deactivatedMeasGapList-r17          SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF MeasGapId-r17  OPTIONAL    -- Cond PreMG

]]

}

<4> Use ToAddMod list to configure the additional gap and gap sharing

MeasGapConfig ::=   SEQUENCE {

gapFR2              SetupRelease { GapConfig }         OPTIONAL,   -- Need M

...,

[[

gapFR1              SetupRelease { GapConfig }         OPTIONAL,   -- Need M

gapUE               SetupRelease { GapConfig }         OPTIONAL    -- Need M

]],

[[

gapUEToAddModList-r17     SEQUENCE (SIZE (1.. maxNrofGapId-1-r17)) OF GapConfig		OPTIONAL,   -- Need N

gapUEToReleaseList-r17    SEQUENCE (SIZE (1.. maxNrofGapId-1-r1)) OF MeasGapId-r17	OPTIONAL,   -- Need N

gapFR1ToAddModList-r17    SEQUENCE (SIZE (1.. maxNrofGapId-1-r1)) OF GapConfig		OPTIONAL,   -- Need N

gapFR1ToReleaseList-r17   SEQUENCE (SIZE (1.. maxNrofGapId-1-r1)) OF MeasGapId-r17	OPTIONAL,   -- Need N

gapFR2ToAddModList-r17    SEQUENCE (SIZE (1.. maxNrofGapId-1-r1)) OF GapConfig		OPTIONAL,   -- Need N

gapFR2ToReleaseList-r17   SEQUENCE (SIZE (1.. maxNrofGapId-1-r1)) OF MeasGapId-r17	OPTIONAL    -- Need N

]]

}

<5> Add gap sharing configuration for each gap pattern

GapConfig ::=                  SEQUENCE {

gapOffset                      INTEGER (0..159),

mgl                            ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp                           ENUMERATED {ms20, ms40, ms80, ms160},

mgta                           ENUMERATED {ms0, ms0dot25, ms0dot5},

...,

[[

refServCellIndicator           ENUMERATED {pCell, pSCell, mcg-FR2}   OPTIONAL   -- Cond NEDCorNRDC

]],

[[

refFR2ServCellAsyncCA-r16      ServCellIndex                        OPTIONAL,   -- Cond AsyncCA

mgl-r16                        ENUMERATED {ms10, ms20}              OPTIONAL    -- Cond PRS

]],

[[

measGapId-r17                   MeasGapId-r17                    OPTIONAL,   -- Cond ConcurrentGap

preConfigInd-r17                ENUMERATED {true}                OPTIONAL,   -- Need R

nscgInd-r17                     ENUMERATED {true}                OPTIONAL,   -- Need R

mgta-r17                        ENUMERATED {ms0dot75}            OPTIONAL,   -- Cond NCSG

mgl-r17                         ENUMERATED {ms1, ms2}            OPTIONAL,   -- Cond NCSG

gapAssociation-r17              MeasGapAssociation-r17           OPTIONAL    -- Need R

gapSharing                      MeasGapSharingScheme             OPTIONAL    -- Need R

]]

}

For gap sharing, rapporteur think the association of gap sharing and multiple gap configuration is easier if we just add the gap sharing configuration within GapConfig. This is different from legacy way that put gap sharing outside gap configuration. So, companies are invited to provide view on this.

Question 4: Companies are invited to provide views on how to add gap sharing configuration for concurrent gap. Do you agree to add gap sharing configuration (MeasGapSharingScheme) in GapConfig?

| Company | Comments |
| --- | --- |

Finally, companies could provide bubble comment to the updated CR. For comment that request more discussion, it can be included in the following table.

Question 5: Companies are invited to provide comments/suggestions on the running CR.

| Company | Comments |
| --- | --- |

NOTE: The procedure text for concurrent gap (and gap sharing) will be updated later.

# 4 Conclusions

Base on the discussion in section 2, we propose the following:

Proposal 1:

# 5 References

[1] R2-2202868, “Introduction of RRC signaling for measurement gap enhancement”, MediaTek

[2] R2-2202877, “Rapporteur resolution for MGE open issues”, MediaTek

[3] R2-2202899, “Report of [Pre117-e][010][MGE] MGE Open Issues Input (MediaTek)”, MediaTek