3GPP TSG RAN WG2#117-e	draft R2-2203663

e-Meeting, 21st February - 3rd March, 2022	rev of R2-2203369

________________________________________________________________________

Title:	draft Reply LS on LTE User Plane Integrity Protection

Response to:	S3-214462/R2-2200153

Release:	17

Work Item:	UPIP_SEC_LTE

Source:	RAN2

To:	SA3

cc:	RAN3, SA2

Contact person:	Chris Pudney (Vodafone)

chris dot pudney at @ vodafone dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

RAN2 thanks SA3 for their LS in S3-214462/R3-220128. RAN2 is taking this information into account (along with the CRs agreed in SA2, CT4 and CT1), but only to the extent that is described by the WID agreed by RAN plenary in RP-213669 which, in particular, restricts the scope to EN-DC capable UEs.

In response to SA3’s request “…to inform SA3 on their final decision with respect to which algorithm code points are to be used”, RAN2 would like to provide the following information:

UPIP for the EPC connected architectures uses NR PDCP and is configured in following way:

- (as is done for legacy LTE UE) an LTE algorithm code point is configured in field integrityProtectionAlgorithm in IE SecurityAlgorithmConfig  in the TS 36.331 SecurityModeCommand message, and this is used to derive KUPint (and also to derive KUPEnc, as for legacy LTE UE).

- The NR algorithm code point (corresponding to the LTE algorithm code point used in the SecurityModeCommand) indicated by the integrityProtAlgorithm included in the securityConfig in the TS 38.331 RadioBearerConfig is used to configure the UP IP algorithm applied by NR PDCP to perform integrity protection.

- The integrityProtection indicated in pdcp-Config in the DRB-ToAddMod(list) in the TS 38.331 RadioBearerConfig is used to activate/deactivate the UP IP using the configured algorithm. UP IP can be activated/disabled only by adding or releasing a DRB.

# 2	Actions

To SA3

ACTION: 	RAN2 politely requests SA3 to take the above information into account.

# 3	Dates of next TSG RAN 2 meetings

Please see the latest information on the RAN 2 calendar.