3GPP TSG-RAN WG2 #117-e	R2-22xxxxx

Online Meeting, Feb 21st – March 3rd, 2022

Agenda Item:	6.3.2

Source:	CATT

Title:	Report of [AT117-e][625][POS] Agenda item 6.3.2 (CATT)

Document for:	Discussion, Decision

Introduction

The below papers have been submitted in the LPP AI 6.3.2 which requires input from companies to identify the support for the corrections.

| R2-2202407 | Corrections on the description of maxNrofSRS-PosResources-1-r16 | CATT |
| --- | --- | --- |
| R2-2202596 | Correction on srs-PosResourceIdList in RRC | Huawei, HiSilicon |

 [AT117-e][625][POS] Agenda item 6.3.2 (CATT)

Scope: Treat documents R2-2202407 and R2-2202596 and conclude on the CRs.

Intended outcome: Agreed CRs (without CB)

Deadline:  Wednesday 2022-03-02 1000 UTC

Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |
| Qualcomm | sfischer@qti.qualcomm.com |
| CATT | lijianxiang@catt.cn |
| Huawei, HiSilicon | yinghaoguo@huawei.com |
| ZTE | pan.yu24@zte.com.cn |
| Xiaomi | lixiaolong1@xiaomi.com |

Discussion

R2-2202407	Corrections on the description of maxNrofSRS-PosResources-1-r16

Reason for change: The description of maxNrofSRS-Resources-1 has already been fixed as “-- Maximum number of SRS resources minus 1.” by deleting “in an SRS resource set”. The maxNrofSRS-PosResources-1-r16 still keeps “in an SRS Positioning resource set.” However the Maximum number of (SRS Positioning) resources is not in a resource set.

So the CR modifies the description of maxNrofSRS-PosResources-1-r16 as below:

maxNrofSRS-PosResources-1-r16 INTEGER ::= 63 -- Maximum number of SRS Positioning resources minus 1.

Question 1: Do Companies Agree with the CR?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes |  |
| CATT | Yes |  |
| Huawei, HiSIlicon | Yes |  |
| Lenovo | Yes but | It is a minor issue so it can be merged with the CR R2-2202596. |
| ZTE | Yes |  |
| Xiaomi | Yes |  |

R2-2202596	Correction on srs-PosResourceIdList in RRC

The CR changes the field description of srs-ResourceIdList and srs-PosResourceIdList, i.e., to remove the restriction for positioning SRS resource set on the number of entries depending on the usage, since there is no such field as usage for positioning SRS resource set.

A part of CR is shown below.

srs-ResourceIdList, srs-PosResourceIdList The IDs of the SRS-Resources/SRS-PosResource used in this SRS-ResourceSet/SRS-PosResourceSet. If this SRS-ResourceSet is configured with usage set to codebook, the srs-ResourceIdList contains at most 2 entries. If this SRS-ResourceSet is configured with usage set to nonCodebook, the srs-ResourceIdList contains at most 4 entries.

Question 2: Do Companies Agree with the CR?

| Company | Change is fine Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes |  |
| CATT | Yes |  |
| Huawei, HiSilicon | Yes |  |
| ZTE | Yes |  |
| Xiaomi | Yes |  |

Conclusion

Based on the discussion in section 2 we propose the following:

TBD

References

[1] R2-2202407 Corrections on the description of maxNrofSRS-PosResources-1-r16	CATT

[2] R2-2202596 Correction on srs-PosResourceIdList in RRC	Huawei, HiSilicon