3GPP TSG-RAN2 Meeting #117-e	R2-220xxxx

Online, 21 February – 03 March 2022

Agenda Item:	9.2.3

Source:		Huawei, HiSilicon

Title:		[Pre117-e][012][IOT-NTN] Control Plane Open Issues (Huawei)

Document for:	Discussion and decision

# Introduction

This is the report of the email discussion on IOT NTN control plane open issues:

- [Pre117-e][012][IOT-NTN] Control Plane Open Issues (Huawei)

Scope: Collect companies views on Control Plane Open Issues

Deadline for comments: Monday 14th February, 23:53 UTC

The document collects companies views on the control plane open issues identified for pre117-e discussion.

# Contact Information

| Company | Name | Email |
| --- | --- | --- |
| Huawei, HiSilicon | Odile Rollinger | odile.rollinger@huawei.com |
| Interdigital | Brian Martin | brian.martin@interdigital.com |
| MediaTek | Abhishek Roy | Abhishek.Roy@mediatek.com |
| Lenovo, Motorola Mobility | Min Xu | xumin13@lenovo.com |
| ZTE | Ting Lu | lu.ting@zte.com.cn |
| NEC | Yuhua chen | Yuhua.chen@emea.nec.com |
| Spreadtrum | Xu Liu | xu.liu1@unisoc.com |
| Intel | Tangxun | xun.tang@intel.com |
| Apple | Pavan Nuggehalli | pnuggehalli@apple.com |
| Transsion Holdings | Wen Wu | wen.wu5@transsion.com |
| Xiaomi | Xiaolong Li | lixiaolong1@xiaomi.com |
| CATT | Xiangdong Zhang | zhangxiangdong@catt.cn |

# Discussion

## OI 2.1: Define a new barring bit for NTN UEs barring

RAN2#116bis-e has agreed:

- We will have the barring bit to prevent terrestrial UEs to use NTN. FFS if we define a new barring bit for NTN UEs barring.

At least, the following points need to be addressed:

- option 1: introduce a new barring bit: which name, how does it work, anything else needed

- option 2: reuse an existing bit: which one, how does it work, anything else needed

Discussion point 1: please provide your views on the above issue

| Company | Option 1/ Option 2 | Comments |
| --- | --- | --- |
| InterDigital | Option1 | Existing barring bits have their own purpose. The usual way to do this is that the UE ignores the existing barring bit, and checks the new barring bit instead. This allows NW to bar legacy UEs and control the new UEs independently using the new barring bit. As for naming, no strong view but something like “cellBarred-NTN-r17” would be aligned with previous new barring bits. |
| MediaTek | Option 1 | A new barring bit will be better and easier to manage. |
| Lenovo, Motorola Mobility | Option 1 |  |
| ZTE | Option 1 | Similar view as InterDigital. |
| NEC | Option1 | Agree with InterDigital Moreover, we could explicitly specify that NTN network shall set the legacy barring bits to barred |
| Qualcomm | Option 1 | It is simpler and cleaner. |
| Nokia | Option 2 | To restrict access from non-NTN UE existing bit can be used. Because legacy UE will only understand existing access barring bits. For access barring NTN UE use of other existing bits can be considered. |
| Spreadtrum | Option 1 | In order to make the barring mechanism work properly, a new barring bit should be introduced. The legacy barred IE can be used for restricting the legacy UE, and the new barring bit works for the NTN capable UE. |
| Intel | option 2 | agree with Nokia that legacy TN UE can only understand existing barring bits. NTN UE can ignore legacy barring bits, and new barring bits can be introduced to prevent NTN UEs. |
| Huawei, HiSilicon | Option 1 | In the meeting, the proposal for option 2 was to use cellReservedForOperatorUse to bar NTN capable UE from accessing a NTN cell. We think this is not suitable as the field is signalled by PLMN. For option 1, the question is whether the intraFreqReselection signalled in a NTN cell applies to both legacy UEs and NTN capable UEs. In our understanding, the main purpose of intraFreqReselection is to deal with congestion on a frequency, thus we think it should be common to both.  add a new barring bit, e.g. cellBarred-NTN-r17, in an extension of cellAccessRelatedInfo to bar NTN UEs to access a NTN cell. |
| Apple | Option 1 |  |
| Transsion Holdings | Option1 | The existing bits may also be used in NTN for their original purpose, if we can make sure the existing bits will not be used in NTN, then we can redefine these bits. The new barring bit name can be cellBarredNTN-r17 broadcast in SIB1, and only works for the NTN UEs. |
| Xiaomi | Option 1 |  |
| CATT |  | If NTN-specific frequency is used, or at the NTN-specific frequency, the current barring bit can be used for NTN UE access control. Otherwise, the current barring bit can be used to prevent TN UEs to use NTN, and then a new barring bit for NTN UEs barring is needed. |

## OI 2.6 If some mechanism is needed to trigger the UE to reacquire the NTN specific SIB in RRC_IDLE

RAN2#116bis-e has agreed:

- Update to serving cell ephemeris information does not affect the system information value tag and does not trigger System information modification procedure. How to trigger re-read of this information is FFS. FFS if the UE shall reacquire the new SIB when SI update is triggered.

- Updates to serving cell ephemeris information are not bound to the BCCH modification period.

- When SI used for UL synch (pre-compensation) is no longer valid, the UE autonomously tunes away and re-aquires the required SI, and then comes back. FFS whether anything additional is needed.

- UE acquires the NTN specific SIB before accessing the cell.

However, handling of the NTN SIB in RRC_IDLE has not been discussed yet

At least, the following points need to be addressed:

- is SIBXX an essential SIB?

- is the UE required to acquire SIBXX in RRC_IDLE? what are the triggers?

- any other aspects

Discussion point 2: please provide your views on the above issue

| Company | Comments |
| --- | --- |
| InterDigital | - it is essential, without having a valid version UE cannot perform e.g. pre-compensation. - UE is required to have a valid version of the SIB before access, therefore acquiring before access this seems to be the only requirement. - How to ensure UE can re-acquire the SIB in time before responding to paging? Does UE therefore need to acquire the SIB in every (e)DRX cycle or can we rely on the validity timer (and UE only needs to acquire before access if the validity timer expires)? |
| MediaTek | - SIBXX is an essential SIB - UE required to acquire SIBXX in RRC_IDLE - The UE can rely on the validity timer and acquire the SIB when the validity timer expires. |
| Lenovo, Motorola Mobility | - SIBXX is essential, at least for TA pre-compensation. - UE needs to acquire SIBXX in IDLE, and at least the validity timer expiration can trigger re-acquiring. - An indication to trigger re-acquiring can be considered, which is controlled by network whenever re-acquiring is necessary. |
| ZTE | - We can agree SIBXX is essential as we think IoT NTN UE needs this information to identify whether a cell is a NTN cell and whether it can camp on this cell. But we assume UE in idle mode doesn’t need to always keep the latest version of SIBXX and therefore no need of keeping (re)acquiring SIBXX. - For TA pre-compensation, it’s enough that UE acquires the SIBXX before accessing the cell. Moreover, we think such (re)acquisition of SIBXX before RACH can be regardless of validity timer. That means, even the validity timer doesn’t expire when UE initiates RACH, UE still need to (re)acquire SIBXX. The reason is, even the validity timer doesn’t expire at the start of RACH, it’s still possible to expire during connection (as it’s unpredictable how long the connection will last). So in order try to avoid expiry of validity timer during connected mode, it’s better for UE to acquire SIBXX before each time RACH. We think this is also needed and enough for paging case. - For UE camping on an a quasi-earth fixed cell, another information in SIBXX, e.g., the timing information on when a serving cell is going to stop serving the area, would be useful for UE to perform neighbor cell measurement and cell reselection. But we assume this information would be stable and think it’s also enough that UE acquire this timing information only once, e.g., when UE firstly camps on the quasi-earth fixed cell. Please note this information is not applicable for moving cell case. |
| NEC | - SIBxx is essential for initial access - Since we agreed “UE acquires the NTN specific SIB before accessing the cell”, it seems we are prepared for extra delay and then UE is not required to acquire SIBxx in RRC_idle other before accessing the cell |
| Qualcomm | - Yes - Only before initiating RACH from IDLE mode, the UE has to acquire the SIBxx. But for other time in IDLE mode, the UE should not be required to acquire the SIBxx as orbital parameters can be used for a long time. But the UE may still follow the validity duration and acquire ephemeris. |
| Nokia | SIBXXX is not essential SIB for camping on the NTN cell. This SIB needs to be acquired prior to access. So UE can start accessing whenever needed ensuring that it read at least once per validity period. SIBXXX is needed prior to accessing the cell in RRC-IDLE state. If the UE is in connected mode and if this SIB needs to be re-acquired the UE should be able to acquire this SIB without leaving RRC-CONNECTED state. |
| Spreadtrum | - SIBxx is an essential SIB. - UE is required to acquire SIBxx before initiating RACH. We think only RACH can trigger acquiring SIBxx regardless whether the validity timer expires or not. |
| Intel | - SIBxx is an essential SIB - UE acquires the NTN specific SIB before accessing the cell |
| Huawei, HiSilicon | a) We have already agreed that the UE is required to acquire SIBXX before accessing the cell.  SIBXX is an essential SIB b) The parameters introduced by RAN1 (ephemeris, TA common parameters, UL synchronisation validity timers, K-mac, K-offset) are for the purpose of TA precompensation or timing adjustment. They are fast changing and needed only during access or during the connection. As RAN2 has not agreed to location based cell (re)selection, the UE does not need ephemeris parameters in RRC_IDLE. Thus the only parameter applicable to RRC_IDLE is t-Service introduced by RAN2 for quasi-fixed cell. This indicates an absolute time and is not supposed to change.  the UE is not required to acquire SIBXX in RRC_IDLE c)  t-service is moved out of SIBXX, e.g. to SIB3 |
| Apple | - SIBx is essential - Only needs to be acquired once prior to access and once after reselection (to obtain t-Service for example) - Final SIB design (including where to put what) can await NR NTN design |
| Transsion Holdings | - It is essential as the NTN specific SIB is used for UL pre-compensation, the UE is required to acquire the NTN specific SIB before trying to access to the NW. - No need to trigger the UE to reacquire the NTN SIB in idle or it is up to UE implementation. |
| Xiaomi | - SIBxx is not essential but required SIB. And need to clarify in 36.331 section 5.2.2.3 as below: an essential SIB for initial aceess. - 5.2.2.3 System information required by the UE The UE shall: 1> ensure having a valid version, as defined below, of (at least) the following system information, also referred to as the 'required' system information: 2> if in RRC_IDLE: 3> if the UE is a NB-IoT UE: 4> the MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB and SystemInformationBlockType1-NB as well as SystemInformationBlockType2-NB through SystemInformationBlockType5-NB, SystemInformationBlockType22-NB, SystemInformationBlockTypeXX-NB (if broadcast); 3> else: 4> the MasterInformationBlock and SystemInformationBlockType1 (or SystemInformationBlockType1-BR depending on whether the UE is a BL UE or the UE in CE) as well as SystemInformationBlockType2 through SystemInformationBlockType8 and SystemInformationBlockType24 (depending on support of the concerned RATs), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC), SystemInformationBlockType29 (only for BL UE or the UE in CE depending on support of resource reservation), SystemInformationBlockTypeXX (only for BL UE or the UE in CE if broadcast); 2> if in RRC_INACTIVE: 3> the MasterInformationBlock and SystemInformationBlockType1 as well as SystemInformationBlockType2 through SystemInformationBlockType8 (depending on support of the concerned RATs), SystemInformationBlockType24 (depending on support of the concerned RATs), SystemInformationBlockType25, SystemInformationBlockTypeXX (only for BL UE or the UE in CE if broadcast);. - Idle UE is required to read SIBxx before RRC establishement. Suggested changes are as follow: - 5.2.2.4 System information acquisition by the UE The UE shall: 1> apply the specified BCCH configuration defined in 9.1.1.1 or BR-BCCH configuration defined in 9.1.1.8; 1> if the procedure is triggered by a system information change notification: 2> if the UE uses an idle DRX cycle longer than the modification period: 3> start acquiring the required system information, as defined in 5.2.2.3, from the next eDRX acquisition period boundary; 2> else 3> start acquiring the required system information, as defined in 5.2.2.3, from the beginning of the modification period following the one in which the change notification was received; NOTE 1: The UE continues using the previously received system information until the new system information has been acquired. 1> if the UE is in RRC_IDLE and enters a cell for which the UE does not have stored a valid version of the system information required in RRC_IDLE, as defined in 5.2.2.3: 2> acquire, using the system information acquisition procedure as defined in 5.2.3, the system information required in RRC_IDLE, as defined in 5.2.2.3; 1> following successful handover completion to a PCell for which the UE does not have stored a valid version of the system information required in RRC_CONNECTED, as defined in 5.2.2.3: 2> acquire, using the system information acquisition procedure as defined in 5.2.3, the system information required in RRC_CONNECTED, as defined in 5.2.2.3; 2> upon acquiring the concerned system information: 3> discard the corresponding radio resource configuration information included in the radioResourceConfigCommon previously received in a dedicated message, if any; 1> following a request from CDMA2000 upper layers: 2> acquire SystemInformationBlockType8, as defined in 5.2.3; 1> neither initiate the RRC connection establishment/resume procedure nor initiate transmission of the RRCConnectionReestablishmentRequest message until the UE has a valid version of the MasterInformationBlock (MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) and SystemInformationBlockType1 (SystemInformationBlockType1-NB in NB-IoT) messages as well as SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT), SystemInformationBlockTypeXX(or SystemInformationBlockTypeXX-NB in NB-IOT, if broadcast) and for NB-IoT, SystemInformationBlockType22-NB;. Changes above can ensure that UE is required to have a valid version of SIBxx but only need to acquire it before RRC establishement. Changes beyond those would not be necessary. |
| CATT | a) SIBXX is an essential SIB. b) UE is required to acquire SIBXX in RRC_IDLE. UE should acquire SIBXX before RACH for TA pre-compensation. |

## OI 2.7 If anything additional is needed on expiry of the UL synchronisation timer

RAN2#116bis-e has agreed:

- When SI used for UL synch (pre-compensation) is no longer valid, the UE autonomously tunes away and re-acquires the required SI, and then comes back. FFS whether anything additional is needed.

In RAN2#116bis-e contributions, there were proposals to have a guard timer for the acquisition of the new SIB and trigger RLF at expiry.

At least, the following points need to be addressed:

- do we need a ‘guard’ mechanism in case the UE does not manage to reacquire the SIB quick enough

- any other aspects

Discussion point 3: please provide your views on the above issue

| Company | Comments |
| --- | --- |
| InterDigital | We don’t think further optimisation is needed at this stage |
| MediaTek | Further optimizations are not needed at this stage. |
| Lenovo, Motorola Mobility | Not needed in this release. |
| ZTE | What’s the specification impacts of such UE behaviour, e.g., “autonomously tunes away and re-acquires the required SI, and then comes back”, is not clear. We think maybe at least it needs to specify kind of UE process as flushing HARQ buffers, clearing configured uplink grants etc. on expiry of the UL synchronisation timer. We are not clear the benefit of a guard timer, so think no need of it. |
| NEC | We also prefer not to introduce “guard” mechanism |
| Qualcomm | (a). If the UE takes forever to acquire SIB when tuning away and fails to acquire the ephemeris, then this will be issue for radio resource reservation. Therefore, guard time mechanism should be defined for triggering RLF and releasing resources in time. |
| Nokia | We think neither the ‘guard’ mechanism nor the trigger RLF at expiry is needed. Instead, based on the RAN2 agreement above, UE will comeback after re-acquiring the required SI. We understand it means when the UE find its validity timer expired, it should be kept in RRC Connected state and come back to service after re-acquire the SI for UL sync recovery. (i.e. when the timer expired, UE should stop UL transmission since its UL sync lost. When the UE UL sync recovered after re-acquiring the new SI, the UE can come back to service and able to perform UL transmission). The mechanism is similar to the maintenance of uplink time alignment when UL synchronization status is “non-synchronised” where it is only the UL synchronization lost, but the DL synchronization is kept. Agree with ZTE the UE behaviour need to be further clarified (e.g. in stage 3). |
| Spreadtrum | It is enough for UE to manage UL sync. No further optimisation is needed. |
| Intel | No further optimisation is needed. |
| Huawei, HiSilicon | a) We support the proposal to have a guard timer for the case where the UE cannot acquire the SIB in a reasonable time. The UE starts the timer when retuning/reconfiguring lower layers to acquire SIBXX and stops it when retuning/reconfiguring lower layers for unicast operation If the timer expires, UE declares RLF and initiates RRC connection re-establishment. b) We think the handling of the validity timer is similar to PUR timing alignment timer, i.e. MAC does not initiate any UL transmission if the timer is not running. Nothing else is needed. Handling of the timer in MAC needs to be specified. |
| Apple | We think this is rare event, no need to super-optimize. No additional mechanism needs to be defined. |
| Transsion Holdings | We think need to set a timer for the UE to re-acquires the required SI, if time out, as the pre-compensation is no longer valid, the UL transmission(including RACH) can’t be success, so there is no need to trigger RLF, it can go to IDLE and punish for a while for power saving. |
| Xiaomi | 1. The timer should be maintained in RRC, instead of MAC (as implemented in current running CR)since the timer is related to SIB validity. Then, after the expiry of the timer, we think RRC need to indicate this to lower layers, so that MAC can suspend UL transmission, and perhaps other behaviours as mentioned by ZTE. 2. We think the behavior when UE is not be able to acquire the SIBxx should be specified, but no need to define a timer. According to current specification, there is no timer for MIB/SIB1 acquisition either, but only the behaviour is defined when UE cannot acquire the MIB/SIB1. We think in this case UE moves to idle mode similar to the behaviour when GNSS fix outdated. Thus, we suggest the following change: - 5.2.2.5 Essential system information missing The UE shall: 1> if in RRC_IDLE, RRC_INACTIVE or in RRC_CONNECTED while T311 is running: 2> if the UE is unable to acquire the MasterInformationBlock (MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT); or 2> if the UE is neither a BL UE nor in CE nor in NB-IoT and the UE is unable to acquire the SystemInformationBlockType1; or 2> if the BL UE or UE in CE is unable to acquire SystemInformationBlockType1-BR or SystemInformationBlockType1-BR is not scheduled; or 2> if the NB-IoT UE is unable to acquire the SystemInformationBlockType1-NB: 3> consider the cell as barred in accordance with TS 36.304 [4]; and 3> perform barring as if intraFreqReselection is set to allowed, and as if the csg-Indication is set to FALSE; 2> else: 3> if the UE is unable to acquire the SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT) and for NB-IoT, SystemInformationBlockType22-NB if scheduled; or 3> if SystemInformationBlockType25 is broadcast and if the UE is connected to 5GC and is unable to acquire the SystemInformationBlockType25: 4> treat the cell as barred in accordance with TS 36.304 [4]; 1> if in RRC_CONNECTED while ul-SyncValidityTimer is not running: 2> if the BL UE or UE in CE or NB-IOT UE is unable to acquire SystemInformationBlockTypeXX (or SystemInformationBlockTypeXX-NB in NB-IOT); 3> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';. |
| CATT | Not needed in this release. |

## OI 2.x: Provision of SIBxx in dedicated signalling at HO

Aspects related to handover have not been discussed yet in RAN2.

It has only been agreed ‘ta-Report-r17 is signaled in radioResourceConfigCommon’ so that it is included in mobilityControlInfo.

Other NTN specific parameters may need to be provided to the eMTC UE at handover.

At least, the following points need to be addressed:

- which NTN specific information is needed in mobilityControlInfo

- which NTN specific SIB, if any, should be provided in RRCReconfiguration for handover

- any other aspects

Discussion point 4: please provide your views on the above issue

| Company | Comments |
| --- | --- |
| InterDigital | At least ephemeris information used for pre-compensation would be needed |
| MediaTek | The ephemeris information, required for L1 pre-compensation, is definitely needed for eMTC. |
| Lenovo, Motorola Mobility | The ephemeris of the target satellite is needed. Besides, like the serving satellite, the epoch time and validity time associated to the ephemeris of the target satellite are also needed. |
| ZTE | We think entire SIBXX can be provided in RRCReconfiguration for handover, as besides the information mentioned in above companies’ comments, the common TA parameters, K_mac etc. are also needed. |
| NEC | We think all serving cell relevant information in SIBxx are relevant to initial access and should be included in handover message |
| Qualcomm | Agree with ZTE. |
| Nokia | We don’t see additional information more than what is needed for NR-NTN. We can wait for NR-NTN agreements related to a). As connected mode duration is very short for IoT-NTN UE, UE can acquire new system information using mechanisms defined currently and also the mechanisms defined for new SIB |
| Spreadtrum | The entire information in SIBxx should be provided in RRCReconfiguration for handover. |
| Intel | neighbour cell’s ephemeris and common TA, and it would be simple to include the entire SIBXX. |
| Huawei, HiSilicon | a) When accessing the new cell at handover, the UE needs all the RAN1 parameters related to UL synchronisation and Timing adjustment, this is typically provided in mobilityControlInfo b) Considering that apart from the RAN1 parameters related to UL synchronisation and Timing adjustment, there is only t-service in SIBXX, which is not applicable to connected mode, we are not sure that SIBxx should also be provided in the RRCReconfiguration message (i.e. in a similar way to SIB1 and SIB2-BR) as this would be duplication of the same information in mobilityControlInfo. |
| Apple | Providing entire SIBxx seems simpler even if there is some redundancy. |
| Transsion Holdings | - the satellite Ephemeris Parameters of the target satellite can be provided for handover - For quasi-earth fix cell, the start-time of target satellite’s coverage can also be provided for handover. |
| Xiaomi | The parameters for initial access should be provided to UE for handover. |
| CATT | Agree with ZTE. |

# Conclusion

# Reference

- R2-2202053 Summary of [Post116bis-e][089][IoT-NTN] Open Issues (Mediatek), RAN2#116bis-e, January 2022