3GPP TSG-RAN WG2 #117-e	R2-22xxxx

Electronic meeting, 2022-02-21 - 2022-03-03

Agenda Item:	9.1.4

Source:	Ericsson

Title:	Report on [Pre117-e][303][NBIOT/eMTC R17] Other open issues 	(Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This document is to lick off below email discussion.

- [Pre117-e][303][NBIOT/eMTC R17] (Ericsson)

Agreements in this area are listed below:

### NB-IoT 16-QAM for unicast in UL and DL

RAN2#113bis-e agreements: Working assumption: For the UE supporting 16-QAM, the L2 buffer size is 12000 bytes. Working assumption: Support of 16-QAM has separate UE capabilities for DL and UL RAN2#114-e agreements: None RAN2#115-e agreements: Confirm the working assumption: The support of 16-QAM uses separate UE capabilities for DL and UL. 16QAM is configured via dedicated signaling separately for UL and DL. A NPUSCH 16QAM activation indication is needed in PUR configuration. RAN2#116-e agreements: Confirm the working assumption of 12000 bytes for DL 16QAM for NB-IoT

### 14 HARQ processes in DL for HD-FDD Cat M1 UEs

RAN2#113bis-e agreements: 14 HARQ activation is configured by dedicated RRC signalling. Working assumption: No change to current L2 buffer size requirement RAN2#114-e agreements: None RAN2#115-e agreements: Confirm the working assumption: No change to current L2 buffer size requirement for HD-FDD Cat M1 UEs supporting 14 HARQ processes in DL.

Paper submitted in RAN2#116bis-e

R2-2200677	On thje open issues for 16QAM for NB-IoT	Nokia, Nokia Shanghai Bells	discussion	Rel-17

R2-2200683	Remaining FFSs on 16QAM for NB-IoT and 1736bits TBS for eMTC	ZTE Corporation, Sanechips	discussion	NB_IOTenh4_LTE_eMTC6-Core

R2-2201078	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson	discussion	Rel-17

R2-2201449	CQI reporting for 16QAM DL	Huawei, HiSilicon	discussion	Rel-17	NB_IOTenh4_LTE_eMTC6-Core

R2-2201448	Introduction of Rel-17 enhancements for NB-IoT and eMTC	Huawei, HiSilicon	draftCR	Rel-17	36.302	16.1.0	B	NB_IOTenh4_LTE_eMTC6-Core

# 2	Open Issue List

The resolution proposed is based upon below

- Each open issue should be associated with suggested treatment/handling.

  - Company input into Pre117-e-offline (i.e. no company tdocs)

  - Company tdocs invited.

  - CR rapporteur handled issue (CR rapporteur will propose resolution as input to next meeting).

  - Other, e.g. immature area, reference to dependency, unclear status etc.

| # Slogan | # Open Issue | # Criticality | # Remark | # Resolution |
| --- | --- | --- | --- | --- |
| # CQI Reporting extension for 16 QAM in msg3 | # Is CQI Reporting extension for 16 QAM in msg3 should be supported? [R2-2200677] | # low | RAN1 agreed that The channel quality report is not supported in Msg3 in connected mode in Rel-17. Further, R2-2200683 suggest 16QAM related channel quality report in Msg3 in idle mode is not supported. We can follow this suggestion and agree that in Rel-17 16QAM related channel quality report in Msg3 in idle mode is not supported. | Company input into Pre117-e-offline (i.e. no company tdocs) |
| # supporting 16 QAM for MT EDT | # Is 16 QAM for MT EDT supported? [R2-2200677] | # low | # Not necessary to consider this optimization in this release; i.e not needed to complete the 16QAM feature in connected mode. | # Company input into Pre117-e-offline (i.e. no company tdocs) |
| # Trigger for 16QAM | # Is trigger for 16QAM needed, if yes which trigger to use | # Medium | legacy Downlink Channel Quality Report Command MAC CE can be reused to trigger the 16QAM related channel quality. | Pre-117e-Offline |
| # Code Points for 16QAM | # How to report CQI for 16QAM [R2-2200677] | # medium | # For connected mode CQI Reporting for 16 QAM use of R bits or unused code-points of NPDCCH-CQI can be considered. R2-2200093 Rapporteur of the TS from MAC and RRC should consider updating. | # Pre-117e-Offline |
| # Implementation of RAN1 parameterlist | # Capturing of RAN1 parameter list and RAN1 agreements | # high | # TS 36.331 Running CR Rapporteur may check the TP R2-2201078 and RAN1 parameter list and take any missing attributes into account | # CR rapporteur handled issue (CR rapporteur will propose resolution as input to next meeting) |
| Field description of npusch-MCS | Is field description update as below suggetsed in R2-2201078 agreeable? npusch-MCS # Index to tables specified in TS 36.213 [23], Table 16.5.1.2-1 and Table 16.5.1.2-2 for single tone and multi tone respectively, that defines modulation and TBS index for NPUSCH for PUR. In case of pur-UL-16QAM-Config is true, multiTone index is used, for the guardband and standalone modes the 16-QAM MCS index is equal to multiTone + 14, for the inband mode the 16-QAM MCS index is equal to multiTone + 11. | # Medium | # Instead of using new ASN.1 bits; PUR-UL-16QAM-Config-NB-r17 ::= SEQUENCE { npusch-MCS-r17 INTEGER (14..21) } # the field description can be updated. TS 36.331 rapporteur can take into consideration | # CR rapporteur handled issue (CR rapporteur will propose resolution as input to next meeting) |
| Stage 2 for 16QAM | How to capture stage2 for 16QAM? | # Medium | # Adopt the CR R2-2201448 | # Pre-117e-Offline |
| TBS Size | ## Support of TBS size 1736 bits with other features | # Low | # Check the below proposals in next meeting. Proposal 2: RAN2 confirm that DL TBS of 1736 bits can be supported in multi-TB scheduling. Proposal 3a: DL TBS of 1736 bits is not supported in SC-PTM. Proposal 3b: DL TBS of 1736 bits is not supported in EDT. | # Company input into Pre117-e-offline (i.e. no company tdocs) |

# 3	Discussion on Open Issue List

## 3.1	CQI Report for16QAM in Msg3

Question 1: Should CQI Reporting extension for 16 QAM in msg3 be supported?

Please provide the comments on the Question here:

| Company | Yes/No |  |
| --- | --- | --- |

## 3.2	16QAM feature support for MT-EDT

Question 2: Is 16 QAM for MT EDT to be supported?

Please provide the comments on the Question here:

| Company | Yes/No |  |
| --- | --- | --- |

## 3.3	Trigger for 16QAM

Question 3: Is trigger for 16QAM needed, if yes which trigger to use?

- legacy Downlink Channel Quality Report Command MAC CE can be reused to trigger the 16QAM related channel quality.

- Any Other

- No trigger needed

Please provide the comments on the Question here:

| Company | OPTIONS (A/B/C) |  |
| --- | --- | --- |

## 3.4	CQI Report for16QAM in Msg3

Question 4: Is CQI Reporting extension for 16 QAM in msg3 should be supported?

Please provide the comments on the Question here:

| Company | Yes/No |  |
| --- | --- | --- |

## 3.5	Code Points for 16QAM

Question 5: How to report CQI for 16QAM?

- For connected mode CQI Reporting for 16 QAM use of R bits or unused code-points of NPDCCH-CQI can be considered.

Please provide your solution on which code points for 16QAM needs to be used and please provide motivation.

| Company | Solution: Which code points |
| --- | --- |

## 3.6	Stage 2 for 16QAM

Question 1: Please review R2-2201448 and provide your comments.

Please provide the comments on the Question here:

| Company | Yes/No |  |
| --- | --- | --- |

## 3.7	TBS Size

Question 1: Do companies agree with the below proposals from paper R2-2200683?

Proposal 2: RAN2 confirm that DL TBS of 1736 bits can be supported in multi-TB scheduling.

Proposal 3a: DL TBS of 1736 bits is not supported in SC-PTM.

Proposal 3b: DL TBS of 1736 bits is not supported in EDT.

Please provide the comments on the Question here:

| Company | Yes/No |  |
| --- | --- | --- |

## 3.7	Any Other

Please provide the comments for anything missing:

| Company |  |
| --- | --- |

# Conclusion