3GPP TSG-RAN WG2 #117-e	R2-22xxxx

Electronic meeting, 2022-02-21 - 2022-03-03

Agenda Item:	9.1.4

Source:	Ericsson

Title:	Report on [Pre117-e][303][NBIOT/eMTC R17] Other open issues 	(Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This document is to lick off below email discussion.

- [Pre117-e][303][NBIOT/eMTC R17] (Ericsson)

Agreements in this area are listed below:

### NB-IoT 16-QAM for unicast in UL and DL

RAN2#113bis-e agreements: Working assumption: For the UE supporting 16-QAM, the L2 buffer size is 12000 bytes. Working assumption: Support of 16-QAM has separate UE capabilities for DL and UL RAN2#114-e agreements: None RAN2#115-e agreements: Confirm the working assumption: The support of 16-QAM uses separate UE capabilities for DL and UL. 16QAM is configured via dedicated signaling separately for UL and DL. A NPUSCH 16QAM activation indication is needed in PUR configuration. RAN2#116-e agreements: Confirm the working assumption of 12000 bytes for DL 16QAM for NB-IoT

### 14 HARQ processes in DL for HD-FDD Cat M1 UEs

RAN2#113bis-e agreements: 14 HARQ activation is configured by dedicated RRC signalling. Working assumption: No change to current L2 buffer size requirement RAN2#114-e agreements: None RAN2#115-e agreements: Confirm the working assumption: No change to current L2 buffer size requirement for HD-FDD Cat M1 UEs supporting 14 HARQ processes in DL.

Paper submitted in RAN2#116bis-e

R2-2200677	On thje open issues for 16QAM for NB-IoT	Nokia, Nokia Shanghai Bells	discussion	Rel-17

R2-2200683	Remaining FFSs on 16QAM for NB-IoT and 1736bits TBS for eMTC	ZTE Corporation, Sanechips	discussion	NB_IOTenh4_LTE_eMTC6-Core

R2-2201078	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson	discussion	Rel-17

R2-2201449	CQI reporting for 16QAM DL	Huawei, HiSilicon	discussion	Rel-17	NB_IOTenh4_LTE_eMTC6-Core

R2-2201448	Introduction of Rel-17 enhancements for NB-IoT and eMTC	Huawei, HiSilicon	draftCR	Rel-17	36.302	16.1.0	B	NB_IOTenh4_LTE_eMTC6-Core

# 2	Open Issue List

The resolution proposed is based upon below

- Each open issue should be associated with suggested treatment/handling.

  - Company input into Pre117-e-offline (i.e. no company tdocs)

  - Company tdocs invited.

  - CR rapporteur handled issue (CR rapporteur will propose resolution as input to next meeting).

  - Other, e.g. immature area, reference to dependency, unclear status etc.

| # Slogan | # Open Issue | # Criticality | # Remark | # Resolution |
| --- | --- | --- | --- | --- |
| # CQI Reporting extension for 16 QAM in msg3 | # Is CQI Reporting extension for 16 QAM in msg3 should be supported? [R2-2200677] | # low | RAN1 agreed that The channel quality report is not supported in Msg3 in connected mode in Rel-17. Further, R2-2200683 suggest 16QAM related channel quality report in Msg3 in idle mode is not supported. We can follow this suggestion and agree that in Rel-17 16QAM related channel quality report in Msg3 in idle mode is not supported. | Company input into Pre117-e-offline (i.e. no company tdocs) |
| # supporting 16 QAM for MT EDT | # Is 16 QAM for MT EDT supported? [R2-2200677] | # low | # Not necessary to consider this optimization in this release; i.e not needed to complete the 16QAM feature in connected mode. | # Company input into Pre117-e-offline (i.e. no company tdocs) |
| # Trigger for 16QAM | # Is trigger for 16QAM needed, if yes which trigger to use | # Medium | legacy Downlink Channel Quality Report Command MAC CE can be reused to trigger the 16QAM related channel quality. | Pre-117e-Offline |
| # Code Points for 16QAM | # How to report CQI for 16QAM [R2-2200677] | # medium | # For connected mode CQI Reporting for 16 QAM use of R bits or unused code-points of NPDCCH-CQI can be considered. R2-2200093 Rapporteur of the TS from MAC and RRC should consider updating. | # Pre-117e-Offline |
| # Implementation of RAN1 parameterlist | # Capturing of RAN1 parameter list and RAN1 agreements | # high | # TS 36.331 Running CR Rapporteur may check the TP R2-2201078 and RAN1 parameter list and take any missing attributes into account | # CR rapporteur handled issue (CR rapporteur will propose resolution as input to next meeting) |
| Field description of npusch-MCS | Is field description update as below suggetsed in R2-2201078 agreeable? npusch-MCS # Index to tables specified in TS 36.213 [23], Table 16.5.1.2-1 and Table 16.5.1.2-2 for single tone and multi tone respectively, that defines modulation and TBS index for NPUSCH for PUR. In case of pur-UL-16QAM-Config is true, multiTone index is used, for the guardband and standalone modes the 16-QAM MCS index is equal to multiTone + 14, for the inband mode the 16-QAM MCS index is equal to multiTone + 11. | # Medium | # Instead of using new ASN.1 bits; PUR-UL-16QAM-Config-NB-r17 ::= SEQUENCE { npusch-MCS-r17 INTEGER (14..21) } # the field description can be updated. TS 36.331 rapporteur can take into consideration | # CR rapporteur handled issue (CR rapporteur will propose resolution as input to next meeting) |
| Stage 2 for 16QAM | How to capture stage2 for 16QAM? | # Medium | # Adopt the CR R2-2201448 | # Pre-117e-Offline |
| TBS Size | ## Support of TBS size 1736 bits with other features | # Low | # Check the below proposals in next meeting. Proposal 2: RAN2 confirm that DL TBS of 1736 bits can be supported in multi-TB scheduling. Proposal 3a: DL TBS of 1736 bits is not supported in SC-PTM. Proposal 3b: DL TBS of 1736 bits is not supported in EDT. | # Company input into Pre117-e-offline (i.e. no company tdocs) |

# 3	Discussion on Open Issue List

## 3.1	CQI Report for16QAM in Msg3

Question 1: Should CQI Reporting extension for 16 QAM in msg3 be supported?

Please provide the comments on the Question here:

| Company | Yes/No |  |
| --- | --- | --- |
| Huawei, HiSilicon | No | For MSG3, UE is configured with QPSK so what would be the benefit to report a smaller range |
| Qualcomm | No | For MSG3 sent in connected mode RAN1 conclusion is not to support 16QAM CQI reporting. For MSG3 sent during idle mode (e.g., during connection establishment or EDT), 16QAM CQI does not apply because UE would not have any 16QAM metrics to report. |

## 3.2	16QAM feature support for MT-EDT

Question 2: Is 16 QAM for MT EDT to be supported?

Please provide the comments on the Question here:

| Company | Yes/No |  |
| --- | --- | --- |
| Huawei, HiSilicon | No | We do not see how the UE would know as MSG4 in EDT uses the MAC/PHY default configuration |
| Qualcomm | No | RAN1 has agreed to not support 16QAM with EDT for both UL and DL. |

## 3.3	Trigger for 16QAM

Question 3: Is trigger for 16QAM needed, if yes which trigger to use?

- legacy Downlink Channel Quality Report Command MAC CE can be reused to trigger the 16QAM related channel quality.

- Any Other

- No trigger needed

Please provide the comments on the Question here:

| Company | OPTIONS (A/B/C) |  |
| --- | --- | --- |
| Huawei, HiSilcion | Not C | We think a trigger is needed for reporting in connected mode, same as in legacy. Whether we can use the legacy trigger depends on RAN1 whether the UE can be asked or not to report one or the other table. |
| Qualcomm | A | No technical reason to have a new trigger. |

## 3.4	CQI Report for16QAM in Msg3

Question 4: Is CQI Reporting extension for 16 QAM in msg3 should be supported?

Please provide the comments on the Question here:

| Company | Yes/No |  |
| --- | --- | --- |
| Huawei, HiSilicon | No | Same as 3.1 |
| Qualcomm | No | See comments to Q1 |

## 3.5	Code Points for 16QAM

Question 5: How to report CQI for 16QAM?

- For connected mode CQI Reporting for 16 QAM use of R bits or unused code-points of NPDCCH-CQI can be considered.

Please provide your solution on which code points for 16QAM needs to be used and please provide motivation.

| Company | Solution: Which code points |
| --- | --- |
| Huawei, HiSilicon | We are not sure about the question. RAN1 has agreed on a new table that RAN4 has incorporated in their specification. There is no need for redefinition of any code points in RAN2 In MAC, we will just need to specify under which conditions one or the other table is used. This is pending on RAN1 discussion. |
| Qualcomm | We don’t think MAC needs to differentiate between reporting legacy reporting values and new reporting values. The RAN1 has designed the new reporting values to include both QPSK and 16QAM CQIs. This means if UE is configured with 16QAM for NPDSCH then UE shall report the new values only and there would be no confusion at the eNB. |

## 3.6	Stage 2 for 16QAM

Question 1: Please review R2-2201448 and provide your comments.

Please provide the comments on the Question here:

| Company | Yes/No |  |
| --- | --- | --- |
| Huawei. HiSilicon | Yes |  |
| Qualcomm | N/A | Draft CR in R2-2201448 looks fine. |

## 3.7	TBS Size

Question 1: Do companies agree with the below proposals from paper R2-2200683?

Proposal 2: RAN2 confirm that DL TBS of 1736 bits can be supported in multi-TB scheduling.

Proposal 3a: DL TBS of 1736 bits is not supported in SC-PTM.

Proposal 3b: DL TBS of 1736 bits is not supported in EDT.

Please provide the comments on the Question here:

| Company | Yes/No |  |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes |  |

## 3.7	Any Other

Please provide the comments for anything missing:

| Company |  |
| --- | --- |

# Conclusion