3GPP TSG RAN WG2#118-e	R2-2206770

e-Meeting, 9th - 20th May, 2022

# 3GPP TSG-SA WG2 Meeting #151E	S2-2204989

Elbonia, 16 - 20 May 2022

Title:	LS On FS_REDCAP_Ph2 option feasibility

Response to:	-

Release:	Rel-18

# Work Item:	FS_REDCAP_Ph2

Source:	SA2

To:	RAN3, RAN2

cc:	CT4, CT1

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 Rel-18 study FS_REDCAP_Ph2 is ongoing and approaching the conclusion phase. The study is focused on one key issue which enables long eDRX>10.24s cycle support for UE in RRC_INACTIVE state. See more details in latest TR 23.700-68 (link).

SA2 discussed mainly two different types of solutions:

A) 	NG-RAN provides UE unreachability information (e.g. the eDRX information) to CN when UE enters RRC_INACTIVE state with long eDRX and CN handles the MT data/signalling while the UE is unreachable. CN triggers MT data/signalling when the UE is considered reachable. For example, see solution#6 in TR 23.700-68 for reference.

B) 	NG-RAN handles MT data/signalling while the UE is RRC_INACTIVE state. In case UE moves out of the RNA area during the unreachable time period and performs resume outside the RNA, the UE context retrieval between NG-RAN nodes and data forwarding are supported via CN when there is no Xn interface. For example, see solution#2 in TR 23.700-68 for reference.

SA2 is starting to consider its conclusion and would like to ask RAN2 and RAN3 on their views on the following aspects:

- NG-RAN providing UE unreachability information to CN for MT data/signalling handling when UE is not reachable in RRC_INACTIVE state.

- NG-RAN can handle a new NG_AP message to trigger RAN paging when UE is in RRC-INACTIVE.

- Including the UE context retrieval with data forwarding handling between NG-RAN nodes via CN.

- NG-RAN buffering capabilities of MT data for the duration of the eDRX cycle.

- NG-RAN’s ability to perform UE context release procedure towards the AMF and locally releases the UE to RRC-IDLE when receiving DL NAS message and the UE is not reachable for a time period longer than 10.28s.

- Alternative to (5): NG-RANs ability to only provide an indication to AMF when receiving DL NAS message and the UE is not reachable for a time period longer than 10.28s. The UE remains in RRC_INACTIVE.

2. Actions:

To RAN3, RAN2

ACTION: 	SA2 kindly asks RAN2 and RAN3 take the above information into consideration and provide their views on the above aspects.

3. Date of Next SA2 Meeting:

SA2#152e	August 17-26, 2022	Electronic