3GPP TSG-RAN WG2 Meeting #118-e                                   R2-xxxxxxx

E-Meeting, May 9th – May 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3120 | rev | 2 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CHO configuration with SCG release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-16, in RAN3 discussions (RAN3 #107-bis-e, RAN3 #108-e meetings), it was decided that the scenario where the SCG is released upon CHO execution, is supported, and changes to the Stage-2 (37.340, 36.300) and Stage-3 (36.423, 38.423) specifications were agreed. However, in the Rel-16 RRC specification 38.331, clarification seems to be needed to indicate that this scenario is supported, i.e., for a UE in MR-DC operation, upon CHO execution, the SCG is released by network signaling. The description of the field condRRCReconfig only indicates that the RRCReconfiguration message contained in it cannot contain the target SCG configuration for CHO. condRRCReconfig The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration, the field daps-Config or the configuration for target SCG for CHO.. |
| Summary of change: |  | Provides text clarification in the description of the field condRRCReconfig to indicate that network signaling supports release of the SCG configuration upon CHO execution for a UE operating in MR-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is potential confusion regarding whether, for a UE operating in MR-DC, the SCG configuration is released upon CHO execution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, - CondReconfigToAddModList |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 2: Indicates that network signaling supports release of the SCG configuration upon CHO execution for a UE operating in MR-DC. Rev 1 and “-“: Provided specification changes indicating that UE autonomously releases SCG configuration upon CHO execution, if operating in MR-DC. This was not agreeable to majority of companies in the email discussion. |  |  |  |  |  |  |  |  |

## ----------Start of the change--------------

### 6.3.2	Radio resource control information elements

## ----------Skipping unchanged part--------------

#### –	CondReconfigToAddModList

The IE CondReconfigToAddModList concerns a list of conditional reconfigurations to add or modify, with for each entry the condReconfigId and the associated condExecutionCond and condRRCReconfig.

CondReconfigToAddModList information element

-- ASN1START

-- TAG-CONDRECONFIGTOADDMODLIST-START

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Cond condReconfigAdd

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...

}

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

-- ASN1STOP

CondReconfigToAddMod field descriptions

condExecutionCond The execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject.

condRRCReconfig The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration, the field daps-Config or the configuration for target SCG for CHO. For CHO, if the UE is configured with MR-DC, the RRCReconfiguration message contained in condRRCReconfig includes mrdc-SecondaryCellGroupConfig set to release.

| Conditional Presence | Explanation |
| --- | --- |
| condReconfigAdd | The field is mandatory present when a condReconfigId is being added. Otherwise the field is optional, need M. |

## ----------End of the change--------------