3GPP TSG-RAN WG2 Meeting #118-electronic	   R2-220xxxx

Online, May 9th - May 20th, 2022

Agenda item:		5.1.4.1

Source:	vivo

Title:	Report of [AT118-e][019][NR1516] CP Miscellanous

Document for:		Discussion and Decision

1 Introduction

This contribution is aimed at reporting the discussion and results of the following offline discussion:

[AT118-e][019][NR1516] CP Miscellanous (vivo)

Scope: Treat R2-2204902, R2-2205428, R2-2205429, R2-2204845, R2-2204846, R2-2205827, R2-2204728, R2-2204729, R2-2204845, R2-2204846, R2-2205827, R2-2204728, R2-2204729, R2-2205503, R2-2205504, R2-2205298, R2-2205299, R2-2205300

Ph1 Determine agreeable parts, Ph2 for agreeable parts agree CRs (offline agreement, CB online only if necessary).

Intended outcome: Report, Agreed CRs

Deadline: Schedule 1

The discussion scope is to gather companies’ views on the contributions [1]-[13]. Companies are invited to provide their views by May 12th (Thursday), 2022, 12:00 UTC for phase-1 discussion.

2 Participants

To facilitate this offline discussion amongst the delegates, would you please fill in your name and the email address in the table below.

| Delegate name | E-mail address |
| --- | --- |
| Yitao Mo (Stephen) | yitao.mo@vivo.com |
| Nokia | amaanat.ali@nokia.com |
| Docomo | masato.taniguchi.mf@nttdocomo.com |
| Sangbum Kim | sb07.kim@samsung.com |
| Mouaffac Ambriss (Qualcomm Inc) | mambriss@qti.qualcomm.com |
| Apple | rrossbach@apple.com |
| Lili Zheng (Huawei, HiSilicon) | zhenglili4@huawei.com |
| Fei Dong (ZTE) | Dong.fei@zte.com.cn |

3 Phase-1 Discussion

3.1 Clarification for Inter-MN HO without SN change

In the previous RAN2 meeting, the need for Stage 3 CR regarding inter-MN handover without SN change was discussed but postponed without consensus. The corresponding agreement is given as follows,

RAN2#117 meeting agreements R2-2202807 Clarification on inter-MN handover without SN change NEC CR Rel-15 38.331 15.16.0 2907 - F NR_newRAT-Core R2-2202808 Clarification on inter-MN handover without SN change NEC CR Rel-16 38.331 16.7.0 2908 - A NR_newRAT-Core [029] Both Postponed

To completely solve this issue which has been discussed for almost one year, the following proposal is given in the contribution [1],

Proposal 1: RAN2 to agree to capture the following in a Chairman notes. - RAN2 confirms that according to the current RRC spec, both fields sourceConfigSCG and scg-RB-Config in CG-ConfigInfo can be sent in the following cases: - SN change procedure - Inter-MN HO with SN change - Inter-MN HO without SN change (Case 0) - Inter-MN HO without SN node change (Case 2)

Q1: Do companies agree with Proposal 1?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| Nokia | Yes | We are fine to capture the scenarios listed in P1 for chair notes |
| vivo | Yes | It makes everything clear. |
| Docomo | Comments | The “content” of the proposal looks correct according to the past discussion, but we still have a concern on the current Stage 3 text, which is not aligned with the proposal and very misleading. sourceConfigSCG Includes all of the current SCG configurations used by the target SN to build delta configuration to be sent to UE, e.g. during SN change. The field contains the RRCReconfiguration message, i.e. including secondaryCellGroup and measConfig. The field is signalled upon change of SN, unless MN uses full configuration option. Otherwise, the field is absent. Could companies accept removing the following part from the description? The field is signalled upon change of SN, unless MN uses full configuration option. Otherwise, the field is absent. With that we can get rid of the misleading part, and the readers (espetially, dev/test people without the context of our long winding discussions) will be able to understand the intention in favor of previously agreed Stage 2 text. |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes |  |

Summary:

3.2 Correction on p-maxNR-FR1 in NR-DC

The current RRC spec specifies that the filed p-maxNR-FR1 is used in (NG)EN-DC and NE-DC. As a result, the power sharing framework for FR1-FR1 NR-DC might not work properly as this field cannot be used to indicate the maximum total transmit power of NR SCG. Thus, the CRs R2-2205428/5429 [2][3] propose that p-maxNR-FR1 shall be also applied to NR-DC, i.e., not only limited to (NG)EN-DC and NE-DC. The corresponding correction is quoted as follows,

p-maxNR-FR1 For (NG)EN-DC and NE-DC, the field indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 1 (FR1) (see TS 38.104 [12]). For NR-DC, it indicates the the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 1 (FR1) (see TS 38.104 [12]) the UE can use in NR SCG.

Q2: Do companies agree with the intention of CR?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| Nokia | Neutral | Proposed change is correct but we are not sure there is any misunderstanding as there is the p-maxUE-FR1 for full FR1 across all cell groups. So not sure why there should be particularly misunderstanding with this one. |
| vivo | No strong view | In our understanding, for NR-DC case, the field p-maxNR-FR1-MCG-r16 is used to indicate the maximum total transmit power that can be used in MCG. Consequently, the field p-maxNR-FR1 is only for SCG. It is quite straightforward. Anyway, no strong view on this clarification. |
| Docomo | Yes | Support. The intention is correct, and literally read, the description looks like it is a total transmit power across “all serving cells”. |
| Samsung | Yes |  |
| Apple | No strong view | It’s already specified, but if companies want clarification, we are ok. |
| Huawei, HiSilicon | Yes |  |

Summary:

3.3 Correction on rrc-ConfiguredUplinkGrant

In the CRs R2-2204845/4846/5827 [4]-[6], it is pointed out that the field description parts for both precodingAndNumberOfLayers and pathlossReferenceIndex are missing within the field rrc-ConfiguredUplinkGrant. So the CRs propose to add the following,

pathlossReferenceIndex Indicates the reference signal used as PUSCH pathloss reference (see TS 38.213 [13], clause 7.1.1).

precodingAndNumberOfLayers Indicates the precoding and number of layers (see TS 38.212 [17], clause 7.3.1.1.2).

Q3: Do companies agree with the intention of CR?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| Nokia | Yes | Propose to merge this to rapporteur CR |
| vivo | Yes (Proponent) | For Rel-15 and Rel-16 specs, we are fine to merge this to rapporteur CR. For Rel-17 spec, we can merge this to the SDT RRC CR as additional SDT-specific field description is needed for those fields. |
| Docomo | Yes |  |
| Samsung | Yes |  |
| Qualcomm Inc | No strong views | Can be merged with rapporteur CR. as a suggestion to reword the description for pathlossReferenceIndex, e.g., "indicates the reference signal index used as PUSCH pathloss reference" |
| Apple | Yes | Agree to these CRs with the addition under b) for R17. - pathlossReferenceIndex: A field description is also missing in the Rel-17 version and this is covered in R2-2205827. - precodingAndNumberOfLayers: Currently the Rel-17 description of this field (in the current h00 version of the RRC spec) references TS 38.213 not TS 38.212. This should be harmonized across releases so that different versions of the RRC spec do not refer different RAN1 specs. In fact the reference to 38.213 seems to be wrong, it should be TS 38.212 [17], clause 7.3.1.1.2. |
| ZTE | No strong views | Can be merged in rapporteur CR. Qualcomm’s suggestion is fine to us. |

Summary:

3.4 Correction on T345 for UAI overheating

In the CRs R2-2204728/4729 [7][8], it is mentioned that the stop conditions for T345 specified in the table in section 7.1.1 are not aligned with the procedure text that is specified in section 5.3.7. To this end, it is proposed that the stop conditions for T345 specified in the table in section 7.1.1 shall be revised from “Upon releasing overheatingAssistance during the connection re-establishment procedure” to “Upon releasing overheatingAssistanceConfig during the connection re-establishment procedure”. More specifically,

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T345 | Upon transmitting UEAssistanceInformation message with overheatingAssistance | Upon releasing overheatingAssistanceConfig during the connection re-establishment procedure, upon initiating the connection resumption procedure, and upon receiving overheatingAssistanceConfig set to release. | No action. |

Q4: Do companies agree with the intention of CR?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| Nokia | Yes | Yes, minor but in fact brings clarity. Can be also captured in the rapporteur CR, as reflects the intended behaviour |
| vivo | Yes |  |
| Docomo | Yes |  |
| Samsung | Yes with comments | Section 7.1.1 is just informative, so it would be good to be merged into Rap CR. |
| Qualcomm Inc | Yes |  |
| Apple | Yes |  |

Summary:

3.5 Need code correction for ReferenceTimeInfo

In NR, upon receiving reference time information in DL information transfer or SIB9, the UE action is to deliver the time to the upper layer, i.e., one shot. However, the need code of referenceTimeInfo-r16 is currently set to Need R, which requires the UE to unnecessarily store the reference time which will be useless after delivering to the upper layer. Thus, the CRs R2-2205503/5504 suggest changing the need code from Need R to Need N in DLInformationTransfer and SIB9, as follows,

referenceTimeInfo-r16               ReferenceTimeInfo-r16               OPTIONAL,   -- Need N

Q5: Do companies agree with the intention of CR?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| Nokia | Neutral | Not sure what is the additional implication to UE to discard v/s store. If it is just a variable storage then maybe change is not so critical. Is there a functionality impact that requires this as some sort of essential correction? If not, then we don’t think any reason to change. |
| vivo | No strong view for DLInformationTransfer No for SIB9 | In our understanding, either implementation leads to Rome. The differences are in storage overhead (e.g. whether the UE needs to store the ASN.1 configuration in the local UE configuration) and storage flush (e.g. the UE needs to flush the local UE configuration when this field is not configured in the next reconfiguration). Anyway, the differences have no impact on functionality, inter-operability, and performance. So, we don’t have a strong view. However, for SIB9, we think the correction is not needed as any field with Need M or Need N in system information shall be interpreted as Need R, according to the current RRC spec. |
| Docomo | No strong view | The second comment from vivo makes sense – would this impact 6.1.2? |
| Samsung | Yes | The alignment of Rel-17 ASN.1. seems correct, but not so essential |
| Apple | No strong view | At least for DLInformationTransfer it would make sense to have a similar behavior between R16 and R17 as was also was discussed in the ASN.1 ad-hoc in the context of I005. So strictly speaking the answer should be Yes for DLInformationTransfer and No for SIB9. But it does not make sense to have different treatment of this parameter depending on whether it was received dedicated signalling or SIB. |

Summary:

3.6 Correction on NR serving frequency results reporting

According to the sub-clause 5.5.5.1 in TS 36.331, for the event A3/A4/A5/B1-NR/B2-NR measurement, if the purpose field is not configured or set to reportLocation, the UE will not include NR serving frequency results. However, for the case that the purpose field is not configured (i.e. general measurements other than sidelink or sensing measurements), the NR serving cell results are also expected to be reported. Thus, to realize the NR serving cell results reporting when the purpose field is not configured, the CRs R2-2205298/5299/5300 suggest the following changes,

1> if the triggerType is set to event; and if the corresponding measObject concerns NR; and if eventId is set to eventB1-NR or eventB2-NR; or 1> if the triggerType is set to event; and if eventId is set to eventA3 or eventA4 or eventA5: 2> if purpose for the reportConfig or reportConfigInterRAT associated with the measId that triggered the measurement reporting is set to a value other than reportLocation or purpose is not configured: 3> set the measResultServFreqListNR to include for each NR serving frequency that the UE is configured to measure according to TS 38.331 [82], if any, the following: 4> set measResultSCell to include the available results of the NR serving cell, as specified in 5.5.5.2; 4> if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas and if eventId is set to eventA3 or eventA4 or eventA5: 5> set measResultBestNeighCell to include the available results, as specified in 5.5.5.2, of the non-serving cell with the highest sorting quantity determined as specified in 5.5.5.3; 3> for each (serving or neighbouring) cell for which the UE reports results according to the previous, additionally include available beam results according to the following: 4> if maxReportRS-Index is configured, set measResultRS-IndexList to include available results, as specified in 5.5.5.2, of up to maxReportRS-Index beams, ordered based on the quantity determined as specified in 5.5.5.3;

Q6: Do companies agree with the intention of CR?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| Nokia | Rel-15, NO unless there is an IODT issue which is there to clarify | We are a bit careful not to update Rel-15 Is this a real issue now coming from IODT? As it needs to be considered this is for Rel-15 apparently (old release) and in our understanding the change is not so fundamental that it would require to now put at risk existing Rel-15 implementations. If 'purpose not configured' is listed explicitly, is it changing something, from the procedural perspective? We think the reporting would happen also today, as 'no purpose' matches the case 'if report purpose is other than reportLocation' so maybe no issue to fix, in fact? We would like to first have common understanding of what the problem really is… |
| vivo | Comments | Similar view with Nokia. We are wondering whether the mentioned case really exists. |
| Docomo | Comments | We agree with the intention that measResultServFreqListNR should be included in “no purpose” case, and “purpose is set to … other that reportLocation” looks a bit tricky. Having said that we can follow the majority considering the timing. |
| Samsung | Yes |  |
| Apple | Comments | We agree with the intention, but not sure whether it will have IODT issue. |
| Huawei, HiSilicon | Yes | From the comments of above companies, it seems everyone agree with the intention that “no purpose configured” case is missing in the current text. Then we don’t understand why RAN2 does not make it clear in the spec. As far as we know, RAN5 also considers this piece of text ambiguous when designing test cases. The corresponding RAN5 papers are in (R5-220106, R5-220107), during offline discussion in RAN5, the following question was raised: for the event-triggered reporting we have following purpose options: reportLocation, sidelink, spare2, spare1 and sensing Using reportLocation is prohibited, setting sidelink/sensing may cause additional complications. Do we have to configure spare then if we want the condition 2 to be satisfied? So form our perspective, RAN2 can refine our spec to spare RAN5 the pain. |

Summary:

4 Conclusion

This discussion report is summarized with final proposals as follows,

5 Reference

- R2-2204902, Confirmation for inter-MN HO without SN change, NEC.

- R2-2205428, Correction on FR1-FR1power control parameters of NR-DC, CATT.

- R2-2205429, Correction on FR1-FR1power control parameters of NR-DC, CATT.

- R2-2204845, Correction on rrc-ConfiguredUplinkGrant in Rel-15, vivo.

- R2-2204846, Correction on rrc-ConfiguredUplinkGrant in Rel-16, vivo.

- R2-2205827, Correction on rrc-ConfiguredUplinkGrant in Rel-17, vivo.

- R2-2204728, Correction on T345 for UAI overheating, OPPO.

- R2-2204729, Correction on T345 for UAI overheating, OPPO.

- R2-2205503, Need code correction for ReferenceTimeInfo, Ericsson.

- R2-2205504, Need code correction for ReferenceTimeInfo, Ericsson.

- R2-2205298, Correction on NR serving frequency results reporting for event-triggered measurement (R15), Huawei, HiSilicon.

- R2-2205299, Correction on NR serving frequency results reporting for event-triggered measurement (R16), Huawei, HiSilicon.

- R2-2205300, Correction on NR serving frequency results reporting for event-triggered measurement (R17), Huawei, HiSilicon.