3GPP TSG-RAN WG2 Meeting #118-e                                         R2-220xxxx

Online, May 9th – May 20th, 2022

Agenda item:	6.1.3.1

Source:	CATT

Title: 	Report of [AT118-e][030][MBS] CP other

WI code:	NR_MBS-Core

Document for:	Discussion and Decision

# Introduction

This document aims at gathering and summarizing companies’ views for the following offline discussion:

[AT118-e][030][MBS] CP other (CATT)

Scope: Treat R2-2204669, R2-2204827, R2-2205749, R2-2204670, R2-2204828, R2-2205249, R2-2205632, R2-2206123, R2-2205626, R2-2206124, R2-2204830, R2-2205627, R2-2204668, R2-2205745

Collect one round of comments, pave the way for on-line agreement (identify agreeable points, discussion points),

Intended outcome: Report

Deadline: For online CB W1 Thursday

# Contact details

| Company |  | Contact details (name, e-mail) |
| --- | --- | --- |
| CATT |  | Rui Zhou(zhourui@catt.cn) |
| Lenovo |  | Mingzeng Dai, daimz4@lenovo.com |
| Huawei, HiSilicon |  | Dawid Koziol (dawid.koziol@huawei.com) |
| Qualcomm |  | Umesh Phuyal (uphuyal@qti.qualcomm.com) |
| Samsung |  | Vinay Kumar Shrivastava (shrivastava@samsung.com) |
| Nokia |  | Jarkko Koskela (jarkko.t.koskela@nokia.com) |
| OPPO |  | wangshukun@oppo.com |
| MediaTek |  | Xiaonan Zhang (Xiaonan.Zhang@mediatek.com) |
| Spreadtrum |  | lifeng.han@unisoc.com |
| Apple | Fangli XU (fangli_xu@apple.com) |  |

# Discussion

## Group paging Handling

### [C006] Correction to UE Behavior on Group Paging Handling

In R2-2204669, it is proposed to specify that when UE receiving group paging that does not contain CN paging, UE should not forward TMGI to upper layers.

In this case, RRC inactive UE directly triggers RRC resume procedure, and thus from this point of view RRC does not expect any trigger from NAS. In this case if TMGI is forwarded to NAS, unexpected procedure (e.g. NAS requests the lower layer to establish an RRC connection) may be triggered by NAS. It is unnecessary and may conflict with ongoing RRC resume procedure.

| TDoc | Proposals |
| --- | --- |
| CATT R2-2204669 | 5.3.2.3 Reception of the Paging message by the UE …… 1> if in RRC_IDLE, for each TMGI included in pagingGroupList, if any, included in the Paging message: 2> if the UE has joined an MBS session indicated by the TMGI included in the pagingGroupList: 3> forward the TMGI to the upper layers; 1> if in RRC_INACTIVE and the UE has joined one or more MBS session(s) indicated by the TMGI included in the pagingGroupList: 2> if none of the ue-Identity included in any of the PagingRecord, if included in the Paging message, matches the UE identity allocated by upper layers: 3> initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set as below: 4> if the UE is configured by upper layers with Access Identity 1: 5> resumeCause is set to mps-PriorityAccess; 4> else if the UE is configured by upper layers with Access Identity 2: 5> resumeCause is set to mcs-PriorityAccess; 4> else if the UE is configured by upper layers with one or more Access Identities equal to 11-15: 5> resumeCause is set to highPriorityAccess; 4> else: 5> resumeCause is set to mt-Access. 2> else: 3> forward the TMGI to the upper layers; 3> perform the actions upon going to RRC_IDLE as specified in 5.3.11; |

Companies are then requested to answer the following question.

Question 1: Do you agree the change proposed in R2-2204669?

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | Yes | According to the spec, RRC inactive UE directly triggers RRC resume procedure if there is only TMGIs in paging message, and thus from this point of view RRC does not expect any trigger from NAS. To avoid unexpected procedure (e.g. NAS requests the lower layer to establish an RRC connection) triggered by NAS, this change makes sense. |
| Lenovo | Yes |  |
| Huawei, HiSilicon | No | This was discussed before and the conclusion was that it makes no harm to provide TMGIs and NAS layer can do whatever it wants with this. It would be safest to check this with CT1, together with other cases where AS interacts with NAS for MBS. We proposed an LS in R2-2206124. |
| Qualcomm | No | In R17 NAS spec, when TMGI is received from RRC, it may or may not trigger Service Request procedure. We should not assume that NAS always triggering SR procedure. In RRC_INACTIVE state, NAS knows RRC state of UE and if TMGI is received from NAS then NAS layer may decide to respond to the page based on TMGI or not. |
| Samsung | No | Purpose of forwarding TMGI to upper layers can be multi-fold e.g. NAS gets early informed about session activation, upper layer/service layer/display can get update. There may be no trigger from NAS to AS in response, that is fine. We think sending LS is okay. |
| Nokia | No but first change is OK | From NAS point of view UE is in CM_CONNECTED state when in RRC_INACTIVE state. Thus any forwarding of TMGI should not trigger any actions from NAS. Thus we do not really see any issue. First change “if in RRC_IDLE” addition seems to be fine though |
| OPPO | Yes | We also think it is different for RRC_IDLE mode and RRC_INACTIVE mode UE. |
| MediaTek | Yes | It makes sense not to send TMGI to NAS if UE can trigger RRC resume, otherwise it may lead to misunderstanding to NAS and then trigger SR procedure. Ok to send a LS to check this behaviour. |
| Spreadtrum | No | The NAS may need to be aware of the TMGI in paging, and maybe we need to check with CT1. |
| Apple | No | Before this clarification/correction, it’s better to check the NAS operation when receiving TMGI in the RRC_INACTIVE state. |

### [V500] Clarification on Group Paging for INACTIVE UE

In R2-2204827, it is proposed to clarify that UE in RRC_INACTIVE only monitor RAN paging using TMGI as CN paging using TMGI is only used to page CM-IDLE UEs.

| TDoc | Proposals |
| --- | --- |
| VIVO R2-2204827 | Proposal 1: Update the description in TS 38.331 clause 4.2.8 that “If configured by upper layers for MBS multicast reception, monitors Paging channel for paging using TMGI” to “If configured by upper layers for MBS multicast reception, monitors Paging channel for RAN paging using TMGI”. Annex: Proposed TP to TS 38.331 4.2.1 UE states and state transitions including inter RAT A UE is either in RRC_CONNECTED state or in RRC_INACTIVE state when an RRC connection has been established. If this is not the case, i.e. no RRC connection is established, the UE is in RRC_IDLE state. The RRC states can further be characterised as follows: …… - RRC_INACTIVE: - A UE specific DRX may be configured by upper layers or by RRC layer; - UE controlled mobility based on network configuration; - The UE stores the UE Inactive AS context; - A RAN-based notification area is configured by RRC layer; - Transfer of unicast data and/or signalling to/from UE over radio bearers configured for SDT; The UE: - Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5); - During SDT procedure, monitors control channels associated with the shared data channel to determine if data is scheduled for it; - While SDT procedure is not ongoing, monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI; - If configured by upper layers for MBS multicast reception, monitors Paging channel for RAN paging using TMGI; |

Companies are then requested to answer the following question.

Question 2: Do you agree the change proposed in R2-2204827?

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | No | For inactive UE, it is still possible to receive a CN paging using TMGI. |
| Lenovo | No | Both CN paging and RAN paging using TMGI should be supported. We prefer to change it as: …for both CN and RAN paging using TMG |
| Huawei, HiSilicon | No | Agree with CATT. It is not appropriate to call it RAN paging. |
| Qualcomm | No | Statement in 4.2.1 is generic and is ok. As long as specific subclause clarify what to do, this section can remain generic. That, we don’t think Lenovo’s suggestion is needed either. Rapporteur had also indicated to reject this RIL and we agree with rapp. |
| Samsung | No | INACTIVE state UE can receive CN paging with TMGI and transits to IDLE state. Based on upper layer initiation, UE can setup a RRC connection and receive multicast service. |
| Nokia | No | Shouldn’t UE be able to receive which ever identity in which ever state? And terminology RAN paging / CN paging is very confusing. We would need to talk about paging with some specific identity instead e.g. Paging with I-RNTI or paging with S-TMSI etc… |
| OPPO | No |  |
| MediaTek | No |  |
| Spreadtrum | No |  |
| Apple | No | The RRC_INACTIVE UE is required to monitor both RAN paging and CN paging. |

### Multicast session start and Paging

In R2-2205749, it is proposed to change the Need code for pagingGroupList and add field description for serviceID, and it is also proposed that UE should report TMGI to upper layers when the when the multicast MRB is established.

| TDoc | Proposals |
| --- | --- |
| Ericsson R2-2205749 | Proposal 1: Need code N is used for pagingGroupList Proposal 2: The following field description is used for the serviceID (similar as in LTE): serviceId Uniquely identifies the identity of an MBS service within a PLMN. The field contains octet 3- 5 of the IE Temporary Mobile Group Identity (TMGI) as defined in TS 24.008 [49]. The first octet contains the third octet of the TMGI, the second octet contains the fourth octet of the TMGI and so on. Proposal 3: When the multicast MRB is established the TMGI is reported to the upper layers. |

Companies are then requested to answer the following question.

Question 3: Do you agree the P1 in R2-2205749?

Proposal 1: Need code N is used for pagingGroupList

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | Yes |  |
| Lenovo | Yes |  |
| Huawei, HiSilicon | Yes | This is already captured in the rapporteur CR in R2-2205938. |
| Qualcomm | ok |  |
| Samsung | Yes |  |
| Nokia | Yes |  |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Spreadtrum | Yes |  |
| Apple | Yes |  |

Question 4: Do you agree the P2 in R2-2205749?

Proposal 2: The following field description is used for the serviceID (similar as in LTE):

serviceId

Uniquely identifies the identity of an MBS service within a PLMN. The field contains octet 3- 5 of the IE Temporary Mobile Group Identity (TMGI) as defined in TS 24.008 [49]. The first octet contains the third octet of the TMGI, the second octet contains the fourth octet of the TMGI and so on.

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | Yes |  |
| Lenovo | Yes |  |
| Huawei, HiSilicon | Yes | This is already captured in the rapporteur CR in R2-2205938. |
| Qualcomm | Ok | Minor editorial: …contains octet 3- 5 of the … -> octet should be octets and extra space after hyphen can be removed in 3- 5. |
| Samsung | Yes |  |
| Nokia | Yes |  |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Spreadtrum | Yes |  |
| Apple | Yes |  |

Question 5: Do you agree the P3 in R2-2205749?

Proposal 3: When the multicast MRB is established the TMGI is reported to the upper layers.

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | No | Not necessary. There is no such requirement” for display purposes” from CT1/SA2. |
| Lenovo | Yes |  |
| Huawei, HiSilicon | Yes | But this is already captured in the specifications like this. We can check this with CT1, if needed, as proposed in the draft LS in R2-2206124. |
| Qualcomm | Yes |  |
| Samsung | Yes | Multicast session activation/deactivation is linked with MRB setup/release in CONNECTED state. It is good to check with LS. |
| Nokia | No | We do not see really a motivation for this? Maybe we do not really understand the intention of this proposal though. Why would AS forward this information to upper layers – anyway if needed by NAS they can always use it as this is UE internal business. |
| OPPO | Yes |  |
| MediaTek |  | Ok to check with LS first |
| Spreadtrum | Yes |  |
| Apple |  | We can check with CT1 on the requirement. |

## Multicast MRB ID change handling

In RAN2#117e meeting, the following agreement is made,

- MRB ID can be changed without releasing/adding MRB (delta config)

The agreement is captured in the 38.331 CR as below. However, it seems that the current procedure does not reflect the agreement correctly.

5.3.5.6.7 Multicast MRB addition/modification The UE shall: 1> for each mrb-Identity value included in the mrb-ToAddModList for which mrb-IdentityNew is included (multicast MRB ID change): 2> update the mrb-Identity to the value mrb-IdentityNew; 1> for each mrb-Identity value included in the mrb-ToAddModList that is not part of the current UE configuration (multicast MRB establishment including the case when full configuration option is used): 2> establish a PDCP entity and configure it in accordance with the received pdcp-Config; …… <omitted> 1> for each mrb-Identity value included in the mrb-ToAddModList that is part of the current UE configuration (prior treating this mrb-ToAddModList): 2> if the reestablishPDCP is set: 3> if drb-ContinueROHC is included in pdcp-Config: 4> indicate to lower layer that drb-ContinueROHC is configured; 3> if drb-ContinueEHC-DL is included in pdcp-Config: 4> indicate to lower layer that drb-ContinueEHC-DL is configured; 3> re-establish the PDCP entity of this multicast MRB as specified in TS 38.323 [5], clause 5.1.2; 2> else, if the recoverPDCP is set: 3> trigger the PDCP entity of this MRB to perform data recovery as specified in TS 38.323 [5]; 2> if the pdcp-Config is included: 3> reconfigure the PDCP entity in accordance with the received pdcp-Config.

During R17 asn.1 review, [C001] [V503][V504][V508] [H002] are raised on this issue. The below papers are also submitted by companies to address this issue.

| TDoc | Proposals/TP |
| --- | --- |
| CATT R2-2204670 | 5.3.5.6.7 Multicast MRB addition/modification The UE shall: 1> for each mrb-Identity value included in the mrb-ToAddModList that is not part of the current UE configuration (multicast MRB establishment including the case when full configuration option is used): 2> establish a PDCP entity and configure it in accordance with the received pdcp-Config; …… <omitted> 1> for each mrb-Identity value included in the mrb-ToAddModList that is part of the current UE configuration (prior treating this mrb-ToAddModList): 2> if the mrb-IdentityNew is included (multicast MRB ID change): 3> update the MRB Identity in current UE configuration to the value mrb-IdentityNew; 2> if the reestablishPDCP is set: 3> if drb-ContinueROHC is included in pdcp-Config: 4> indicate to lower layer that drb-ContinueROHC is configured; 3> if drb-ContinueEHC-DL is included in pdcp-Config: 4> indicate to lower layer that drb-ContinueEHC-DL is configured; 3> re-establish the PDCP entity of this multicast MRB as specified in TS 38.323 [5], clause 5.1.2; 2> else, if the recoverPDCP is set: 3> trigger the PDCP entity of this MRB to perform data recovery as specified in TS 38.323 [5]; 2> if the pdcp-Config is included: 3> reconfigure the PDCP entity in accordance with the received pdcp-Config. |
| VIVO R2-2204828 | Proposal 1: Remove the first bullet 1 in TS 38.331 clause 5.3.5.6.7, and add the operation of updating the mrb-Identity to the value mrb-IdentityNew in the loop of “for each mrb-Identity value included in the mrb-ToAddModList that is part of the current UE configuration”. Proposal 2: Add a note in TS 38.331 clause 5.3.5.6.7 that “UE is not expected to receive the mrb-ToAddModList in which an entry carrying mrb-IdentityNew is put behind an entry carrying only mrb-Identity”. 5.3.5.6.7 Multicast MRB addition/modification The UE shall: 1> for each mrb-Identity value included in the mrb-ToAddModList that is part of the current UE configuration (prior treating this mrb-ToAddModList): 2> update the mrb-Identity to the value mrb-IdentityNew if included; 2> if the reestablishPDCP is set: 3> if drb-ContinueROHC is included in pdcp-Config: 4> indicate to lower layer that drb-ContinueROHC is configured; 3> if drb-ContinueEHC-DL is included in pdcp-Config: 4> indicate to lower layer that drb-ContinueEHC-DL is configured; 3> re-establish the PDCP entity of this multicast MRB as specified in TS 38.323 [5], clause 5.1.2; 2> else, if the recoverPDCP is set: 3> trigger the PDCP entity of this MRB to perform data recovery as specified in TS 38.323 [5]; 2> if the pdcp-Config is included: 3> reconfigure the PDCP entity in accordance with the received pdcp-Config. 1> for each mrb-Identity value included in the mrb-ToAddModList that is not part of the current UE configuration (multicast MRB establishment including the case when full configuration option is used): 2> establish a PDCP entity and configure it in accordance with the received pdcp-Config; 2> if an SDAP entity with the received tmgi does not exist: 3> establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1; 3> if an SDAP entity with the received tmgi did not exist prior to receiving this reconfiguration: 4> indicate the establishment of the user plane resources for the tmgi to upper layers; NOTE 1: When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. It does that e.g. by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity. NOTE 2: In this specification, UE configuration refers to the parameters configured by NR RRC unless otherwise stated. NOTE 3: In this specification, UE is not expected to receive the mrb-ToAddModList in which an entry carrying mrb-IdentityNew is put behind an entry carrying only mrb-Identity. |
| Nokia, Huawei R2-2205249 | Annex A: TP of modified procedural text for handling each element of mrb add mod list separately 5.3.5.6.7 Multicast MRB addition/modification The UE shall for each element in the order of entry in the list the mrb-ToAddModList: 1> if mrb-Identity value included in the mrb-ToAddModList that is part of the UE configuration: 2> if mrb-Identity value included in the mrb-ToAddModList for which mrb-IdentityNew is included (multicast MRB ID change): 3> update the mrb-Identity to the value mrb-IdentityNew; 2> if the reestablishPDCP is set: 3> if drb-ContinueROHC is included in pdcp-Config: 4> indicate to lower layer that drb-ContinueROHC is configured; 3> if drb-ContinueEHC-DL is included in pdcp-Config: 4> indicate to lower layer that drb-ContinueEHC-DL is configured; 3> re-establish the PDCP entity of this multicast MRB as specified in TS 38.323 [5], clause 5.1.2; 2> else, if the recoverPDCP is set: 3> trigger the PDCP entity of this MRB to perform data recovery as specified in TS 38.323 [5]; 2> if the pdcp-Config is included: 3> reconfigure the PDCP entity in accordance with the received pdcp-Config. 1> else if mrb-Identity value included in the mrb-ToAddModList that is not part of the UE configuration (multicast MRB establishment including the case when full configuration option is used): 2> establish a PDCP entity and configure it in accordance with the received pdcp-Config; 2> if the multicast MRB was configured with the same tmgi prior to receiving this reconfiguration message: 3> associate the established multicast MRB with the corresponding tmgi; 2> else: 3> indicate the establishment of the multicast MRB(s) and the tmgi of the established multicast MRB(s) to upper layers; 2> if an SDAP entity with the received tmgi does not exist: 3> establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1; NOTE 1: When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. It does that e.g. by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity. NOTE 2: In this specification, UE configuration refers to the parameters configured by NR RRC unless otherwise stated. Annex B: TP for retaining existing style with required modifications 5.3.5.6.7 Multicast MRB addition/modification The UE shall: 1> for each mrb-Identity value included in the mrb-ToAddModList for which mrb-IdentityNew is included (multicast MRB ID change): 2> update the mrb-Identity in the current UE configuration and the mrb-Identity in the mrb-ToAddModList to the value mrb-IdentityNew; 1> for each mrb-Identity value included in the mrb-ToAddModList that is not part of the current UE configuration (multicast MRB establishment including the case when full configuration option is used): 2> establish a PDCP entity and configure it in accordance with the received pdcp-Config; 2> if the multicast MRB was configured with the same tmgi prior to receiving this reconfiguration message: 3> associate the established multicast MRB with the corresponding tmgi; 2> else: 3> indicate the establishment of the multicast MRB(s) and the tmgi of the established multicast MRB(s) to upper layers; 2> if an SDAP entity with the received tmgi does not exist: 3> establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1; 1> for each mrb-Identity value included in the mrb-ToAddModList that is part of the current UE configuration: 2> if the reestablishPDCP is set: 3> if drb-ContinueROHC is included in pdcp-Config: 4> indicate to lower layer that drb-ContinueROHC is configured; 3> if drb-ContinueEHC-DL is included in pdcp-Config: 4> indicate to lower layer that drb-ContinueEHC-DL is configured; 3> re-establish the PDCP entity of this multicast MRB as specified in TS 38.323 [5], clause 5.1.2; 2> else, if the recoverPDCP is set: 3> trigger the PDCP entity of this MRB to perform data recovery as specified in TS 38.323 [5]; 2> if the pdcp-Config is included: 3> reconfigure the PDCP entity in accordance with the received pdcp-Config. 1> for each mrb-Identity value included in the mrb-ToAddModList that is not part of the current UE configuration (multicast MRB establishment including the case when full configuration option is used): 2> establish a PDCP entity and configure it in accordance with the received pdcp-Config; 2> if the multicast MRB was configured with the same tmgi prior to receiving this reconfiguration message: 3> associate the established multicast MRB with the corresponding tmgi; 2> else: 3> indicate the establishment of the multicast MRB(s) and the tmgi of the established multicast MRB(s) to upper layers; 2> if an SDAP entity with the received tmgi does not exist: 3> establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1; NOTE 1: When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. It does that e.g. by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity. NOTE 2: In this specification, UE configuration refers to the parameters configured by NR RRC unless otherwise stated. |
| ZTE R2-2205632 | - For the deployment scenarios of PDCP SN sync based on SN on NG-U, the MRB ID and PDCP SN length are synced by means of network implementation. - MRB ID is of the same length of DRB, i.e., no need to extend MRB ID length. - One multicast MRB is uniquely identified by MRD ID and the MBS session ID (i.e., TMGI). - Do not support the MRB ID modification on the fly, i.e., MRB ID modification without release/add MRB explicitly. |

Companies (CATT,VIVO,Nokia/Huawei) propose different solutions to correctly capture the agreement in the 38.331 spec. On the other hand, it seems that ZTE proposes to override the RAN2 agreement (i.e. MRB ID can be changed without releasing/adding MRB (delta config)).

Therefore, there are following options to address the MRB ID change issue,

Option 1: TP from CATT (as in R2-2204670)

Option 2: TP from VIVO (as in R2-2204828)

Option 3: TP (Annex A) from Noika, Huawei(as in R2-2205249)

Option 4: TP (Annex B) from Noika, Huawei(as in R2-2205249)

Option 5: Do not support the MRB ID modification on the fly (as in R2-2205632),which means to override the previous agreement(i.e. MRB ID can be changed without releasing/adding MRB (delta config)), i.e.

Companies are then requested to answer the following question,

Question 6: Which option do you prefer to address the MRB ID change issue?

| Company | Preferred option (option1,2,3,4,5) | Comments / justification |
| --- | --- | --- |
| CATT | Option 1 | Option 1 is simplest and can address the MRB ID change issue well. |
| Lenovo | Option 3 | Not sure the note in option 2 is needed. |
| Huawei, HiSilicon | Option 4 (preferred), option 2 or option 3 | Option 1 will still not work in some cases, e.g. if a new MRB is added with an MRB ID which is already used, but will be changed to another one. Out of options 2-4, we prefer option 4 as it does not require MRBs to be provided in a specific order by the network (as in Option 3) and it does not repeat the specifications text unnecessarily (as Option 2). Option 5 is against the agreement and it should not be pursued at this stage. |
| Qualcomm | Either Option 2 or option 3 with edits, further checking needed | For Option 1 will not work in case new MRB is added with and already used MRB ID. For Option 2: NOTE 3 can be moved right after the first new text bullet 2> as that is more relevant there. Appreciate Nokia’s comprehensive paper, however: Option 3: can be confusing on what the "UE configuration" means -- does this mean before starting to process the list, or at any given instant including already processed entries? Option 4: does not seem to work, because the first bullet already updates MRB ID, then how/what does UE compare whether the MRB config existed or not? E.g. taking example 1, after this step old 29 would be 30 and old 30 would be 29. Then how to process second step? Option 5: We disagree to reverting the agreements reached after long discussions. Better to fix the procedural text as needed to achieve the intended behaviour. |
| Samsung | Option 4 | But seems further checking needed |
| Nokia | Option 2,3 or 4 (we are not sure if option 1 works) | Any of the options 2, 3 or 4 is fine for us. |
| OPPO | Option 5 | What is the scenario of changing the MRB ID value? |
| MediaTek | Option1 or 3 |  |
| Spreadtrum | Option 2,3 or 4 |  |

In R2-2204828, it is also proposed that only when UE establishes an SDAP for a TMGI, UE informs the establishment of user plane resources for the TMGI.

| TDoc | Proposals/TP |
| --- | --- |
| VIVO R2-2204828 | Proposal 3: Only when UE establishes an SDAP for a TMGI, UE informs the establishment of user plane resources for the TMGI. #### Annex: Proposed TP to TS 38.331 …… 5.9.3.3 Broadcast MRB establishment Upon a broadcast MRB establishment, the UE shall: 1> establish a PDCP entity and an RLC entity in accordance with MRB-InfoBroadcast for this broadcast MRB included in the MBSBroadcastConfiguration message and the configuration specified in 9.1.1.7; 1> receive DL-SCH on the cell where the MBSBroadcastConfiguration message was received for the MBS broadcast service for which the broadcast MRB is established and using g-RNTI and mtch-SchedulingInfo (if included) in this message for this MBS broadcast service; 1> configure the physical layer in accordance with the mbs-SessionInfoList, searchSpaceMTCH, pdsch-ConfigMTCH, applicable for the broadcast MRB, as included in the MBSBroadcastConfiguration message; 1> if an SDAP entity with the received tmgi does not exist: 2> establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1. 2> if an SDAP entity with the received tmgi did not exist prior to receiving this reconfiguration: 3> indicate the establishment of the user plane resources for the tmgi to upper layers; |

Companies are then requested to answer the following question,

Question 7: Do you agree P3 and corresponding TP in R2-2204828?

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | Yes |  |
| Lenovo | Yes | It depends on whether SDAP entity is needed for multicast session. |
| Huawei, HiSilicon | Perhaps yes | This would be more aligned with the handling for DRBs/PDU sessions. |
| Qualcomm | ok |  |
| Samsung | Yes |  |
| Nokia | Maybe | No strong view from us. This seems to be correct although difficult to see how UE would behave wrongly with old text either. Note that there’s also a discussion on SDAP handling (Q12 of #034). |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Spreadtrum | Yes |  |
| Apple | Yes |  |

## Other topics

### [H091]Corrections for GroupConfig structure

Currently in the Group-Config type specified for MBS within MAC-CellGroupConfig IE, two different types of identifiers are used to denote the identity of the particular group config. The type of identifier that is used depends on whether G-RNTI or G-CS-RNTI is configured within Group-Config IE (which in turn depends on whether the Group-Config is within g-RNTI-ConfigToAddModList-r17 or within g-CS-RNTI-ConfigToAddModList-r17).

In R2-2206123, company thinks that this results in using a CHOICE structure for choosing the identifier which is then used for managing the lists of G-RNTI and G-CS-RNTI configurations. This is rather an unusual practice and it also is not required to have two different identifiers for these two listst as a single identifier will equally serve the purpose.so it is proposed that:

- The Group-Config structure is modified so that it relies on a single identifier for list management.

- Group-Config type is renamed as MBS-RNTI-SpecificConfig to better express its purpose.

The text proposals are as below,

| TDoc | Proposals |
| --- | --- |
| Huawei R2-2206123 | -MAC-CellGroupConfig The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX. MAC-CellGroupConfig information element -- ASN1START -- TAG-MAC-CELLGROUPCONFIG-START MAC-CellGroupConfig ::= SEQUENCE { drx-Config SetupRelease { DRX-Config } OPTIONAL, -- Need M schedulingRequestConfig SchedulingRequestConfig OPTIONAL, -- Need M bsr-Config BSR-Config OPTIONAL, -- Need M tag-Config TAG-Config OPTIONAL, -- Need M phr-Config SetupRelease { PHR-Config } OPTIONAL, -- Need M skipUplinkTxDynamic BOOLEAN, ..., [[ csi-Mask BOOLEAN OPTIONAL, -- Need M dataInactivityTimer SetupRelease { DataInactivityTimer } OPTIONAL -- Cond MCG-Only ]], [[ usePreBSR-r16 ENUMERATED {true} OPTIONAL, -- Need R schedulingRequestID-LBT-SCell-r16 SchedulingRequestId OPTIONAL, -- Need R lch-BasedPrioritization-r16 ENUMERATED {enabled} OPTIONAL, -- Need R schedulingRequestID-BFR-SCell-r16 SchedulingRequestId OPTIONAL, -- Need R drx-ConfigSecondaryGroup-r16 SetupRelease { DRX-ConfigSecondaryGroup } OPTIONAL -- Need M ]], [[ enhancedSkipUplinkTxDynamic-r16 ENUMERATED {true} OPTIONAL, -- Need R enhancedSkipUplinkTxConfigured-r16 ENUMERATED {true} OPTIONAL -- Need R ]], [[ intraCG-Prioritization-r17 ENUMERATED {enabled} OPTIONAL, -- Cond LCH-PrioWithReTxTimer drx-ConfigSL-r17 SetupRelease { DRX-ConfigSL } OPTIONAL, -- Cond Mode1AndDRX-Only drx-ConfigExt-v1700 SetupRelease { DRX-ConfigExt-v1700 } OPTIONAL, -- Cond DRX schedulingRequestID-BFR-r17 SchedulingRequestId OPTIONAL, -- Need R schedulingRequestID-BFR2-r17 SchedulingRequestId OPTIONAL, -- Need R schedulingRequestConfig-v1700 SchedulingRequestConfig-v1700 OPTIONAL, -- Need M --Editor’s note: extension done according to A4.3.5 offsetThresholdTA ENUMERATED{ms05, ms1, ms2, ms3, ms4, ms5, ms6 ,ms7, ms8, ms9, ms10, ms11, ms12, ms13, ms14, ms15} OPTIONAL, -- Need R --Editor’s note: may need to add here parameter TimingAdvanceSR g-RNTI-ConfigToAddModList-r17 SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17 OPTIONAL, -- Need N g-RNTI-ConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-GroupConfigId-r17 OPTIONAL, -- Need N g-CS-RNTI-ConfigToAddModList-r17 SEQUENCE (SIZE (1..maxG-CS-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17 OPTIONAL, -- Need N g-CS-RNTI-ConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxG-CS-RNTI-r17)) OF MBS-GroupConfigId-r17 OPTIONAL, -- Need N allowCSI-SRS-Tx-MulticastDRX-Active-r17 BOOLEAN ]] } DataInactivityTimer ::= ENUMERATED {s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180} MBS-RNTI-SpecificConfig-r17 ::= SEQUENCE { mbs-RNTI-SpecificConfigId-r17 MBS-RNTI-SpecificConfigId-r17; groupCommon-RNTI CHOICE { g-RNTI RNTI-Value, g-CS-RNTI RNTI-Value }, drx-ConfigPTM-r17 SetupRelease { DRX-ConfigPTM-r17 } OPTIONAL, -- Need M harq-FeedbackEnablerMulticast-r17 ENUMERATED {dci-enabler, enabled} OPTIONAL, -- Need S harq-FeedbackOptionMulticast-r17 ENUMERATED {ack-nack, nack-only} OPTIONAL, -- Cond HARQFeedback pdsch-AggregationFactorMulticast-r17 ENUMERATED {n2, n4, n8} OPTIONAL -- Cond G-RNTI } MBS-RNTI-SpecificConfigId ::= INTEGER (0..maxG-RNTI-1-r17) -- TAG-MAC-CELLGROUPCONFIG-STOP -- ASN1STOP mbs-RNTI-SpecificConfigId An identifier of the RNTI specific configuration for MBS multicast. |

Companies are then requested to answer the following question,

Question 8: Do you agree the change proposed in R2-2206123?

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | Yes |  |
| Lenovo | Yes |  |
| Huawei, HiSilicon | Yes (proponent) |  |
| Qualcomm | Ok with intent. See comments | Intent of using a single IE for the list elements is ok, but the TP has several issues that needs to be fixed. E.g. what is the need of the following CHOICE? groupCommon-RNTI CHOICE { g-RNTI RNTI-Value, g-CS-RNTI RNTI-Value }, Where is MBS-GroupConfigId-r17 defined? Etc. |
| Samsung | Yes with correction | Replace MBS-GroupConfigId-r17 by MBS-RNTI-SpecificConfigId Also note that maxG-CS-RNTI-r17 is 8 and maxG-RNTI-r17 is 16 |
| OPPO | Yes |  |
| MediaTek | Yes | But we should wait for RAN1’s conclusion on the final value of maxG-RNTI-r17/ maxG-CS-RNTI-r17 |
| Spreadtrum | Yes |  |
| Apple | Yes |  |

### [H001, H005, Z608, C005] Discussion on multicast MRB and DRB in RRC

In R2-2205626, several proposals are proposed on how to treat multicast MRB in

- connection management

- priority of multicast MRB in MII.

| TDoc | Proposals |
| --- | --- |
| ZTE R2-2205626 | Observation 1 Based on the resource model defined by SA2, there will always be a DRB if there is MRB configured by network. Proposal 1 The conditionalReconfiguration for CHO or CPA is configured regardless of the existence of multicast MRB. Proposal 2 RRC Connection suspension can be issued regardless of the existence of multicast MRB. Proposal 3 The prioritization over unicast bearer applies to multicast MRB too, that is, if indicated by UE, the reception of broadcast services is prioritized compared to unicast bearer and also multicast MRB. |

For P1 in R2-2205626, regarding the co-existence of CHO and MRB it has been agreed in RAN2#116bis meeting as following,

RAN2 will not do additional work to support CHO for UEs for which MRB is configured in R17.

In rapporteur’s understanding, whether there are spec impacts is not clear if P1 is agreed.

Companies are then requested to answer the following question,

Question 9: Do you agree P1 in R2-2205626?

Proposal 1	The conditionalReconfiguration for CHO or CPA is configured regardless of the existence of multicast MRB.

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | No | RAN2 has agreed that RAN2 will no specify anything to support CHO for UEs for which MRB is configured in R17 |
| Lenovo | Yes | DRB will be always present |
| Huawei, HiSilicon | Yes | Agree with the proposal, but the intention in the Tdoc seemed to be the opposite. In any case, RAN2 agreed not to modify CHO for the sake of MBS, but there is no reason to forbid it artificially. The change is already captured in the rapporteur CR. |
| Qualcomm | Yes | Even if UE is configured with Muticast MRB, NW can still configure CHO, but no specific enhancement needed for Multicast MRB handling during CHO. |
| Samsung | No | There seems no spec impact |
| Nokia | No | Handling of this coordination is up to NW |
| OPPO | No | If CHO is configured with MRB, it means there will be no MRB in the target. But, the service continuity should be ensured also. |
| MediaTek | Probably No |  |
| Spreadtrum | Yes | The CHO can be configured at least for DRB. |
| Apple | Yes |  |

Question 10: Do you agree P2 in R2-2205626?

Proposal 2	RRC Connection suspension can be issued regardless of the existence of multicast MRB.

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | Yes |  |
| Lenovo | Yes | DRB will be always present |
| Huawei, HiSilicon | Yes | Agree with the proposal as such, but the intention in the Tdoc seemed the opposite. In any case, the current specs are OK. |
| Qualcomm | No | We note that Observation 1 in the paper is not correct. There can be MRB without any DRB. Then, if there is configured MRB and UE actively receiving Multicast data, gNB should not release UE into INACTIVE state. |
| Samsung | Yes |  |
| Nokia | No | We don’t see that anything needs to be captured on this one. This can be left up to NW implementation |
| OPPO | Yes |  |
| MediaTek | No | Agree with Qualcomm |
| Spreadtrum | Yes |  |
| Apple | Yes |  |

In MBS interest indication message, priority between the reception of all listed MBMS frequencies and the reception of any unicast bearer  can be included,according to the RAN2 agreement,

The UE reports the following MBS interest information (as LTE SC-PTM):

- MBS frequency list (20/24)

- priority between the reception of all listed MBMS frequencies and the reception of any unicast bearer (23/24)

- TMGI list (24/24)

For P3 in R2-2205626,it is proposed to extend the priority to cover multicast MRB too.

Question 11: Do you agree P3 in R2-2205626?

Proposal 3	The prioritization over unicast bearer applies to multicast MRB too, that is, if indicated by UE, the reception of broadcast services is prioritized compared to unicast bearer and also multicast MRB.

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | No | UE preference on the priority of multicast reception may be different from unicast, we think it is not suitable to treat multicast reception same as unicast reception. |
| Lenovo | Yes |  |
| Huawei, HiSilicon | Yes | We agree with the proposal and it is already captured like this in the specifications in some places, but this addition is missing from some places so the rapp CR will have to be updated to consider this. |
| Samsung | No | Multicast is targeted for critical/low latency services. UE may have interest in broadcast and express priority over unicast, but it does not mean priority over multicast. |
| Nokia | Yes | Otherwise we see no way for NW to know if UE prefers to receive multicast over broadcast. |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Spreadtrum | Yes |  |
| Apple | Yes |  |

### LS to CT1 on AS-NAS layer interactions for MBS

In R2-2206124, it is proposed to send LS to CT1 to confirm whether the AS to NAS layer indications as mentioned below are needed and/or whether there are any other events concerning MBS at AS layer which the NAS layer should be informed about.

| TDoc | Proposals |
| --- | --- |
| Huawei R2-2206124 | RAN2 is currently reviewing the procedures in RAN2 specifications related to MBS and one of the doubts that arose during the process was related to whether some of the information should be passed to NAS layers or not. Currently, according to RAN2 specifications, AS layer will indicate the following events related to MBS session: - When the UE receives a Paging message including a TMGI for a multicast MBS sessions which the UE has previously joined, the UE will forward the TMGI to upper layers (for both UE in RRC_IDLE and RRC_INACTIVE states). - When MRB(s) is/are established for either MBS broadcast or MBS multicast, the UE will notify upper layers about this (and include TMGI to identify the session the notification concerns). - When MRB(s) is/are released for either MBS broadcast or MBS multicast, the UE will notify upper layers about this (and include TMGI to identify the session the notification concerns). RAN2 respectfully asks CT1 to confirm whether the AS to NAS layer indications as mentioned above are needed and/or whether there are any other events concerning MBS at AS layer which the NAS layer should be informed about. |

Companies are then requested to answer the following questions,

Question 12: Do you agree to send LS to CT1 to confirm the AS-NAS layer interactions for MBS?

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | Yes | It is beneficial to confirm with CT1 |
| Lenovo |  | We are fine for sending a LS to CT1. MRB v.s. MBS session needs to be clarified, e.g. the first MRB of a MBS session is established, the AS will notify the upper layer with TMGI. It also depends on the discussion with SDAP issue. |
| Huawei, HiSilicon | Yes (proponent) | It would be good to clarify with CT1 if there are doubts. Also OK to include the question suggested by Lenovo. |
| Qualcomm |  | We thought it was clear in RAN2, but ok to confirm with CT1 if majority prefers. |
| Samsung | Yes | Ok to check with a LS |
| Nokia | No | We see no need. There is no limit of handling any information from AS in NAS regardless if it is forwarded in procedural text. |
| OPPO | Yes |  |
| MediaTek | Yes | Fine to check with CT1 |
| Spreadtrum | Yes | It is ok to send an LS to CT1. |
| Apple | Yes |  |

### [V533] Correction on Logical Channel Setup for PTM Transmission

In R2-2204830, company thinks that logicalChannelIdentityExt is only applied to PTM according to RAN2 agreement. So it is proposed to update the field description of logicalChannelIdentityExt as below,

| TDoc | Proposals |
| --- | --- |
| vivo R2-2204830 | Proposal 1: Update the field description of logicalChannelIdentityExt to “Extended logical channel ID used commonly for the MAC logical channel and for the RLC bearer for PTM reception. If this field is configured, the UE shall ignore logicalChannelIdentity”. Proposal 2: If proposal 1 is agreed, RAN2 adopts the TP in the Annex. |

Companies are then requested to answer the following question,

Question 13: Do you agree the change proposed in R2-2204830?

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | Yes |  |
| Lenovo | Yes |  |
| Huawei, HiSilicon | Yes | The intention was misunderstood by the RRC CR rapporteur when replying to this RIL in R2-2206120. We agree with this clarification. |
| Qualcomm | ok |  |
| Samsung | Yes |  |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Spreadtrum | Yes |  |
| Apple | Yes |  |

### Miscellaneous correction to TS 38331

In R2-2205627, miscellaneous corrections to TS 38331 are proposed,

| TDoc | Proposals |
| --- | --- |
| ZTE R2-2205627 | Change 1: In 5.3.5.2, and 5.3.8.1, the initiation of RRC reconfiguration and release can be done regardless of configured multicast MRB. Change 2: In 5.3.5.6.7, 5.9.3.3, 5.9.3.4 SDAP related procedure was deleted. Change 3: In 5.9.4, the priority between broadcast and unicast/multicast was clarified. The prioritization is applied to both unicast and multicast. change 4: In 6.3.2, to achieve better power efficiency and scheduling flexibility, put the parameter allowCSI-SRS-Tx-MulticastDRX-Active in Group-Config instead of per UE config. change 5: in the MRB-Identity, the length of MRB ID is changed to 5bits, in the RadioBearerConfig, TMGI is indicated in the the MRB release; in the RLC-BearerConfig, TMGI is added in the served MRB. |

Companies are then requested to answer the following questions,

Question 14: Do you agree the corrections proposed in R2-2205627?

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | Chang 1:No Chang 2:No Chang 3:No Chang 4:No Chang 5:No | Change 1: Disagree. We think the current text is correct. Change 2: Disagree. SDAP entity is needed to handle MBS data according to 37.324 Change 3: Disagree, same comments as Q11. change 4: Disagree, we think it is not motivated to make parameter allowCSI-SRS-Tx-MulticastDRX-Active on a per multicast DRX basis. change 5: Disagree.it overrides the RAN2 agreement(“Extend MRB ID space beyond current 32 limit and up to 512.”) |
| Huawei, HiSilicon | See next column | Change 1: Disagree. The current specifications is OK. It is true that in most of the cases DRB will be configured, but MRB-only configuration is also valid, e.g. during congestion. Change 2: Disagree, SDAP entity is needed for MBS. Change 3: Agree as commented in Q11. Change 4: Disagree, we think it is OK the way it is currently captured. Change 5: Disagree, it contradicts the agreement from the previous meeting. |
| Qualcomm | No | The CR has several issues, but mostly covered by other papers in any case. Also related to answers in Q9/Q11. In general, agree with CATT comments. |
| Samsung | No | Disagree with changes 1 to 5 |
| Nokia | No to all | Similar comments to CATT |
| OPPO | No |  |
| MediaTek |  | Agree with change 3 as comment to Q11 Disagree with change 1 2 4 5 |
| Spreadtrum | No to all |  |

## 38.304 corrections

### Scenario on setting frequencies to be of the lowest priority

In RAN2#115e meeting, the following agreement was reached,

- The UE may consider cell reselection candidate frequencies at which it cannot receive the MBS service to be of the lowest priority during the MBS session, as LTE SC-PTM. (25/25)

And the agreement is captured in 38.304 CR as below,

5.2.4 Cell Reselection evaluation process 5.2.4.1 Reselection priorities handling …… <omitted> If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s), the UE may consider cell reselection candidate frequencies at which it can not receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as the SIB20 is provided by the cell on the MBS frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell.

During previous CR update discussions, some companies mentioned that it is still not clear on what scenarios UE should consider frequencies to be of the lowest priority during the MBS session.

Since this function mainly reuses the LTE mechanism, so we can take the corresponding text in 36.304 as a reference,

//TS 36.304 5.2.4 Cell Reselection evaluation process 5.2.4.1 Reselection priorities handling …… If the UE is capable either of MBMS Service Continuity or of SC-PTM reception and is receiving or interested to receive an MBMS service provided on a downlink only MBMS frequency, on a frequency used by dedicated MBMS cells, on a frequency used by FeMBMS/Unicast-mixed cells as defined in TS 36.300 [2], or on a frequency belonging to PLMN different from its registered PLMN, the UE may consider cell reselection candidate frequencies at which it can not receive the MBMS service to be of the lowest priority during the MBMS session TS 36.300 [2], as long as the above mentioned condition 1) is fulfilled for the cell on the MBMS frequency which the UE monitors or this cell broadcasts SIB1-MBMS and as long as the above mentioned condition 2) is fulfilled for the serving cell. NOTE 2: Example scenarios in which the previous down-prioritisation may be needed concerns the cases where camping is not possible, while the UE can only receive this MBMS frequency when camping on a subset of cell reselection candidate frequencies, e.g. the MBMS frequency is a downlink only carrier, the MBMS frequency is used by dedicated MBMS cells, the MBMS frequency is used by FeMBMS/Unicast-mixed cells TS 36.300 [2], or the MBMS frequency belongs to a PLMN different from UE's registered PLMN.

It seems also necessary to add similar clarification in 38.304. Therefore, in R2-2204668, it is proposed to add NOTE to clarify the scenarios on setting frequencies to be of the lowest priority during the MBS session,

| TDoc | Proposals |
| --- | --- |
| CATT,CBN R2-2204668 | 5.2.4 Cell Reselection evaluation process 5.2.4.1 Reselection priorities handling …… If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s), the UE may consider cell reselection candidate frequencies at which it can not receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as the SIB20 is provided by the cell on the MBS frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell. NOTE: Example scenarios in which the previous down-prioritisation may be needed concerns the cases where camping is not possible, while the UE can only receive the MBS broadcast service when camping on a subset of the cell reselection candidate frequencies, e.g. the MBS broadcast frequency is a downlink only carrier, or the MBS broadcast frequency belongs to a PLMN different from UE's registered PLMN. |

Companies are then requested to answer the following question,

Question 15: Do you agree the change proposed in R2-2204668?

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | Yes | We think it is essential to clarify it, or for the intended scenario on setting frequencies to be of the lowest priority during the MBS session, it may not implemented correctly at UE side. |
| Lenovo | Yes |  |
| Huawei, HiSilicon | Yes | We think such clarification is useful, similarly as currently captured in 36.304. |
| Qualcomm | No | In our view, this detailed note is not needed. |
| Samsung | No strong view |  |
| Nokia | No | Note does not really bring any clarity and actually we do not see really any added value to existing text with the note. Additionally note uses some odd terminology e.g. “down prioritizing” etc… |
| OPPO | Yes |  |
| MediaTek | Yes | To align with LTE |
| Spreadtrum | Yes | We think the clarification is helpful. |
| Apple |  | We are not sure whether the DL only frequency for MBS broadcast service is supported in this release. |

### Correction to frequency prioritization

| TDoc | Proposals |
| --- | --- |
| Ericsson R2-2205745 | Proposal 1: Clarify (e.g. in a NOTE) that the UE no longer considers the frequency of the highest priority when the MCCH does not include the MBS broadcast service the UE is interested in. Proposal 2: Clarify in NOTE 7 what is up to UE implementation concerning frequency prioritization. Observation 1: Stop prioritizing the MBS frequency does not resolve the possible congregation on the MBS frequency. Proposal 3: Clarify in a NOTE that when the UE stops prioritizing the MBS frequency, the UE triggers inter-frequency measurements for cell reselection to another frequency. |

For P1 and P3 in R2-2205745, they are enhancements on stopping frequency prioritization. The rapporteur understands that spec impact on stopping frequency prioritization has been extensively discussed during the WI phase, and there is no additional spec impact identified according to below agreements,

- When the conditions for frequency prioritization are no longer met, the UE should stop prioritizing the frequency of this cell (e.g. when the cell reselected by the UE due to frequency prioritization for MBS stops providing SIBx etc.). FFS whether there is additional TS impact.

- There is no additional TS impact on stopping frequency prioritization.

Companies are then requested to answer the following question,

Question 16: Do you agree that P1 in R2-2205745?

Proposal 1: Clarify (e.g. in a NOTE) that the UE no longer considers the frequency of the highest priority when the MCCH does not include the MBS broadcast service the UE is interested in.

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | No | We think there is no additional spec impact on stopping frequency prioritization according to previous discussions and RAN2 agreements |
| Lenovo | No | Agree with rapporteur’s view that there is no spec impact identified. |
| Huawei, HiSilicon | No | The proposal is incorrect. Even when the session is currently not provided (i.e. not included in MCCH), the UE should still camp on the frequency and monitor for session start notification. |
| Qualcomm | Yes |  |
| Samsung | No |  |
| Nokia | No | same view with CATT |
| OPPO | Yes | Agree with Huawei. |
| MediaTek | No |  |
| Spreadtrum | No | same view with CATT |
| Apple | No |  |

Question 17: Do you agree that P3 in R2-2205745?

Proposal 3: Clarify in a NOTE that when the UE stops prioritizing the MBS frequency, the UE triggers inter-frequency measurements for cell reselection to another frequency.

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | No | Same comments as Q16 |
| Lenovo | No | Agree with rapporteur’s view that there is no spec impact identified. |
| Huawei, HiSilicon | No | It is unnecessary in our opinion. Higher priority frequencies will be measured anyway. |
| Qualcomm | No | It is natural UE behaviour. |
| Samsung | No |  |
| Nokia | No | We should not have unnecessary notes. There will be requirements defined in RAN4 which will dictate UE requirements – how UE achieves those is up to UE implementation. |
| OPPO | No |  |
| MediaTek | No |  |
| Spreadtrum | No |  |
| Apple | No |  |

For P2 in R2-2205745, it is proposed to clarify the NOTE 7 in 38.304 further,

5.2.4 Cell Reselection evaluation process 5.2.4.1 Reselection priorities handling …… If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled: 1) The cell reselected by the UE due to frequency prioritization for MBS is providing SIB20; 2) Either: - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.346 [20], or - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service. NOTE 7: It is up to UE implementation how to use information in USD to determine whether/how to do the frequency prioritization for specific frequency/frequencies included in USD.

NOTE 7 was added according to the RAN2#116bis-e agreement as below,

- UE can prioritize the frequency indicated in USD when SIBy is provided in the cell but does not provide the frequency mapping for the concerned service.

- It is up to UE implementation how to use information in USD (e.g. with other explicit knowledge) to determine whether to (or how to) do the frequency prioritization for specific frequency/frequencies in USD.

The rapporteur understands that NOTE 7 is aligned well with the agreement already.

Companies are then requested to answer the following question,

Question 18: Do you agree to clarify NOTE 7 further, as proposed in P2 of R2-2205745?

Proposal 2: Clarify in NOTE 7 what is up to UE implementation concerning frequency prioritization.

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| CATT | No strong view | We are not sure if there is something to clarify further, but we follow the majority view. |
| Lenovo | No | Agree with rapporteur’s view that the existing NOTE 7 is aligned well with the agreement already |
| Huawei, HiSilicon | Yes | We agree the current text is rather unclear. We should clarify that it is up to UE implementation to decide which frequency to prioritize in case USD provides more than one frequency. |
| Qualcomm | No |  |
| Samsung | No |  |
| Nokia | No | We did not see actual TP in the paper. What is proposed? As long as there is no clear proposal we cannot support. |
| OPPO | No |  |
| MediaTek | No |  |
| Spreadtrum | No |  |
| Apple | No |  |

## Other issues

Question 19: Any other CP open issues?

| Company | Comments / justification |
| --- | --- |

# Summary

Based on the discussion above, we propose:

# Reference

[1] R2-2204669	[C006] Correction to UE Behavior on Group Paging Handling	CATT	CR	Rel-17	38.331	17.0.0	2991	-	F	NR_MBS-Core

[2] R2-2204827	[V500] Clarification on Group Paging for INACTIVE UE	vivo	discussion	Rel-17	NR_MBS-Core

[3] R2-2205749	Multicast session start and Paging	Ericsson	discussion	Rel-17	NR_MBS-Core

[4] R2-2204670	[C001] Modificaitons towards the MRB ID Change Procedure	CATT	CR	Rel-17	38.331	17.0.0	2992	-	F	NR_MBS-Core

[5] R2-2204828	[V503][V504][V508] Correction on MRB Handling	vivo	discussion	Rel-17	NR_MBS-Core

[6] R2-2205249	[V503][H002] MRB identity change procedural text issue	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_MBS-Core	Late

[7] R2-2205632	[C001, H002, v503, Z609] MRB ID scope and its modification on the fly	ZTE, Sanechips	discussion	Rel-17	NR_MBS-Core

[8] R2-2206123	Corrections for GroupConfig structure (RIL: H091)	Huawei, HiSilicon	draftCR	Rel-17	38.331	17.0.0	F	NR_MBS-Core

[9] R2-2205626	[H001, H005, Z608, C005] Discussion on multicast MRB and DRB in RRC	ZTE, Sanechips	discussion	Rel-17	NR_MBS-Core

[10] R2-2206124	Draft LS on AS-NAS layer interactions for MBS	Huawei, HiSilicon	LS out	Rel-17	NR_MBS-Core

[11] R2-2204830	[V533] Correction on Logical Channel Setup for PTM Transmission	vivo	discussion	Rel-17	NR_MBS-Core

[12] R2-2205627	Miscellaneous correction to TS 38331	ZTE, Sanechips	CR	Rel-17	38.331	17.0.0	3106	-	F	NR_MBS-Core

[13] R2-2204668	Correction to 38.304 for MBS	CATT, CBN	CR	Rel-17	38.304	17.0.0	0237	-	F	NR_MBS-Core

[14] R2-2205745	Frequency prioritization	Ericsson	discussion	Rel-17	NR_MBS-Core