3GPP TSG-RAN WG2 Meeting #118 electronic	R2-2206506

Online, May 9 – 20th, 2022

Agenda Item:	6.10.1.1

Source: 	Thales

Title:  	Summary of [AT118-e][119][NTN] Coarse UE location info (Thales)

Document for:	Discussion and Decision

# Introduction

This document aims to summarize the following discussion.

[AT118-e][119][NTN] Coarse UE location info (Thales)

Scope: Discuss the coarse UE location information format (e.g. whether to use the most significant bits of GNSS longitude/latitude or also consider the proposals in R2-2205574) and the reporting mechanism (e.g. reuse the mechanism for reporting commonLocationInfo)

Intended outcome: Summary of the offline discussion with list of proposals

Deadline (for companies' feedback):  Thursday 2022-05-19 08:00 UTC

Deadline (for rapporteur's summary in R2-2206506):  Thursday 2022-05-19 10:00 UTC

Proposals marked "for agreement" in R2-2206506 not challenged until Thursday 2022-05-19 20:00 UTC will be declared as agreed via email by the session chair.

This offline discussion aims at defining the coarse location format as outcome of RAN2#118-e.

# Discussion

The following proposals on the coarse location format in [1] are submitted to the discussion.

## 2.1 Which definition to start from ?

Proposal 1	When defining a coarse UE location representation format, use the definition of EllipsoidPoinWithAltitude in TS 37.355 and round the coordinates to fewer bits to achieve a suitable accuracy.

Question 4.5.1: Do companies agree with the proposal above ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |

## 2.4 Altitude ?

Proposal 2	Make it configurable whether an indication of the altitude should be included in a report of a coarse UE location.

Question 4.5.2: Do companies agree with the proposal above ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |

## 2.3 Compensation scheme for different latitudes

Proposal 3	Compensate for the different longitude density at different latitudes, so that a reported coarse UE location cannot be too accurate.

Question 4.5.3: Do companies agree with the proposal above ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |

## 2.4 Representation format of coarse UE location information

| Latitude range in degrees () | Latitude range represented with 11 bits | Number of bits representing the longitude | Longitude range | Number of steps a “latitude circle” is divided into |
| --- | --- | --- | --- | --- |
| 0 - 59.978022 | 0 - 1364 | 13 | 4095 | 8191 |
| 59.978022 - 75.538462 | 1365 - 1718 | 12 | 2047 | 4095 |
| 75.538462 - 82.835165 | 1719 - 1884 | 11 | 1023 | 2047 |
| 82.835165 - 86.439560 | 1885 - 1966 | 10 | 511 | 1023 |
| 86.439560 - 88.197802 | 1967 - 2006 | 9 | 255 | 511 |
| 88.197802 - 89.120879 | 2007 - 2027 | 8 | 127 | 255 |
| 89.120879 - 89.560440 | 2028 - 2037 | 7 | 63 | 127 |
| 89.560440 - 89.824176 | 2038 - 2043 | 6 | 31 | 63 |
| 89.824176 - 89.912088 | 2044 - 2045 | 5 | 15 | 31 |
| 89.912088 - 89.956044 | 2046 | 4 | 7 | 15 |
| 89.956044 - 90 | 2047 | 3 | 3 | 7 |

Proposal 4	Adopt the above table for the definition of the representation format of a coarse UE location.

Question 4.5.4: Do companies agree with the proposal above ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |

## 2.5 fields of coarse UE location information

Proposal 5	For representing a coarse UE location, use 12 bits for the latitude (where one bit indicates “north”/”south”), 3-13 bits for the longitude (where the number of bits depends on the latitude), and 4 bits for the optional altitude (where one bit is used to indicate “height”/”depth”).

Question 4.5.5: Do companies agree with the proposal above ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |

## 2.6 ASN.1 code

Two options are proposed for ASN.1 code of the coarse UE location information

Option 1: “CoarseEllipsoidPointWithOptionalAltitude”

CoarseEllipsoidPointWithOptionalAltitude ::= SEQUENCE {

latitudeSign                   ENUMERATED {north, south},

degreesLatitude               INTEGER (0..2047),          -- 11 bit field

degreesLongitude              CHOICE {

longForLatRange1              INTEGER (-4095..4095),  -- For latitudes 0-1364. 13 bit field.

longForLatRange2              INTEGER (-2047..2047),  -- For latitudes 1365-1718. 12 bit field.

longForLatRange3              INTEGER (-1023..1023),  -- For latitudes 1719-1884. 11 bit field.

longForLatRange4              INTEGER (-511..511),     -- For latitudes 1885-1966. 10 bit field.

longForLatRange5              INTEGER (-255..255),     -- For latitudes 1967-2006. 9 bit field.

longForLatRange6              INTEGER (-127..127),     -- For latitudes 2007-2027. 8 bit field.

longForLatRange7              INTEGER (-63..63),       -- For latitudes 2028-2037. 7 bit field.

longForLatRange8              INTEGER (-31..31),       -- For latitudes 2038-2043. 6 bit field.

longForLatRange9              INTEGER (-15..15),       -- For latitudes 2044-2045. 5 bit field.

longForLatRange10             INTEGER (-7..7),         -- For latitude 2046. 4 bit field.

longForLatRange11             INTEGER (-3..3)          -- For latitude 2047. 3 bit field.

},

altitudeDirection			ENUMERATED {height, depth}    OPTIONAL,

altitude					INTEGER (0..7)    OPTIONAL  -- 3 bit field. Step size: 4.096 km.

}

Option 2: ASN.1 code is divided into two IEs:

- one with altitude included (tentatively denoted as “CoarseEllipsoidPointWithAltitude”)

- and one without the altitude included (tentatively denoted as “CoarseEllipsoidPoint”).

CoarseEllipsoidPointWithAltitude ::= SEQUENCE {

latitudeSign				   ENUMERATED {north, south},

degreesLatitude               INTEGER (0..2047),          -- 11 bit field

degreesLongitude              CHOICE {

longForLatRange1               INTEGER (-4095..4095),  -- For latitudes 0-1364. 13 bit field.

longForLatRange2               INTEGER (-2047..2047),  -- For latitudes 1365-1718. 12 bit field.

longForLatRange3               INTEGER (-1023..1023),  -- For latitudes 1719-1884. 11 bit field.

longForLatRange4               INTEGER (-511..511),     -- For latitudes 1885-1966. 10 bit field.

longForLatRange5               INTEGER (-255..255),     -- For latitudes 1967-2006. 9 bit field.

longForLatRange6               INTEGER (-127..127),     -- For latitudes 2007-2027. 8 bit field.

longForLatRange7               INTEGER (-63..63),       -- For latitudes 2028-2037. 7 bit field.

longForLatRange8               INTEGER (-31..31),       -- For latitudes 2038-2043. 6 bit field.

longForLatRange9               INTEGER (-15..15),       -- For latitudes 2044-2045. 5 bit field.

longForLatRange10              INTEGER (-7..7),         -- For latitude 2046. 4 bit field.

longForLatRange11              INTEGER (-3..3)          -- For latitude 2047. 3 bit field.

},

altitudeDirection			ENUMERATED {height, depth},

altitude					INTEGER (0..7)               -- 3 bit field. Step size: 4.096 km.

}

CoarseEllipsoidPoint ::= SEQUENCE {

latitudeSign               ENUMERATED {north, south},

degreesLatitude           INTEGER (0..2047),          -- 11 bit field

degreesLongitude          CHOICE {

longForLatRange1          INTEGER (-4095..4095),  -- For latitudes 0-1364. 13 bit field.

longForLatRange2          INTEGER (-2047..2047),  -- For latitudes 1365-1718. 12 bit field.

longForLatRange3          INTEGER (-1023..1023),  -- For latitudes 1719-1884. 11 bit field.

longForLatRange4          INTEGER (-511..511),     -- For latitudes 1885-1966. 10 bit field.

longForLatRange5          INTEGER (-255..255),     -- For latitudes 1967-2006. 9 bit field.

longForLatRange6          INTEGER (-127..127),     -- For latitudes 2007-2027. 8 bit field.

longForLatRange7          INTEGER (-63..63),       -- For latitudes 2028-2037. 7 bit field.

longForLatRange8          INTEGER (-31..31),       -- For latitudes 2038-2043. 6 bit field.

longForLatRange9          INTEGER (-15..15),       -- For latitudes 2044-2045. 5 bit field.

longForLatRange10         INTEGER (-7..7),         -- For latitude 2046. 4 bit field.

longForLatRange11         INTEGER (-3..3)          -- For latitude 2047. 3 bit field.

}

}

Question 4.5.6: Which option (see above) do companies prefer for ASN.1 code of coarse UE location information ?

•	Option 1: CoarseEllipsoidPointWithOptionalAltitude IE

•	Option 2: CoarseEllipsoidPointWithAltitude IE + CoarseEllipsoidPoint IE

| Company | Option 1 / Option 2 | Comments/Suggestions |
| --- | --- | --- |

## 2.7 Reporting mechanism

Question 4.5.7: Which reporting mechanisms do companies prefer

- Option 1: After AS security is established, gNB can obtain a GNSS-based location information from the UE using existing signalling method, i.e., by configuring includeCommonLocationInfo in the corresponding reportConfig

- Option 2: other

| Company | Option 1 / Option 2 | Comments/Suggestions |
| --- | --- | --- |

# 3. Summary and Proposals

# 4. References

- R2-2205574	Coarse location format	Ericsson

# Contact information

| Company | Delegate contact |
| --- | --- |
| COMPANY_NAME | NAME (email@address.com) |
| Thales | nicolas.chuberre@thalesaleniaspace.com |