3GPP TSG-RAN WG2 #119-e	R2-22xxxxx

Electronic Meeting, Aug 17th – 29th, 2022

Agenda Item:	6.0.2

Source:	Ericsson

Title:	[AT118-e][013][NR17] RRC I (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

The following document summarizes the following email discussion:

[AT119-e][013][NR17] RRC I (Ericsson)

Scope: Treat R2-2207776, R2-2208654, R2-2207267, R2-2207002, R2-2207006, R2-2207013, R2-2208141 (if available), and R2-2208133 (MINT in [6.24.3])

Determine agreeable parts, For agreeable parts, agree CRs.

Intended outcome: Report, Agreed CRs, LS out if applicable

Deadline: Schedule 1

A first round with Deadline for comments W1 Friday Aug 19th 1400 UTC to settle scope what is agreeable etc.

A Final round with Final deadline W2 Thursday Aug 25th 1200 UTC to settle details / agree CRs etc.

Companies are invited to fill in contact details.

| Company | Contact details |
| --- | --- |
| Samsung | Seungri Jin (seungri.jin@samsung.com) |
| ZTE | Eswar Vutukuri (eswar.vutukuri@zte.com.cn) Yu Liu (liu.yu3@zte.com.cn) |
| Qualcomm Incorporated | Masato Kitazoe (mkitazoe@qti.qualcomm.com) |
| Lenovo | Hyung-Nam Choi (hchoi5@lenovo.com) |

# 3	Discussion, First round

## 3.1	UE handling of cell-specific parameters provided in dedicated signalling

R2-2207776	UE handling of cell-specific parameters provided in dedicated signalling	Huawei, HiSilicon	discussion	Rel-17	TEI17, NR_MBS_enh-Core

Q1. Do companies agree with the intention of the propsal and TP of the document above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No | From our understanding, ServingCellConfig is definetly confiugred per UE after checking the UE capabilities. In that sense, BWP-DownlinkCommon and/or BWP-UplinkCommon configured in ServingCellConfig are not the exceptional case of ”inability to comply with RRCReconfiguration” what specified for ServingCellConfigCommon. |
| Huawei, HiSilicon | Yes | Proponent |
| Qualcomm Incorporated | No | We are not convinced by the MBS use case explained in the document. |

## 3.2	Correction on UERadioPagingInformation and UERadioPagingInfo container

R2-2208654	Correction on UERadioPagingInformation and UERadioPagingInfo container	Ericsson	CR	Rel-17	38.331	17.1.0	3460	-	F	NR_newRAT-Core, NR_redcap-Core

Q2. Do companies agree with the intent of the CR above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | This change is NBC, but we think it is needed. |
| Huawei, HiSilicon | No | This CR seems not correct, if it changes the UE radio capabilty singaling. It should be discussed/reviewed by RedCap session at least. The way of changing is NBC for inter-node message. The IEs in UERadioPagingInformation-v1700-IEs are not UE’s paging capability signaling. It is for the inter-node message, which includes the assist information about UE capability. gNB gets this information so that gNB can do optimized paging based on the UE paging capability information. numberOfRxRedCap-r17 ENUMERATED {one, two} OPTIONAL, hd-FDDRedCap-r17 ENUMERATED {supported} OPTIONAL, The related UE radio capability were already captured in the spec as maxNumberMIMO-LayersPDSCH and halfDuplexFDD-TypeA-RedCap-r17. |
| Qualcomm Incorporated | No | At least we do not see the current specification is broken. Too late to do this type of cleanup after ASN.1 freeze. |

## 3.3	Unified TCI state with deactivated SCG

R2-2207002	Corrections to initiation upon reception of RAN paging and T380 Expiry	Samsung Electronics Co., Ltd	draftCR	Rel-17	38.331	17.1.0	NR_newRAT-Core, NR_SmallData_INACTIVE-Core

R2-2207267	Unified TCI state with deactivated SCG	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core, NR_FeMIMO-Core

Q3. Do companies agree with the intent of the CR above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes, but | From our understanding, unified TCI state with SCG deactivation could be configured together because there has no restrictions what RAN1/RAN2 made. If this is true, we tend to agree the proposals to correct the tci-info. For the detail ASN.1 changes proposed in this draft CR, NBC corrections in terms of ASN.1 are proposed. We agree this is cleaner approach but it requires more RAN2 concensus. |
| Huawei, HiSilicon | Yes/No | NBC proposal (ASN.1 NBC change). The proposed changes to the field description are ok but don't see the need for any ASN.1 change, the network can provide the MAC CE for UL TCI states after SCG activation. Otherwise, it should be a new UE capability and added in a BC way. |
| Qualcomm Incorporated | No | The CR presents one possible way to implement the unified TCI state in the tci-info framework (and some details need to be looked at), but there can be other ways to do this, e.g. not having ”indicated TCI state” in RRC configuration, but rely on DCI after SCell/PSCell activation. To us, it is too late to discuss all the details for release-17. We propose to leave the feature combination unsupported in release-17. |

## 3.4	Corrections to initiation upon reception of RAN paging and T380 Expiry

R2-2207002	Corrections to initiation upon reception of RAN paging and T380 Expiry	Samsung Electronics Co., Ltd	draftCR	Rel-17	38.331	17.1.0	NR_newRAT-Core, NR_SmallData_INACTIVE-Core

Q4. Do companies agree with the intent of the CR above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes (Proponent) | Scenario: RRC initiate resumption procedure in RRC_INACTIVE. While the resumption is ongoing, T380 expires. Case 1: resumption is initiated for SDT - Normative text is added in specification to prohibit initiating the second resumption procedure while the first one is ongoing. Case 2: resumption is not initiated for SDT - No text (neither normative text nor NOTE) is added in specification to prohibit initiating the second resumption procedure while the first one is ongoing - RAN2#113bis Agreement: The UE should not start the 2nd RRC resumption procedure when there is a RRC resumption procedure ongoing (no spec change required) In our understanding, in both the cases, second resumption procedure should not be initiated while the first one is ongoing. The specification should be consistent in handling both the cases. Specifying normative text in one case and not in another is misleading. Its gives an impression that only in case 1, second resumption is prohibited while the first one is ongoing. |
| Huawei, HiSilicon | No | This was discussed during RAN2#113 meeting and it was agreed that the UE does not initiate second resume procedure during an ongoing one but we also agreed not to capture this. Then, it was dioscussed also for SDT and the agreement was confirmed while it was decided to capture this for SDT explicitly as SDT procedure is expected to last longer than „normal“ resume. |
| Qualcomm Incorporated | Yes | While we are aware of the previous RAN2 decision, we think it makes sense to have the specification aligned between SDT case and non-SDT case. Otherwise readers may misunderstand that different behaviours are expected. |
| ZTE | Yes | The same views as Qualcomm. Can be merged to rapporteur CR. |

## 3.5	MsgA PUSCH resource release upon T304 expiry for SCG

R2-2207006	MsgA PUSCH resource release upon T304 expiry for SCG	Samsung Electronics Co., Ltd	draftCR	Rel-17	38.331	17.1.0	NR_newRAT-Core

Q5. Do companies agree with the intent of the CR above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes (Proponent) | Upon T304 expiry of MCG, RRC (section 5.3.5.8.3) releases dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured. The same operation should also be performed uponT304 expiry of SCG, as the according to MAC and RRC R17 spec cfra-TwoStep is supported for reconfigurationWithSync of both MCG and SCG. |
| Huawei, HiSilicon | Yes | Seems this change addresses an omission. Coversheet should be improved, to make it a real CR to be approved. |
| ZTE | Yes |  |
| Qualcomm Incorporated | Yes |  |

## 3.6	Corrections to MBS paging monitoring during the SDT procedure

R2-2207013	Corrections to MBS paging monitoring during the SDT procedure	Samsung Electronics Co., Ltd	draftCR	Rel-17	38.331	17.1.0	NR_SmallData_INACTIVE-Core, NR_MBS-Core

Q6. Do companies agree with the intent of the CR above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes (Proponent) | In the SDT WI it was agreed that UE does not monitor paging message while the SDT procedure is ongoing. UE only monitors SI update/emergency notifications. According to current spec, in RRC_INACTIVE, for MBS multicast reception, UE monitors a Paging channel for paging using TMGI. TMGIs are included in paging message. So this basically means that UE monitors paging message in RRC_INACTIVE. Considering that SDT procedure can be initiated by UE which is also supports MBS, it should be clarified that While SDT procedure is not ongoing, UE in RRC_INACTIVE monitors a Paging channel for paging using TMGI for multicast reception. |
| Huawei, HiSilicon | Yes | This has already been in the MBS RRC rapporteur CR. Suggest to discuss in MBS session directly. |
| ZTE | Yes | Can be merged with MS RRC CR if necessary |
| Qualcomm Incorporated | Yes | Should avoid duplicated discussion in multiple sessions/email discussions. |

## 3.7	Miscellaneous non-controversial corrections Set XV

R2-2208141	Miscellaneous non-controversial corrections Set XV	Ericsson	CR	Rel-17	38.331	17.1.0	3362	-	F	NR_newRAT-Core	Late

Q7. Do companies agree with the intent of the CR above?

(Companies are invited to indicate further typos etc. We are also all aware that many editorials are taken care of in WI CRs.)

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| Huawei, HiSilicon |  | There is a NBC change, by removing the extentsion marker in IE RedCap-ConfigCommonSIB-r17. We don’t agree to introduce this NBC change for no good reason. |
| Qualcomm Incorporated | No | We do not support ASN.1 NBC without sufficiently good reason. It is unfortunate NBC was hidden in misc. correction type CR. |
| ZTE | Yes,but | ASN.1 NBC change is not expected. |
| Lenovo | Yes | Referring to chairman’s guidance, ASN.1 NBC changes are not precluded as long as there are agreements on them. And in RAN2#118-e RAN2 agreed to remove extension markers from IEs which are used in broadcast signaling to avoid potential overhead in the future. The following minor issues should be fixed as well: - IE RadioLinkMonitoringConfig: field name should start with lowercase letter. [[ Beamfailure-r17 BeamFailureDetection-r17 OPTIONAL -- Need R ]] - In IE BandNR: in field name parrallelPRS-MeasRRC-Inactive-r17 the redundant “r” should be removed. - MeasAndMobParametersCommon IE: Add suffix “-r17” for the capabilities nr-NeedForGapNCSG-reporting and eutra-NeedForGapNCSG-reporting. [[ -- R4 19-2 Concurrent measurement gaps concurrentMeasGap-r17 CHOICE { concurrentPerUE-OnlyMeasGap-r17 ENUMERATED {supported}, concurrentPerUE-PerFRCombMeasGap-r17 ENUMERATED {supported} } OPTIONAL, -- R4 19-1 Network controlled small gap (NCSG) nr-NeedForGapNCSG-reporting ENUMERATED {supported} OPTIONAL, eutra-NeedForGapNCSG-reporting ENUMERATED {supported} OPTIONAL, -- R4 19-1-1 per FR Network controlled small gap (NCSG) ncsg-MeasGapPerFR-r17 ENUMERATED {supported} OPTIONAL, |

## 3.8	Correction to MINT – applicableDisasterInfoList

R2-2208133	Correction to MINT - applicableDisasterInfoList	Ericsson	CR	Rel-17	38.331	17.1.0	3359	-	F	TEI17

Q8. Do companies agree with the intent of the CR above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | This was a mistake when implementing MINT. |
| Samsung | Yes | We agree with the proposed change. |
| Huawei, HiSilicon |  | Editorial, can be merged into the rapportuer CR. |
| Qualcomm Incorporated | Yes |  |
| ZTE | Yes |  |
| Lenovo | Yes but | The following CR cover page issues should be fixed: - CR title: add tag “[MINT]”. - Reason for change: the number of the third entry should be “3”. Entry 1 of plmn-IdentityInfoList: plmn-IdentityInfoList{PLMN A; PLMN B} Entry 2 of plmn-IdentityInfoList: plmn-IdentityInfoList{PLMN C} Entry 2 of plmn-IdentityInfoList: plmn-IdentityInfoList{PLMN D, PLMN E} In the sentence below typos should be fixed, i.e. it should say “sharing the”. Disaster information is indicated as a list, the intention was that the network should be able to signal different disaster information for the different networks that share the cell, similar to how different networks sharin ghte cell could have different cell identity, tracking area code, etc. The sentence below is incomplete. We suggest to add the text in red color. This error makes it impossible to provide disaster information for the scenario shown above as it is unclear for which of the (more than one) plmn-IdentityList the disaster roaming information applies. |

# 5	Conclusion

To be added.