CR3GPP TSG-RAN WG2 Meeting #119-e	R2-220xxxx

# Electronic, August 17 – 29, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | xxx | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to IAB topology defintion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The present IAB topology definition is: IAB topology: The unison of all IAB-nodes and IAB-donor-DUs that are interconnected via BH links and terminate F1 and/or RRC at the same IAB-donor-CU. 1. Change: This definition misses to unambigiously assign the descendent nodes of a single-connected boundary node to an IAB-topology. - The single-connected boundary node belongs to the non-F1-terminating IAB-donor’s topology since its IAB-MT has RRC connectivity with this IAB-donor and since the parent node belongs to this IAB-donor’s topology. - The single-connected boundary node does not belong to the F1-terminating IAB-donor’s topology since it does not have a BH link to a parent node that terminates at this IAB-donor. - The child node of the boundary node has F1 and RRC connections to the F1-terminating IAB-donor, but its BH link does not connect to parent node of this donor. It it therefore not clear to which IAB-topology the child node belongs. - The grand child node and lower-tier descendant nodes of the boundary nodes also have F1 and RRC connections to the F1-terminating IAB-donor, but the topology of their parent nodes is undefined. It is therefore not clear to which IAB-topology these grand-child and descendent nodes belong. For dual-connected boundary nodes, this unambiguity does not exist. In this case, based on this definition, descendent nodes are assigned to the F1-terminating IAB-donor’s topology. 2. Change: According to this definition the IAB-nodes and IAB-donor-DUs terminate F1 and/or RRC at the same IAB-donor-CU. This wording is technically incorrect since IAB-nodes, IAB-donor-DUs and IAB-donor-CUs may terminate RRC and/or F1, but it is not possible for the IAB-node or iAb-donor-DU do terminate RRC and/or F1 at the IAB-donor-CU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Change: The descendent nodes of a single-connected boundary node should belong to the F1-terminating IAB-donor’s topology in alignment to the descendent nodes of a dual-connected boundary node. This is achieved via a small change of the IAB topology’s defintion. 2. Change: Proper rewording of the defintion to capture that the IAB-donor-CU terminates the RRC and/or F1 of the IAB-nodes and IAB-donor-DUs in question. Impact analysis Impacted functionality: BAP sub-layer routing/re-routing and BH RLC channel mapping after inter-donor partial migration and inter-donor recovery. Inter-operability: In absence of this change, inter-operability between IAB-nodes and IAB-donor is not properly supported since interpretation of IAB-topology is ambiguous for BAP sub-layer routing/re-routing and BH RLC channel mappings. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | BAP sub-layer routing/re-routing and BH RLC channel mapping may occur incorrectly since IAB-topology indicator cannot is not incorrectly interpreted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

>>> Unchanged parts skipped

IAB topology: The unison of all IAB-nodes and IAB-donor-DUs  whose F1 and/or RRC connections terminate at the same IAB-donor-CU.

>>> Unchanged parts skipped

End of Changes