3GPP TSG-RAN WG2 Meeting #119e                                                                  R2-22xxxxx

Online, August 17th- August 29th, 2022

Agenda item:	6.24.1

Source:	vivo

Title:	Summary for email discussion [AT119-e][022][NR17] DC Location Report (vivo)

Document for:	Discussion and Decision

1 Introduction

This document is a report on the following email discussion:

[AT119-e][022][NR17] DC Location Report (vivo)

Scope: Treat R2-2206951, R2-2207613, R2-2207135, R2-2207136, R2-2207138, R2-2207614, R2-2208370, R2-2208371, Determine agreeable parts. For the agreeable parts, agree CRs.

Intended outcome: Report, Agreed CRs (if any), LS out (if applicable)

Deadline: Schedule 1

The deadline Schedule 1 for this email discussion is copied from Chair notes:

A first round with Deadline for comments W1 Friday Aug 19th 1400 UTC to settle scope what is agreeable etc

A Final round with Final deadline W2 Thursday Aug 25th 1200 UTC to settle details / agree CRs etc.

For all discussions: Additional deadlines check points etc if needed are defined by the Rapporteur of each discussion respectively. In case some parts of an email discussion need more time, doesn’t converge, need on-line treatment, then please contact the chair.

2	Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| vivo(rapporteur) | Xiaodong Yang | yangxiaodong5g@vivo.com |

3	Discussion

In this document, this DC location report topic is from the following contributions.

[1] R2-2207613	Remaining issues on DC location report for Rel-17	vivo	discussion	Rel-17	NR_RF_FR2_req_enh2-Core

[2] R2-2207135	Discussion on DC location for more than 2 UL CCs	OPPO	discussion	Rel-17	NR_RF_FR2_req_enh2-Core

[3] R2-2207136	CR on UE capability for DC location for more than 2 UL CCs	OPPO	CR	Rel-17	38.306	17.1.0	0759	-	B	NR_RF_FR2_req_enh2-Core

[4] R2-2207138	CR on DC location for more than 2 UL CCs	OPPO	CR	Rel-17	38.331	17.1.0	3219	-	B	NR_RF_FR2_req_enh2-Core

[5] R2-2207614	LS to RAN4 on DC location	vivo	LS out	Rel-17	NR_RF_FR2_req_enh2-Core	To:RAN4

[6] R2-2208370	Discussion on the update to endorsed CRs for DC location report for more than 2CC	Huawei, HiSilicon	discussion	Rel-17	NR_RF_FR2_req_enh2

[7] R2-2208371	Introduction of DC location reporting for more than 2CCs	Huawei, HiSilicon	CR	Rel-17	38.331	17.1.0	3097	2	B	NR_RF_FR2_req_enh2	R2-2206650

For the topic of DC location report in case of more than 2 CC are configured for UE, an LS [1] was sent from RAN4 to ask RAN2 to define the signalling during RAN2#116bis e-meeting. In RAN2#116bis e-meeting, the following agreements were achieved:

For default DC location derivation, the UE signals:

- 1. the choice of frequency component, among {Activated CC, Configured CC, Activated BWP, Configured BWP}.

- 2. the choice of UL and/or DL for frequency component, among {UL, DL, Edge most frequencies among any DL and UL}

The network specifies the radio resource configuration (including BWP / CC activation state) for which the UE is requested to report the offset to default DC location. FFS how the radio resource configuration is specified.

Introduce a new release-17 network request for the extended DC location reporting for more than 2 UL CCs.

Upon a new release-17 network request, the UE reports the extended DC location reporting for more than 2 UL CCs, i.e. the release-17 network request does not trigger the reporting of reportUplinkTxDirectCurrent and reportUplinkTxDirectCurrentTwoCarrier-r16.

Furthermore, an LS was sent to RAN4 based on the discussion and another LS [2] is received from RAN4 as the reply during RAN2#118 e-meeting. Based on the LS, the following agreements were further achieved and a CR [3] was endorsed as baseline for further work.

[045] P1: DC location (frequency component option and offset to default) is signalled per CC group. For determination of each CC group, the UE signals the lowest CC/serving cell and the highest CC/serving cell. These parameters are signalled in RRCReconfigurationComplete and RRCResumeComplete.

[045] P2: Network request for the extended DC location reporting for more than 2 UL CCs is introduced in CellGroupConfig.

[045] P3: As part of the network request, the network indicates the combinations of UL and DL CC state of each serving cell per UL intra-band CA component of the configured CA band combination. “CC state” indicates CC activation state and the active BWP of the activated CC.

[045] P4: For UE capability signalling, wait for RAN4 conclusion on the UE feature list.

[045] P5: FFS: DC location reporting for a band combination in which multiple UL intra-band CA components with >2CCs and with <=2CCs are included.

In this meeting, RAN4 sent another LS [4] to ask RAN2 to take RAN4 conclusions into consideration in signaling design.

3.1 Clarify the calculation of the default DC location

In  R2-2207613, the company identifies it is unclear whether UE can calculate default DC location for different CC/BWP combinations based on different frequency components in a CC group.

Question 1: What’s your understanding of the calculation of the default DC location for different CC/BWP combinations in a CC group?

- Understanding 1: UE is allowed to calculate default DC location for different CC/BWP combinations based on different frequency components in a CC group;

- Understanding 2: UE can only calculate default DC location for different CC/BWP combinations based on same frequency components in a CC group.

| Company | Option | Comments |
| --- | --- | --- |

In  R2-2207613, the company presents two ways to report offsetToDefault if frequency component type is only allowed to be same for different CC/BWP combinations in a CC group and if the frequency component type is configuredCarrier or configuredBWP.

Qusetion 2: If the answer to Q1 is understangding 2, if the frequency component type is configuredCarrier or configuredBWP, which option do you prefer for reporting offsetToDefault?

- Option 1: offsetValue is used, further clarify that all requested CC combinations associated DC location offset are offsetValue;

- Option 2: offsetlist is used and the value of each entity in this list is same;

- Option 3: Other

| Company | Option | Comments |
| --- | --- | --- |

3.2 Analyses of LS R2-2206951

RAN4 has sent an LS [4] to ask RAN2 to take RAN4 conclusions into consideration in signaling design. Three companies have analyzed the LS and given corresponding proposals.

Offset range

Relevant proposals of companies on the offset range are listed below.

| Tdoc No. | Relevant Proposals | Source |
| --- | --- | --- |
| R2-2207613 | Proposal 3: The offset range is (-20000, 20000) both for FR1 and FR2. | vivo |
| R2-2207135 | Proposal 6: the value range of default DC location should be (-20480~20480) for FR1 and FR2. | OPPO |
| R2-2208370 | Proposal 1: The offset value is update to (-19800 to 19800). Proposal 2: The lowest SCS in the reported CC group is used as the offset granularity for offset report. | Huawei |

All three companies gave suggested offset ranges, but the suggested ranges are inconsistent.

In order to form a common view on offset range, companies are encouraged to answer the following question.

Question 3: For the offset range, which option do you prefer for FR1 and FR2?

- Option 1: (-20000, 20000)

- Option 2: (-20480~20480)

- Option 3: (-19800 to 19800)

- Option 4: Other

| Company | Option | Comments |
| --- | --- | --- |

In addition, in R2-2208370, company proposes the lowest SCS in the CC group instead of in the CA configuration is used as the offset granularity considering that the offset is signaled per CC group.

In order to form a common view on offset granularity, companies are encouraged to answer the following question.

Question 4: Do company agree that the lowest SCS in the CC group is used as the offset granularity?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Applicability update and compatibility with R15/R16 reporting scheme

Relevant proposals on the applicability update and compatibility with R15/R16 reporting scheme are listed below.

| Tdoc No. | Relevant Proposals | Source |
| --- | --- | --- |
| R2-2207613 | Proposal 4: Modify the name and description of R17 IE for DC location report (i.e., UplinkTxDirectCurrentMoreCarrierList) to make the R17 DC location reporting scheme also applies for 2CCs and single UL carrier within CA. Proposal 5: If single CC is included in a CC group, one of the following options can be adopted: - Option 1: UE sets the value of servCellIndexHigher same as the value of servCellIndexLower; - Option 2: UE only sets the value of servCellIndexLower, the servCellIndexHigher IE is absent. Proposal 6: Add a NOTE in the RRC reconfiguration procedure and RRC connection Resume procedure as follows: - NOTE: For a UL CA configuration in one band combination, if multiple DCs exist there is only one scheme will be used for these DC location reporting, for example, only Rel-17 reporting scheme can be used throughout different UL CC groups on the same band. | vivo |
| R2-2207135 | Proposal 1: if there is more than 2 UL CCs in at least one intra-band component, only Rel17 solution is applicable for the band. Proposal 1a: if UE support R17 scheme for all bands within same band combination and network decide to apply Rel17 solution for at least one band, then Rel17 solution should be applied for the whole band combination. Proposal 2: if there is no more than 2 UL CCs in any intra-band component, it is up to network to decide whether R15, R16 or R17 solution, whichever is applicable. | OPPO |
| R2-2208370 | Proposal 3: only one DC location report scheme among R15/16/17 is used for a CA configuration and the network indicate which scheme is to be used based on UE’s capability. | Huawei |

Referring to contributions above,  all companies express similar views, i.e., only one DC location report scheme among R15/16/17 is used for a CA configuration and the network indicate which scheme is to be used. In addition, in R2-2207613,company suggests that CR should be updated to make the R17 DC location reporting scheme also applies for 2CCs and single UL carrier within CA.

In order to form a common view on this issue, companies are encouraged to answer the following questions.

Question 5: Do company agree that only one DC location report scheme among R15/16/17 is used for a CA configuration and the network indicate which scheme is to be used?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Question 6: Do company agree that CR should be updated to make the R17 DC location reporting scheme also applies for 2CCs and single UL carrier within CA?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

DL CC(s) clarification

Relevant proposals on DL CC(s) clarification are listed below.

| Tdoc No. | Relevant Proposals | Source |
| --- | --- | --- |
| R2-2207613 | Proposal 7: For DL CC(s) clarification, the existing signaling structure is sufficient. | vivo |
| R2-2207135 | Proposal 4: In the spec it should be clarified that DL-only FR2 frequency spectrum is not used to calculate the default DC location | OPPO |

Two companies above show different views, one thinks it is needed to be clarified that DL-only FR2 frequency spectrum is not used to calculate the default DC location while the other thinks it does not need.

In order to form a common view on this issue, companies are encouraged to answer the following questions.

Question7: For DL CC(s) clarification, which option do you prefer?

- Option 1: It should be clarified that DL-only FR2 frequency spectrum is not used to calculate the default DC location

- Option 2: There is no need to clarify that DL-only FR2 frequency spectrum is not used to calculate the default DC location

| Company | Option | Comments |
| --- | --- | --- |

CC group for multiple DCs reporting

Relevant proposals on CC group for multiple DCs reporting are listed below.

| Tdoc No. | Relevant Proposals | Source |
| --- | --- | --- |
| R2-2207613 | Proposal 8: For CC group for multiple DCs reporting, the existing signaling structure is sufficient. | vivo |
| R2-2207135 | Proposal 3: In the field description of CC group, it should be clarified that in Rel17 there is only one UL CC for FR1 band. | OPPO |
| R2-2208370 | Proposal 4: we shall not limit the CC groups so that each group contains only one UL CC as suggested by RAN4, but we shall update the CR to allow that a CC group only contains only CC. Proposal 5: The UE shall report only one CC group/DC location for an intra-band CC combination with one active uplink carrier in case the default DC location is derived from active CC/BWP. | Huawei |

Since RAN4 indicates the restriction that each group contains only one UL CC, two companies think the existing signaling structure is flexible enough to support all the cases of CC group for multiple DCs reporting, while the other one company think it should be clarified that in Rel17 there is only one UL CC for FR1 band. Furthermore, one of the companies thinks we shall update the CR to allow that a CC group only contains only CC.

In order to form a common view on this issue, companies are encouraged to answer the following questions.

Question 8: For the CC group for multiple DCs reporting, which option do you prefer?

- Option 1: There is no need to limit the CC groups so that each group contains only one UL CC;

- Option 2: It should be clarified that in Rel-17 there is only one UL CC for FR1 band.

| Company | Option | Comments |
| --- | --- | --- |

Question 9: Do company agree that CR is needed to be updated to allow that a CC group only contains only CC?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

In addition, RAN4 also suggests to only report one DC location in case NW deactivates or de-configures UL CC based the assumption that only two UL CCs are supported in Rel-17. Considering that there might be more CCs supported in future, in R2-2208370, company suggest the UE shall report only one CC group/DC location for an intra-band CC combination with one active uplink carrier in case the default DC location is derived from active CC/BWP.

Question 10: Do company agree that UE shall report only one CC group/DC location for an intra-band CC combination with one active uplink carrier in case the default DC location is derived from active CC/BWP?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

shift7dot5kHz

Relevant proposals on adding shift7dot5kHz are listed below.

| Tdoc No. | Relevant Proposals | Source |
| --- | --- | --- |
| R2-2207613 | Proposal 9: Capture shift7dot5kHz in Rel-17 DC location report signalling, shift7dot5kHz is associated with each offsetValue. | vivo |
| R2-2207135 | Proposal 5: shift7dot5kHz should be added per DC location. | OPPO |
| R2-2208370 | Proposal 6: add shift7dot5kHz to the baseline CR. | Huawei |

Referring to contributions above,  all companies think shift7dot5kHz should be added to the baseline CR, two of these companies propose that shift7dot5kHz is associated with each offsetValue.

In order to form a common view on this issue, companies are encouraged to answer the following questions.

Question 11: Do company agree that shift7dot5kHz should be added to the baseline CR?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Question 12: If the answer to Q10 is yes, do company agree that shift7dot5kHz is associated with each offsetValue?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

3.3 UE capability

In R2-2207135, company think UE capability 17-5 in [5] doesn’t cover single UL CC case. However, The LS [4] says “the R17 DC location reporting scheme is also applicable for 2CCs and single UL carrier within CA”. So the company proposes UE supporting RAN4 UE capability 17-5 can report default DC location also for single UL CC case and this should be clarified for 306 CR.

In order to form a common view on this issue, companies are encouraged to answer the following questions.

Question 13: Do company agree that RAN4 UE capability 17-5 can report default DC location also for single UL CC case and this should be clarified for 306 CR?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

3.4 LS to RAN4

Although above issues are discussed in RAN2, some of the issues should be finally checked by RAN4. As an example, the understanding of calculation of the default DC location for different CC/BWP combinations in a CC group (i.e., Q1) depends on RAN4. As another example, RAN4 indicates that “limit the CC groups so that each group contains only one UL CC”, one company thinks this limitation is only applicable for FR1 band not for FR2 band. From rapporteur’s perspective, RAN4 should further clarify the applicable scenarios. As another example, the issue on UE capability has an impact on RAN4 In R2-2207135, which also needs to be notified to RAN4.

Regarding whether to send an LS to RAN4 and what contents should be included in LS , companies are encouraged to answer the following question.

Question 14: Do company agree to send an LS to RAN4?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Question 15: If the answer to Q14 is yes, what contents do you think to be contained in the LS?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

4 Conclusion

TBD.

5 Reference

- R2-2200117, LS on DC location for >2CC (R4-2119965; contact: Qualcomm)

- R2-2204506, Reply LS on DC location for >2CC (R4-2206602; contact: Qualcomm)

- R2-2206650, Introduction of DC location reporting for more than 2CCs

- R2-2206951, LS on DC location for intra-band CA (R4-2210782; contact: vivo)

- R4-2211190	LS on Rel-17 RAN4 UE feature list for NR