3GPP TSG-RAN WG2 Meeting #119	R2-220xxxx

Online, 17 – 26 August 2022

Agenda item:	6.21.1

Source:	Chair (MediaTek Inc)

Title:	Comments on New proposals

WID/SID:	TEI17

Document for:	Discussion and Decision

1	Introduction

This document is to collect comments on the following proposals:

[AT119-e][038][NRTEI17] Comments on New proposals (Chair)

Scope: Collect a round of comments on the new TEI proposals in R2-2208241, R2-2207434, R2-2208430, R2-2208668, R2-2207938 in order to determine if any of these could be agreeable.

Intended outcome: Report for CB W2 Friday

Deadline: W2 Thursday 1800 UTC

2	Discussion

2.1	Inclusion of the CSI reports in MDT framework

R2-2208241	Inclusion of the CSI reports in MDT framework	Nokia, Nokia Shanghai Bell, Deutsche Telekom, Verizon	discussion	Rel-17	TEI17, NR_ENDC_SON_MDT_enh-Core	R2-2206144

Proposal 1: UE CSI reporting is added to the MDT framework to allow for advanced network optimization procedures as part of TEI17.

Proposal 2: RAN2 is asked to agree to TS37.320 changes as provided in the Annex.

| Company | Support / CanAccept / Oppose / Question | Comments and justification for the expressed opinion (and/or question). |
| --- | --- | --- |
| Chair |  | Chair initial comments (open ended): no impact to the UE, impact to the base-station. CSI can map reasonably to achievable L1 bitrate so it may be useful. Could be a low hanging fruit, but may have impact in other groups, so not clear if TEI is the best WI? |
| Nokia | Support | Proponent |
| LG | CanAccept | As the CSI reporting is already there, it can be used for MDT without any UE impact. No strong view. |

2.2	SDAP end-marker in RLC UM

R2-2207434	SDAP end-marker in RLC UM	 Apple, Futurewei, Spreadtrum, FGI, Asia Pacific Telecom, T-Mobile USA, ZTE Corporation	discussion	Rel-17	TEI17	R2-2205679

Proposal 1: RAN2 to study how the handling of PDUs carrying SDAP end-marker or RDI can be enhanced to mitigate against potential message loss in RLC UM in Rel-17.

| Company | Support / CanAccept / Oppose / Question | Comments and justification for the expressed opinion (and/or question). |
| --- | --- | --- |
| Chair |  | Chair initial comments (open ended): This was discussed in Rel-15 and was left as is, assuming an impl timer will resolve the situation, and indeed the timer brings some delay. Proposal to study is not optimal for TEI type work, but can check the level of interest. |
| Apple | Support (Proponent) | Moving on from Rel-15 the NR feature set has been enhanced to support new services, many of which facilitate a lower end-to-end latency. As a consequence, later releases are expected to use RLC UM more widely. For example, RLC UM on a dedicated DRB is a likely deployment in use-cases such as IIoT, URLLC, advanced interactive services and gaming in Rel-17. When a DRB is configured with RLC UM, the end-marker in UL or the RDI in DL can get lost and introduce unwanted side-effects. • In the UL, a lost end-marker may cause the gNB to buffer data for a long time, hampering user experience and adding delay. • In the DL, a lost RDI packet from the gNB would cause the UE not updating the QoS flow to DRB mapping. Thus, the UE would continue sending UL packets of the QFI on the old DRB while the gNB buffers packets waiting for the end-marker. Therefore, we would like to see if companies are open to revisit some of the SDAP procedures with the aim to reduce unintended delays in scenarios where the RDI or end-marker can get lost. Our intention is to check how to potentially address these issues. We are open to a range of solution options. By “study” we mean that suitable approaches could be sorted in an email discussion, but no extensive study phase. Depending on which option is chosen we do not think a timer is necessarily needed. Some of the options can be lightweight without too much impact on specs and implementation. A quick comparison of the solution options: • Option 2 may be straightforward by having the UE to (provisionally) repeat the end-marker, which is not allowed in current specifications. • Option 1 could latch upon the survival time feature. Alternatively, RAN2 could simply increase the reliability of the initial transmission. • Option 3 perhaps is a clean approach, but it may need a new DL control PDU or some form of an ACK, so the spec impact could be slightly higher. • Option 4 is also relatively straightforward at least from a UE perspective, and to identify when to repeat the RDI it could be up to network implementation. |
| Nokia | Can Accept | We are fine with having the end marker on the old bearer for both RLC AM and RLC UM, but we would question the need to improve its reliability for RLC UM. In general, we believe it would be more important to give the old bearers (or the packets of that QoS flow in the old bearer) the same priority as the new bearer so that the delay impacts of the packets from the relocated QoS flow remaining in the old bearer are minimised. |
| LG | Oppose | This issue was discussed in Rel-15, and it was concluded that no enhancement is needed for loss of SDAP end marker. Even if the SDAP end marker is lost, the network implementation can handle the buffered data, e.g. using reordering timer in the network side. |

2.3	Remote access issue

R2-2208430	Discussion on remote access issue	CMCC, vivo, Huawei	discussion	Rel-18	TEI

Proposal 1: RAN2 is kindly asked to confirm the remote access issue, that UE may access to a remote cell due to the radio signal transmitted and reflected by atmospheric and seawater with tiny propagation loss at some certain temperature and humidness conditions.

Proposal 2: RAN2 is kindly asked to confirm that the current implementation solutions cannot completely address the remote access issue.

Proposal 3: RAN2 is kindly asked to discuss the potential solutions, e.g.:

–	Introduce a TA threshold in the system information to indicate the maximum Timing Advance to access the cell.

–	If the received TA is larger than TA threshold, UE should apply Qoffsettemp for R criteria when UE goes back to idle/inactive mode. No change to the existing RA procedure.

| Company | Support / CanAccept / Oppose / Question | Comments and justification for the expressed opinion (and/or question). |
| --- | --- | --- |
| Chair |  | Chair initial comments (open ended): In my experience, this kind of “strangeness” in the network is quite normal, i.e. for deployments that has significant shadowing effects or open water etc, the best coverage for a UE may indeed be by a distant cell rather than the closest base-station, but RAN2 usually assumes that from radio perspective (interference etc) it is good to keep the UE on the best cell, distant or not. For very distant cells, it has been recognized in the past that there are issues with the resulting camping or connection being very unstable. For this purpose it is possible today to exclude or deprioritize cells from UE evaluation, by neighbor cell lists. |
| Nokia | Question | A solution affecting UE implementation is only a partial fix. Furthermore how can we ensure that TA is long in this case as the error scenario is straightline ? In general we are not convinced that chiba lake issues (which this is) cannot be solved by qoffesttemp. The most straightforward solution would be to fix the tracking app in order log the location of the UE. |
| LG | Oppose | Agree with chair’s comment. In addition, we think this kind of region-specific issue is not a normal case, and it should be solved by implementation not by standard. |

2.4	DRX correction for DCI controlled bundling

R2-2208668	Correction to DRX operation with bundling controlled in the DCI	Ericsson, Nokia, T-Mobile USA, Verizon, Docomo	discussion	Rel-17

Proposal 1	Introduce a configurable parameter drx-LastTransmissionUL that enables the start of the drx-HARQ-RTT-TimerUL after the end of the last transmission (within a bundle) instead of after the end of the first transmission (within a bundle).

Proposal 2	Introduce a new UE capability and corresponding RRC signalling for drx-LastTransmissionUL.

| Company | Support / CanAccept / Oppose / Question | Comments and justification for the expressed opinion (and/or question). |
| --- | --- | --- |
| Chair |  | Chair initial comments (open ended): This was briefly discussed before, with a mix or positive and negative comments. Anyway the proposal is limited, clear and could be implemented asap (as required for TEI) - in case there is more support / less objections now. |
| Apple | CanAccept | We agree that the proposed handling may reduce UE active time in cases where the HARQ RTT timer is (much) shorter than the time required to transmit all (many) TB repetitions. However, there are also other aspects to consider: 1. The updated handling is targeting a subset of services/scenarios only, but a UE may have multiple active services of different characteristics at the same time. 2. The network can configure the HARQ RTT timer to a value that is longer than the max TB repetition time, e.g., in a typical setup. In this case, the updated handling proposed may lead to a slight increase in UE power consumption since it removes opportunities for micro-sleep in between TB repetitions. 3. From a UE perspective, we would like to be able to terminate TB repetitions early, for example, there is no point continuing TB repetitions if already the first transmission was successful. Moreover, if the TB was not received correctly then the NACK would typically come after the last TB. Since there is a capability and Rel-17 is a new release we can accept if majority of companies supports, otherwise, we’d tend to rely on the legacy handling. |
| Nokia | Proponent |  |
| LG | Oppose | The network sets the repetition number based on estimation of transmission success probability. However, it is just a probability. It does not mean that only the last transmission can be succeeded. For example, even if the network sets Repetition=8 to ensure 99% of success probability, the UE can succeed in the first or second transmission. If the HARQ RTT Timer starts after the first transmission according to legacy, the network can send Cancellation to the UE to skip the UL transmission from the 5th repetition. However, in the proposal, the HARQ RTT Timer always starts after the last transmission, which means that the UE uselessly performs 5th to 8th transmission. We think skipping UL transmission is much helpful for UE power saving than skipping PDCCH monitoring, because TX requires more power than RX. Skipping only one UL TX may be better than skipping PDCCH monitoring for multiple subframes. In addition, we are not sure how the network can always perfectly control the repetition number. The repetition number depends on radio condition that can be getting worse or better due to some reasons (e.g., UE moving fast, unpredictable interference, etc), which is unpredictable by the network. |

2.5	Priority based inter-freq measurement reporting

R2-2207938	Priority based inter-freq measurement reporting	Apple	discussion	Rel-17	TEI17

Moved from 6.21.2

Proposal: Consider a reporting priority based mechanism to address the problem.

| Company | Support / CanAccept / Oppose / Question | Comments and justification for the expressed opinion (and/or question). |
| --- | --- | --- |
| Chair |  | Chair initial assessment (open ended): I understand that there is some interest for this, but as RAN4 is the group for detailed measurement behavior, it is somewhat difficult to see that this should be a RAN2-only topic (in order to arrive at a function that makes sense). Same comment applies for the referenced R2-2202436. |
| Ericsson | CanAccept if… | We are generally fine with enhancing the inter-frequency measurement configuration and reporting and we would like also to clarify that in our contribution in R2-2205832 submitted in the last RAN2 meeting we propose solutions that are confined only to RAN2 and do not require any RAN4 work. According to this, we are ok to discuss this but also the solutions proposed in R2-2205832 should be part of the discussion. |
| Nokia | Question | Seems like a RAN4 topic. |
| BT | CanAccept including R2-2205832 proposals | Currently, UEs perform HO measurements in a vendor specific way. That difficult the network engineering and its maintenance, e.g. load balancing when HO can cause an undesirable extra. As Ericsson mention, our contribution R2-2205832 in last RAN2 proposes several RAN2 self-contained solutions. For this problem and being realistic, we need to avoid any impact on RAN4. We understand this topic is sensible for UE vendors and chipset manufacturers. For that reason, our idea is to discuss a signalling based solution that suit everyone. We are fine to discuss Apple proposal but in addition, we consider R2-2205832 proposals needs to be added to the discussion. |

3	Conclusion

TBD