3GPP TSG-RAN WG2 Meeting # 119bis-e	R2-2210484

# Electronic, 10th – 19th October 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3547 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the necessity of SIB19 in NTN cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-09-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN2#118 meeting, it was agreed that SIB19 is essential for NTN cell but it was not captured in the spec. - RAN2 understands that SIB19 is essential for NTN cell, and it could be up to UE implementation if UE cannot acquire SIB19. FFS if a note is needed in the spec for this. In LTE spec, the LTE SIB31 is introduced in LTE IOT NTN, which has the same situation as the NR SIB19 in NR NTN. In LTE RRC, the SIB31 is the essential SIB for NTN cell, and the procedural text specifies the UE behavior when the essential SIB31 is missing (see below). Therefore, if there is no any description on the necessity of the SIB19 for NTN connection in the spec, this will cause misunderstanding. <SIB31 description in TS36.331> |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a NOTE to clarify the UE operation when the SIB19 cannot be acquired before initiating the RRC connection in NTN cell. NOTE: Whether to bar the cell or perform the cell reselection to other cells is up to UE implementation if UE cannot acquire the SIB19. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case that UE cannot acquire the SIB19 in NTN cell, the UE may camp on the NTN cell but cannot initiate the RRC connection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.2.5	Essential system information missing

The UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if the UE is unable to acquire the MIB:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform barring as if intraFreqReselection is set to allowed;

2>	else if the UE is unable to acquire the SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	if the UE is a RedCap UE:

4>	peform barring as if intraFreqReselectionRedCap is set to allowed;

3>	else:

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20].

NOTE:	Whether to bar the cell or perform the cell reselection to other cells is up to UE implementation if UE cannot acquire the SIB19.