3GPP TSG-RAN WG2 #119bis-e	R2-22xxxxx

Online, 10th – 19th Oct, 2022

Agenda Item:	8.18

Source:	ZTE Corporation

Title:	Report of [AT119bis-e][016][NR18] DSS enhancement (ZTE)

Document for:	Discussion, Decision

# Introduction

This document is the report of the following offline discussion:

- [AT119bis-e][016][NR18] DSS enhancement (ZTE)

Scope: Treat R2-2209314, R2-2210636, R2-2210133, R2-2210297, R2-2210586, R2-2210587, Determine agreeable parts, Open points etc

Intended outcome: Report, Agreeable CRs if applicable.

Deadline: For CB W1 Fri

# Contact Information

| Company | Name | Email address |
| --- | --- | --- |
| ZTE | LiuJing | liu.jing30@zte.com.cn |
| Huawei, HiSilicon | Chong Lou | louchong@huawei.com |
| vivo | Jianhui Li | jianhui.li@vivo.com |
| Xiaomi | Yumin Wu | wuyumin@xiaomi.com |

# Discussion

For reference, the objectives of Rel-18 DSS WID (in RP-213575) are shown below:

The following objectives shall be included for improvement of NR spectrum efficiency for LTE-NR co-existence (RAN1): - Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1] - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources. - Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1, RAN2]

## Work plan for Rel-18 DSS

R2-2210636	Work plan for Rel18 WI on Enhancement of NR Dynamic spectrum sharing (DSS)	Ericsson	discussion

The rapporteur of WI (Ericsson) has provided the work plan for Rel-18 DSS, the RAN2 work plan is copied/pasted below:

#extracted from R2-2210636

| RAN2 meeting | Work plan |
| --- | --- |
| RAN2#119bis-e, October 2022 | Support for two overlapping CRS rate matching patterns - Identify spec impact and endorse draft CRs/TPs to introduce support for two overlapping CRS rate matching patterns. |
| RAN2#120, November 2022 | Support for two overlapping CRS rate matching patterns - Finalize any remaining issues with draft CRs/TPs to introduce support for two overlapping CRS rate matching patterns and agree on final CRs. Note: RAN2 may eventually also handle additional RAN2 impacts at a later stage e.g. RRC parameters provided by RAN1 for this WI towards the end of the release, as usual. |

Companies are invited to show your views to the RAN2 work plan.

Q1: Do companies agree with RAN2 work plan in R2-2210636?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei, HiSilicon | See comments | We are in general fine with the work plan, but it is worthy indicating that, considering RAN1 has finished this WI, RAN2 discussion should be triggered by RAN1 (either by LS/RRC parameter inputs) and hence any potential RAN1 impact should be avoided, i.e. restrained in RAN2 scope. |
| vivo | Yes | Agree with Huawei that RAN2 should not trigger discussion that may cause potential RAN1 impact, unless triggered by RAN1 LS. |
| Xiaomi | Yes |  |

## Two overlapping LTE-CRS patterns

Regarding the objective on two overlapping LTE-CRS patterns, RAN2 receives the following LS from RAN1.

R2-2209314	LS to RAN2 on two overlapping LTE-CRS patterns in Rel-18 DSS (R1-2208194; contact: ZTE)	RAN1	LS in	Rel-18	NR_DSS_enh	To:RAN2

This LS includes the RAN1 agreements made on two overlapping LTE-CRS patterns and their expected RAN2 spec changes, based on the LS, companies provided draft CRs/TPs.

R2-2210297	Discussion on two overlapping LTE-CRS patterns in Rel-18 DSS	ZTE Corporation, Sanechips, Ericsson	discussion	Rel-18	NR_DSS_enh-Core

R2-2210586	Clarification on the DSS UE capability	Xiaomi	CR	Rel-16	38.306	16.10.0	0818	-	F	TEI16

R2-2210587	Clarification on the DSS UE capability	Xiaomi	CR	Rel-17	38.306	17.2.0	0819	-	A	TEI16

### TS 38.331 CR

In R2-2210297, it proposes the following changes to Rel-18 TS 38.331:

#Change 1: Introduce lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 to ServingCellConfig, add corresponding field description.

ServingCellConfig ::=               SEQUENCE {

*** skip non-related part***

lte-CRS-PatternList1-r16            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

lte-CRS-PatternList2-r16            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

*** skip non-related part***

[[,

lte-CRS-PatternList3-r18            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

lte-CRS-PatternList4-r18            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

]]

}

lte-CRS-PatternList3 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. The network does not configure this field and lte-CRS-ToMatchAround, or this field and lte-CRS-PatternList1, or this field and lte-CRS-PatternList2 simultaneously.

lte-CRS-PatternList4 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3. The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and the field lte-CRS-PatternList3 is configured.

#Change 2: Update the field description of crs-RateMatch-PerCORESETPoolIndex based on RAN1 LS.

crs-RateMatch-PerCORESETPoolIndex Indicates how UE performs rate matching when both lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 are configured or when both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are configured as specified in TS 38.214 [19], clause 5.1.4.2.

Q2: Do companies agree with above change 1 and 2 in R2-2210297?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes | Proponent |
| Huawei, HiSilicon | Yes | It is aligned with RAN1 LS |
| vivo | Yes | It is aligned with RAN1 LS |
| Xiaomi | Yes |  |

### TS 38.306 CR

For TS 38.306 CR, companies provided CR of different spec versions, the CR provided in R2-2210297 is intended to correct only Rel-18 spec, while the CR provided in R2-2210586/R2-2210587 are based on Rel-16/17 specs.

Based the RAN1 LS (see below) and RAN1 discussion, rapporteur understands the intention is to add the pre-requisite from Rel-18, so it does not impact Rel-16 and Rel-17 specs.

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16 in Rel-18 ASN.1.

Companies are invited to show your views.

Q3: Do companies agree the condition needs to be added to Rel-18 spec, no need to change Rel-16/Rel-17 specs?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes | Based on the information we got from our RAN1 colleagues, RAN1 agree to add the condition starting with Rel-18 version, they do not intend to update Rel-16/Rel-17 specs so that legacy UEs are not impacted. |
| Huawei, HiSilicon | Yes | Agree with the moderator, no need to change legacy spec. |
| vivo | Yes | Receive same input from our RAN1 colleagues as ZTE comments. |
| Xiaomi | No | The new Rel-18 DSS feature would anyway require a new UE capability bit which shall not be mixed up with the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16. The RAN1 LS is to clarify the legacy UE feature overlapRateMatchingEUTRA-CRS-r16 which was not clearly reflected in the RAN2 specification, without changing any legacy function. To reflect the correction understanding of the legacy UE feature overlapRateMatchingEUTRA-CRS-r16 in Rel-18 ASN.1, the change should start from Rel-16. Otherwise we might have Rel-16 UE supporting overlapRateMatchingEUTRA-CRS-r16, but not supporting multiDCI-MultiTRP-r16. The implementation team will not look into the Rel-18 specification (Note that this will not be provided before the Rel-18 ASN.1 frozen.) to implement a Rel-16 feature. |

For the detailed wording of the condition, companies proposed different changes.

Option 1: proposed in R2-2210297:

| overlapRateMatchingEUTRA-CRS-r16 Indicates whether the UE supports two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz SCS overlapping with a LTE carrier. If the UE supports this feature, the UE needs to report multipleRateMatchingEUTRA-CRS-r16. In this release of the specification, the UE indicating support of this feature shall indicate support of multiDCI-MultiTRP-r16. | Band | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

Option 2: proposed in R2-2210586/R2-2210587:

| overlapRateMatchingEUTRA-CRS-r16 Indicates whether the UE supports two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz SCS overlapping with a LTE carrier. If the UE supports this feature, the UE needs to report multipleRateMatchingEUTRA-CRS-r16 and multiDCI-MultiTRP-r16. | Band | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

Rapporteur thinks there is no big difference between Option1 and Option 2, but if answers “Yes” to Q3, then Option 1 is preferred, because it is aligned with other pre-requisites which are only applicable from a specific release.

Q4: Which wording change (Option1 or Option2) do you prefer?

| Company | Opt1 or Opt2 | Comments |
| --- | --- | --- |
| ZTE | Option 1 | Proponent. We think the condition is only needed for Rel-18 and future release of UEs, so it is better to make the spec clear by highlighting “in this release of the specification”. |
| Huawei, HiSilicon | Opt 1 |  |
| vivo | Opt 1 |  |
| Xiaomi | Option 2 | Option 1 does not work at all. See reasons provided above. |

Note: based on the feedbacks from companies, proponent company can provide ‘real’ draft CRs for endorsement.

## PDCCH on CRS

Regarding the following objective, one company provides contribution with proposals.

- Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1]

  - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources.

R2-2210133	RRC configuration and UE capability for PDCCH on CRS	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_DSS_enh

Proposal 1: A PDCCH candidate with REs overlapping with LTE CRS REs as configured to the UE is either monitored or dropped according to explicit RRC configuration or a determination based on the transition to RRC Connected state:

- Different PDCCH candidates / aggregation levels of a search space can be configured to be processed/dropped when they collide with LTE CRS

- Any PDCCH aggregagtion levels employed or higher than the aggregationlevels employed in the CCS during the transition from RRC Idle to RRC connected state are assumed to be monitored

Proposal 2: A PDCCH candidate with REs overlapping with LTE CRS REs as configured to the UE (and configured to be monitored) is tagged with information on what the gNB does to the PDCCH REs colliding with the LTE CRS REs

- Different PDCCH candidates / aggregation levels of a search space can be configured differently

- The configuration is either a relative power differentce of CRS RE and PDCCH RE (inf and -inf indicating that one is present and the other is punctured), or as binary (overlapping PDCCH RE present / not present)

Based on the RAN1 discussion, rapporteur thinks this objective (together with above two proposals) are under RAN1’s discussion. From RAN2 perspective, we can wait for RAN1’s conclusion first and update our spec if needed.

Q5: Do companies agree to wait for RAN1 about the RRC configuration and UE capability for PDCCH on CRS (Objective 1 in WID)?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes (wait for RAN1) | Based on the information we got from our RAN1, these proposals are now discussed in RAN1 this meeting, under [110bis-e-R18-Others-01]. We think RAN2 can wait for RAN1, if there is RAN2 impact, RAN1 will inform RAN2 as usual. |
| Huawei, HiSilicon | Yes | It is RAN1 issue, which should be discussed first in RAN1. If RAN1 cannot conclude anything, RAN2 has no point to specify anything relevant. |
| vivo | Yes | As commented in Q1, we prefer it being discussed in RAN1 first. |
| Xiaomi | Yes | Agree with ZTE that RAN1 is discussing the same proposals. |

If answers “No” to above question, then companies are invited to show your views to the proposals.

Q6: If answers “No” to Q5, do companies agree with Proposal 1 and Proposal 2?

| Company | Yes or No | Comments |
| --- | --- | --- |

# Conclusion

TBD

# References

- R2-2209314	LS to RAN2 on two overlapping LTE-CRS patterns in Rel-18 DSS (R1-2208194; contact: ZTE)	RAN1	LS in	Rel-18	NR_DSS_enh	To:RAN2

- R2-2210636	Work plan for Rel18 WI on Enhancement of NR Dynamic spectrum sharing (DSS)	Ericsson	discussion

- R2-2210133	RRC configuration and UE capability for PDCCH on CRS	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_DSS_enh

- R2-2210297	Discussion on two overlapping LTE-CRS patterns in Rel-18 DSS	ZTE Corporation, Sanechips, Ericsson	discussion	Rel-18	NR_DSS_enh-Core

- R2-2210586	Clarification on the DSS UE capability	Xiaomi	CR	Rel-16	38.306	16.10.0	0818	-	F	TEI16

- R2-2210587	Clarification on the DSS UE capability	Xiaomi	CR	Rel-17	38.306	17.2.0	0819	-	A	TEI16