3GPP TSG-RAN WG2 Meeting #120		R2-2211353

Toulouse, France, 14th - 18th November 2022

Source:	CATT

Title:	RACH enhancement for SON

Agenda Item:	8.13.6

Document for:	Discussion and Decision

Introduction

In last meeting, some agreements are made for the RACH partitioning, and SgNB RACH report for MR-DC scenarios has no time to discuss online and only a post email is provided. The agreements are listed as below [1]:

Agreements:

For RACH report about RACH partitioning information

1	Agree to add the following parameters into RACH report for RACH partitioning:

-	Feature or the combination of features that triggered the RACH

-	Used feature combination

In this contribution, we analyse the appropriate MR-DC scenarios and the solution for SgNB RACH report, and also the issues on RACH report for RACH partitioning.

Discussion

SgNB RACH report for MR-DC scenarios

RAN2 has reached an agreement that only focus on the leftover issues for completing the whole work which partly done in R17 in RAN3. An LS to RAN3 [2] is also sent to RAN3 to ask the support of (NG)EN-DC and NE-DC scenarios for clarification.

From the RAN3 reply LS [3], and to check with RAN3 specification and the discussion in R17, it is clear that NE-DC scenario is not in the scope of RAN3 RACH report optimization consideration. To fulfill the RAN3 SgNB RACH report requirement, and to align the RAN2 and RAN3 procedures, it is suggest supporting the SgNB RACH report of (NG)EN-DC scenario in R18.

Observation 1: NE-DC scenario is not in the RAN3 discussion scope of RACH report in R17.

Proposal 1: Study and support the SgNB RACH report of EN-DC and NG-EN-DC scenarios in R18.

Additionally, available indicator to the SN node RACH report is suggested by some companies in RAN2. But the RAN3 reply LS [3] has confirmed that both the MN and the SN RACH information are reported to the MN node. From RAN2 point of view, the UE may not always be connected to a SN node, so to report all information to the MN node is reasonable.

A network solution has been assumed in RAN3 that for indicating the RACH occurrence from gNB-DU to gNB-CU or the potential availability of RA report from SN to MN, some indicators may be needed between network nodes, therefore it is not necessary to introduce the available indicator or reporting to the SN node from the UE side.

Observation 2: RAN3 LS has confirmed both the MN and the SN RACH information are reported to the MN node.

Proposal 2: UE reports both the MN and the SN RACH information to the MN node for EN-DC and NG-EN-DC scenarios similar as the NR-DC scenario, and no available indicator to SN is needed.

For the EN-DC and NG-EN-DC scenarios, in RAN2 the same UE behaviour will be performed for UE RACH report in the air-interface which only impact TS36.331.

Current RACH Report in TS36.331 only supports simple parameters report such as the number of preambles and whether the contention is detected (For normal UE).

RACH-Report-r16 ::=					SEQUENCE {

numberOfPreamblesSent-r16			NumberOfPreamblesSent-r11,

contentionDetected-r16				BOOLEAN

}

RACH-Report-v1610 ::=	SEQUENCE {

initialCEL-r16 					INTEGER (0..3),

edt-Fallback-r16					BOOLEAN

}

To support the SgNB RACH information and report to the MN, the UE should include the NR SgNB RACH information in LTE specification, the format could be an NR container. Two detailed options could be considered:

- MN(LTE) could be enhanced to decode the NR container to get the target cell ID, and then send the RACH information to the correct SgNB (in which the RACH occurred);

- UE send the cell ID in LTE format together with the NR container to the MN. The MN does not need to decode the NR container, and send the container to the correct SgNB based on the cell ID. This option needs UE to report multiple containers for different SgNBs/cells respectively.

Compare with the 2 options, it is simpler to use option2 to assist the MN sending the NR container to the correct SN node which does not need the updated MN to decode the NR container [4].

Proposal 3: Includes NR container in LTE RACH Report to enhance the SgNB UE RACH Report for EN-DC and NG-EN-DC scenarios.

Proposal 4: Includes the cell ID in which the RACH occurred in LTE format together with the NR container to assist the network forwarding the SgNB RACH report content for EN-DC and NG-EN-DC scenarios.

Since in LTE, the RA report is almost an immediate report after UE has connected to a cell, the is no variable about RA report. Therefore to support the SgNB RA report mechanism, an additional Variable should be introduced into TS36.331.

Proposal 5: A new UE variable about SgNB RACH report information should be introduced in TS36.331.

For NR-DC scenario, there is no additional UE capability introduced for the SgNB RACH report since the RACH information of MN and SN use the same encode format and share the same mechanism and the UE variable. But for EN-DC and NG-EN-DC scenarios, if the NR container is introduced, additional UE capability may be needed to identify whether the inter-RAT record and report of RACH information could be supported.

Proposal 6: Additional capability may be needed for NR RACH Report enhancement in LTE for EN-DC and NG-EN-DC scenarios.

RACH report about RACH partitioning information

In RAN2119bis-e meeting, some preliminary agreements were made on RACH report about RACH portioning information.

Agreements:

For RACH report about RACH partitioning information

1	Agree to add the following parameters into RACH report for RACH partitioning:

-	Feature or the combination of features that triggered the RACH

-	Used feature combination

It is beneficial to report the feature/feature combination that triggers the RACH and used in the RACH procedure. But there are some other information can be reported to the network for further optimization.

RACH resource configuration for RACH partitioning

In Rel-17 RACH partitioning, multiple RACH resources are configured for different RACH partitioning in order to enable early identification of the RACH feature or RACH feature combination on the network side. But it may be happened that the UE triggers one feature or feature combination, but there is no corresponding RACH resource. Based on current spec procedure, the UE will select another RACH resource to initiate RACH procedure based on the used feature combination. So, it is useful to report the RACH resources for the configured RACH feature/feature combination, i.e., the RACH resource configuration for RACH partitioning.

Proposal 7: The UE reports the RACH resource configuration for feature/feature combination for SON.

FeaturePriorities for RACH partitioning

In RACH partitioning procedure, he UE will select the RACH resources with a feature which has the highest priority among the features applicable to the RACH procedure. For example, when the SDT (Small data transmission), Msg3 repetition and NSAG (Network Slice AS Group) 1 are applicable to the RACH procedure, the UE will compare the featurePriorities of each feature. And assuming the featurePriorities for NSAG1 is the feature with highest priority, the UE will select the RACH resource corresponding to NSAG1 feature. Hence, it is useful to report the featurePriorities to the network to figure out why the mismatch happens between triggered feature/feature combination that triggered in the RACH and the feature/feature combinations that used in the RACH.

Hence, we propose that:

Proposal 8: The UE indicates the featurePriorities information for feature /feature combination.

Conclusion

According to the analysis in section 2, we propose:

For SgNB RACH report for MR-DC

Observation 1: NE-DC scenario is not in the RAN3 discussion scope of RACH report in R17.

Proposal 1: Study and support the SgNB RACH report of EN-DC and NG-EN-DC scenarios in R18.

Observation 2: RAN3 LS has confirmed both the MN and the SN RACH information are reported to the MN node.

Proposal 2: UE reports both the MN and the SN RACH information to the MN node for EN-DC and NG-EN-DC scenarios similar as the NR-DC scenario, and no available indicator to SN is needed.

Proposal 3: Includes NR container in LTE RACH Report to enhance the SgNB UE RACH Report for EN-DC and NG-EN-DC scenarios.

Proposal 4: Includes the cell ID in which the RACH occurred in LTE format together with the NR container to assist the network forwarding the SgNB RACH report content for EN-DC and NG-EN-DC scenarios.

Proposal 5: A new UE variable about SgNB RACH report information should be introduced in TS36.331.

Proposal 6: Additional capability may be needed for NR RACH Report enhancement in LTE for EN-DC and NG-EN-DC scenarios.

For RACH report about RACH partitioning

Proposal 7: The UE reports the RACH resource configuration for feature/feature combination for SON.

Proposal 8: The UE indicates the featurePriorities information for feature /feature combination.

Reference

- RAN2#119bis-e Meeting Report-V1.docx

- R2-2209024 LS on SN RACH report status in R17 RAN2

- R2-2211164/R3-226053 Reply LS on SN RACH report status in R17 RAN3

- R2-2200394 Specification Impact of SgNB RACH Report on TS38.331 and TS36.331 CATT