3GPP TSG-RAN WG2 #120	R2-2211953

Toulouse, France, November 2022

Agenda Item:	8.3.2

Source:	OPPO

Title:	Discussion on DTX/DRX mechanism

Document for:	Discussion, Decision

Introduction

This paper will provide our considerations on DTX/DRX mechanism, for which RAN2#119bis-e meeting reached the following agreements.

=>	RAN2 assumes that the options above are possible for gNB DTX/DRX behavior and discuss UE RAN2 behavior/impact during the DTX/DRX.

=>	For the purpose of our discussion we will focus on a single UE behavior at any point in time.  FFS if we allow multiple configuration of NW DRX/DTX behaviors.

=>	Periodic DTX is assumed as a baseline.  The gNB provides indication to UE about NW DTX mode/configuration via dedicated dynamic L1/L2 signaling.

=>	Dynamic L1/L2 group signalling from NW to provide NW DTX mode/configuration is also considered in RAN2

=>	It is beneficial to align UE DRX with network DTX and DRX alignment among multiple UEs.  Details are FFS, including UE transmission/reception behavior during DTX.  RAN2 to study the alignment.

Discussion

In the RAN2#119bis-e meeting, periodic Cell DTX is assumed as a baseline. The network can provide an indication of Cell DTX via dynamic L1/L2 signalling. As described in TR 38.864 and supported by many companies in the TR discussion, the periodic Cell DTX/DRX is configured by the gNB via RRC signalling. Accordingly, one aspect to be clarified is the relationship between L1/L2 signalling and RRC configuration [2], i.e.

Whether L1/L2 signalling can be used to configure the DTX/DRX pattern, or only be used to activate the RRC-configured DTX/DRX pattern:

-	Alt-1: Allow both periodic pattern (configured by RRC) and one-shot pattern (configured by L1/L2 signalling)

-	Alt-2: Allow periodic pattern, configured by RRC and activated by L1/L2 signalling

- On Cell DTX/DRX, one aspect to be clarified is the relationship between L1/L2 signalling and RRC configuration.

In our understanding, both alternatives are applicable. In detail,

- Alt-1, i.e. independent operation between L1/L2 signalling and RRC configuration:

In our view, if Cell DTX/DRX pattern is aperiodic, or the gNB just would like to have a one-shot pattern, or the gNB wants to temporally adapt the DRX/DTX pattern, the gNB can indicate Cell DTX/DRX pattern by dynamic signalling. Such Cell DTX/DRX pattern indicated by L1/L2 signalling may indicate the time that the UE enters the Cell DTX/DRX non-active mode and how long the UEs kept in that non-active mode. Otherwise, if it is a periodic Cell DTX/DRX pattern, it is natural that gNB configures Cell DTX/DRX pattern via RRC reconfiguration. Such Cell DTX/DRX pattern indicated by RRC may contain the Cell DTX/DRX periodicity, the start offset and the duration of active/non-active mode.

- Alt-2, i.e. combined operation between L1/L2 signalling and RRC configuration:

In the RAN1’s evaluation, several network power states are considered including Deep sleep, Light sleep, Micro sleep, Active UL and Active DL. For each individual network power state, RAN1 assumes individual transition delay between the active mode and non-active mode and the corresponding network energy-saving gain (See table below). The different transition delays in our understanding imply different Cell DTX/DRX patterns. Thus, the gNB may use RRC signalling to configure the UEs with multiple Cell DTX/DRX configurations for different network power states respectively and then activate one of them by using L1/L2 signalling (L1/L2 signalling needs to include Cell DTX/DRX configuration index to be activated). As a result, the UE can use the activated Cell DTX/DRX configuration for transmission/reception.

Agreement

For the purpose of evaluation, adopt the following as BS power consumption model. These entries for this table is per reference configuration set.

- FFS: One or multiple values for relative power and transition time.

| Power state | Characteristic | Relative Power | Additional transition energy3 | Total transition time |
| --- | --- | --- | --- | --- |
| Deep sleep1 | There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. | P1=1 | E1 | T1 |
| Light sleep | There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. (P2>P1) | P2 | E2 | T2 |
| Micro sleep | There is neither DL transmission nor UL reception. Immediate transition is assumed for network energy saving study purpose from or to a non-sleep state. | P3 | 0 | 0 |
| Active DL | There is only DL transmission. | P4 | NA | NA |
| Active UL | There is only UL reception. FFS: Whether multiple P5 values are needed to address low power UL mode | P5 | NA | NA |
| Note 1: Depending on implementations, there could be a state that the power is lower than deep sleep and requires larger total transition time, e.g. hibernating sleep or Quasi-off, which is not explicitly modeled in this study for evaluation purpose. Note 3: Unit in relative power times duration. FFS: Details on how transition energy is defined. |  |  |  |  |

Another method is that a Cell DTX/DRX configuration is configured via RRC and activated only when receiving the L1/L2 signalling, we are not sure whether it is really needed, since in our understanding the gNB can directly de-configure or re-configure such Cell DTX/DRX configuration if this Cell DTX/DRX configuration is not suitable any more.

RAN2 allows multiple sets of Cell DTX/DRX configuration in RRC.

RAN2 confirms both alternatives below can be considered for Cell DTX/DRX configuration indication.

- Alt1: Allow both periodic pattern (configured by RRC) and one-shot pattern (configured by L1/L2 signalling).

- Alt2: Allow multiple periodic patterns configured by RRC and one of them is activated by L1/L2 signalling.

Periodic Cell DTX/DRX pattern in the RRC configuration includes the periodicity, the start offset and the duration of active/non-active mode. Additionally, if multiple Cell DTX/DRX patterns are supported, the index of the Cell DTX/DRX pattern is also indicated in the RRC configuration.

for Alt1, Cell DTX/DRX pattern in L1/L2 signalling includes the start offset and the duration of non-active mode.

In the RAN2#119bis-e meeting, RAN2 assumes the following options for Cell DTX/DRX behaviour and would further study the corresponding UE behaviour/impact.

- Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods.

- Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals)

- Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

- Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

Compared with the above table on the BS power consumption model which is evaluated in RAN1, one potential behaviour is no information to be transmitted or received between the gNB and the UE during Cell DTX/DRX non-active periods. In other words, nothing interacts between the UE and the gNB during these periods. From our perspective, Example 1 seems more aligned with this intended behaviour, but we are not sure if Example 1 covers turning off both the SIB and SSB. If not, we prefer RAN2 to take this behaviour as an additional example.

RAN2 clarifies whether Example 1 covers no transmission of SIB and SSB.

Typically, the UE should follow the way that the gNB is expected when Cell DTX/DRX is configured/enabled, no matter whether the dedicated UE DRX is configured or not. In other words, the UE behaviour on Cell DTX overrides the UE behaviour on UE DRX. It means that the Cell DTX non-active period will be always the inactive time for UE (in case DRX is configured) and UE DRX active time is only possible in the Cell DTX active period.

In the case that Cell DTX/DRX is configured/enabled, RAN2 assumes that the UE behaviour is aligned with gNB expected Cell DTX/DRX behaviour, i.e. the Cell DTX non-active period will be always the inactive time for UE (in case the UE DRX is configured) and UE DRX active time is only possible in the Cell DTX active period.

Conclusion

We have the following observations:

Observation 1	On Cell DTX/DRX, one aspect to be clarified is the relationship between L1/L2 signalling and RRC configuration.

We have the following proposals:

Proposal 1	RAN2 allows multiple sets of Cell DTX/DRX configuration in RRC.

Proposal 2	RAN2 confirms both alternatives below can be considered for Cell DTX/DRX configuration indication.

•	Alt1: Allow both periodic pattern (configured by RRC) and one-shot pattern (configured by L1/L2 signalling).

•	Alt2: Allow multiple periodic patterns configured by RRC and one of them is activated by L1/L2 signalling.

Proposal 3	Periodic Cell DTX/DRX pattern in the RRC configuration includes the periodicity, the start offset and the duration of active/non-active mode. Additionally, if multiple Cell DTX/DRX patterns are supported, the index of the Cell DTX/DRX pattern is also indicated in the RRC configuration.

Proposal 4	for Alt1, Cell DTX/DRX pattern in L1/L2 signalling includes the start offset and the duration of non-active mode.

Proposal 5	RAN2 clarifies whether Example 1 covers no transmission of SIB and SSB.

Proposal 6	In the case that Cell DTX/DRX is configured/enabled, RAN2 assumes that the UE behaviour is aligned with gNB expected Cell DTX/DRX behaviour, i.e. the Cell DTX non-active period will be always the inactive time for UE (in case the UE DRX is configured) and UE DRX active time is only possible in the Cell DTX active period.

Reference

[1] RP-213554  New SI: Study on network energy savings for NR

[2] R2-2211066, Report of [POST119bis][303][NES] TP on NW DTX/DRX (Huawei/Apple)

Annex – TP for Cell DTX/DRX

### 6.1.1	Cell DTX/DRX

- 6.1.1.x	Higher layer procedures

Cell DTX/DRX is applied to at least UEs in RRC_CONNECTED state. A periodic Cell DTX/DRX (i.e., active and non-active periods) can be configured by gNB via RRC signalling. Below examples on Cell DTX/DRX behaviour during non-active periods are assumed to be possible options, and the UE behaviour/impact will be studied:

- Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods.

- Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals)

- Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

- Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

The study will focus on UE behavior when at any point in time the cell activates a single DTX/DRX configuration.

Editor's note: FFS if multiple sets of Cell DRX/DTX configuration are allowed.

The Cell DTX mode/configuration can also be indicated to the UE via dynamic L1/L2 signalling. The dynamic L1/L2 signalling at least supports UE dedicated indication. Whether UE group common signalling is also supported will be further studied. In the case that multiple sets of Cell DTX/DRX are configured in RRC signalling, one of them can be activated via L1/L2 signalling.

It is beneficial to align UE DRX with Cell DTX and DRX alignment among multiple UEs. The alignment mechanism will be studied. In the case that Cell DTX/DRX is configured/enabled, the UE behaviour is aligned with gNB expected Cell DTX/DRX behaviour, i.e. the Cell DTX non-active period will be always the inactive time for UE (in case the UE DRX is configured) and UE DRX active time is only possible in the Cell DTX active period.

Editor's note: FFS details of alignment, including UE transmission/reception behavior during DTX.

- 6.1.1.y	Assistance information from UE side

Editor's note: will be updated once more agreements are made.

#### 6.1.1.z  Impacts on network interfaces

Editor's note: will be updated once more agreements are made.