3GPP TSG-RAN WG2 Meeting #120	R2-2212403

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.7.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Further on network verified UE location

WID/SID:	NR_NTN_enh - Release 18

Document for:	Discussion and Decision

# 1	Introduction

During RAN2#119bis the following was agreed in respect to network verified UE location

Agreements:

- RAN2 assumes that the network is able to compute possible UE locations independently from the GNSS location reported by UE

- RAN2 assumes that the UE location verification procedure can be triggered by the CN and it is up to the CN to decide when to trigger the procedure

- RAN2 should consider in priority the NGSO case with earth moving and earth fixed beams for the definition of the UE location verification procedure

- Multi-connectivity involving multiple NTN NG-RAN nodes or NTN NG-RAN node and TN NG-RAN node is not part of the Rel-18 study on UE location verification

- RAN2 assumes that the verification of the consistency (within 5-10 km) between the actual reported UE location with the UE location(s) computed by the network is up to the 5GC. (this doesn’t mean that RAN2 has nothing to do for this WI objective)

This Tdoc further discuss the topic, in respect to the challenges of not having multi-connectivity, as well as the ambiguous denotation that it is up to CN to trigger the verification.

# 2	Heading 1

## 2.1	Methods

Although the methods of calculating the position used to verify the UEs reported GNNS position are being under discussion in RAN1, some challenges can already be foreseen to have RAN2 to handle. As most positioning methods like multi-RTT, DL/UL-TDOA relies on triangulation and require at least 3 reference points, a problem arise when using only satellite. This issue occur as the three points are on a single orbit, i.e. on a single line, in the end leading to two potential position points or areas.

As mentioned in RAN2#119bis, this can be problematic when being used for determining for example the location of a UE in a country as can be seen in the examples in Figure 1. In the Figures an area of south Denmark and the Northern part of Germany can be seen with different orbits (yellow lines). The real location of the UE is the blue star (in Denmark). The green and red stars are the points which also are found when using any of the above-mentioned positioning methods and the network does not know which one the one is true. In other words, a malicious UE can copy its location to the mirror-point on the other side of the orbital plane. In the examples below 3 of those are in another country (Germany, the green points), while one (the red point) is still in the same country (Denmark).

Observation

Proposal

Figure 1 examples of the real location (blue star) and mirror points (green and red points) in case of multi-RTT and UL/DL-TDOA. Country boarder is marked with yellow thin line 4-5 grid lines from the bottom of the map.

Generally, the so called “mirror points” are relevant in only a subset of the network verified UE location procedure, which we can therefore split up in 3 scenarios:

- Reporting UE reporting GNSS positioned on or very close to the orbital line, meaning no solution should be necessary.

      - In such case, the two mirror points are very close together i.e., the distance is less than the required verification distance of 5-10 km apart, so no differentiation is needed, as the required accuracy for the verification will not be (has no need to be) able to distinguish the two positions.

- The UE is reporting a GNSS position with a distance from the centre smaller than the cell radius, but larger than verification distance (5-10km), thus requiring the network to verify the location in case of i.e., nearby borders etc.

      - In this case, the UEs reported location may be in either of the two mirror-points within the cell, and thus if the given cell covers a restrictive area, such as a country border, the network verification is required to ensure correct network behaviour.

- The UE is reporting the GNSS position ‘much’ further than a cell radius away from the cell centre, and verification may not be required/easily obtainable, as the UE would most likely not connect to the given cell, but another one, causing the scenario to be closer to scenario 2.

Proposal 1: RAN2 to agree that the mirror point issue exists and that RAN2 should work on solutions.

For NTN, it is most likely that each satellite will cover several(i.e. 38.821) cells, with different beams such that fewer satellites are required for the same coverage, and each of these cells will have its own unique cell ID, which will be known to the CN through signalling from gNB. To eliminate the mirror point, other/additional information is required, while still being based on input from the UE. One possible solution could be to utilise neighbour cell measurements within the verification procedure, to at least eliminate the mirror point issue discussed.

Observation 1: A single satellite provides multiple cells through separate beams.

Observation 2: Each cell will have its own unique cell ID known to the gNB.

Proposal 2: Neighbour cell measurements may be utilised in the UE location verification procedure to at least eliminate the mirror point issue.

Network verified UE location is mainly used for verification and should thereby rely on trustworthy information. To ensure the validity the network may control the verification procedure in terms of i.e., number of required network and/or UE measurements in case the UE is placed further away from the orbital line. Through this control, the network should be able to ensure the validity of the information provided by the UE, as well as ensuring the requirement(s) for the uncertainty area to be calculated with enough precision to verify the results.

Observation 3: The precision of the network position estimate is based on the quality and/or quantity of the measurements obtained.

Observation 4: If the network controls neighbour measurements, verification can be trusted more.

Proposal 3: RAN2 to agree the network should have the ability to at least control the measurement procedure used for the UE position verification procedure at least by configuring measurement duration and periodicity.

Proposal 4: Discuss whether/how network should be able to configure the measurements i.e. by requesting UE to measure from multiple cells.

Some disadvantages of including multiple cells within the measurement procedure are increased battery consumption of the UE, as well as more overhead/complexity of the message used to transmit the measurements to the network. A potential solution to this, could be to allow the UE to process the cell measurements based on certain rules from the network, and hereby compress the measurements before transmitting.

Proposal 5: RAN2 to discuss whether/how neighbour cell measurements can be processed at UE side to limit overhead.

# 3	Conclusion

This documents has made the following observations and proposals:

Proposal 1: RAN2 to agree that the mirror point issue exists and that RAN2 should work on solutions.

Observation 1: A single satellite provides multiple cells through separate beams

Observation 2: Each cell will have its own unique cell ID known to the gNB

Proposal 2: Neighbour cell measurements may be utilised in the UE location verification procedure to at least eliminate the mirror point issue

Observation 3: The precision of the network position estimate is based on the quality and/or quantity of the measurements obtained.

Observation 4: If the network controls neighbour measurements, verification can be trusted more.

Proposal 3: RAN2 to agree the network should have the ability to at least control the measurement procedure used for the UE position verification procedure at least by configuring measurement duration and periodicity.

Proposal 4: Discuss whether/how network should be able to configure the measurements i.e. by requesting UE to measure from multiple cells.

Proposal 5: RAN2 to discuss whether/how neighbour cell measurements can be processed at UE side to limit overhead