3GPP TSG-RAN WG2 #120	Tdoc R2-2212794

Toulouse, France, 14th-18th November

Agenda Item:	6.17.2

Source:	Ericsson

Title:	Misc clarifications for feMIMO RRC

Document for:	Discussion

# 1	Reply LS on active TCI state list for UL TCI

| R2-2211152 | LS in | Reply LS on active TCI state list for UL TCI (R1-2210719; contact: Nokia) |
| --- | --- | --- |

The LS contains the following responses from RAN1 to RAN4 and RAN2 is in CC:

Question 1: What is the relationship between qd and active UL (or joint) TCI state list? How does network configure the RS resource indexes qd?

Answer 1: According to 38.213, “the RS index for obtaining the downlink pathloss estimate for PUSCH,

PUCCH, and SRS transmission is provided by PL-RS associated with or included in the indicated TCIState or

UL-TCIstate except for SRS transmission that is not provided followUnifiedTCIstateSRS”

[...]

”if followUnifiedTCIstateSRS is not provided for a SRS resource set and for a SRS resource from the SRS

resource set, [...] a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by

PL-RS associated with or included in the TCIState or UL-TCIState of an SRS resource with

lowestSRS-ResourceId in the SRS resource set”

Question 2: Is UE expected to maintain the pathloss RSs of all the pathloss reference RS in the active UL (or

joint) TCI list?

Answer 2: A UE is expected to maintain the pathloss RSs of all active UL (or joint) TCI states. But the total

number of different path-loss RSs of all active UL (or joint) TCI states is not expected to be larger than 4.

Question 3: What is the UE behavior when number of active UL (or joint) TCI states is larger than 4 if

corresponding pathloss RSs need to be maintained?

Answer 3: The UE is not required to maintain more than 4 PL-RS. The same PL-RS can be used in

more than one UL or Joint TCI states.

The second response looks like it should be captured in RRC. The pathloss reference RSs are configured in IE BWP-UplinkDedicated according to the IPA CR for feMIMO in R2-2212781. The limitation indicated in the LS could be stated in the field description of pathlossReferenceRSToAddModList  in IE BWP-UplinkDedicated e.g. as follows:

pathlossReferenceRSToAddModList

A list of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for path loss estimation for PUSCH, PUCCH and SRS for unified TCI state operation. If unifiedTCI-StateType is not configured for the serving cell, the network does not configure any RS in this list and deletes them all if there was any previously configured. Up to 4 different reference signals is configured for a UE across all serving cells.

RAN2 to add restriction in field description of pathlossReferenceRSToAddModList that only 4 different RS can be configured for a UE.

# 2	CSI resource configuration

The channel part for CSI resources is configured with field nzp-CSI-RS-SSB which may include CSI-RS resources or SSB resources or both.

The nzp-CSI-RS-ResourceSets are configured in IE CSI-resourceConfig with field nzp-CSI-RS-ResourceSetListList and the SSB resources with csi-SSB-ResourceSetList and csi-SSB-ResourceSetListExt.

#### –	CSI-ResourceConfig

The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet.

CSI-ResourceConfig information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIG-START

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]]

}

-- TAG-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

CSI-ResourceConfig field descriptions

bwp-Id The DL BWP which the CSI-RS associated with this CSI-ResourceConfig are located in (see TS 38.214 [19], clause 5.2.1.2.

csi-IM-ResourceSetList List of references to CSI-IM resources used for CSI measurement and reporting in a CSI-RS resource set. Contains up to maxNrofCSI-IM-ResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise (see TS 38.214 [19], clause 5.2.1.2).

csi-ResourceConfigId Used in CSI-ReportConfig to refer to an instance of CSI-ResourceConfig.

csi-SSB-ResourceSetList, csi-SSB-ResourceSetListExt List of references to SSB resources used for CSI measurement and reporting in a CSI-RS resource set (see TS 38.214 [19], clause 5.2.1.2). If groupBasedBeamReporting-v1710 is configured in the IE CSI-ReportConfig that indicates this CSI-ResourceConfig as resourceForChannelMeasurement, the network configures 2 resource sets, which may be two NZP CSI-RS resource sets, two CSI SSB resource sets or one NZP CSI-RS resource set and one CSI-SSB resource set (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B: - if the list has one CSI-SSB resource set, this resource set is indicated by a resource set indicator set to 1, while the resource set indicator of the NZP CSI-RS resource set is 0; - if the list has two CSI-SSB resource sets, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1.

nzp-CSI-RS-ResourceSetList List of references to NZP CSI-RS resources used for beam measurement and reporting in a CSI-RS resource set. Contains up to maxNrofNZP-CSI-RS-ResourceSetsPerConfig resource sets if resourceType is 'aperiodic'. Otherwise, contains 1 resource set when groupBasedBeamReporting-v1710 is not configured in IE CSI-ReportConfig. If groupBasedBeamReporting-v1710 is configured and resourceType is set to 'periodic' or 'semipersistent', then the network configures 2 resource sets, which may be two NZP CSI-RS resource sets, two CSI SSB resource sets or one NZP CSI-RS resource set and one CSI-SSB resource set (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B, the following applies: - if the list has one NZP CSI-RS resource set, this resource set is indicated by a resource set indicator set to 0; - if the list has two NZP CSI-RS resource sets, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1.

resourceType Time domain behavior of resource configuration (see TS 38.214 [19], clause 5.2.1.2). It does not apply to resources provided in the csi-SSB-ResourceSetList.

From the current field description of nzp-CSI-RS-ResourceSetList, it is not clear that when groupBasedBeamReporting-v1710 is not configured, nzp-CSI-RS-ResourceSetList contains 1 resource set only when if resourceType is set to 'periodic' or 'semipersistent'. Hence, we suggest to clarify this e.g. as follows:

nzp-CSI-RS-ResourceSetList

List of references to NZP CSI-RS resources used for beam measurement and reporting in a CSI-RS resource set. Contains up to maxNrofNZP-CSI-RS-ResourceSetsPerConfig resource sets if resourceType is 'aperiodic'. Otherwise, contains 1 resource set when groupBasedBeamReporting-v1710 is not configured in IE CSI-ReportConfig and resourceType is set to 'periodic' or 'semipersistent'. If groupBasedBeamReporting-v1710 is configured and resourceType is set to 'periodic' or 'semipersistent', then the network configures 2 resource sets, which may be two NZP CSI-RS resource sets, two CSI SSB resource sets or one NZP CSI-RS resource set and one CSI-SSB resource set (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B, the following applies:

- if the list has one NZP CSI-RS resource set, this resource set is indicated by a resource set indicator set to 0;

- if the list has two NZP CSI-RS resource sets, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1.

RAN2 to clarify in the  field description of nzp-CSI-RS-ResourceSetList that when groupBasedBeamReporting-v1710 is not configured, nzp-CSI-RS-ResourceSetList contains 1 resource set only when if resourceType is set to 'periodic' or 'semipersistent'.

In Release-16 and earlier the csi-SSB-ResourceSetList could contain only one SSB. In Release 17 this is extended by csi-SSB-ResourceSetListExt. The extension can only be used when groupBasedBeamReporting-v1710 is configured. Further, the csi-SSB-ResourceSetListExt can only be configured with csi-SSB-ResourceSetList. This clarification is preferred as the original list is within a CHOICE structure and Ext list is on “main level”. Hence, we suggest to add these clarifications e.g. as follows:

csi-SSB-ResourceSetList, csi-SSB-ResourceSetListExt

List of references to SSB resources used for CSI measurement and reporting in a CSI-RS resource set (see TS 38.214 [19], clause 5.2.1.2). The csi-SSB-ResourceSetListExt provides additional references if csi-SSB-ResourceSetList is included and groupBasedBeamReporting-v1710 is configured.

If groupBasedBeamReporting-v1710 is configured in the IE CSI-ReportConfig that indicates this CSI-ResourceConfig as resourceForChannelMeasurement, the network configures 2 resource sets, which may be two NZP CSI-RS resource sets, two CSI SSB resource sets or one NZP CSI-RS resource set and one CSI-SSB resource set (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B:

- if the list has one CSI-SSB resource set, this resource set is indicated by a resource set indicator set to 1, while the resource set indicator of the NZP CSI-RS resource set is 0;

- if the list has two CSI-SSB resource sets, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1.

RAN2 to clarify the field description of csi-SSB-ResourceSetList, csi-SSB-ResourceSetListExt that csi-SSB-ResourceSetListExt can be configured only when if csi-SSB-ResourceSetList is included and groupBasedBeamReporting-v1710 is configured.

# 3	reportQuantity in IE CSI-ReportConfig

In Release-17 an extention for reportQuantity was specified in IE CSI-ReportConfig. However, the field description has not been updated for Release 17 extenion

reportQuantity-r17                  CHOICE {

cri-RSRP-Index-r17                  NULL,

ssb-Index-RSRP-Index-r17            NULL,

cri-SINR-Index-r17                  NULL,

ssb-Index-SINR-Index-r17            NULL

}

reportQuantity

The CSI related quantities to report. see TS 38.214 [19], clause 5.2.1. If the field reportQuantity-r16 is present, UE shall ignore reportQuantity (without suffix).

RAN2 to add in field description of reportQuantity in IE CSI-ReportConfig that If the field reportQuantity-r17 is present, UE shall ignore reportQuantity-r16 and reportQuantity (without suffix).

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to add restriction in field description of pathlossReferenceRSToAddModList that only 4 different RS can be configured for a UE.

Proposal 2	RAN2 to clarify in the  field description of nzp-CSI-RS-ResourceSetList that when groupBasedBeamReporting-v1710 is not configured, nzp-CSI-RS-ResourceSetList contains 1 resource set only when if resourceType is set to 'periodic' or 'semipersistent'.

Proposal 3	RAN2 to clarify the field description of csi-SSB-ResourceSetList, csi-SSB-ResourceSetListExt that csi-SSB-ResourceSetListExt can be configured only when if csi-SSB-ResourceSetList is included and groupBasedBeamReporting-v1710 is configured.

Proposal 4	RAN2 to add in field description of reportQuantity in IE CSI-ReportConfig that If the field reportQuantity-r17 is present, UE shall ignore reportQuantity-r16 and reportQuantity (without suffix).