3GPP TSG-RAN WG2 Meeting #121	R2-2300253

Athens, Greece, February 27- March 3, 2023

Agenda Item:	8.16.2

Source:	NEC

Title:	AIML Model Transfer Requirements during Handover

Document for:	Discussion, Decision

# Introduction

In the last meeting, RAN2 assumed the model ID is used to identify an AI/ML model and also agreed to study CP-based and UP-based model transfer/delivery.

Agreement @RAN2#120:

R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery.

R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed).

For model transfer/delivery for AI/ML models (for the target use cases of this SI), RAN2 to study CP-based, UP-based solutions

There was also long email discussions organized for both model transfer/delivery and data collection (with focus on monitoring and training).

This paper discusses the potential AIML Model Transfer during UE mobility.

# Discussion

## Mobility Scenarios considering AIML support

During handover, there are different possibilities for the AIML capability from the network perspective. For example, the two gNBs (i.e., source gNB and target gNB) may be deployed by two different vendors, each using different AIML model for a AIML use case e.g., beam management. In addition, the source gNB and target gNB may be deployed by two different vendors, one is equipped with AIML technology, and the other is not, in which case, the Base station capabilities are different. The source gNB and target gNB may be deployed with different version of software by the same vendor, in which case, the Base station capabilities to run AIML algorithm are different.

From the UE perspective, the UE may move from one cell to another cell without AI/ML model change for a particular feature, or vice versa. The UE may stop the applicability of an AIML model for such feature when moving from one cell to another, or change to a different AI/ML model for the feature.

During UE mobility, the following scenarios should be supported by the UE:

- from non-AIML supporting gNB to AIML supporting gNB;

- from AIML supporting gNB to non AIML supporting gNB

- from one AIML supporting gNB to another AIML supporting gNB

In any case, in order to maximize the efficiency of the utilization of AIML algorithm, it is expected that the UE and the network should align the utilization of the AIML model for a particular feature.

Proposal-1: During UE mobility, the following scenarios should be supported by the UE:

- from non-AIML supporting gNB to AIML supporting gNB;

- from AIML supporting gNB to non AIML supporting gNB;

- from one AIML supporting gNB to another AIML supporting gNB.

In the following discussion, the third scenario is firstly assumed for this paper, but the first and the second scenarios should be also considered later.

## Requirements for AIML operation during handover

It is important to clarify the requirements for AIML operation during handover, in order to allow the UE to continue to use the AIML algorithm for a particular feature as much as possible. This may be very useful to ensure a consistent UE performance during the UE movement. Meanwhile, as the UE is expected to align with the network (gNB) on the operation of AIML algorithm, it would be quite essential to clarify if the UE is required to be ready to run the AIML algorithm as supported by the target gNB. For example, if the target gNB uses a different AIML for beam management from the source gNB, should the UE be ready to run the corresponding AIML model immediately after its access to the target gNB due to handover operation. This requirement should be clarified by RAN2.

Proposal-2: RAN2 to discuss if the UE is required to be ready to run the corresponding AIML model as supported by the target gNB immediately after its access to the target gNB due to handover operation.

## AIML capability exchange for mobility

In order to prepare the handover between source gNB and target gNB for the UE, source gNB and target gNB can exchange its AIML capabilities before the handover (e.g., during Xn setup). This may be carried by new Xn interface based message. With this information exchange, the source gNB will know if the target gNB will continue the AIML algorithm for a particular feature after the handover and may decide if there is a need to make necessary context transfer. In some scenarios, both source gNB and target gNB may support the AIML based operation for the same feature but may run different model. In this case, source gNB and target gNB may need to exchange its supported AIML model for such feature.

Proposal-3: RAN2 to assume that AIML capability exchange between the source gNB and target gNB is performed for UE mobility before handover and confirm with RAN3.

## AIML model transfer during handover

During the handover procedure, the target gNB may hold the AIML model by itself. It is also possible that the target gNB does not hold the AIML model, but there is a third party entity that hosts the AIML model. This third party entity can be another network entity or OTT server, depending on the AIML architecture assumptions. For example, in some centralized AIML model deployment architecture, a specific server may be deployed to train the AIML model and to host and maintain the AIML model, which is to be used within the whole wireless network.

In this case, RAN2 and RAN3 needs to discuss how the target gNB indicates to the UE the source of the AIML model, since the UE should download the corresponding AIML model as the target gNB before that can be used between the UE and the gNB.

Proposal-4: RAN2 to discuss how the UE downloads the AIML model (as supported by target gNB) from a (centralized) entity during handover.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: During UE mobility, the following scenarios should be supported by the UE:

- from non-AIML supporting gNB to AIML supporting gNB;

- from AIML supporting gNB to non AIML supporting gNB;

- from one AIML supporting gNB to another AIML supporting gNB.

Proposal-2: RAN2 to discuss if the UE is required to be ready to run the corresponding AIML model as supported by the target gNB immediately after its access to the target gNB due to handover operation.

Proposal-3: RAN2 to assume that AIML capability exchange between the source gNB and target gNB is performed for UE mobility before handover and confirm with RAN3.

Proposal-4: RAN2 to discuss how the UE downloads the AIML model (as supported by target gNB) from a (centralized) entity during handover.

# Reference

- R2-22NNNNN Summary of RAN2 post-meeting discussion on AIML on model transfer/delivery

- R2-22NNNNN Summary of RAN2 post-meeting discussion on data collection

- TR37.817