3GPP TSG-RAN WG2 #121	R2-2301003

Athens, Greece, Feb 27 – Mar 3, 2023

Agenda item:	8.13.4

Source:	Nokia, Nokia Shanghai Bell

Title:	SPR and SHR generation and reporting

WID/SID:	NR_ENDC_SON_MDT_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

At RAN2#120 the following agreements with respect to SPR configuration and retrieval were made:

- Only MN can retrieve the SPR from the UE.

- Regarding the configuration of the UE, RAN2 agree following options depending on which of nodes initiates SPR, i.e.:

  - For the MN-initiated PSCell Change/Addition, MN sends the SPR config to the UE

  - For the SN-initiated PSCell Change, the source-SN sends the Successful PSCell Change configuration within the container through MN.

  - T304 trigger needs to be configured by the target SN node.

Furthermore, some more SPR enhancements (other than LS-related discussions) were agreed as well:

Agreements:

- UE stores both SPCR and SHR configuration (one for each type at most) if received from NW.

- UE can send the (stored) SPR to gNB. FFS how long UE keeping SPR is FFS.

- Only the latest successful PSCell change/addition is reported by the UE.

- Random access related information is included in SPR at least when the SPR is triggered due to T304 exceeds the configured threshold. Other conditions are FFS.

- UE records/reports PCell SHR and PSCell SPR separately

RAN2#119bis e also agreed on how the SPR is to be retrieved by the network:

B.SPR is fetched via UE Information Request/Response procedure.

In this contribution, we discuss enhancements for SPR/SHR regarding the generation of the reports and SPR reporting mechanism to the network.

# 2	Discussion

## 2.1	SPR generation trigger

The application of SPR for conditional PSCell addition and change (CPAC) was considered in the discussion at the last meeting RAN2#120 with focus on optimization of the conditional reconfiguration of a PSCell addition/change.

When conditional PSCell change is configured, the UE starts evaluation of the prepared target PSCells according to the configured execution condition. Within the RLM process of the serving PSCell the signal quality is monitored by means of lower layer indications. If signal quality deteriorates during the evaluation phase that out-of-sync indications from lower layers are triggered and timer T310 is started, the mobility procedure is jeopardized to fail. If the execution condition of the conditional PSCell change is met before T310 expires, the UE avoids RLF. The conditional PSCell change was successful without making the network aware how risky cell change was. For such cases, SHR has been agreed as addition means for MRO, and SPR for the PSCell changes in NR-DC context is the natural extension of SHR.

Since only crucial issues during the cell change procedure should be addressed with SPR (as it is for SHR), a certain degree of outage, which is measured by exceeding a percentage threshold of the failure criterion resulting in an RLF, should have been reached; e.g., the parameter thresholdPercentageT310, the UE was configured with beforehand.

Observation 1: Crucial outages exceeding a certain percentage of failure criterion during the mobility procedure are triggering an SPR.

### 2.1.1	SPR for “almost too late” CPC execution

The case that the UE escapes with CPC execution from a RLF while T310 running (as depicted in Figure 1) is the intended case from MRO enhancement perspective. SPR was triggered by passing thresholdPercentageT310 (here exemplarily is configured with 40% of T310 duration) and CPC execution happens before T310 expires. Even though the CPC was successful, the SPR provides information of an “almost too late” execution, which can be used for MRO refinements.

Observation 2: SPR of outages where CPC execution rescued the UE are relevant MRO refinements.

Figure 1: SPR triggered while UE rescued by CPC execution and successful handover

However, it can also happen that RLM stops the timer T310 itself, since radio signal quality returns to reliable in-sync condition (i.e., lower layers are in-sync indications) by passing the Qin threshold after the SPR was triggered by passing thresholdPercentageT310. In this case, the UE has a robust link quality at the moment of the CPC execution (see Figure 2) and, therefore the SPR was not created when RLF almost happened as above. Therefore, the reported SPR will be wrongly interpreted and categorized as “almost too late” execution.

Observation 3: A SPR is created even though T310 is stopped by RLM and might be therefore wrongly interpreted.

Observation 4: The current SPR (also SHR) does not allow a distinction of the case when T310 has almost expired when CPC executed from the case when T310 has been stopped before CPC executed.

Figure 2: SPR has been triggered but not related to the cell change

Proposal 1: RAN2 agrees that current SPR procedure (as well as SHR procedure) does not provide means to distinguish the case when T310 has almost expired when CPC executed from the case when T310 has been stopped before CPC executed.

Proposal 2: RAN2 agrees to means how SPR (as well as SHR procedure) should be enhanced to enable the distinction of the case when T310 has almost expired when CPC executed from the case when T310 has been stopped before CPC executed.

## 2.2	SPR reporting mechanism

According to RAN2#119bis-e agreement the UEInformationRequest/UEInformationResponse mechanism will be used to fetch the SPR from the UE. The UE shall send the SPR report if available in the UEInformationResponse only when triggered by the UEInformationRequest. The UEInformationRequest with respect to SPR is sent to the UE only if UE has previously indicated the availability of SPR to the network.

SPR availability could be indicated using legacy mechanism such in the case of SHR, where the report availability is sent to the network via the RRCReconfigurationComplete message. However, in the dual connectivity scenario (as depicted in Fig. 3) during a PSCell change the RRCReconfigurationComplete message is sent to MN right after receiving the RRCConnectionReconfiguration and before the PSCell is accessed (successfully or not). At that stage the UE cannot know in advance if any of the configured thresholds for SPR generation have already been exceeded and thus cannot generate SPR.

Figure 3: SN change procedure – MN initiated (TS 37.340 Fig. 10.5.2-1)

Observation 5: SPR availability cannot be indicated to the network by using RRCReconfigurationComplete message as the latter is sent to MN too early.

Proposal 3: RAN2 discusses a different mechanism (other than indicating it in RRCReconfigurationComplete message) to indicate SPR availability to the network.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Crucial outages exceeding a certain percentage of failure criterion during the mobility procedure are triggering an SPR.

Observation 2: SPR of outages where CPC execution rescued the UE are relevant MRO refinements.

Observation 3: A SPR is created even though T310 is stopped by RLM and might be therefore wrongly interpreted.

Observation 4: The current SPR (also SHR) does not allow a distinction of the case when T310 has almost expired when CPC executed from the case when T310 has been stopped before CPC executed.

Proposal 1: RAN2 agrees that current SPR procedure (as well as SHR procedure) does not provide means to distinguish the case when T310 has almost expired when CPC executed from the case when T310 has been stopped before CPC executed.

Proposal 2: RAN2 agrees to means how SPR (as well as SHR procedure) should be enhanced to enable the distinction of the case when T310 has almost expired when CPC executed from the case when T310 has been stopped before CPC executed.

Observation 5: SPR availability cannot be indicated to the network by using RRCReconfigurationComplete message as the latter is sent to MN too early.

Proposal 3: RAN2 discusses a different mechanism (other than indicating it in RRCReconfigurationComplete message) to indicate SPR availability to the network.