3GPP TSG-RAN WG2 Meeting #121	R2-2301924

Athens, Greece, Feb 27th – Mar 3th, 2023

Agenda Item	8.2.2

Source	KT Corp.

Title		Discussion on Anchor UE (Re)discovery (Re)Selection for sidelink positioning

Document for	Discussion and Decision

WID/SID	NR_pos_enh2

1	Introduction

At the last RAN2 #120 meeting, the following proposals were agreed upon.

Agreements:

Proposal 13 (modified)	RAN2 confirm that from RAN2 perspective, it is feasible to send at least the following positioning signaling for groupcast/broadcast (in addition to unicast):

•	SL positioning capability (5)

•	SL positioning assistance data (6)

Location information is not excluded and can be further considered in normative work.

Proposal 14 (modified)	RAN2 to further discuss in normative work:

- the security issues (e.g., requirements for ciphering and/or integrity) on specific information of SL positioning capability and assistance data in groupcast/broadcast and consult to SA2 and SA3.

- the use cases for applying groupcast/broadcast.

LS to SA2/SA3 to indicate the agreement, that we are aware of SA2’s security concern, and inquire what security constraints would apply to transmission of SL positioning capability and distribution of assistance data by groupcast/broadcast.  Inquire of SA2 if they have identified groupcast/broadcast use cases.

Agreements:

Proposal 2 (modified)	RAN2 to confirm either of UEs including target UE and one or multiple anchor UEs may be OOC in partial coverage scenarios, but with at least one UE being in coverage. How to enable the procedures/signaling for supporting SL positioning in partial coverage will be further discussed in normative work.

Proposal 12	RAN2 to discuss the details of functionalities of LMF for supporting SL positioning in normative work.

Agreements:

UE roles are not captured in the diagram of the positioning architecture.  Can discuss in normative work if some information is needed in stage 2 in association with the architecture (e.g., a NOTE with the figure).

RAN2 confirm the intention to follow SA2 architecture design.

RAN2 will not work on LTE PC5 in the study item.  RAN2 leave it to RAN to determine if LTE PC5 is in scope of a future WI.

Agreement:

From RAN2 perspective, if it is determined to support group positioning, it is feasible to perform at least ranging with the estimate calculation at multiple UEs.

And according to TR 23.700-86 [2], there are many solutions for sidelink positioning especially for Reference/Anchor UE discovery, LMF/server UE functionality and Inter-PLMN Sidelink Positioning. In sidelink positioning, Anchor UE Discovery procedure and Selection is very important to guarantee and achieve high accuracy and stable positioning performance.

In sidelink positioning, to decide on which UEs are anchor UEs for sidelink positioning (a.k.a. Anchor UE selection) is very important to perform successful sidelink positioning operation and to achieve desired accuracy.

In this paper, we consider how to discovery and select Anchor UE well, and provide a method for successful sidelink positioning.

2	Discussion

2.1		Anchor UE (Re)Discovery

Anchor UE Discovery for sidelink positioning purpose will use to ProSe (Proximity Service) procedure which is defined in TS 23.303 and TS 23.204. However, current ProSe based discovery procedure cannot cover the below problem situations, which can occur during candidate Anchor UE Discovery. Figure 1 shows that candidate Anchor UE Discovery failure due to absence of candidate Anchor UE nearby Target UE. Figure 2 also shows that Discovery failure due to not supported SLPP or fast moving candidate Anchor UE type such as vehicle. And even if, too few candidate Anchor UE is discovery by Target UE, the discovery procedure will ends as shown in Figure 3.

Figure 1. Candidate Anchor UE Discovery Failure due to absence of candidate Anchor UE nearby Target UE

Figure 2. Candidate Anchor UE Discovery Failure due to not supported SLPP or fast-moving candidate Anchor UE type

Figure 3. Discovery Procedure End even if too few discovered Candidate Anchor UEs

Observation 1.	In case of Candidate Anchor UE Discovery Failure, there is no requested operation of the target/anchor UE.

Observation 2.	Even when the number of discovered Candidate Anchor UEs is remarkably small, the ProSe Discovery procedure ends.

Observation 3.	When transmitting the initial Discovery MSG at full power, the target/anchor UE consumes unnecessary power.

Observation 4.	In out-of-coverage scenario (public safety service), when PC5 link power loss occurs caused by environmental factors (rain, obstacles, etc.), Target/Anchor UE operation is required to discover for candidate Anchor UEs.

In other words, in order to solve the above situations, it is necessary for the gNB/LMF to (Pre)-configure a pre-defined parameter rather than to depend on the UE implementation. Below table 1 is the example (Pre)-configuration parameters for Discovery message transmission modification which should be performed by Target UE for discover candidate Anchor UE.

Target UE can increase their transmission power when they can not find out any candidate anchor UEs. Target UE can allocate more RBs for Discovery message in resource pool. And Target UE can apply low MCS index. And Target UE can cause candidate Anchor UE to do soft combining by repeatedly sending a discovery message equivalent to it previously sent. As another method, Target UE may consider a method of delivering feedback information in order to inform candidate Anchor UEs of its discovery importance to nearby Target UE.

Proposal 1.	RAN2 to discuss following configurations for candidate anchor UE Discvoery;

- Discovery MSG Tx Power

- N_RBDiscovery (Allocate more RBs for Discovery MSG Tx Resource Pool))

- MCS Index (Apply Low MCS Index)

- Soft Combining Indicator

- Feedback Information (Ramped up power, N_RBDiscovery, # of Activated Anchor UE)

Table 1. The example (Pre-)Configuration for Discovery MSG Tx Modification

| Parameter | Attribute (Ex) |
| --- | --- |
| Discovery MSG Tx Power | 2dBm |
| N_RBDiscovery (Allocate more RBs for Discovery MSG Tx Resource Pool) | RB 25 |
| MCS Index (Apply Low MCS Index) | Level 4 |
| Soft Combining Indicator | ON |
| Feedback Information | Ramped up power N_RBDiscovery # of Activated Anchor UE |

Figure 4. Proposed solution for modifying Target UE’s discovery message Tx structure

In here, we can think about the criteria when to modify Target UE’s discovery Tx structure. The timing of modifying Target UE’s discovery Tx structure may be considered based on the number of discovered anchor UEs or the number of activated/selected anchor UEs. For example, SL-TDOA (Time Difference of Arrival) method needs at least three anchor UEs which should be distributed around Target UE for high accuracy. So, the maximum number of discovered Anchor UE and the maximum number of activated Anchor UE can be configured based on positioning method. However, if Target UE transmits Discovery message indefinitely, battery consumption of the target UE may increase. So, it can be considered as a method of limiting the maximum number of discovery message re-transmissions. Below table 2 shows an example (Pre-)Configuration for discovery procedure end conditions.

Table 2. An example (Pre-)configuration for discovery procedure Re-Trigger/End condition

| Event | Parameter |
| --- | --- |
| Discovery Complete (End) | Max_N_Anchor UEDiscovered (Max # of Discovered Candidate Anchor UE) |
|  | Max_N_Anchor UEActivated (Max # of Activated (Selected) Anchor UE) |
|  | Max_N_Discovery_Re_Tx (Max # of Discovery MSG Retry Tx) |

Proposal 1.	RAN2 to discuss following considerations for Discovery Complete (End) conditions;

- The maximum number of Discovery Anchor UE

- The maximum number of Activated Anchor UE

- The maximum number of Discovery message retry transmission

2.2		Anchor UE (Re)Selection

In general, Target UE discovery procedure will be started after receiving a sidelink positioning request message from LMF or itself. After discovery, below procedures will be proceeded.

Anchor UE Selection, PC5 Establishment, SLPP Capability Transfer, Sidelink Positioning Method Determination, SLPP Assistance Data Exchange, SLPP Location Information Transfer, SL-PRS Tx/RX, Measurement Report, Location Calculation

However, activated or selected Anchor UE may disappear at any time during the above procedures. It means that sidelink positioning cannot be performed or its accuracy can be degraded considerably.

Observation 5.	Activated or Selected Anchor UE may be disappear during the SLPP procedures.

Thus, Target UE discovery procedure should be re-triggered if the number of activated/selected Anchor UE is not enough for sidelink positioning. Table 3 is an example (Pre)configuration for discovery procedure Re-Trigger. The parameters can be configured based on positioning method, QoS(Quality of Server) class, Target/Anchor UE Type, Use cases (public safety, IioT, V2X etc.).

Table 3. An example (Pre-)configuration for discovery procedure Re-Trigger

| Event | Parameter |
| --- | --- |
| Discovery Procedure Re-Trigger | Min_N_Anchor UEActivated (# of Activated (Selected) Anchor UE) |

Proposal 2.	RAN2 to discuss the parameters for Discovery procedure Re-trigger condition, the number of activated/selected Anchor UE

3	Conclusion

Observations:

Observation 1.	In case of Candidate Anchor UE Discovery Failure, there is no requested operation of the target/anchor UE.

Observation 2.	Even when the number of discovered Candidate Anchor UEs is remarkably small, the ProSe Discovery procedure ends.

Observation 3.	When transmitting the initial Discovery MSG at full power, the target/anchor UE consumes unnecessary power.

Observation 4.	In out-of-coverage scenario (public safety service), when PC5 link power loss occurs caused by environmental factors (rain, obstacles, etc.), Target/Anchor UE operation is required to discover for candidate Anchor UEs.

Observation 5.	Activated or Selected Anchor UE may be disappear during the SLPP procedures.

Proposals:.

Proposal 1.	RAN2 to discuss following considerations for Discovery Complete (End) conditions;

- The maximum number of Discovery Anchor UE

- The maximum number of Activated Anchor UE

- The maximum number of Discovery message retry transmission

Proposal 2.	RAN2 to discuss the parameters for Discovery procedure Re-trigger condition, the number of activated/selected Anchor UE

4	References

[1] 3GPP TR 23.700-86 v1.2.0, “Study on Architecture Enhancement to support Ranging based services and sidelink positioning (Release 18),” Nov. 2022.