3GPP TSG-RAN2 Meeting #121	R2-2301950

# Athens, Greece, 27th February – 3rd March, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.340 | CR | 0031 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the inter-CU routing and header rewriting for eIAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The header rewriting operation may be performed by a boundary IAB-node for inter-CU routing by 5.2.1.5. In addtion, it may also be performed by a descendant node of boudnary IAB-node for re-routing by 5.2.1.3. Therefore, it is a general operation for eIAB node, not limited to the boundary IAB-node. - In R17 topology redudancy scenario, the boudnary IAB-node can be configured with two BAP addresses by MN and SN. Therefore, the configuration of IAB-node's BAP address via RRC can be either one or two BAP addresses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 4.2.2, remove “for a boundary IAB-node,” - In 4.5., add “(es)” in The IAB-node's BAP addresses via RRC. Impact analysis Impacted functionality: BAP Inter-operability: - If the IAB-MT is implemented according to this CR but the network is not, there is no inter-operability issue forseen. - If the network is implemented according to this CR but the IAB-MT is not, there is no inter-operability issue forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The descendant IAB node may not be able to perform the header rewriting for re-routing case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

### 4.2.2	BAP entities

On the IAB-node, the BAP sublayer contains one BAP entity at the MT function and a separate collocated BAP entity at the DU function. On the IAB-donor-DU, the BAP sublayer contains only one BAP entity. Each BAP entity has a transmitting part and a receiving part.

NOTE: The modelling of BAP entities does not restrict internal implementation of IAB-nodes, i.e. the exact modelling of BAP sublayer may vary for different IAB-node implementations.

The transmitting part of the BAP entity has a corresponding receiving part of a BAP entity at the IAB-node or IAB-donor-DU across the BH link.

Figure 4.2.2-1 shows one example of the functional view of the BAP sublayer. This functional view should not restrict implementation. The figure is based on the radio interface protocol architecture defined in TS 38.300 [2].

In the example of Figure 4.2.2-1, the receiving part on the BAP entity delivers BAP PDUs to the transmitting part on the collocated BAP entity. Alternatively, the receiving part may deliver BAP SDUs to the collocated transmitting part. When passing BAP SDUs, the receiving part removes the BAP header and the transmitting part adds the BAP header with the same BAP header content as carried on the BAP PDU header prior to removal. Passing BAP SDUs in this manner is therefore functionally equivalent to passing BAP PDUs, in implementation. The following specification therefore refers to the passing of BAP Data Packets.

Besides, BAP entity generates, delivers/receives BAP Control PDU(s) as described in clause 6.1.2. BAP Control PDU can only be exchanged between peer BAP entities across the BH link.

In addition to the functions shown in Figure 4.2.2-1, the transmitting part of the BAP entity may perform BAP header rewriting operation, in accordance with clause 5.2.1.

Figure 4.2.2-1. Example of functional view of BAP sublayer

Next Change

## 4.5	Configurations

The configuration of the BAP entity includes:

-	The IAB-node's BAP address(es) via RRC.

-	The IAB-donor-DU's BAP address via F1AP.

-	Mapping from next hop BAP address to downstream egress link via F1AP.

-	Mapping from next hop BAP address to upstream egress link via RRC.

-	Mapping from upper layer traffic to BAP routing ID in BAP header via F1AP and RRC.

-	The BAP routing entries via F1AP.

-	Mapping to egress BH RLC channels via F1AP and RRC.

-	Flow control feedback type(s) to be provided, if any, via RRC.

-	Inter-donor-DU re-routing enabling/disabling via F1AP.

BH RLC channels are configured via RRC on the IAB-MT, and via F1AP on the IAB-DU/IAB-donor-DU.

For F1AP configurations, the following mapping, which are derived from the original F1AP signaling, are used in procedure:

-	Uplink Traffic to Routing ID Mapping Configuration.

-	Downlink Traffic to Routing ID Mapping Configuration.

-	BH Routing Configuration.

-	BH RLC Channel Mapping Configuration.

-	Uplink Traffic to BH RLC Channel Mapping Configuration.

-	Downlink Traffic to BH RLC Channel Mapping Configuration.

-	Header Rewriting Configuration.

End of Change